Ethernet Frame Calculations

Introduction

Frames per second (FPS) or Packets per second (PPS) is a common method of rating the throughput performance of a network device. You should always research this when purchasing a new device for your network. The more frames per second a device can forward the better off your network as a whole will be. There are other factors in which you need to research.

Understanding how to calculate frames per second can provide a lot of insight on how the Ethernet system functions, assists you in network design, and is useful when purchasing new network devices.

The Calculation

In this section we will calculate the theoretical maximum frames per second of an Ethernet (10Mbps), Fast Ethernet (100Mbps), and Gigabit Ethernet (1Gbps) segment. The calculation is considered theoretical because it requires a network segment without collisions and a standard packet length. Collisions are a normal part of an Ethernet system and are not considered bad but they would affect the results. Also Ethernet data size relies on the upper layer application, so it is very unlikely to find a stream of packets with exactly the same size in an Ethernet network.

The minimum frame payload is 46 Bytes (dictated by the slot time of the Ethernet LAN architecture). The maximum frame rate is achieved by a single transmitting node which does not therefore suffer any collisions. This implies a frame consisting of 72 Bytes with a 9.6 microsecond inter-frame gap (corresponding to 12 Bytes at 10Mbps).

| >Frame part | >Size |
| --- | --- |
| MAC Preamble | 7 Bytes |
| Start Fram Delimiter (SFD) | 1 Bytes |
| Destination MAC Address | 6 Bytes |
| Source MAC ardes | 6 Bytes |
| MAC Type or Length | 2 Bytes |
| PayLoad (Network PDU) | IP Header; 20 Bytes. TCP Header; 20 Bytes. Data/Padding; 6 Bytes. Total; 46 Bytes. |
| Frame Check Sequence | 4 Bytes |
| Interframe Gap (9.6 microseconds) | 12 Bytes |
| Total physical size of Frame | 84 Bytes |

The maximum number of frames per second is:

Ethernet Data Rate (bits per second) / Total Frame physical size (bits)

Ethernet = 10,000,000 / (84 X 8) = 14,880 frames per second

Fast Ethernet = 100,000,000 / (84 X 8) = 140,880 frames per second

Gigabit Ethernet = 1,000,000,000 / (84 X 8) = 1,488,000 frames per second

Note: The Interframe Gap timing is set to 96 bit times for all types of Ethernet. Bit time is defined as the amount of time required to send one data bit on the network. The table below defines the bit times and the corresponding Interframe Gap for each Ethernet medium.

| Ethernet Type | Bit time | Interframe Gap |
| --- | --- | --- |
| Ethernet (10M) | 100 nanoseconds | 9.6 microseconds |
| Fast Ethernet (100M) | 10 nanoseconds | 960 nanoseconds |
| Gigabit Ethernet (1G) | 1 nanosecond | 96 nanoseconds |

(1 Milisegundo=0,001 seg ,  1 Microsegundo=0,000,001 seg, 1 nanosegundo = 0,000,000,001 seg)

Example 1: Calculate the maximum frame rate of a node on an Ethernet LAN.

The minimum frame payload is 46 Bytes (dictated by the slot time of the Ethernet LAN architecture). The maximum frame rate is achieved by a single transmitting node which does not therefore suffer any collisions. This implies a frame consisting of 72 Bytes (see table above) with a 9.6 µs inter-frame gap (corresponding to 12 Bytes at 10 Mbps). The total utilised period (measured in bits) corresponds to 84 Bytes.

| Frame Part | Minimum Size Frame |
| --- | --- |
| Inter Frame Gap (9.6µs) | 12 Bytes |
| MAC Preamble (+ SFD) | 8 Bytes |
| MAC Destination Ardes | 6 Bytes |
| MAC Source Address | 6 Bytes |
| MAC Type (or Length) | 2 Bytes |
| Payload (Network PDU) | 46 Bytes |
| Check Sequence (CRC) | 4 Bytes |
| Total Frame Physical Size | 84 Bytes |

Calculation of number of bit periods occupied by smallest size of Ethernet frame

The maximum number of frames per second is:

Ethernet Data Rate (bits per second) / Total Frame Physical Size (bits)

= 10 000 0000 / (84 x 8 )

= 14 880 frames per second.

In practice, this exceeds the forwarding capacity of many routers and bridges (typically 1000's of frames per second). This is however not usually a concern, since most Ethernet networks carry packets with a range of lengths and usually transmit a significant proportion of maximum sized frames (the maximum rate of transmission of maximum sized frames is only 812 frames per second). If the forwarding capacity is momentarilly exceeded due to a large number of small frames, the bridge/router will simply discard the excess frames which it can not process (this is allowed, since Ethernet provides only a best effort link layer service).

Example 2: Calculate the maximum throughput of the link layer service provided by Ethernet

The maximum frame payload is 1500 Bytes, this will offer the highest throughput, when the frames are transmitted by only one node on the network (i.e. there are no collisions) To calculate the throughput provided by the link layer, one must first calculate the maximum frame rate for this size of frame.

| Frame Part | Maximum Size Frame |
| --- | --- |
| Inter Frame Gap (9.6µs) | 12 Bytes |
| MAC Preamble (+ SFD) | 8 Bytes |
| MAC Destination Address | 6 Bytes |
| MAC Source Address | 6 Bytes |
| MAC Type (or Length) | 2 Bytes |
| Payload (Network PDU) | 1500 Bytes |
| Check Sequence (CRC) | 4 Bytes |
| Total Frame Physical Size | 1538 Bytes |

Calculation of number of bit periods occupied by largest size of Ethernet frame

The largest frame consists of 1526 Bytes (see table above) with a 9.6 µs inter-frame gap (corresponding to 12 Bytes at 10 Mbps). The total utilised period (measured in bits) therefore corresponds to1538 Bytes.

The maximum frame rate is:

Ethernet Data Rate (bits per second) / Total Frame Physical Size (bits) = 812.74 frames per second

The link layer throughput (i.e. number of payload(carga util) bits transferred per second) is:

Frame Rate x Size of Frame Payload (bits) = 812.74 x (1500 x 8) = 9 752 880 bps.

This represents a throughput efficiency of 97.5 %.

Example 3: One node transmits 100 Byte frames at 10 frames per second, another transmits 1000 Byte frames at 2 frames per second, calculate the utilisation of the Ethernet LAN.

The utilisation is the percentage of time the physical link is transmitting data. This calculation will assume that the transmissions do not collide. (Thisd may need to be reviewed if the utilisation is greater than about 10%).

The maximum frame payload is 1500 Bytes, this will offer the highest throughput. To calculate the throughput provided by the link layer, one must first calculate the maximum frame rate for this size of frame.

| Frame Part | Frame 1 | Frame 2 |
| --- | --- | --- |
| Inter Frame Gap (9.6µs) | 12 Bytes | 12 Bytes |
| MAC Preamble (+ SFD) | 8 Bytes | 8 Bytes |
| MAC Destination Address | 6 Bytes | 6 Bytes |
| MAC Source Address | 6 Bytes | 6 Bytes |
| MAC Type (or Length) | 2 Bytes | 2 Bytes |
| Payload (Network PDU) | 100 Bytes | 1000 Bytes |
| Check Sequence (CRC) | 4 Bytes | 4 Bytes |
| Total Frame Physical Size | 138 Bytes | 1038 Bytes |

Calculation of number of bit periods occupied by largest size of Ethernet frame

The two frames occupy respectively a period corresponding to 138 Bytes and 1038 Bytes (including the 9.6 µs inter-frame gap). The total number of bits which are utilised in 1 second are therefore be:

(Frame Size 1 x Frame Rate 1) + (Frame Size 2 x Frame Rate 2)

= (138 x 8 x 10) + (1038 x 8 x 2)

= 27648 bits

This represents a utilisation of :

(27648 / 10 000 0000) x 100 = 0.28 %

(N.B. 0.28 % << 10%, therefore it was permissable to neglect the effect of collisions.)

Reference:

http://www.packetshack.org

http://www.erg.abdn.ac.uk/users/gorry/course/lan-pages