The Three gorges dam is near near Santouping Township in Yichang City, Hubei Province. The functions include flood control, power generation, improved navigation. The construction of dam will cause the changes of River Chang Jiang downstream in quantity and quality. These changes also affect the channel morphology and ecology of downstream.

The quantity of discharge downstream will become constant and regular. To control flooding, there are  reservoirs . The reservoir's normal water level will be 175 meters and the storage capacity will be 39.3 billion cubic meters, of which the storage capacity for flood control will be 22.15 billion cubic meters. The greatest rate of discharging floodwater will be 100,000 cubic meters per second. The floodwater from the upper reaches of the river with a flow rate of 87,600 cubic meters per second will be blocked by the reservoir and discharged at 56,700 cubic meters per second.

The discharge downstream will be clearer. Because of dam and reservoirs, the sediment will be deposited before reaching downstream. Also, the dissolved oxygen will reduce. Because of the dam, the downstream become more peaceful. So, the oxygen in air cannot go into river by the river rapid movement, such as turbulent flow.

There will occur industrial pollution.  The dam will improve navigation and power generation. The dam attracts people to develop the industries. In 1999, Chongqing discharged 902.2 million tons of industrial waste water, 82.89 percent of which was treated, with 66.38 percent reaching the national standard for discharged waste water. The top three polluting industries were the chemical, food, tobacco and beverage processing, and ferrous metal smelting and forging industries.

Also, it attracts people to live here because there is electricity provided. Therefore, the domestic sewage increased. Each year, Chongqing produced2.19 million tons of domestic waste and dumps about 21.7 million tons of waste on the river bank.

Morever, the farming activities in downstream increase because the farmers from upstream move to downstream. The reason is their house and farm will be over wash after finishing the construction. The increasing no. of farming activities cause more pollutions by using pesticide and chemical fertilizer. . In 1999,a total of 132,200 tons of fertilizer were used in the area, averaging 526.87 kg per hectare.

Last year, 808 vessels at the Three Gorges were monitored and another 8,755 vessels were surveyed for the discharge of sewage water. Results showed that the vessels discharged 779,000 tons of sewage water into the Three Gorges, only 74.1 percent of which was treated. Among the 147.14 tons of discharged pollutants, 57.8 percent were petroleum pollutants, representing a 48 percent increase over the previous year. While most of the pollution was caused by passenger ships in 1998, cargo ships contributed to most of the pollution last year. Twenty pollution accidents caused by vessels occurred there in 1999, discharging 11.8 tons of diesel oil and 100 tons of polluted water, and dumping 20 bags of garbage into the Three Gorges section of the Yangtze. Though the number of pollution accidents was not as many as that in 1998, the impact was more serious.

In addition, because the discharge become more peaceful, the pollutants cannot be removed easily, the pollution becomes more serious.

In river morphology, the delta will be more erosion. Because the water loading become less, less sediment go to delta causing less deposition. And the delta always occur erosion because of wave energy, therefore, the delta occur more erosion. Also, the channel of downstream become deepen and widen because there is less water loading in the channel, therefore, the energy of river become stronger ,the erosion occur. The floodplain occur less deposition because of less sediment in the downstream. The groundwater level rise because of the discharge of downstream become more peaceful, therefore, the velocity of the river become slower. Near the ground surface, because the groundwater level rise, the mineral from the ground also rise to the surface. The salinization occurs.

In ecology, because the salinization occurs, the land become insuitable for farming. Also, because the pollution becomes more serious, it affects the wilding. In land plant, There is a total of 6,388 species of higher plants in the area, 188 of which are under key State protection and 57 of which are rare and near extinction. There are more than 8,000 ancient trees, 29 of which will be submerged in water after the dam is completed. In land animals, . Animals and birds close to extinction include mandarin ducks and golden monkeys. It has not yet been determined how the mandarin ducks will fare after the construction of the reservoir because their living environment will have changed quite a bit. Also, in finding fish resources and environment, The fishing output in 1999 in the area was 3,612 tons, down 67.16 percent from 1998. A test of the water quality on the river shows that the fishing waters were polluted to some extent, with the copper and petroleum content exceeding set limits. All of these show that the effects of the dam in ecology.

However, the three gorges dam is not just bring the bad effect for the environment. For example, the hydropower station cause less pollution than using the oil or coal. It is not easy to say this project is good or not.

.