glove_init(mode, function)

puts the glove into a given mode; the following are defined so far:

HIRES -- high-resolution

LORES -- low-resolution

IHIRES -- interrupt-driven high-resolution

ILORES -- interrupt-driven low-resolution

Note that the code as supplied always uses high resolution, but

will still accept LORES and ILORES as valid modes; glove_init()

will return the mode *actually* selected (for forwards and

backwards compatability).

The function, if not NULL, is a pointer to a function to be called

from the interrupt handler every time the glove position changes.

glove_quit()

Basically just restores the interrupt vectors that were taken over

by glove_init() if an interrupt mode (IHIRES or ILORES) was used;

must be called before exiting, so glove_init() uses atexit() to

set this.

glove_read(&glov)

Reads the current glove data; this may be called at any time (no

need to poll) if in interrupt mode.  Returns non-zero if the data

has changed since the last time you called glove_read().

glove_ready()

Always returns 1 in interrupt modes; otherwise, polls the glove and

returns 1 if it's ready.  Not needed in interrupt mode.

glove_delay()

Used only in polled mode, this routine should be called between

calls to glove_ready() to make sure we aren't confusing the glove's

processor by bugging it too often.

The glove data structure (glove_data) now contains an "nmissed" field,

which is the number of samples we've missed between calls to glove_read().

Note that deglitching and dehysterisis are done in the interrupt handler,

so the caller never needs to worry about it.

--

Bernie Roehl, University of Waterloo Electrical Engineering Dept

Mail: broehl@sunee.waterloo.edu OR broehl@sunee.UWaterloo.ca

BangPath: watmath!sunee!broehl

Voice:  (519) 885-1211 x 2607 [work]