Welcome to Virtual Realities.  You have received Virtual Realities, Virtua-Track-HeadMouse.  This system

includes the program disk, the Virtua-Track, the Virtua-Track

connector/adaptor cord, and the electric adaptor.

INSTALLATION:

1) Verify you have all of the necessary components, listed above.

2) Place the Virtua-Track on your head,

the Virtua-Track will be in the back, adjust the head strap as needed.

3) Plug the Virtua-Track connector/adaptor cord into the Virtua-Track,

making sure the arrow is pointing up and the connector/adaptor cord is pointing down.

Actually, this cord will only fit tightly if connected as mentioned above.

4) Take the other end of the connector/adaptor cord and plug the electric

adaptor into the back of it.

5) Now plug the connector/adaptor cord into a serial port on your

computer.

6) Plug the electric adaptor into an electrical socket.

7) Load the program disk into your computer. Double click on the MY COMPUTER icon, then

double click on the A drive (or your 3 1/2" disc drive). This will bring up an

installation screen. If you are ready to install the software, click on the setup

button.

8) If you are going to use the Virtua-Track as a pointer only (thus utilizing

the existing mouse to click on the icon pointed to by the Virtua-Track),

click on the Virtua-Track setup button only.

OR

9) If you are going to use the Virtua-Track as a pointer and mouse, click

on the Virtua-Track setup button, complete the installation, then come back to this menu

and click on the MouseTool 3 install button and complete that installation.

NOW you should be ready to go.  Please read the attached sheet for

fine tuning instructions and hints on how to utilize the Virtua-Track

fully.

Tips & Instructions for use of Virtua-Track

- The Virtua-Track is easiest to use if Windows is set up with an extra large scheme as

follows: Right-click on the desktop, select Properties, go to the Appearance tab when the

window opens. Under Appearance, there is a setting called Scheme. In the drop down menu,

select Windows Standard (large) or (extra large). This will make it much easier to 'click'

on smaller icons.

- The headband is adjustable by using the knob on the headtracker. By turning the knob

right or left, you can reduce or increase the size of the headband for a comfortable but

secure fit.

- The headband is normally worn with the tracker module in the rear. It will not operate

correctly when upside down.

- The cable assembly has an extra cable on it labeled AUX, which goes to an auxiliary

power supply of 5VDC and 225mA.

- When the Virtua-Track is installed, a window will open up to configure it:

Click on Calibrate, and follow the on-screen instructions to calibrate the Virtua-Track to

the comfortable limits of your head motion. The filter settings control how the mouse

moves in response to your head movements.

Filters:

Filters affect the way your Virtua-Track relates head movement to cursor movement.

1. Jitter affects the mouse resolution and jumpiness. The higher the number, the lower the

mouse resolution, but the less jumpy the mouse is.

2. Magnetic and Tilt are adjustable from 1 to 7. Magnetic affects the strength of hardware

filtering done on the earth's magnetic field, and should generally be set at 5.

3. Tilt affects damping in the nodding (elevation) direction, and should generally be set

at 3.

4. Kallmann Filter

FC is a function that changes the weight of the samples. The filter gives more importance

to the most recent mouse sample, and less to the previous samples. By decreasing FC, you

can cause the filter to give more importance to older samples, and by increasing FC you

can make the most recent samples more important.

Interval indicates the sampling period in milliseconds. HeadMouse analyzes tracking data

for the 40 most recent periods. A typical setting is 50, or a total of 2 seconds. Filter

Curve (FC) selects the time-sensitivity curve as shown below. Each curve is labeled with

a number to be entered into the window FC. Curve 0.10 nearly averages the data over the

last 40 periods, and the cursor will be very steady, but respond very slowly to a change

in head position. Curve 1 weights the data linearly  decreasing with time: 100% of the

most recent period, 50% of the data from 20 periods ago, to 0% of the data from 40 periods

ago. Curve 40 takes almost all its data from the most recent periods: the cursor will

respond very quickly to a change in head position, but will jump around considerably in

response to heartbeat or body tremor.

MouseTool 3

MouseTool is a program for inputting "mouse clicks" without a physical click device

(switch closure, suck and puff switch, etc.). The program allows you to "click" at a

cursor location simply by keeping your mouse at that location for a (user-definable)

period of time.

When MouseTool has been installed and run, you will see a small box as pictured above:

Click on the green light to disable the automatic clicking while you configure MouseTool 3.

To re-enable the automatic clicking, click on the red light, and automatic clicking will

be enabled again.

The dwell-actuate time is adjusted under the Options button. The program has excellent

Help files