World War Two
 Armour Penetration Statistics

by John Salt

(formatted by Valeriy Potapov)

1999

# Foreword

This document, which I hope will prove useful to WW2 wargamers, gives penetration performance details of WW2 anti-tank weapons.

I have assembled these from over sixty sources, mostly from my own library. All are in the public domain. Those books no longer in print should be easy to buy from a good book search service such as amazon.com, or to borrow from a good library. The documents cited from the Public Records Office, Kew, are available for inspection there to anyone with a Reader’s ticket.

Where page numbers are cited, tables usually appear in the original source. In other cases, it has been necessary to extract and tabulate data spread through the body of the text. Sometimes it has been necessary to reduce information presented as graphs or polar diagrams to tabular form, and some loss of accuracy is inevitable in the process. Given the inherent imprecision of all these figures, however, this does not much matter.

Sources referring to the post-war period have been cited where they cover weapons developed during the war. Some entries refer to weapons never introduced into service, such as the British 32-pounder and Burney guns with "Wallbuster" (HESH) shell, American experimental 90mm guns, and a variety of German developments; these are included for interest.

To be useful, armour penetration figures should specify not only the gun, range and penetration achieved, but also the ammunition nature and model fired, the angle of impact, and the type of armour attacked. Some few sources provide all this information; most do not. An additional annoyance is that some sources are obviously mistaken or misprinted in places. This has been noted under the heading "Comments and Corrections" where I have been able to spot the error.

The customary angle of impact when quoting penetration performance is 30º from the vertical, but some American weapons are quoted for 20º. German practice was to measure angles from the horizontal, so where 60º is stated, it is usually safe to assume that 30º from the vertical is intended. In action, unless firing from exactly right-angles to the target vehicle, there will be an additional angular component depending on the target tank’s facing. A table for rough conversions from sloped armour to an equivalent thickness of vertical plate is given as one of the extracts from WO 185/118; it will be seen that well-sloped armour seems to be more effective than would be indicated by a simple cosine calculation.

It is also traditional to quote penetration against homogenous armour. Performance against face-hardened plate, especially for smaller weapons firing uncapped rounds, can differ greatly. Armour hardness may vary considerably. For example, according to Kenneth Macksey's "The Guinness Book of Tank Facts and Feats" (Guinness Superlatives, Enfield, 1972), British homogenous armour at the outbreak of WW2 came in "soft" (machineable) and "hard" grades with Brinnel hardnesses of 380 and 450 respectively, while Vickers Cemented armour had a Brinnel hardness of 600 on the front and 400 on the back. One of the papers extracted from WO 185/118 gives the hardness of armour on the Tiger. It seems that there is a certain velocity band which face-hardened plate protects against best, and for projectiles arriving outside this velocity band, face-hardened plate is less effective than the same thickness of homogenous armour.

Where ammunition nature is not specified, it may be possible to make an educated guess based on the muzzle velocity, if the source quotes one, or simply by applying common sense to the magnitude of the penetration figure given — the widely-quoted 140mm penetration at 500m credited to the Soviet 57mm ZIS-2 can hardly apply to a standard AP round, for example. One source shows a significant difference in performance between APHE and APC or APCBC rounds for the same gun, and it can sometimes be difficult to decide which is meant. The nationality of the weapon can serve as a guide; the Germans never used solid steel AP, preferring APHE; the Russians seem to have had a strong preference for APHE; the Americans seem to have been happy with both; and the British never used APHE after the 3-pounder. The question is further complicated by the fact that rounds are sometimes misleadingly designated; for example, some American rounds designated APC are in fact APCBC. In the name of accuracy, some of these, and Panzergranate 39 for guns above 5cm calibre, should probably be designated APCBCHE, but they rarely are. For complete accuracy, the precise model number needs to be known. Sometimes even this is not enough, as the table from Hunnicutt's "Sherman" shows; there is an appreciable difference between the "early" and "late" 90mm M82 APC, due to "hotter" loading. Some sources do not help by propagating errors; many sources credit the 17-pdr firing APDS with 231mm penetration at 1000 yards on armour sloped at 30º, but it seems tolerably obvious that this figure really refers to armour at 0º. It is rare for original sources to be accurately credited.

Even when all these sources of variation are accounted for, authoritative sources can seem to disagree. Consider the German 75mm PaK40, a common and well-documented weapon, firing PzGr 39 against homogenous armour at 30º at 500 metres (or 600 yards, which is near enough). Even discarding the highest and lowest observations as outliers, we find a variation between 90mm and 106mm, a difference of 17% of the lower figure. This highlights the point that armour penetration is far from being a deterministic phenomenon, and giving penetrative performance in millimetres (and even in some cases half-millimetres) suggests a degree of precision that does not really exist. Performance in the field, of course, is subject to very much greater variation. As Shelford Bidwell says in "Gunners at War", "All simplified tables showing the performance of anti-tank guns are misleading". The Bovington booklet "Fire and Movement" says "Chance also causes discrepancies — for instance, a normally certain penetration may be prevented by the deflection away of a projectile on a lifting-lug projecting from a turret; conversely, a normally certain immunity may be removed if that same type of projection deflects a projectile towards the armour at a different angle".

Using penetration tables takes no account either of behind-armour effect following penetration, nor of non-penetrating damage. In principle, penetrating shell with a bursting charge should have better behind-armour effect than solid shot, but whether this matters in calibres above about 50mm seems highly questionable. The most supremely futile attempt to improve behind-armour effectiveness must surely be the German inclusion of a small tear-gas pellet in the AP bullet for the PzB 39 anti-tank rifle. It should still be borne in mind that most of these figures generally indicate the thickness of armour the projectile will just penetrate; the greater the degree to which the armour is overmatched, presumably, the greater will be the damage potential of the penetrating projectile. Hollow-charge weapons are a special case, in that the projectile itself does not penetrate the armour, but rather the jet formed by its charge does. Hollow charge weapons of the WW2 epoch were reputed to have less-than-ideal behind-armour effects. As the Bovington "Fire and Movement" booklet points out, "If the projectile is not an appreciable overmatch for the target, however, the penetration can be small enough to do little damage". One of the tables in WO 185/178 makes an interesting attempt to quantify behind-armour effects by estimating the weight of metal shot into the tank by a penetrating hit.

Penetration tables also take no account of non-penetrating damage. The running gear of all vehicles can be vulnerable to AP or HE fire from nearly all calibres. Extremely large calibre rounds can cause catastrophic damage without needing to penetrate the target; a Soviet 122mm shell, for example, might completely remove the turret of its target. Germany and Russia began using welded construction in the 1930s, but Britain and America continued to use rivets until the beginning of WW2. A shot hitting an exposed rivet-head could cause the rivet shank to break off and be projected into the tank, causing damage. The British continued to use rivets in the Cromwell until the end of the war, but these were tapered to prevent this happening.

Finally, the last few pages contain a few tables dealing with the still more vexed question of hit probabilities. This includes a small section taken from a WW2 Operational Research study on the hit-and-kill probabilities of two British anti-tank guns on some German tank targets. This is principally interesting in that it shows the strong dependence of target facing on weapon effectiveness.

Thanks to Tony Williams and Ernest Ashworth for the information they contributed to this collection. Anyone with comments, corrections or additions is invited to e-mail them to John.Salt@Brunel.ac.uk. Please be sure to include title, author, publisher and year of publication for any source you refer to.

23 April 1999

# Contents

Foreword	2

Contents	4

Armour Penetration	6

Barker, 1979	6

Bidwell, 1976	6

Blackburn, 1998	6

Bovington Museum, 1975	7

Braun, 1977	8

Chamberlain, Doyle & Jentz, 1978	9

Chamberlain & Ellis, 1969	10

Chamberlain & Gander, 1974	10

Ellis et al, 1962	11

Ellis, 1993	12

Farrar-Hockley, 1976	13

Featherstone, 1973	13

Fleischer, 1994	14

Forty, 1979	15

Foss, 1974	15

Funcken & Funcken, 1976	16

Gander & Chamberlain, 1977	16

Gander & Chamberlain, 1978	16

Godfrey, 1984	19

Grove, 1976	19

Handbook on the Italian Army	20

Hogg, 1975	20

Hogg, 1976	21

Hogg, 1977	22

Hogg, 1978	23

Hogg, 1997	24

Hogg & Weeks, 1977	25

Hunnicut, 1978	25

Isby, 1974	26

Isby, 1979	26

Kent, 1990	26

Kosnett, 1977	26

Labbett & Brown, 1988	27

Lucas, 1988	27

Lucas, 1994	27

Macksey, 1988	27

Messenger, 1982	28

Moore, 1991	29

Orgill, 1971	30

Perrett, 1990	30

Playfair, 1956	30

Quarrie, 1989	30

TM 30-410	31

TM-E 30-451	32

Unknown	32

Vanags-Baginskis & Watanabe, 1993	33

von Senger und Etterlin, 1971	34

Weeks, 197?	35

Wiener, 1978	35

Williams, 1999	35

Woodman, 1991	36

WO 171/336, 30 Corps "G" War Diary	37

WO 185/15, Anti-tank grenades.	37

WO 185/118, DDG/FV(D) Armour plate experiments.	37

WO 185/122, Churchill tank with 17 pdr gun.	38

WO 185/133, Tungsten Carbide Production.	39

WO 185/145, New infantry anti-tank weapon.	39

WO 185/171, Armour plate experiments.	39

WO 185/178, Tank armament versus armour.	39

WO 185/182, Recoilless weapons.	42

WO 185/194, Tank and anti-tank armament.	42

WO 190/706, German Army infantry weapons.	42

WO 194/598, Interim damage assessment report on 20mm, 25mm and 30mm calibre cannon attacks against APC targets.	43

WO 219/2785, SHAEF equipment and supply returns.	43

WO 219/2801, SHAEF documents on tanks.	43

WO 219/2806, Appendix G to SHAEF/16652/GCT/Arty	43

WO 279/200, The Development of Artillery Tactics and Equipment	44

WO 291/385, "The Performance of the Infantry Sledge under Fire."	45

WO 291/741, "Comparison of the performance of 75mm and 76mm tank gun ammunition."	45

WO 291/1143, "Effectiveness of British and Russian tanks."	45

Zaloga, 1979	46

Zaloga & Grandsen, 1984	46

Zaloga & Ness, 1998	47

Zaloga & Sarson, 1993	47

Hit probabilities	48

Featherstone, 1973	48

Isby, 1979	48

WO 291/171, "Effectiveness of British anti-tank guns"	48

WO 291/180, "Accuracy of anti-tank gunnery."	51

WO 291/1263, "Firing Trials, 17pdr Sherman."	52

WO 291/2398, "The hit chance of WOMBAT against moving targets."	52

# Armour Penetration

## Barker, 1979

"Japanese Army Handbook 1939–1945", A. J. Barker, Ian Allan, 1979, page 95. Armour at normal, armour type not specified, range not specified.

| Weapon | Penetration |
| --- | --- |
| 37mm ATk gun | 25 |
| 47mm ATk gun | 51 |

Comments and corrections

The original gives penetration in inches.

The failure to specify range makes these figures less than informative, but we might assume 500 yards.

## Bidwell, 1976

"Artillery Tactics 1939–1945", Shelford Bidwell, Almark, 1976, page 72. Armour at 30º slope, type unspecified. Ranges in yards.

| Weapon | Ammo | 250 | 500 | 540 | 1000 | 2000 |
| --- | --- | --- | --- | --- | --- | --- |
| US or Ger 37mm |  |  | 36 |  | 27 |  |
| 2 pdr |  | 58 | 53 |  | 40 |  |
| Russian 45mm |  |  | 60 |  | 38 |  |
| Russian 57mm |  |  |  | 140 |  |  |
| 25 pdr |  |  | 62 |  | 54 |  |
| German 50mm |  |  | 65 |  | 53 |  |
| 88mm FLAK |  |  | 112 |  | 103 | 86 |
| 6 pdr |  |  | 75 |  | 63 |  |
|  | APDS |  | 146 |  |  |  |
| German 75mm |  |  | 171 |  | 130 |  |
| Russian 76mm |  |  | 90 |  | 83 |  |
| 17 pdr |  |  | 123 |  | 113 |  |
|  | APDS |  |  |  | 231 |  |

Comments and corrections

This book is one of Almark's "Mechanics of War" series. The original table has the last four rows misaligned; the error is corrected here. Many of these figures are also given on page 234 of Bidwell's "Gunners at War", A& AP, 1970, which states the slope as 30º and gives the 2 000 yard figure for the 88mm (possibly intended to be for the 17-pdr, as the formatting is unclear, but assumed to be for the 88mm by reference to the War Office document that appears to be Bidwell's source).

The figures for APDS are included as notes, with no range given, but the main text attributes 231mm penetration to the 17-pdr with APDS at 1000 yards, which seems a lot, but agrees with Hogg's "The Guns of World War II". It seems likely that this figure refers to 0º slope.

The figures for the Russian 57mm at 540 yards and the 2-pdr at 250 yards are added from the main text.

Bidwell lists the 37mm as "British", as well as US and German.

The "Russian" 76mm shown here is better known as the PaK 36(r) in German service.

The German 75mm entry carries the comment "A very good gun, but short lived." and the footnote "supplies of tungsten carbide for its special shot ran out", from which it seems safe to conclude that the weapon referred to is the PaK 41 Gerlich gun, rather than the PaK40.

## Blackburn, 1998

"The Guns of Normandy", George Blackburn, Constable, 1998, page 496. Penetration in millimetres, 30º slope, armour type and range unstated:

| Weapon | Penetration |
| --- | --- |
| 6 pdr | 63 |
| 88mm KwK 36 | 102 |
| 88mm KwK 43 | 164 |
| 75mm KwK 42 | 110 |
| 17 pdr plain shot | 113 |
| 17 pdr sabot | 231 |
| 75mm | 68 |

Comments and corrections

These figures seem about right for 1000 yards. The source wrongly states that the 6-pdr was mounted in the Churchill VII.

## Bovington Museum, 1975

"Fire and Movement", RAC Tank Museum, Bovington, 1975, pages 22–25. "Penetration v. homogenous armour at 30º, at ranges in yards". The armour is machineable quality.

| Weapon | Ammo | 100 | 500 | 1000 | 1500 | 2000 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-pr Mks IX and X | AP |  |  | 40 |  |  |  |
|  | APCBC |  | 53 | 49 | 44 | 40 |  |
|  | SV |  | 88 | 72 | 60 | 48 |  |
| 3-pr Mk II | APHE |  |  | 25 |  |  |  |
| 6-pr Mk 3 or 5 | APCBC |  | 87 | 80 | 73 | 67 |  |
|  | APDS |  | 131 | 117 | 103 | 90 |  |
| 75mm Mk V | APC |  | 68 | 61 | 54 | 47 |  |
|  | APCBC |  | 103 | 94 | 86 | 78 |  |
| 77mm Mk 2 | APCBC |  | 120 | 110 | 100 | 90 |  |
|  | APDS |  | 182 | 165 | 148 | 130 |  |
| 17-pr Mk 2 | APCBC |  | 125 | 118 | 110 | 98 |  |
|  | APDS |  | 187 | 170 | 153 | 135 |  |
| 37mm M6 | APC |  | 46 | 42 | 40 | 37 |  |
| 75mm M2 and M3 | APCBC |  |  | 62 | 48 | 40 |  |
|  | APC |  | 70 | 59 | 55 | 50 |  |
| 76mm M1A1 or | APCBC |  | 94 | 89 | 81 | 76 |  |
| M1A2 | HVAP |  | 158 | 134 | 117 | 99 |  |
| 90mm M3 | APCBC |  | 126 | 120 | 114 | 105 |  |
|  | HVAP |  | 221 | 200 | 177 | 154 |  |
| 47mm mod 37 | APC | 55 | 43 | 29 | 23 |  |  |
| 20mm L65 | AP | 31 | 22 |  |  |  |  |
|  | APCR | 49 | 35 |  |  |  |  |
| 37mm L45 | APC |  | 30 |  |  |  |  |
|  | APCR | 68 | 43 | 22 |  |  |  |
| 50mm L42 | APC |  | 56 | 47 | 39 |  |  |
|  | APCR |  | 66 | 42 | 26 |  |  |
| 50mm L60 | APC |  | 61 | 50 |  |  |  |
|  | APCR |  | 86 | 55 | 32 |  |  |
| 75mm L24 | APCBC |  | 46 | 41 | 35 |  |  |
| 75mm L43 | APCBC |  | 84 | 72 | 62 | 53 |  |
| 75mm L48 | APCBC |  | 90 | 79 | 70 | 62 |  |
| 75mm L70 | APCBC |  | 141 | 121 | 104 | 89 |  |
| 88mm L56 | APCBC |  | 110 | 101 | 93 | 84 |  |
|  | APCR |  | 126 | 103 | 85 | 70 |  |
| 88mm L71 | APCBC |  | 182 | 167 | 153 | 139 | 127 |
| 128mm L55 | APC |  | 175 | 150 | 132 | 120 |  |
|  | APCBC |  | 215 | 202 | 190 | 178 |  |
| 37mm mod 94 | APHE |  | 32 | 25 |  |  |  |
| 47mm Type 1 | AP |  | 59 | 45 | 34 |  |  |
| 75mm 94 or 1 | AP |  | 62 | 53 | 46 |  |  |
| Sov 76 L41 | APCBC | (estd) | 56 | 50 | 45 | 40 |  |
| Sov 85 L53 | APCBC | (estd) | 103 | 94 | 86 | 77 | 64 |
| Sov 100 L46 | APCBC | (estd) | 130 | 120 | 110 | 100 | 95 |
| Sov 122 L43 | APCBC | (estd) | 140 | 130 | 120 | 110 | 100 |

Comments and corrections

Hollow charge rounds shown are the British 95mm How Mk 1 (110mm), US 105mm how M4 (100mm) and German 75mm L24 (43mm). The last of these seems remarkably ungenerous, but agrees with the figure given in Hogg's "German Artillery of World War 2" for plate at 30º. Dates for introduction of different kinds of ammunition are given as: 2-pr AP Sep 39; APCBC May 42; SV Jan 43; 6-pr APC, APCBC Oct 42; APDS Jun 44; 17-pr AP, APC, APCBC early 43; HE Jun 44; APDS Summer 44.

## Braun, 1977

"Handbuch der Flugzeug Bordwaffenmunition", 1977, collected by Matthias Braun. Ranges in metres, angles given from the horizontal.

| Gun | Cartridge | Target | Attack | Range | 90º | 60º | 30º |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MG17 | SmK v | 140-150 | D | 100 | 12 | 8 | 4 |
|  |  |  | D | 300 | 9 | 6 | 3 |
|  |  |  | D | 600 | 6 | 3.5 | 1.5 |
|  |  |  | I | 100 | 4 | 3 | 3 |
| 13mm | Pzgr Patr L'spur | 140-150 | D | 100 | 17 | 10 | 4.5 |
|  |  |  | D | 300 | 11 | 7 | 3 |
|  |  |  | D | 600 | 5.5 | 4 | 2 |
|  |  |  | I | 100 | 16 | 9.5 | 4 |
|  |  |  | I | 300 | 9 | 6.5 | 3 |
|  |  |  | I | 600 | 5.5 | 4 | 2 |
| 15mm | Pzgr Patr L'spur | 140-150 | D | 300 | 25 | 18 | 11.5 |
|  |  |  | D | 400 | 19 | 15 | 9.5 |
|  |  |  | D | 600 | 14 | 10.5 | 6.5 |
|  |  |  | I | 100 | 15 | 11.5 |  |
|  |  |  | I | 300 | 11.5 | 10 |  |
| 15mm | H-Pzgr Patr | 140-160 | D | 100 | 48 | 26.5 | 12 |
|  |  |  | D | 300 | 42 | 20 | 9.5 |
|  |  |  | D | 600 | 31.5 | 15.5 | 7 |
| 2cm | Pzgr Patr FFM | 150 | D | 100 | 21.5 | 12 | 7 |
|  |  |  | D | 300 | 17 | 9.5 | 5.5 |
|  |  |  | D | 600 | 12 | 7 | 4 |
|  |  |  | I | 100 | 12.5 | 8.5 | 5 |
|  |  |  | I | 300 | 11 | 8 | 4.5 |
|  |  |  | I | 600 | 8 | 5.5 | 3 |
| 2cm | Pzgr Patr 151 | 150 | D | 100 | 24 | 12 | 8 |
|  |  |  | D | 300 | 23 | 11.5 | 7 |
|  |  |  | D | 600 | 15 | 7 | 4 |
|  |  |  | I | 100 | 13 | 10 | 6.5 |
|  |  |  | I | 300 | 12 | 9 | 5 |
|  |  |  | I | 600 | 8.5 | 7 | 4 |
| 3cm | Pzgr Patr L'spur | 140-150 | D | 300 | 32 | 26 | 19 |
|  |  |  | D | 600 | 27 | 21.5 | 14 |
| 3cm | H Pzgr Patr L'spur | 100 | D | 100 | 100 | 58 |  |
|  |  |  | D | 300 | 95 | 52 |  |
|  |  | 150 | D | 100 | 78 | 47 |  |
|  |  |  | D | 300 | 74 | 42 |  |
| 3.7cm | H Pzgr Patr Flak 18 | 150 | D | 100 | 140 | 68 |  |
|  |  |  | D | 600 | 95 | 47 |  |

Comments and corrections

"Attack" type shows either indirect attack (I), after pentrating a 3mm thick plate of dural at 20º placed at 1.5m from the main armour, or direct attack (D) with no such encumbrance.

"Target" shows the "festigkeit", which I take to mean literally "strength", of the target in Kg per square mm. I take this to be a measure of hardness, but do not know how it compares to the Brinnell scale.

These results are read by eye from graphs, and rounded to the nearest half millimetre.

Thanks to Tony Williams for supplying this document, which summarises results collected from 1936 to 1945 at the "Luftwaffenmunitionsentwicklungsabteilung der Deutschen Wehrmacht" at Erprobungsstelle E6, Rechlin.

## Chamberlain, Doyle & Jentz, 1978

Appendix I, "German Tank Armament ", in: "Encyclopedia of German Tanks of World War Two", first edition, Peter Chamberlain, Hilary L. Doyle & Thomas L. Jentz, A&AP 1978, page 245.

"Penetration of Homogenous Armour Plate at 30º from Vertical", ranges in metres.

| Weapon | Ammunition | 100 | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- | --- |
| 7.92mm MGs | Patr SmK | 8 | 3 |  |  |  |
| 2cm KwK or FlaK | PzGr | 20 | 14 | 9 |  |  |
|  | PzGr40 | 49 | 20 |  |  |  |
| sPzB41 | PzGr41 | 60 | 40 | 19 |  |  |
| 3.7cm KwK or Pak35/36 | PzGr | 34 | 29 | 22 | 19 |  |
|  | PzGr40 | 64 | 34 |  |  |  |
| 3.7cm KwK34(t) | PzGr | 37 | 31 | 26 | 22 |  |
| 3.7cm KwK38(t) | PzGr | 41 | 35 | 29 | 24 |  |
|  | PzGr40 | 64 | 34 |  |  |  |
| 3.7cm FlaK 43 | PzGr18 | 35 | 28 | 21 | 17 |  |
| 4.7cm PaK(t) | PzGr36(t) | 54 | 48 | 41 | 35 |  |
|  | PzGr40 | 100 | 59 |  |  |  |
| 5cm KwK38 | PzGr39 | 54 | 46 | 36 | 28 | 22 |
|  | PzGr40 | 96 | 58 |  |  |  |
| 5cm KwK39 | PzGr39 | 67 | 57 | 44 | 34 | 26 |
|  | PzGr40 | 130 | 72 | 38 |  |  |
| 5cm PaK38 | PzGr39 | 69 | 59 | 48 | 38 | 29 |
|  | PzGr40 | 130 | 72 | 38 |  |  |
| 7.5cm KwK37 or StuK37 | K Gr rot Pz | 41 | 39 | 35 | 33 | 30 |
| 7.5cm KwK40 or | PzGr39 | 98 | 91 | 82 | 72 | 63 |
| StuK40 L43 | Pzgr40 | 126 | 108 | 87 | 69 |  |
| 7.5cm PaK39, KwK40 or | PzGr39 | 106 | 96 | 85 | 74 | 64 |
| StuK40 L48 or PaK40 | PzGr40 | 143 | 120 | 97 | 77 |  |
| 7.5cm Kwk or StuK42 | PzGr39/42 | 138 | 124 | 111 | 99 | 89 |
|  | PzGr40/42 | 194 | 174 | 149 | 127 | 106 |
| 7.62cm PaK36(r) | PzGr39 | 98 | 90 | 82 | 73 | 65 |
|  | PzGr40 | 135 | 116 | 94 | 75 | 58 |
| 8.8cm FlaK18 or 37 | PzGr | 97 | 93 | 87 | 80 | 72 |
|  | PzGr39 | 127 | 117 | 106 | 97 | 88 |
| 8.8cm KwK36 | PzGr39 | 120 | 110 | 100 | 91 | 84 |
|  | PzGr40 | 171 | 156 | 138 | 123 | 110 |
| 8.8cm KwK or PaK43 | PzGr39-1 | 203 | 185 | 165 | 148 | 132 |
|  | PzGr40/43 | 237 | 217 | 193 | 171 | 153 |
| 10cm K18 | PzGr rot | 164 | 149 | 133 | 119 | 109 |
| 10.5cm leFH18 or StuH42 | PzGr | 63 | 59 | 54 | 50 | 46 |
| 12.8cm K40 | PzGr | 201 | 176 | 150 | 132 | 120 |
| 12.8cm PaK44 | PzGr | 189 | 166 | 143 | 127 | 117 |
|  | PzGr43 | 187 | 178 | 167 | 157 | 148 |

Hollow charge rounds, penetration independent of range:

| Calibre | 7.5cm | 7.5cm | 7.5cm | 10.5cm | 10.5cm | 10.5cm | 15cm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ammo | Gr38 H1/A | Gr38 H1/B | Gr38 H1/C | Gr39 H1/A | Gr39 H1/B | Gr39 H1/C | Gr39 H1/A |
| Penetration | 70 | 75 | 100 | 80 | 90 | 100 | 160 |

Comments and corrections

No comments.

## Chamberlain & Ellis, 1969

"British and American Tanks of World War II", Peter Chamberlain and Chris Ellis, A&AP, 1969, pages 202–207. Ranges in yards, armour type not specified, at 30º.

| Weapon | Ammo | Range | Penetration |
| --- | --- | --- | --- |
| 2pdr Mk IX, X & XA | AP | 500 | 57 |
|  | APCBC | 500 | 57.5 |
| 6 pdr Mk 3 |  | 500 | 81 |
| 6 pdr Mk 5 |  | 500 | 83 |
| 75mm Mk V, VA |  | 500 | 68 |
| 17pdr Mk II, IV, VI, VII |  | 500 | 120 |
|  | APDS | 500 | 186 |
| 77mm Mk II |  | 500 | 109 |
| 95mm How Mk 1 | HEAT | 500 | 110 |
| 37mm M6 |  | 500 | 48 |
| 75mm M2 |  | 500 | 60 |
| 75mm M3 |  | 500 | 70 |
| 76mm M1, M1A1, | APC | 1000 | 88 |
| M1A1C, M1A2 | HVAP | 1000 | 133 |
| 90mm M3 | APC | 1000 | 120 |
|  | HVAP | 1000 | 195 |

Comments and corrections

This seems to be the source for the gun performance table on page 55 of Malcolm Bellis' splendid privately-published "Datafile" booklet, "British Tanks and Formations 1939–45". Other books that use these figures, in whole or in part, are Donald Featherstone's "Tank Battles in Miniature" and Terry Gander and Peter Chamberlain's Airfix Magazine guide no. 17, "British Tanks of World War 2", Patrick Stevens Ltd, 1976.

This source states that APCBC for the 2pdr was introduced in September 1942, APDS for the 6pdr in early 1944, and for the 17pdr in 1944.

The authors state that the American 90mm was inferior in penetrative power and accuracy the 17pdr and similarly inferior to the 88mm.

## Chamberlain & Gander, 1974

Source: "Anti-Tank Weapons" Peter Chamberlain and Terry Gander, MacDonald and Jane's, 1974. Ranges in units specified, armour type not specified.

| Weapon | Ammunition | Range | Angle | Penetration |
| --- | --- | --- | --- | --- |
| Skoda 47mm vz 36 |  | 700 yds | 30º | 51 |
| 2.8cm sPzB 41 |  | 400 yds | 30º | 56 |
| 3.7cm PaK 35/36 | AP | 400 yds | 30º | 38 |
|  | AP40 | 400 yds | 30º | 49 |
| 4.2cm lePaK 41 | AP | 500 yds | 30º | 72 |
| 5cm PaK 38 | AP | 500 yds | 30º | 61 |
|  | AP40 | 500 yds | 30º | 86 |
| 7.5cm PaK 40 | AP | 500 yds | 30º | 106 |
|  | AP40 | 500 yds | 30º | 115 |
| 7.5cm PaK 41 |  | 500 yds | 30º | 171 |
| 7.5cm PaK 97/38 |  | 900 yds | 30º | 60 |
| 7.62cm PaK 36(r) | AP | 500 yds | 30º | 98 |
|  | AP40 | 500 yds | 30º | 118 |
| 8.8cm PaK 43 |  | 500 yds | 30º | 226 |
| Panzerfaust klein | HEAT |  | 30º | 140 |
| Panzerfaust gross | HEAT |  | 30º | 200 |
| R PzB 43 | HEAT | 150 m |  | 210 |
| R PzB 54 & 54/1 | HEAT | 200 m |  | 210 |
| 7.92mm Pz B 38, 39 |  | 100 m | 60º | 30 |
| Gross Pz Gr 46 | HEAT | 200 yds |  | 90 |
| It 37/45 |  | 400 yds | 30º | 38 |
| It 47/32 M35 |  | 550 yds | 0º | 43 |
| 37mm Type 94 |  | 1000 yds | 30º | 24 |
| 37mm Type 97 |  | 400 yds |  | 38 |
| 47mm Type 1 |  | 500 yds | 30º | 51 |
| 20mm ATR Type 97 |  | 270 yds |  | 30 |
| wz 35 Marosczek |  | 300 m |  | 20 |
| Solothurn M SS 41 |  | 100m | 60º | 30 |
| Solothurn s18-1100 |  | 300 m | 30º | 15-18 |
| 2 pr |  | 500 yds | 30º | 53 |
| 6 pr |  | 1000 yds | 20º | 69 |
| 17 pr |  | 1000 yds | 30º | 130 |
| Boys ATR |  | 300 m | 0º | 21 |
| 37mm M3A1 | AP | 1000 yds | 20º | 25 |
|  | APC | 1000 yds | 20º | 53 |
| 57mm M1 |  | 1000 yds | 20º | 69 |
| 3-in M5 |  | 1000 yds | 20º | 98 |
| 2.36 in RL M1, M9 | HEAT | 700 yds | 0º | 119 |
| Rifle Gren M9 | HEAT |  |  | 101 |
| 37mm M30 |  | 400 yds | 30º | 38 |
| 45mm M32 |  | 1000 yds | 30º | 38 |
| 45mm M42 |  | 300 m |  | 95 |
| 57mm M41, 43 |  | 500m |  | 140 |
| 100mm M1944 |  | 450m |  | 192 |
| PTRD, PTRS |  | 500m |  | 25 |
| VPGS 1940 |  | 65 yds |  | 30 |

Comments and corrections

It should be fairly obvious where performance is given for APCR ammunition, e.g. for the Soviet 45mm M42 and 57mm M41. I suspect the 60º angles given for two anti-tank rifles means 60º from the horizontal, and so means the same as 30º from the vertical. The claimed difference in performance between US 37mm AP and APC is hard to believe.

## Ellis et al, 1962

"Victory in the West, Volume I: The Battle of Normandy", Maj. L F Ellis with Capt G R G Allen, Lt-Col A E Warhurst and ACM Sir James Robb, HMSO, 1962, page 549. "Penetration against homogenous armour plate at 30º angle of attack", ranges in yards.

| Gun | Projectile | 100 | 500 | 1000 | 2000 |
| --- | --- | --- | --- | --- | --- |
| 75mm Mark V | APCBC | 74 | 68 | 60 | 47 |
| US 57mm | APC |  | 81 | 64 | 50 |
| 6-pdr Mark V | APCBC | 93 | 87 | 80 | 67 |
| US 3-in or 76mm | APC | 109 | 99 | 89 | 73 |
| US 90mm | APC | 123 | 113 | 104 | 87 |
| 6-pdr Mark V | DS ('Sabot') | 143 | 131 | 117 | 90 |
| 17-pdr Mark II | APCBC | 149 | 140 | 130 | 111 |
|  | DS ('Sabot') | 221 | 208 | 192 | 161 |
| 75mm KwK or PAK 40 | APCBC | 99 | 92 | 84 | 66 |
| 88mm KwK 36 | APCBC | 120 | 112 | 102 | 88 |
| 75mm KwK 42 | APCBC | 138 | 128 | 118 | 100 |
| 88mm KwK 43 | APCBC | 202 | 187 | 168 | 137 |
| 128mm PaK 44 | APCBC |  | 212 | 202 | 182 |

Comments and corrections

This is the first volume of the British official history of the NW Europe campaign.

Elsewhere in the text it is stated that "a small quantity" of 6-pdr DS ammunition was available for the landings, and that "it was late in August before there was any for the 17-pdr." The Panzerfaust and Panzerschreck are mentioned: "these could penetrate 200mm of armour within a hundred yards or so but were rather of a 'do-or-die' nature."

The figures given here for the 75mm Mark V, 17-pdr without sabot, 75mm KWK 40 and 42 and 88mm KWK 36 are repeated by Max Hastings in his "Overlord", Michael Joseph, 1984. Hastings fails to state the armour type and slope and ammunition type used, and perpetuates an error in Ellis by misidentifying the Sherman's 75mm gun as the British Mk V, when it is of course the US M3.

The figures Hastings uses for the German guns at 1000 yards, and the Sherman's gun at all ranges, are also quoted in Chapter 3 of Patrick Delaforce's "The Black Bull: From Normandy to the Baltic with the 11th Armoured Division", Alan Sutton Publishing, 1993.

## Ellis, 1993

"The World War II Databook", John Ellis, Aurum, 1993, page 304. Ranges in yards, armour type and slope not stated.

| Weapon | Ammo | 250 | 500 | 750 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 37mm SA18 |  |  |  |  | 30 |  |  |
| 47mm SA37 |  |  |  | 80 |  |  |  |
| 37mm L46 |  | 32 | 28 |  |  |  |  |
|  | PzGr 40 | 40 | 28 |  |  |  |  |
| 50mm L42 |  | 56 | 53 | 46 | 40 | 28 | 22 |
|  | PzGr 40 | 83 | 60 | 42 |  |  |  |
| 50mm L60 |  | 67 | 61 | 56 | 50 | 38 | 29 |
|  | PzGr 40 | 109 | 77 | 46 |  |  |  |
| 75mm L24 |  |  | 46 | 42 | 41 | 33 | 30 |
|  | Hl |  | 75 | 75 | 75 |  |  |
| 75mm L43 |  |  | 92 |  | 82 | 72 | 63 |
|  | PzGr 40 |  | 108 |  | 87 | 69 |  |
| 75mm L48 |  |  | 96 |  | 85 | 74 | 64 |
|  | PzGr 40 |  | 120 |  | 97 | 77 |  |
| 88mm L56 |  |  | 110 |  | 100 | 91 | 84 |
|  | PzGr 40 |  | 156 |  | 138 | 123 | 110 |
| It 20mm |  | 29 | 24 |  |  |  |  |
| It 37mm |  |  |  |  | 32 |  |  |
| It 47mm |  |  | 48 (400) | 38 | 32 |  |  |
| Jap 20mm ATR |  | 30 (270) |  |  |  |  |  |
| 37mm type 94 |  |  |  |  | 24 |  |  |
| 47mm type 1 |  |  | 51 |  |  |  |  |
| Marosczek |  | 20 (300) |  |  |  |  |  |
| 37mm wz 36 |  | 40 (100) | 33 |  | 26 |  |  |
| Boys ATR |  | 21 (300) |  |  |  |  |  |
| 2pdr |  | 58 | 52 | 46 | 40 |  |  |
|  | HV shot | 64 | 57 | 51 | 45 |  |  |
| 37mm |  | 58 | 53 | 48 | 47 |  |  |
| 6pdr |  |  | 79 | 72 | 65 | 52 |  |
| 17pdr |  |  |  | 120 | 113 | 96 | 82 |
| 75mm M2 | M72 |  | 61 |  | 53 | 46 | 38 |
|  | M61 |  | 66 |  | 61 | 56 | 51 |
| 75mm M3 | M61 |  | 66 |  | 61 | 56 | 51 |
| 76mm |  |  |  |  | 98 |  |  |
| 25pdr |  |  | 63 | 58 | 54 |  |  |
| Sov 37mm |  |  | 38 (400) |  |  |  |  |
| Sov 45mm |  |  | 80 |  | 50 |  |  |
| Sov 57mm |  |  | 140 |  |  |  |  |
| 76mm F-34 | DS AP |  | 92 |  | 60 |  |  |
| 85mm D-5 |  |  | 138 |  | 100 |  |  |
| 100mm D-10 | HE AP |  | 195 |  | 185 |  |  |
| 122mm M-30 | HE AP |  | 145 | 145 | 145 |  |  |

Comments and corrections

The figures for the Soviet 45mm and 57mm are obviously with APCR ammunition, although this is not stated in the original.

The idea that the French 47mm has better penetration than the 6pdr is extremely hard to believe.

It also rather hard to believe that the US 75mm guns M2 and M3 have the same performance with M61.

The original shows penetration figures for the German 75mm L24 with a Panzergranate 40 round; presumably the hollow charge round is meant. This has been corrected here, as has the spelling of "Marosczek".

Identical figures for the US, UK, USSR, German and Italian weapons listed above appear in Table 62 of the same author's "Brute Force", André Deutsch, 1990.

## Farrar-Hockley, 1976

"Infantry Tactics 1939-1945", Anthony Farrar-Hockley, Almark, 1976, page 41. Ammunition and armour type not specified, range in yards.

| Weapon | Range | Slope | Penetration |
| --- | --- | --- | --- |
| 2 Pounder | 1000 | 30º | 40 |

Comments and corrections

This is another title from Almark's "Mechanics of War" series.

## Featherstone, 1973

"Tank Battles in Miniature: A wargamer's guide to the Western Desert Campaign 1940–1942", Donald Featherstone, Patrick Stevens Ltd, 1973, pages 141–143. "At 50º–90º angle-of-impact gun can penetrate maximum armour thickness of: (in millimetres)", ranges in yards, armour and ammunition types unspecified.

| Weapon | Angle | 200 | 400 | 600 | 1000 | 1600 | 2000 | 2600 | 3000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37mm | 50º | 56 | 50 | 42 | 26 |  |  |  |  |
|  | 90º | 65 | 58 | 51 | 29 |  |  |  |  |
| 75mm M2 | 50º | 66 | 61 | 55 | 45 | 28 | 19 |  |  |
|  | 90º | 76 | 70 | 64 | 53 | 33 | 23 |  |  |
| 75mm M3 | 50º | 77 | 69 | 62 | 50 | 33 | 22 |  |  |
|  | 90º | 87 | 80 | 73 | 59 | 39 | 25 |  |  |
| 2 pdr | 50º | 56 | 50 | 42 | 26 |  |  |  |  |
|  | 90º | 66 | 58 | 51 | 29 |  |  |  |  |
| 6 pdr | 50º | 93 | 85 | 79 | 60 | 36 | 20 |  |  |
|  | 90º | 108 | 99 | 92 | 70 | 43 | 24 |  |  |
| 25 pdr | 50º | 80 | 71 | 66 | 52 | 31 | 19 | 10 | 5 |
|  | 90º | 89 | 73 | 68 | 58 | 38 | 26 | 19 | 15 |
| 20/55 | 50º | 32 | 24 | 16 |  |  |  |  |  |
| KwK | 90º | 35 | 28 | 19 |  |  |  |  |  |
| sPzB 41 | 50º | 54 | 50 | 38 | 12 |  |  |  |  |
|  | 90º | 63 | 57 | 45 | 15 |  |  |  |  |
| 37/45 | 50º | 49 | 40 | 32 | 16 |  |  |  |  |
| KwK | 90º | 61 | 52 | 43 | 26 |  |  |  |  |
| 37/45 | 50º | 55 | 47 | 39 | 22 |  |  |  |  |
| PaK | 90º | 64 | 55 | 46 | 29 |  |  |  |  |
| 47mm | 50º | 55 | 57 | 53 | 46 | 35 | 27 | 12 |  |
| PaK | 90º | 69 | 66 | 62 | 54 | 41 | 30 | 14 |  |
| 50/60 | 50º | 120 | 98 | 80 | 55 | 33 | 18 | 0 |  |
| PaK | 90º | 142 | 129 | 112 | 84 | 54 | 35 | 5 |  |
| 50/60 | 50º | 67 | 63 | 59 | 50 | 34 | 23 | 10 |  |
| KwK | 90º | 89 | 82 | 75 | 61 | 40 | 28 | 12 |  |
| 75/24 | 50º | 50 | 45 | 38 | 27 | 12 |  |  |  |
| KwK | 90º | 60 | 53 | 45 | 30 | 14 |  |  |  |
| 75/43-48 | 50º | 100 | 93 | 87 | 80 | 63 | 52 | 35 | 24 |
| KwK | 90º | 115 | 107 | 102 | 93 | 74 | 63 | 50 | 39 |
| 75/46 | 50º | 115 | 109 | 104 | 94 | 79 | 72 | 53 | 40 |
| PaK | 90º | 144 | 138 | 132 | 121 | 103 | 91 | 70 | 59 |
| 88/56 | 50º | 118 | 112 | 108 | 101 | 90 | 82 | 71 | 63 |
| KwK | 90º | 130 | 134 | 122 | 107 | 94 | 86 | 76 | 68 |

On p. 95, the following figures for the 88mm FlaK are given, against homogenous armour at 30º:

| Ammunition | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- |
| PzGr 39 | 111 | 102 | 93 | 86 |
| PzGr 40 | 157 | 141 | 126 | 114 |

It is also stated that the hollow-charge Gr 39 Hl penetrates 90mm at all ranges.

Comments and corrections

These figures were calculated by R. D. Miles, and include many more range bands than there is space to show here. Some figures matching those from Chamberlain & Ellis are also given on pp 96-97. In the main table, angles seem to be given in German style, from the horizontal. The figures are given in pairs; it is not wholly clear that the lower is at 50º and the higher at 90º, but that is how they have been interpreted here. The strange behaviour of the lower figure for the 47mm is not accounted for. From their higher penetration, it seems that PaKs have been credited with APCR ammunition, although their performance does not seem to fall off as rapidly as one might expect with increasing range.

## Fleischer, 1994

"Panzerfaust and other German Infantry Anti-Tank Weapons", Wolfgang Fleischer, Schiffer, 1994, page 47 (figure for 3.7cm PaK from text). Ranges in metres, armour type and slope not specified.

| Weapon | Range | Penetration |
| --- | --- | --- |
| 3.7cm PaK | 500 | 29 |
| AP rifle bullet | 100 | 8 |
| ATk rifle gren 30 | 40 | 50 |
| Large ATk rifle gren | 80 | 80 |
| ATk Rifle Gren 46 | 60-80 | 90 |
| ATk rifle gren 61 | 60-80 | 125 |
| ATk rifle gren GGP |  | 45 |
| Wurfkörper 326 HL/LP |  | 50 |
| Panzerwurfkörper 42 LP | 75 | 80 |
| Panzerbüsche 38 or 39 | 300 | 25 |
| Granatbüsche 39 | 80 | 80 |
| Panzerbüsche 35(P) | 100 | 30 |
| Panzerbüsche 783(r) or 784(r) | 100 | 30 |
| ATk hand gren 41 | 10-15 | 30 |
| ATk throwing mine (short) | 25 | 150 |
| ATk hand mine 3 | 0 | 130 |
| ATk hand mine 4 | 0 | 150 |
| Hollow charge 3 | 0 | 140 |
| Hollow charge 3.5 | 0 | 180 |
| T-mine 35 | 0 | 80-100 |
| Clustered charge 3Kg | 0 | 60 |
| Faustpatrone | 70 (max) | 140 |
| Panzerfaust klein | 30 | 140 |
| Panzerfaust gross (30m) | 30 | 200 |
| Panzerfaust 60 m | 60-75 | 200 |
| Panzerfaust 100 m | 100 | 200 |
| Panzerfaust 150 m | 150 | 220 |
| Panzerfaust 250 m | 250-300 | 220 |
| Raketenpanzerbüsche 54 or 54/1 | 150-180 | 160 |
| Raketenpanzerbüsche 10.5 cm |  | 220 |

Comments and corrections

The ranges are maximum effective ranges, so kinetic-energy weapons (ATRs here) may not have the performance indicated at exactly the range indicated. Panzerfaust 150 was undergoing troop trials at the end of the war. Neither Panzerfaust 250 nor 10.5cm Raketenpanzerbüsche were ever issued.

## Forty, 1979

"U.S. Army Handbook" (first edition), George Forty, Ian Allan, 1979, pages 99 and 107. Ammunition and armour type unspecified, ranges in yards

| Weapon | #### Range | Angle | Penetration (inches) | Penetration (mm) |
| --- | --- | --- | --- | --- |
| 37mm M3A1 | 1000 | 20º | 2.1 | 53 |
| 57mm M1 | 1000 | 20º | 2.7 | 69 |
| 3in M5 | 1000 | 20º | 3.85 | 98 |
| M1 or M9 Bazooka |  | 90º | 4.7 | 119 |

Comments and corrections

The conversion from inches in the original has been done into centimetres, using assuming 2.5cm to the inch, giving figures of 5.25cm, 6.75cm and 9.63cm respectively for the guns, and 11.75cm for the Bazooka. The figures given here use the correct conversion factor of 25.4mm to one inch.

Evidently the 90º is from the horizontal; the 20º, however, must be from the vertical.

"The Imperial War Museum Book of the Desert War 1940–1942", Adrian Gilbert, BCA 1992, page 134, gives a figure of 2.7in penetration at 1000 yards for the 6-pounder.

## Foss, 1974

"Artillery of the World" (first edition), Christopher Foss, Ian Allan, 1974. Ranges in metres, armour type and slope not specified.

| Weapon | Calibre | Ammunition | Range | Penetration |
| --- | --- | --- | --- | --- |
| 6 Pounder | 57mm |  | 475 | 118 |
| 17 Pounder | 76mm |  | 1000 | 222 |
| M-1943 D-1 | 152mm | Semi-AP | 1000 | 82 |
| M-1938 M-10 | 152mm | Semi-AP | 1000 | 82 |
| M-1937 ML-20 | 152mm | APHE | 1000 | 124 |
| M-1931/37 A-19 | 122mm | APHE | 1000 | 190 |
| M-1938 M-30 | 122mm | HEAT |  | 200 |
| M-1944 D-10 | 100mm | APHE | 1000 | 185 |
| M-1945 D-44 | 85mm | APHE | 1000 | 102 |
|  |  | HVAP | 1000 | 130 |
| M-1942 ZIS-3 | 76mm | APHE | 500 | 69 |
|  |  | HVAP | 500 | 92 |
|  |  | HEAT |  | 120 |
| M-1943 ZIS-2 | 57mm | APHE | 500 | 106 |
|  |  | HVAP | 500 | 140 |
| M-1942 | 45mm | HVAP | 500 | 66 |
| M-1939 (LAA) | 37mm | AP | 500 | 46 |
| ZPU-1 | 14.5mm | API | 500 | 32 |
| M-101A1 How | 105mm | HEAT | 1500 | 102 |
| M-1 LAA | 40mm |  | 900 | 50 @ 30º |

Comments and corrections

D-1, M-10 and M-30 are howitzers; A-19, D-10 and all smaller calibre pieces are guns; ML-20 is a gun-howitzer. The 45mm M-1942 is the long-barrelled (L/66) version of the weapon.

The M-1939 KS-12 and M-1944 KS-18 85mm AA guns are credited with the same penetration performance as the M-1945 D-44.

Foss quotes 380mm penetration for the 100mm D-10 HEAT round, but this sounds like post-war ammunition [Definitely, this is post-war ammunition, developed in 1946-47 – Valeriy Potapov]. The 120mm quoted for the ZIS-3 76mm ammunition is more believable for WW2 ammunition, but still more generous than the figure stated by Zaloga.

Although the ZPU-1 is a post-war weapon, it is mentioned here because the KPV 14.5mm HMG fire the same cartridge as the wartime PTRS and PTRD anti-tank rifles.

It is not clear what the significance of the range figure is for the US 105mm howitzer HEAT round, but perhaps this represents the maximum effective engagement range.

The performance given for the U.S. M-1 LAA gun – the familiar 40mm Bofors – seems quite optimistic.

The 1976 second edition of the book amends none of the figures cited here.

## Funcken & Funcken, 1976

Source: "Arms and Uniforms: The Second World War, Vol. 4", Liliane and Fred Funcken, Ward Lock, 1976. Ammunition type and armour type and slope not stated, ranges in yards.

| Weapon | Range | Penetration |
| --- | --- | --- |
| US 37mm | 1000 | 38 |
| PTRD | 333 | 38 |
| sPzB41 | 440 | 51 |
| German ATk grenades | 100–200 | 63–127 |
| PIAT | 100 | 102 |
| Jap hollow charge grenade | 0 | 76 |
| Lunge mine | 0/90º | 152 |
|  | 0/60º | 102 |
| Stielgranate | 200 | 152 |

Comments and corrections

Penetration figures in the original are given in inches.

## Gander & Chamberlain, 1977

Airfix Magazine guide 26, "American Tanks of World War 2", Terry Gander and Peter Chamberlain, Patrick Stevens Ltd, 1977.

Ranges in yards, ammunition type and armour type and slope unspecified.

| Weapon | Range | Penetration |
| --- | --- | --- |
| 37mm gun M6 | 500 | 61 |
| 75mm gun M2 | 500 | 60 |
| 75mm gun M3 | 500 | 70 |
| 3-inch gun M7 | 1000 | 80 |
| 76mm gun M1 | 1000 | 88 |
| 90mm gun M3 | 1000 | 120 |

Comments and corrections

It is not clear why Chamberlain did not use the figures from "British and American Tanks of World War II", which he co-authored with Chris Ellis.

These figures seem to show that the 37mm is as good as the 75mm M2 at 500 yards, and that there is a noticeable performance difference between the 76mm and 3-inch guns.

## Gander & Chamberlain, 1978

"Small Arms, Artillery and Special Weapons of the Third Reich", Terry Gander and Peter Chamberlain, Macdonald and Jane's, 1978. Armour type unstated, ranges in metres.

| Weapon | Ammunition | #### Range | Penetration @ 0º | Penetration @ 30º |
| --- | --- | --- | --- | --- |
| 7.92 mm PzB 38, 39 |  | 300 |  | 25 |
| 7.92mm M SS 41 |  | 300 | 20 |  |
| 13mm EW 141 |  | 300 |  | 25 |
| 2.8 cm sPzB41 | PzGr 42 | 100 | 94 | 69 |
|  |  | 200 | 86 | 65 |
|  |  | 300 | 79 | 60 |
|  |  | 400 | 72 | 56 |
|  |  | 500 | 66 | 52 |
|  |  | 600 | 60 | 48 |
|  |  | 700 | 54 | 44 |
|  |  | 800 | 49 | 41 |
| 3.7 cm Pak 35/36 | PzGr | 200 | 56 | 42 |
|  |  | 400 | 51 | 38 |
|  |  | 500 | 48 | 36 |
|  |  | 600 | 46 | 34 |
|  | PzGr 40 | 100 | 79 | 68 |
|  |  | 200 | 72 | 61 |
|  |  | 300 | 65 | 55 |
|  |  | 400 | 58 | 49 |
|  | StielGr 41 | 200 | 180 |  |
| 4.2 cm Pak 41 | PzGr 41 | 0 | 124 | 95 |
|  |  | 250 | 105 | 83 |
|  |  | 500 | 87 | 72 |
|  |  | 750 | 70 | 62 |
|  |  | 1000 | 60 | 53 |
| 5 cm Pak 38 | PzGr | 0 | 99 | 73 |
|  |  | 250 | 88 | 67 |
|  |  | 500 | 78 | 61 |
|  |  | 750 | 69 | 56 |
|  |  | 1000 | 61 | 50 |
|  |  | 1200 | 53 | 45 |
|  |  | 1500 | 47 | 40 |
|  | PzGr 40 | 0 | 165 | 143 |
|  |  | 250 | 141 | 109 |
|  |  | 500 | 120 | 86 |
|  |  | 750 | 101 | 69 |
|  |  | 1000 | 84 | 55 |
|  |  | 1250 | 70 | 44 |
| 7.5 cm Pak 40 | PzGr 39 | 0 | 149 | 121 |
|  |  | 500 | 135 | 106 |
|  |  | 1000 | 121 | 94 |
|  |  | 1500 | 109 | 83 |
|  |  | 2000 | 98 | 73 |
|  | PzGr 40 | 0 | 176 | 137 |
|  |  | 500 | 154 | 115 |
|  |  | 1000 | 133 | 96 |
|  |  | 1500 | 115 | 80 |
|  |  | 2000 | 98 | 66 |
|  |  | 2500 | 83 | 53 |
| 7.5 cm Pak 41 | PzGr 41 | 0 | 245 | 200 |
|  |  | 250 | 226 | 185 |
|  |  | 500 | 209 | 171 |
|  |  | 750 | 192 | 157 |
|  |  | 1000 | 177 | 145 |
|  |  | 1250 | 162 | 133 |
|  |  | 1500 | 149 | 122 |
|  |  | 1750 | 136 | 111 |
|  |  | 2000 | 124 | 102 |
| 7.62 cm Pak 36(r) | PzGr 39 | 0 | 133 | 108 |
|  |  | 500 | 120 | 98 |
|  |  | 1000 | 108 | 88 |
|  |  | 1500 | 97 | 79 |
|  |  | 2000 | 87 | 71 |
|  |  | 2500 | 78 | 64 |
| 8.8 cm Pak43, 43/1 | PzGr 39 | 0 | 225 | 198 |
|  |  | 500 | 207 | 182 |
|  |  | 1000 | 190 | 167 |
|  |  | 1500 | 174 | 153 |
|  |  | 2000 | 159 | 139 |
|  |  | 2500 | 145 | 127 |
|  | PzGr 40/43 | 0 | 311 | 265 |
|  |  | 500 | 274 | 226 |
|  |  | 1000 | 241 | 192 |
|  |  | 1500 | 211 | 162 |
|  |  | 2000 | 184 | 136 |
|  |  | 2500 | 159 | 114 |
| 12.8 cm Pak 44 |  | 500 | 219 |  |
|  |  | 1000 | 202 |  |
|  |  | 1500 | 187 |  |
| 8 cm PAW 600 | Hollow charge | 750 | 140 |  |
| 7.5/5.5 cm Pak 44 |  | 2500 |  | 120 |
| 10 cm PAW 600 | Hollow charge |  |  | 200 |
| Hammer |  | 500 |  | 100 |
| 3.7 cm Pak 36(p) |  | 400 |  | 37 |
|  |  | 600 | 40 |  |
| 4.5 cm Pak 184(r) |  | 900 |  | 38 |
| 7.92mm PzB 35(p) |  | 300 | 25–33 |  |
| 13.9mm PzB 782(e) |  | 300 | 21 |  |
| 14.5mm PzB |  | 100 | 30 |  |
| 783 or 784(r) |  | 300 | 27.5 |  |
|  |  | 500 | 25 |  |
| 2cm PzB 785(s) |  | 100 |  | 20–22 |
|  |  | 300 |  | 15–18 |
| Panzerfaust klein |  | 30 |  | 140 |
| Panzerfaust 30 |  | 30 |  | 200 |
| Panzerfaust 60 |  | 80 |  | 200 |
| Panzerfaust 100 |  | 100 |  | 200 |
| Panzerfaust 150 |  | 150 |  | 200 |
| Panzerfaust 250 |  | 200 |  | 200 |
| RPzB 54, 54/1 |  |  |  | 160 |
| 2.5cm Pak 112(f) (L/72) |  | 600 | 50 |  |
| 2.5cm Pak 113(f) (L/77) |  | 600 | 50 (not confirmed) |  |
| 4.7 cm Pak 36(t) |  | 1200 | 60 (homogenous) |  |
| 4.7cm Pak 181 or 183(f) |  | 200 | 80 |  |
| 4 cm Pak 192(e) |  | 183 | 60 |  |
|  |  | 365 | 55 |  |
|  |  | 548 | 51 |  |
| 5.7 cm Pak 208(r) |  | 500 | 140 |  |

Comments and corrections

This wonderful book, in achieving compendious coverage of weapons of the Third Reich, includes a large number of non-German weapons. Those given here include:

| Country of origin | German designation | Original designation |
| --- | --- | --- |
| Poland | 7.92mm PzB 35(p) | wz 35 Maroszcek |
|  | 3.7 cm Pak 36(p) | wz 36 |
| Russia | 14.5mm PzB 783(r) | PTRD-41 |
|  | 14.5mm PzB 784(r) | PTRS-41 |
|  | 4.5 cm Pak 184(r) | 45mm PTP M-1930 |
|  | 4.5 cm Pak 184/1(r) | 45mm PTP M-1932 |
|  | 5.7 cm Pak 208(r) | 57mm PTP M-1941 ZiS-2 |
|  | 7.62 cm Pak 36(r) | 76mm M-1936 |
| England | 13.9mm PzB 782(e) | 0.55" Boys Mk I |
|  | 4 cm Pak 192(e) | OQF 2 pr |
| Switzerland | 2cm PzB 785(s) | Solothurn s18-1100 |
| France | 2.5cm Pak 112(f) (L/72) | SA-L mle 1934 |
|  | 2.5cm Pak 113(f) (L/77) | SA-L mle 1937 |
|  | 4.7cm Pak 181 or 183(f) | SA mle 1937 or 1939 |
| Czechoslovakia | 4.7 cm Pak 36(t) | Skoda PUV vz 36 |

There was no 45-mm gun in 1930. I believe 4.5 cm Pak 184(r) refers to the 45 mm ATG M1932/37, therefore the 4.5 cm Pak 184/1(r) refers to 45 mm ATG M1942. PTP is Russian abbreviation, it means anti-tank gun (protivotankovaya pushka). The original (Soviet) designation for the 7.62 cm Pak 36(r) is "76 mm Divisional Gun Model 1936 F-22" – Valeriy Potapov.

The Czech 47mm was mounted in the Panzerjäger 1.

The Polish 37mm is the Bofors model, widely used by other nations, including Britain.

The Maroszcek ATR was also in Italian service.

The penetration quoted for the Soviet 57mm is presumably with APCR.

Of the German weapons listed, the following were experimental, and did not see service during the war:

| 13mm EW 141, Gerät 318 | 1938: Intended for RK 9 and VK 601 recce cars |
| --- | --- |
| 7.5/5.5 cm Pak 44 | Under development 1942-45 |
| 10 cm PAW 600, PWK 10H64 | Work started 1944 |
| Hammer, Panzertod, Panzerschreck | Work started Oct 1943, suspended most of 1944. |
| Panzerfaust 150 and 250 | At trials stage by end of war |

The authors state that APC was introduced with the 5cm Pak 38, and APCBC with the 7.5cm Pak 40.

Mention is made of PzGr 40 being used in "the early days" of the invasion of Russia.

These authors give the date of the decision to give priority for tungsten supplies to the machine-tool industry as 1941.

## Godfrey, 1984

"The Pusan Perimeter", Maj. F A Godfrey, in: "The Korean War: History and Tactics", Ed. David Rees, Orbis, 1984, page 24. Penetration at 500 metres, ammunition type and armour type and slope not specified.

| Weapon | Penetration |
| --- | --- |
| 85mm in T-34 | 114 |
| 76mm in M4A3 Sherman | 110 |

Comments and corrections

The accompanying graphic suggests that these figures are for an impact angle of 30º.

These look like APCR performance figures in both cases.

## Grove, 1976

"World War II Tanks", Eric Grove, Orbis, 1976. Armour type unstated, conventional armour-piercing ammunition unless otherwise stated, ranges in yards.

| Weapon | Slope | Range | Penetration |
| --- | --- | --- | --- |
| 20mmm KWK 30 | 0º | 500 | 24 |
| 75mm L/43 KWK 40 | 30º | 500 | 89 |
| PaK 36(r) | 30º | 500 | 83 |
| ...with PzGr 40 | 30º | 500 | 118 |
| 88mm KWK 36 | 30º | 500 | 112 |
| 75mm KWK 42 | 30º | 1000 | 120 |
| 88mm PaK 43 | 30º | 500 | 226 |
| 88mm KWK 43 | 30º | 500 | 182 |
| 2-pounder | 30º |  | 57 |
| 3-pounder | 30º |  | 27 |
| 77mm | 30º | 500 | 109 |
| 17-pounder | 30º | 500 | 120 |
| M-40 76mm L/41 | 30º | 500 | about 65 |
| D-5T85 (M-44) 85mm | 30º | 1000 | 95 |
| D-10S 100mm | 0º | 1000 | 160 |
| 75mm M2 | 30º | 500 | 60 |
| 75mm M3 | 30º | 500 | 70 |
| 76mm M1A1C or M2 | 30º | 500 | 94 |
| 3-in M7 | 30º | 500 | over 100 |

Comments and corrections

The difference between the performance stated for the two "long" 88s is presumably due to the figure for the PaK43 applying to APCR. The muzzle velocities quoted tend to support this view.

## Handbook on the Italian Army

"Handbook on the Italian Army", Athena Books, 1983, pages 106 and 156. Ranges in yards, armour type and ammunition unspecified.

| Weapon | Range | Angle | Penetration |
| --- | --- | --- | --- |
| 20mm Breda | 500 | 0º | 25 |
|  |  | 30º | 15 |
| Solothurn ATk rifle | 550 | 15º | 25 |
| Hotchkiss 25mm L/72 | 100 |  | 40 |
| 37mm L/35 | 400 | 30º | 38 |

Comments and corrections

This source is a facsimile edition of a 1943 original.

## Hogg, 1975

"German Artillery of World War 2", Ian V. Hogg, A&AP, 1975. Ranges in metres, penetration of homogenous armour.

| Weapon | Ammo | Angle | 100 | 250 | 500 | 1000 | 1500 | 2000 | 3000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SPzB41 | PzGr 41 | 0º | 94 |  | 66 |  |  |  |  |
|  |  | 30º | 69 |  | 52 |  |  |  |  |
| Pak 36 | PzGr | 0º | 65 |  | 48 |  |  |  |  |
|  |  | 30º | 50 |  | 36 |  |  |  |  |
|  | PzGr 40 | 0º | 79 |  | 50 |  |  |  |  |
|  |  | 30º | 68 |  | 40 |  |  |  |  |
| PJK 41 | PzGr41 | 0º | 120 |  | 87 | 60 |  |  |  |
|  |  | 30º | 90 |  | 72 | 53 |  |  |  |
| Pak 38 | PzGr 38 | 0º |  | 88 | 78 | 61 |  |  |  |
|  |  | 30º |  | 67 | 61 | 50 |  |  |  |
|  | PzGr 40 | 0º |  | 141 | 120 | 84 |  |  |  |
|  |  | 30º |  | 109 | 86 | 55 |  |  |  |
| Pak 40 | PzGr 39 | 0º | 148 |  | 132 | 116 | 102 |  |  |
|  |  | 30º | 120 |  | 104 | 89 | 76 |  |  |
|  | PzGr 40 | 0º | 175 |  | 154 | 133 | 115 |  |  |
|  |  | 30º | 135 |  | 115 | 96 | 80 |  |  |
| Pak 41 | PzGr 41 HK | 0º |  | 226 | 209 | 177 | 149 | 124 |  |
|  |  | 30º |  | 185 | 171 | 145 | 122 | 102 |  |
| Pak 36(r) | PzGr 39 rot | 0º |  |  | 120 | 108 | 97 | 87 |  |
|  |  | 30º |  |  | 98 | 88 | 79 | 71 |  |
|  | PzGr 40 | 0º |  |  | 158 | 130 | 106 | 84 |  |
|  |  | 30º |  |  | 118 | 92 | 71 | 55 |  |
| Pak 43 | PzGr 39 | 0º |  |  | 207 | 190 | 174 | 159 |  |
|  |  | 30º |  |  | 182 | 167 | 153 | 139 |  |
|  | PzGr 40 | 0º |  |  | 274 | 241 | 211 | 184 |  |
|  |  | 30º |  |  | 226 | 192 | 162 | 136 |  |
| Pak 44 | PzGr 43 | 30º |  |  |  | 230 |  | 200 | 173 |
| 10.5 cm | PzGr rot | 0º |  |  | 67 | 62 | 59 |  |  |
| le FH 18 |  | 30º |  |  | 56 | 52 | 49 |  |  |
| Flak 36 | PzGr | 30º |  |  | 110 | 105 |  |  |  |
|  | PzGr 40 | 30º |  |  | 126 | 103 |  |  |  |
| Flak 41 | PzGr 39/1 | 30º |  |  |  | 202 |  |  |  |
| Flak 39 | Pz Sprgr | 30º |  |  |  | 140 |  |  |  |

Hollow charge rounds, same at all ranges:

| Weapon | Ammo | Angle | Penetration |
| --- | --- | --- | --- |
| 7.5cm le IG 18 (or LG 40) | 7.5cm I Gr 38 Hl | 30º | 45 |
| 10.5cm le FH 18 (or LG40) | 10.5cm Gr 39 rot Hl/A | 0º | 100 |
|  |  | 30º | 70 |
| Flak 36 | 8.8cm Hl Gr 39 |  | 165 |
| Pak 36 | Stielgranate 41 |  | 180 |
| Pak 38 | Stielgranate 42 |  | 180 |
| PAW 600 | 8cm W Gr H1 |  | 140 |

Comments and corrections

The author notes that there are 4 models of hollow charge shell for the leFH 18; the Gr 39 Hl, Hl/A, Hl/B and Hl/C. The experimental 7.5cm Pak 44 has a claimed performance of 120mm at 2500 metres.

Stielgranate 41 has max range 300m; Stielgranate 42 has max recommended range 150m.

PAW 600 has max range 750m. PzGr43 for the 128mm PaK44 is a standard APCBC round.

Some penetration figures are quoted as being at 3º, but it seems clear that this is a misprint for 30º.

The text refers to the "the 1942 ban on tungsten" halting production of PzGr 40 for the Pak 40. In the same author's "The guns 1939–1945", Macdonald, 1970, he gives the date of the Führer's order that tungsten was no longer to be used for weapons as June 1942.

## Hogg, 1976

"The Guns of World War II", Ian V. Hogg, Macdonald and Jane's, 1976. Ammunition type and armour type unstated, ranges in yards.

| Weapon | Ammunition | Range | Angle | Penetration |
| --- | --- | --- | --- | --- |
| 2-pounder |  | 1000 |  | 50 |
| Ger 75mm |  | 1000 |  | 94 |
| 88mm |  | 1000 | 30º | 105 |
|  | PzGr 40 | 500 |  | 126 |
|  | Hollow charge |  |  | 165 |
| Pak 36(r) |  | 1000 |  | 108 |
|  | PzGr 40 | 1000 |  | 130 |
| US 90mm M1–3 |  | 1000 |  | 109 |
|  | HVAP | 1000 |  | 173 |
| US 90mm T54 | HVAP | 1000 |  | 300 |
| US 90mm T8 | HVAP | 1000 |  | 250 |
| 17-pounder |  | 1000 |  | 110 |
|  | APDS | 1000 |  | 231 |
| Ger 105 FH 18/40 |  | 1000 |  | 62 |
|  | PzGr 39TS | 1000 |  | 80 |

Comments and corrections

The 90mm T54 and T8 were experimental only.

In this book, Hogg says tungsten supplies for ammunition dried up "after 1943".

## Hogg, 1977

"The Encyclopedia of Infantry Weapons of World War II", Ian V. Hogg, A&AP, 1977, page 190 and from the text in the "Grenades" chapter. Ranges in units specified, armour type not specified.

| Weapon | Ammunition | Range | Angle | Penetration |
| --- | --- | --- | --- | --- |
| 6 pounder Mk 2 |  | 1000 yds | 30º | 74 |
| 37mm M3A1 |  | 1000 yds | 0º | 54 |
| 57mm RCL M18 | HEAT |  |  | 75 |
| 75mm RCL M20 | HEAT |  |  | 100 (est) |
| 57mm M1 |  | 1000 yds | 20º | 68 |
| 45mm M42 | AP | 500 m | 30º | 50 |
|  | APCR | 500 m | 30º | 54 |
| 57mm M43 | AP | 500 m | 30º | 86 |
|  | APCR | 500 m | 30º | 100 |
| 2.8cm s PzB 41 | APCNR | 500 m | 30º | 52 |
| 3.7cm PaK 36 | AP | 500 m | 30º | 36 |
|  | APCR | 500 m | 30º | 40 |
| 4.2 cm PJK 41 | APCNR | 500 m | 30º | 72 |
| 5 cm PaK 38 | AP | 1000 m | 30º | 50 |
|  |  | 500 m | 30º | 61 |
|  |  | 250 m | 0º | 88 |
|  | APCR | 1000 m | 30º | 55 |
|  |  | 500 m | 30º | 86 |
|  |  | 250 m | 0º | 141 |
| 7.5 cm PaK 40 | AP | 1000 m | 30º | 89 |
|  |  | 500 m | 30º | 104 |
|  | APCR | 1000 m | 30º | 96 |
|  |  | 500 m | 30º | 115 |
| 7.5 cm PaK 41 | APCNR | 2000 m | 30º | 171 |
|  |  | 1000m | 30º | 145 |
|  |  | 500m | 30º | 102 |
| 8cm PAW 600 | HEAT | 850 yds | 0º | 140 |
| 8.8cm PaK 43 | AP | 2000 m | 30º | 167 |
|  |  | 1000m | 30º | 139 |
|  | APCR | 2000 m | 30º | 192 |
|  |  | 1000m | 30º | 136 |
| 37mm Model 94 | AP | 500m | 30º | 32 |
| 47mm Model 01 | AP | 700m | 30º | 70 |
| Boys ATR |  | 500m | 0º | 20 |
| PTRS, PTRD |  | 500m | 0º | 25 |
| PzB 38, 39 |  | 300m | 30º | 25 |
| S 18-1000 |  | 300m | 0º | 35 |
| Type 97 |  | 200m | 0º | 12 |
| WZ/35 |  | 300m | 0º | 20 |
| RPG-43 | HEAT |  |  | 76 (homogenous) |
| Grenade, rifle, 68 | HEAT | 100m | 0º | 50 |
| ATk grenade M9 | HEAT |  |  | 40 |
| ATk gren M9A1 | HEAT |  |  | 60 |
| PIAT | HEAT | 100 yds |  | 75 (approx) |
| 2.36 in RL | HEAT | 400 yds |  | 80 (approx) |
| Haft Hohlladung 3Kg | HEAT |  |  | 110 |
| Gross/Klein Gew PzGr | HEAT | 100m |  | 40 |
| Gew PzGr 46mm | HEAT | 100m |  | 90 |
| SS Gew PzGr 61mm | HEAT | 200m |  | 125 |
| Panzerfaust | HEAT | 30, 60, 100, 150m | 30º | 200 |
| R PzB 54 | HEAT | 150m | 0º | 100 (approx) |
| RW 43 Puppchen | HEAT | 750m | 0º | 100 (approx) |

Comments and corrections

Hogg himself points out the difficulty of believing both the Russian and the Japanese figures for their 47mm guns.

Barry Gregory and John Batchelor's "Airborne Warfare 1941–1945", Phoebus, 1979, also gives "approximately 75mm" as the penetration of the PIAT, with no further qualification.

The 50mm penetration figure for the British No. 68 rifle grenade is repeated in Hogg's "Illustrated Encyclopedia of Ammunition", The Apple Press, 1985, although again without specifying armour type or slope.

## Hogg, 1978

"British & American Artillery of World War 2", Ian V. Hogg, A&AP, 1978. Armour type in most cases not stated, but assumed to be homogenous. Ranges in yards.

| Weapon | Ammo | #### Armour | Angle | 100 | 400 | 500 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2pdr | AP Mk 10T |  | 30º |  |  |  | 42 |
| 6pdr | AP Mk 1-7T |  | 30º |  |  |  | 74 |
|  | APCBC Mk 9T |  | 30º |  |  |  | 88 |
|  | APDS Mk 1T |  |  |  |  |  | 146 |
| 6pdr 6cwt AA | APC Mk 3T |  | 30º |  |  |  | 75 |
| 17pdr | AP Mk 3T |  | 30º |  |  |  | 109 |
|  | APC Mk 4T |  | 30º |  |  |  | 118 |
|  | APDS Mk 1T |  | 30º |  |  |  | 231 |
| 25pdr | AP Mk 1T-6T, 8T |  | 0º |  | 70 |  |  |
| 3in 20cwt AA | AP Mk 2T |  | 30º |  |  |  | 84 |
| 3.7in AA | AP Mk 5T |  | 30º |  |  |  | 117 |
| 37mm M3 | AP M74 |  | 0º |  |  | 36 |  |
|  | APC M51B1/2 |  | 0º |  |  | 61 |  |
| 57mm M1 | AP M70 |  | 20º |  |  |  | 73 |
|  | APC M86 | H | 20º |  |  |  | 73 |
|  |  | FH | 20º |  |  |  | 79 |
| 75mm M1916 | APC M61 | H |  |  |  | 70 |  |
| 3in or 76mm | AP M79 | H | 0º |  |  |  | 100 |
|  |  | FH |  |  |  |  | 70 |
| 90mm | AP M77 | H |  |  |  | 142 |  |
|  | APC M82 | H |  |  |  | 130 |  |
|  |  | FH |  |  |  | 140 |  |
| 105mm T8 | APC | H |  | 210 |  |  |  |
| 37mm AA | AP M59A1 | H |  |  |  | 25 |  |
|  |  | FH |  |  |  | 48 |  |
| 40mm AA | AP M18A1 | H | 0º |  |  | 52 | 42 |

Hollow charge rounds, same at all ranges, armour type and angle unspecified:

| Weapon | Ammo | Penetration |
| --- | --- | --- |
| 105mm howitzer M1 | HEAT M67 | 115 |
| 75mm pack howitzer M3A1 | HEAT M66 | 89 |
| 3.7in mountain howitzer | HEAT 3.7in Mk 1 | 63 |
| Smith gun | Bomb, SB, HEAT, Mk 1 | 63 (approx) |

Comments and corrections

The 105mm T8 was experimental.

The 6pdr 6cwt AA gun, Britain's effort at an "intermediate" AA gun, never entered service.

The 3.7in howitzer HEAT was developed in 1942 for the Indian Army.

The figures claimed for British ATk guns at 1000 yards seem much more believable for 500.

Conversely, the figures stated for the US 90mm are unbelievably poor.

It does not seem likely that the 37mm M59A1 is really almost twice as good against face-hardened as homogenous armour.

In this book, Hogg states that 6pdr APDS was issued in June 1944, although in "The Illustrated Encyclopedia of Ammunition", Apple Press, 1985, he says on p. 58 that it "appeared" in 1944, and on p. 151 that it was "first introduced for the 6-pdr" in 1943. In the same book, he refers to "the grave shortage of tungsten which affected Germany from 1943 onwards".

## Hogg, 1997

Source: "Tank Killers", Ian Hogg, Pan, 1997. Ranges in yards.

| Weapon | Ammo | Range | #### Angle | Armour | Penetration |
| --- | --- | --- | --- | --- | --- |
| sPzB41 |  | 545 |  |  | 66 |
|  |  | 600 |  | Homogenous | 60 |
| 6-pdr | APDS | 1000 |  |  | 146 |
| Boys ATR |  | 330 |  |  | 21 |
| Marocszek |  | 330 |  |  | 20 |
| PzB 39 |  | 110 |  |  | 33 |
| s18-1100 |  | 330 |  |  | 27 |
| PTRS, PTRD |  | 545 |  |  | 25 |
|  |  | 545 | 30º |  | 20 |
| Panzerfaust |  |  | 30º |  | 140 |
| 37mm Bofors |  | 1000 |  |  | 20 |
| PaK 36 |  | 545 |  |  | 48 |
| 25mm SA-L 34 |  | 545 |  |  | 40 |
| 47mm SA-37 |  | 875 |  |  | 70 |
| 2-pounder |  | 1000 | 30º |  | 42 |
|  |  |  |  |  | 51 |
| PaK 38 |  | 1000 |  |  | 60 |
|  | PzGr 40 | 1095 |  |  | 84 |
| PaK 40 |  | 110 |  |  | 116 |
|  | PzGr 40 | 1095 |  |  | 133 |
| lePaK 41 |  | 545 |  |  | 87 |
| PaK 41 |  | 2185 |  |  | 124 |
| 25-pounder |  | 440 |  |  | 70 |
| 17-pounder | AP | 1000 |  |  | 109 |
|  | APC | 1000 |  |  | 118 |
|  | APDS | 1000 | 30º | Face-hardened | 231 |
|  | APDS | 1000 |  |  | 230 |
| PaK 43 |  | 2185 | 30º |  | 159 |
|  | PzGr 40/43 | 2185 | 30º |  | 184 |
| PaK 44 |  | 1000 |  |  | 200 |
|  |  | 3280 |  |  | 173 |
| Sov 57mm |  | 545 |  |  | 140 |
| 76mm M-42 |  | 545 |  |  | 69 |
| 85mm D-44 |  | 545 |  |  | 100 |
|  | APCR | 545 |  |  | 113 or 130 |
| Bazooka |  | 109 |  |  | 178 |
| 100mm DS-3 | APCR | 1095 |  |  | 185 |
|  | APCR | 1095 |  |  | 180 |
| 37mm M3A1 |  | 545 |  |  | 64 |
| 3-in M5 |  | 1000 |  |  | 100 |
| US 105mm |  | 1000 |  |  | 210 |
| 57mm RCL |  |  |  |  | 76 |
| 75mm RCL |  |  |  |  | 92 |
| US 90mm |  | 1000 |  |  | 122 |
|  | HVAP | 1000 | 0º |  | 230 |
| Sov 122mm |  | 1095 |  |  | 160 |
| Rotkäppchen |  | 1095 | 30º |  | 200 |
| 30mm MK101 | APCR | 330 |  |  | 75 |

Comments and corrections

Hogg gives the introduction dates for APDS as June 1944 for the 6-pdr and August 1944 for the 17-pdr, and dates the German tungsten decision to "early 1942". The ambiguity about the Soviet 85mm D-44 occurs in the original. The Rotkäppchen ATGW and the US 105mm never saw service.

## Hogg & Weeks, 1977

"Military Small Arms of the 20th century", Ian V. Hogg and John Weeks, A&AP, 1977, pages 266 to 273. Armour type unspecified, ranges in metres.

| Weapon | Range | Angle | Penetration |
| --- | --- | --- | --- |
| Mauser T-Gewehr | 200 | 0º | 25 |
| PzB 38 or 39 | 100 | 30º | 30 |
| PzB 41 | 250 | 0º | 30 |
| Boys ATR | 300 | 0º | 21 |
| wz 35 Marosczek | 300 | 0º | 20 |
| PTRD or PTRS | 500 | 0º | 25 |

Comments and corrections

The original gives and angle of 90º for the penetration performance of the PzB 41, changed to 0º here for consistency with the other entries.

This source states that tungsten-cored bullets were available for the Marosczek, that these were copied by the Germans, that in 1940 the Boys was issued with a tungsten-cored bullet with a plastic/aluminium body, and that Soviet ATRs originally fired a boat-tailed steel bullet which was replaced by a non-streamlined tungsten-cored round. Unfortunately, neither this nor any other source consulted distinguishes between the penetration performance of tungsten and steel ammunition.

## Hunnicut, 1978

"Sherman, A History of the American Medium Tank", R. P. Hunnicutt, Presidio Press, 1978, pages 559–570. Ranges in yards; armour type (FH = Face-hardened, H = Homogenous) as shown at 30º.

| Weapon | Ammo | Type | 250 | 500 | 750 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37mm M6 | APC | H |  | 53 |  | 46 | 40 | 35 |
|  | M51 | FH |  | 46 |  | 40 | 38 | 33 |
| 2 pdr IX-X | APHV/T | H | 64 | 57 | 51 | 45 |  |  |
|  | AP/T | H | 58 | 52 | 46 | 40 |  |  |
| 6 pdr III & V | APCBC-T | H |  | 81 |  | 74 | 63 | 56 |
|  |  | FH |  | 76 |  | 74 | 68 | 63 |
| 75mm M2 | APC | H |  | 60 |  | 55 | 51 | 46 |
|  | M61 | FH |  | 69 |  | 60 | 55 | 48 |
|  | APC | H |  | 60 |  | 53 | 46 | 38 |
|  | M72 | FH |  | 58 |  | 46 | 33 | 25 |
| 75mm M3 | APC | H |  | 66 |  | 60 | 55 | 50 |
| or M6 | M61 | FH |  | 74 |  | 67 | 60 | 54 |
|  | APC | H |  | 76 |  | 63 | 51 | 43 |
|  | M72 | FH |  | 66 |  | 53 | 41 | 33 |
|  | HVAP T45 | H |  | 117 |  | 97 | 79 | 64 |
| 76mm M1 and | APC M62 | H |  | 93 |  | 88 | 82 | 75 |
| 3-in M7 | HVAP M93 | H |  | 157 |  | 135 | 116 | 98 |
|  | AP M79 | H |  | 109 |  | 92 | 76 | 64 |
| 17 pdr IV, VII | APCBC Mk VIII T | H |  | 140 |  | 130 | 120 | 111 |
|  | APSV/DS | H |  | 208 |  | 192 | 176 | 161 |
| 25 pdr II, III | AP-T chg 3 | H |  | 63 | 58 | 54 |  |  |
| 90mm M2 | APC M82(early) | H |  | 120 |  | 112 | 104 | 96 |
|  | APC M82(late) | H |  | 129 |  | 122 | 114 | 106 |
|  | HVAP M304 | H |  | 221 |  | 199 | 176 | 156 |
|  | AP T33 | H |  | 119 |  | 117 | 114 | 109 |
| 155mm M1918 | AP | H |  | 127 |  | 119 |  |  |
|  | M112B1 | FH |  | 109 |  | 102 |  |  |
| 155mm M1, M2 | AP | H |  | 160 |  | 152 |  |  |
|  | M112B1 | FH |  | 135 |  | 130 |  |  |

Comments and corrections

The HVAP T45 round for the 75mm was experimental only.

The 105mm howitzer M2A1 and M4 fires HEAT M67, which penetrates 102mm of homogenous armour at 0º at all ranges.

Hunnicutt says "a few" 17 pdr APDS became available in Aug 44, but that the early rounds were "somewhat erratic", and less accurate than APCBC.

## Isby, 1974

"Patrol! Modern Infantry Tactics, 1914–74", David C. Isby, in: Strategy & Tactics No. 46, Simulations Publications, Inc., Sept/Oct 1974, page 35. Armour type and slope not specified, ranges in metres.

| Weapon | Range | Penetration |
| --- | --- | --- |
| M9 2.36" Bazooka | 100 | 100 |
| Panzerfaust 60 | 80 | 203 |
| Panzerfaust 44-1 | 200 | 320 |

Comments and corrections

The original notes the PF-44 as "In svce., 1944 to date", but the penetration figure looks as if it applies to a post-WW2 warhead.

## Isby, 1979

"Panzer Battles: The Evolution of Mechanized Warfare, 1939–1979", David C. Isby, in: Strategy & Tactics No. 73, Simulations Publications, Inc., March/April 1979, page 9. Armour type and slope not specified, ranges in metres.

| Vehicle | Ammo | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- |
| Grant | APC | 66 | 61 | 56 | 51 |
| Pz IIIj | AP | 61 | 48 | 38 |  |

Comments and corrections

It is assumed that the weapons intended are the 75mm M2 for the Grant and the 50mm L/60 KwK 39 for the Pz III J.

## Kent, 1990

"German 7,9mm military ammunition, 1888-1945", Daniel W Kent, privately published, 1990. Armour type not specified, ranges in yards.

| Cartridge | Range | Angle | Penetration |
| --- | --- | --- | --- |
| 7.9x57 SmKH |  | 0º | 18-19 |
|  |  | 30º | 13.5 |
| 7.9x94 SmKRs L'Spur | 100 | 0º | 32 |
|  | 300 | 0º | 25 |

Comments and corrections

Thanks to Tony Williams for providing this information.

The original gives performance at 90º and 30º; presumably the first is from the horizontal and the second from the vertical. Pentration for the L'Spur cartridge is given in inches in the original.

Mr. Kent seems to take an enthusiastic and practical direct approach to his subject. He has confirmed the brittleness of a tungsten-carbide core by shattering it with a hammer, and verified the continued potency of the tear-gas element in German ATR rounds by personally experiencing its effects.

## Kosnett, 1977

"Highway to the Reich: Operation Market-Garden 17–26 September 1944", Phil Kosnett, in: Strategy & Tactics No. 61, Simulations Publications, Inc., March/April 1977, page 32. Ammunition type and armour type and slope not specified, ranges in metres.

| Weapon | Range | Penetration | Weapon | Range | Penetration |
| --- | --- | --- | --- | --- | --- |
| Panzerfaust | 30 | 175 | Ger 75/48 | 500 | 125 |
| Panzerschreck | 400 | 100 | Ger 75/70 | 500 | 155 |
| PIAT | 50 | 100 | Ger 88/56 | 500 | 117 |
| Bazooka | 400 | 150 | 2-pdr | 500 | 55 |
| PAK 40 | 500 | 154 | US 75/40 | 500 | 96 |
| PAK 43 | 500 | 274 | 6-pdr/US 57mm | 500 | 80 |
| Ger 50/60 | 500 | 75 | 17-pdr | 500 | 120 |

Comments and corrections

No other source is so generous as to give Panzerschreck a range of 400m.

The 6pdr is mis-designated as "6-lb" in the original, and the German 75/70 and 88/56 mis-designated 75/90 and 88/42 respectively.

## Labbett & Brown, 1988

"Technical Ammunition Guide Series 2 Pamphlet 8, British Anti-Tank Rifle ammunition 1917-1945", P Labbett and F A Brown, Labbett-Brown, 1988. Proof penetration by 7 out of 10 bullets at 100 metres range and 20º angle of strike, armour type unspecified:

| Boys anti-tank rifle firing: | Penetration |
| --- | --- |
| Cartridge SA Armour Piercing .55 inch W Mark 1 | 16 |
| Cartridge SA Armour Piercing .55 inch W Mark 2 | 19 |

Comments and corrections

An APCR round approved "for record purposes only" in Spetember 1945 and not adopted was considered to give as good a performance at 500m as the W Mark 2 did at 50m. Has this round been adopted, it would almost certainly have been designated W Mark 3.

Thanks to Tony Williams for providing this information.

## Lucas, 1988

"Storming Eagles", James Lucas, Grafton Books, 1988. Armour type unstated, range in yards.

| Weapon | Range | Slope | Penetration |
| --- | --- | --- | --- |
| Panzerfaust |  |  | 200 |
| sPzB 41 | 400 | 30º | 53 |
| 7.5cm LG40 |  | 30º | 50 |

Comments and corrections

No comments.

## Lucas, 1994

"The Last Year of the German Army", James Lucas, BCA, 1994. page 82. Penetration in millimetres, ranges in metres, armour type and slope unstated:

| Weapon | 500 | 1000 | 2000 | 2500 |
| --- | --- | --- | --- | --- |
| 88mm KwK L/71 | 205 | 186 | 154 | 140 |

Comments and corrections

No comments.

## Macksey, 1988

"Tank versus Tank", Kenneth Macksey, Guild Publishing, 1988. Ranges in metres at which "high-velocity projectiles fail to penetrate" the specified target at 30º.

| Weapon | Ammo | #### Target | Armour | Range |
| --- | --- | --- | --- | --- |
| 40mm (2-pdr) | AP | Pz Kw III | 30 | 1200 |
| 47mm (Somua) |  | " | 30 | 1000 |
| 37mm (Pz Kw III) | APC | Matilda II | 70 | 0 |
|  |  | Somua | 60 | 200 |
| 88mm L/56 FlaK | AP | Matilda II | 70 | 2000 |
|  |  | Somua | 60 | 2000 |
| 50mm L60 | APC | T34/76B | 60 | 500 |
|  |  | KV1 | 90 | 0 |
|  |  | Sherman M4A1 | 85 | 350 |
| 75mm L48 | APCBC | T34/76B | 60 | 2000 |
|  |  | KV1 | 90 | 500 |
|  |  | Sherman M4A1 | 85 | 800 |
| Sov 76mm | APCBC | Pz Kw IIIJ | 57+20 | 200 |
|  |  | Pz Kw IVF | 50 | 1000 |
| 75mm L/40 | APC | Pz Kw IIIJ | 57+20 | 0 |
|  |  | Pz Kw IVF | 50 | 2000 |
|  |  | Tiger I | 110 | 0 |
|  |  | Panther D | 120 | 0 |
| 57mm L50 (6-pdr) | APCBC | Tiger I | 110 | 200 |
|  |  | Panther D | 120 | 0 |
| 85mm L/53 | APCBC | Tiger I | 110 | 500 |
|  |  | Panther D | 120 | 100 |
| 88mm L/56 | APCBC | Sherman M4A1 | 84 | 1500 |
|  |  | Churchill IV | 88 | 1400 |
|  |  | T34/85 | 75 | 1400 |
| 75mm L/70 | APCBC | Sherman M4A1 | 84 | 2000+ |
|  |  | Churchill IV | 88 | 2000+ |
|  |  | T34/85 | 75 | 2000+ |
|  |  | Sherman (76) | 105 | 1600 |
|  |  | Comet | 101 | 1500 |
|  |  | Pershing | 145 | 600 |
|  |  | JS2 | 160 | 400 |
| 88mm L/71 | APCBC | Sherman (76) | 105 | 2500+ |
|  |  | Comet | 101 | 2500+ |
|  |  | Pershing | 145 | 1800+ |
|  |  | JS2 | 160 | 1200 |
| 76mm L/55 | HVAP | Panther D | 120 | 1500 |
|  |  | Tiger II | 185 | 0 |
| 76mm L/50 (77mm) | APDS | Panther D | 120 | 1500 |
|  |  | Tiger II | 185 | 500 |
| 90mm L/53 | HVAP | Panther D | 120 | 2000+ |
|  |  | Tiger II | 185 | 1300 |
| 122mm L/43 | APCBC | Panther D | 120 | 1500 |
|  |  | Tiger II | 185 | 1200 |

Comments and corrections

Each table bears the weird legend "Range which high-velocity projectiles fail to penetrate vary according to range and angle of impact".

On p. 129, the 17pdr is cited as penetrating 118mm with APCBC, 170mm with APDS; and the 88mm, 101mm with APCBC, 103mm with APCR, against homogenous armour at 1000m and 30º.

The British introduction of APC is dated to May 1942.

## Messenger, 1982

"The Unknown Alamein", Charles Messenger, Ian Allan, 1982, pp 29–31. Armour type and impact angle unstated, ranges in yards.

| Weapon | Range | Penetration |
| --- | --- | --- |
| 2pdr | 1000 | 40 |
| 6pdr | 1500 | 52 |
| 25pdr | 1000 | 54 |
| US 37mm | 1000 | 46 |
| US 75mm | 2000 | 38 |
| Ger 20mm | 500 | 25 |
| Ger 37mm | 500 | 28 |
| 50mm (short) | 750 | 46 |
| 50mm (long) | 1000 | 50 |
| 75mm (short) | 1000 | 41 |
| 75mm (long) | 2000 | 62 |
| 76.2mm | 2000 | 53 |
| 88mm | 2000 | 83 |
| It 20mm Breda | 500 | 24 |
| It 47/32 | 1000 | 32 |

Accompanying the above figures is a graph showing penetration of 30º armour against range in yards. The curves shown are almost exclusively linear; the following approximate figures can be read off:

| Weapon | 250 | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- |
| Short 50mm | 59 | 53 |  |  |  |
| 2pdr |  | 54 | 42 |  |  |
| US 37mm | 59 | 55 | 46 |  |  |
| Long 50mm |  | 62 | 50 |  |  |
| US 75mm |  | 63 | 55 | 47 |  |
| 25pdr |  | 64 | 56 |  |  |
| 6pdr |  | 79 | 66 | 54 | 40 |
| 75mm L43 |  | 89 | 79 | 69 | 59 |
| 88mm |  | 111 | 103 | 93 | 84 |

Comments and corrections

Note that the figures given on the graph do not always exactly match those given in the text.

The author indicates that a hollow-charge round was available for the long 75mm gun, with a penetration of approx. 75mm.

## Moore, 1991

"Panzer Bait: With the 3rd Royal Tank Regiment 1940–1944", William Moore, Leo Cooper, 1991. Armour type and slope not specified, range in yards.

| Weapon | Ammunition | Range | Penetration |
| --- | --- | --- | --- |
| 50mm | PzGr40 | 440 | 78 |
| It 47mm in M13, M14 |  | 400 | 48 |
| 88mm |  | 2000 | 83 |
| 75mm in Mk IV special |  | 1000 | 70 |
| Improved 75mm |  | 1000 | 84 |
| Panther 75mm |  | 2000 | 100 |

Comments and corrections

It is fairly obvious which weapons are referred to here; the 50mm is the PaK 38, and the 88mm is the FlaK 36. Of the 75mm guns, the Mk IV special gun is the L/43, the improved version is the L/48, and the Panther's gun the L/70.

This source says that in Spring 1941, British technical intelligence estimated the penetrative performance of the 88mm as being in the same class as the 50mm, 80–90mm at 400 yards; equally oddly, they overestimated the performance of the "short" 75mm (L/24) as being 100mm at 600 yards.

Moore also states that it was only by early summer 1942 that face-hardened plate was identified on German tanks, and that time "Only the Honey had capped rounds and these were not entirely satisfactory".

## Orgill, 1971

"T-34, Russian Armour", Douglas Orgill, Macdonald, 1971. Armour type unstated, ammunition type unstated, ranges in yards.

| Weapon | 500 | 1000 | "about a mile" |
| --- | --- | --- | --- |
| 76mm L/41 | 69 |  | 54 |
| 100mm |  | 160 |  |
| 88mm L/71 | 182 |  |  |

Comments and corrections

I assume "about a mile" means 1500 yards.

## Perrett, 1990

"Tank Warfare", Bryan Perrett, A&AP, 1990, page 59. Armour type and slope not specified, at 1000 yards.

| Weapon | Penetration |
| --- | --- |
| 122mm L/43 in IS-II | 185 |

Comments and corrections

The author also states that the German 75mm L/43 "could penetrate 89mm armour set back at 30 degrees", but this time without specifying the range.

Elsewhere he makes the rather dubious statement that "The IS-II was regarded as more than a match for the up-gunned Pz Kpfw IV and the Panther, but could be penetrated by the Tiger E at 1,900 yards". Given that he credits the IS-II with 160mm of armour, I assume that Tiger B was intended.

On page 62, Perrett states that the Germans began using APCR in 1941, the Russians in 1942 and the Americans in 1944; that the British introduced APDS in 1944; and that HEAT began reaching the battlefield in 1942.

## Playfair, 1956

"The Germans come to the Help of their Ally (1941)", Volume II of the Official History "The Mediterranean and Middle East, Maj-Gen I. S. O. Playfair, HMSO, 1956. Penetration of homogenous plate at 30º, ranges in yards.

| Weapon | Ammo | 200 | 250 | 400 | 500 | 750 | 1000 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pak 35/36 | PzGr | 42 |  | 38 |  |  |  |  |
|  | PzGr 40 | 61 |  | 49 |  |  |  |  |
| Pak 38 | PzGr |  | 67 |  | 61 | 56 | 50 |  |
|  | PzGr 40 |  | 109 |  | 86 | 69 | 55 |  |
| 50mm L42 | PzGr |  | 54 |  | 49 | 44 | 39 |  |
|  | PzGr 40 |  | 83 |  | 66 | 53 | 42 |  |
| Flak 36 |  |  |  |  | 112 |  | 103 | 83 |
| 2-pdr | AP |  | 58 |  | 52 | 46 | 40 |  |

Comments and corrections

The accompanying text reminds the reader that "in battle the conditions are so variable that, except in extreme cases, only broad forecasts can be made".

The author cites DAK ammunition returns as showing that 13% of ammunition for ATk guns was at first PzGr40. "They asked for a great deal more, but their demands were not met in full".

## Quarrie, 1989

Source: "Encyclopaedia of the German Army in the 20th Century", Bruce Quarrie, Patrick Stephens Ltd, 1989. Armour type unspecified, angle of impact 30º, ranges in metres.

| Weapon | Ammo | 500 | 1000 | 2000 | 3000 |
| --- | --- | --- | --- | --- | --- |
| sPzB41 | APCNR | 50 |  |  |  |
| PaK 35/36 | PzGr 39 | 36 |  |  |  |
|  | PzGr 40 | 55 |  |  |  |
| le PaK 41 | APCNR | 70 | 50 |  |  |
| 4.7 PaK 36(t) |  | 55 |  |  |  |
| PaK 38 | PzGr 39 | 61 | 50 |  |  |
|  | PzGr 40 | 86 | 55 |  |  |
| PaK 40 | PzGr 39 | 104 | 89 |  |  |
|  | PzGr 40 | 115 | 96 |  |  |
| PaK 41 | APCNR | 171 | 145 |  |  |
| PaK 97/38 |  | 100 |  |  |  |
| PaK 36(r) | PzGr 39 | 98 | 88 | 71 |  |
|  | PzGr 40 | 118 | 92 | 55 |  |
| PaK 43 | PzGr 39/43 | 182 | 167 | 139 |  |
|  | PzGr 40/43 | 226 | 192 | 136 |  |
| PaK 44 |  |  | 230 | 202 | 173 |
| 2cm 30 |  | 20 |  |  |  |
| 3.7cm L45 |  | 30 |  |  |  |
| 5cm L42 |  | 56 |  |  |  |
| 5cm L60 |  | 61 |  |  |  |
| 7.5cm L24 |  | 41 |  |  |  |
| 7.5cm L43 |  | 89 |  |  |  |
| 7.5cm L48 |  | 104 |  |  |  |
| 7.5cm L70 |  | 141 |  |  |  |
| 8.8cm L56 |  | 110 |  |  |  |
| 8.8cm L71 |  | 182 |  |  |  |

| Weapon | Range | Angle | Penetration |
| --- | --- | --- | --- |
| Stielgranate 41 | 300 | 30º | 180 |
| PzB 38/39 | 100 | 90º | 30 |
|  | 300 | 90º | 25 |
| RPzB 54 | 120 | 30º | 100 |
| Panzerfaust | 30, 60, 100, 150 | 30º | 140 |
| Haft-Hohlladung | 0 |  | 110 |
| Panzerwurfmine | 18 |  | 64 |
| Gew PzGr | 45 |  | 20 |
| Gross Gew PzGr | 90 |  | 33 |
| Gross PzGr 46 | 200 |  | 90 |
| Gross PzGr 61 | 200 |  | 126 |
| SchussGranate P-40 | 90 |  | 37 |
| Pappmine | 0 |  | 22 |
| Panzer Schnell | 0 |  | 26 |
| Holzmine 42 | 0 |  | 23 |
| Sprengriegel 43 | 0 |  | 35 |
| Tellermine 29–43 | 0 |  | 24 |
| Topfmine | 0 |  | 25 |
| R-mine 43 | 0 |  | 17 |
| LPZ | 0 |  | 10 |
| Eismine 42 | 0 |  | 8 |

Comments and corrections

Mine figures are from US TM-9-1900, German Explosive Ordnance, 1953.

## TM 30-410

"Handbook on USSR Military Forces", HERO, 1978?, pages IX 120 and XI 91–100. Angle unspecified, ranges in metres.

| Weapon | Ammunition | Armour type | Range | Penetration (mm) |
| --- | --- | --- | --- | --- |
| ShKAS 7.62mm | B-32 API | cement | 200 | 7–8 |
|  | BV-46 AP-T |  | 200 | 6 |
| 12.7mm MG | B-22 API | cement | 200 | 20 |
|  | BSF-46 AP-phos | "strong" | 200 | 20 |
|  | BST AP-T |  | 200 | 15 |
| 20mm ShVAK | BS API |  | 1000 | 25 |
| LMG rocket mine | Hollow charge |  | any | 102 |

Comments and corrections

This source is a facsimile edition of a 1945 original, TM 30-430. Probably "cement" means cemented, that is, face-hardened armour.

## TM-E 30-451

"Handbook on German Military Forces", 1945, section VII, pages 22–100. Armour type unspecified except where indicated, ranges in yards.

| Weapon | Ammunition | #### Range | Angle | Penetration |
| --- | --- | --- | --- | --- |
| 7.5cm LG40 | Hollow-charge |  | 30º | 50 |
| 2.8cm sPzB41 | AP | 400 | 30º | 53 |
| 3.7cm Pak | AP40 | 400 | 30º | 49 |
|  | Stick bomb | 150 (mvng target) |  | 152 (approx) |
| 4.2cm le Pak 41 | AP | 700 | 30º | 68 |
| 5cm Pak 38 | AP | 1000 | 30º | 56 |
| 7.5cm Pak 40 | APCBC | 1000 | 30º | 102 |
| 7.5cm Pak 41 | AP | 1000 | 30º | 130 |
| 7.62cm Pak 36(r) | APCBC | 1000 | 30º | 83 |
| 8.8cm Pak 43, 43/41 | APCBC | 500 | 30º | 184 |
|  | APCBC | 1000 | 30º | 169 |
|  | APCBC or AP40 | 1500 | 30º | 130 |
| 12.8cm PaK 44 | APC | 500 | 30º | 172 |
|  | APCBC | 500 | 30º | 212 |
|  | APC | 1000 | 30º | 148 |
|  | APCBC | 1000 | 30º | 200 |
| SS Gew PzGr 46 | Hollow charge | "long" |  | 90 |
|  |  | "short" (18 ft) |  | 70 |
| SS Gew PzGr 61 | Hollow charge | 220 |  | 126 |
|  |  | "short" (18 ft) |  | 100 |

Comments and corrections

This source is a facsimile edition of the original.

Obviously, the AP rounds listed for the sPzB41, le Pak 41 and PaK 41 Gerlich guns are APCNR.

The figures for the Pak 36, PaK 36(r), Pak 41, Pak 43 and Pak 44 specify homogenous armour.

## Unknown

Pages 186-187 of an unknown small-format (8 × 5½ inches) hardback, possibly about anti-tank artillery, as no tank-only guns are listed. Ranges in metres, armour type and slope not specified.

| Weapon | Ammunition | Range | Penetration |
| --- | --- | --- | --- |
| 2 pounder | AP | 1000 | 42 @ 30º |
|  | APCNR | 1000 | 57 |
| 6 pounder | AP | 1000 | 74 |
|  | APCBC | 1000 | 88 |
|  | APDS | 1000 | 136 |
| 17 pounder | AP | 1000 | 109 |
|  | APCBC | 1000 | 119 |
|  | APDS | 1000 | 200 |
| 37mm M3 | APHE | 500 | 30 |
|  | APC | 500 | 50 |
| 57mm M1 | AP | 1000 | 70 |
|  | APCHE | 1000 | 70 |
| 3 inch M5 | APHE | 1000 | 85 |
| 90mm M1 | APHE | 1000 | 110 |
| It 47/32 Model 37 | AP | 1000 | 26 |
| It 90/53 AA gun | AP | 1000 | 101 |
| Jap 37mm Type 97 | APHE | 500 | 35 |
| Jap 47mm Type 1 | AP | 1000 | 40 |
| 45mm M1937 | AP | 1000 | 38 |
| 45mm M1942 | APHE | 500 | 50 |
|  | APCR | 500 | 54 |
| 57mm M1943 | APHE | 500 | 86 |
|  | APCR | 500 | 100 |
| 100mm M1944 | AP | 500 | 186 |
|  | APHE | 500 | 153 |
|  | APCR | 500 | 181 |
| 2.8 cm SPzB41 (Taper) | APCNR | 500 | 52 |
| 3.7 cm PAK 36 | APHE | 500 | 36 |
|  | APCR | 500 | 40 |
| 4.2 cm PJK41 (Taper) | APCNR | 1000 | 53 |
| 5 cm PAK 38 | APHE | 1000 | 50 |
|  | APCR | 1000 | 55 |
| 7.5 cm PAK 40 | APHE | 1000 | 89 |
|  | APCR | 1000 | 96 |
| 7.5 cm PAK 41 (Taper) | APCNR | 1000 | 145 |
| 7.62 cm PAK 36(r) | APHE | 1000 | 88 |
|  | APCR | 1000 | 92 |
| 8.8 cm PAK 43, 43/1 | APHE | 1000 | 167 |
|  | APCR | 1000 | 192 |
| 12.8cm PAK 44 | APHE | 1000 | 230 |

Comments and corrections

Note the significantly inferior penetrative performance of APHE to AP for the US 37mm, and of APHE to AP for the Soviet 100mm. Unfortunately, Hogg's "British & American Artillery of World War 2" considers the M74 AP round inferior to the M51B1 APC (APHE). Failure to specify the ammunition nature being used may be one of the main reasons for disagreement between sources.

## Vanags-Baginskis & Watanabe, 1993

"Aggressors Volume 1: Tank Buster vs. Combat Vehicle", Alex Vanags-Baginskis and Rikyu Watanabe, Airlife, 1993. Armour type not specified, ranges in metres.

| Weapon | Ammo type | #### Range | Angle | Penetration |
| --- | --- | --- | --- | --- |
| 3-inch rocket | solid AP rocket | 460 |  | 100 |
| 45mm M-1932 L/45 |  | 500 | 0º | 50 |
| 57mm L/73 |  | 500 | 0º | 140 |
| BK 3.7 | AP | 100 | 60º | 58 |
|  | APCR | 100 | 60º | 120 |
| 30mm MK 101 | APCR | 100 |  | 103 |
|  |  | 300 |  | 75 |
| BK 7.5 (Pak 40L) |  | 1000 |  | 130 |
| Panzerfaust | Hollow charge |  |  | 200 |
| Panzerschreck II | Hollow charge | 100-120 |  | 120 |
| Panzerblitz II (Pb II) | Hollow charge | 500 |  | 110 |
| PbIII R4/HL | Hollow charge |  | 0º | 160 |
| PTAB 2.5-1.5 | 1.5Kg ATk bomb |  | 0º | 60 |
| Förstersonde | 45mm SG113A |  |  | 48 |

Comments and corrections

It is assumed that the 60º slope specified is in the German style, from the horizontal.

The Panzerblitz II only just made it into service before the end of the war, serving for five or six weeks with I(Pz)/SG9.

The Förstersonde, an unusual weapon fired by an electro-magnetic probe, did not quite make it. The penetration figure for the Förstersonde is based on the observation, on page 40, that trials at Rechlin showed it could just penetrate the 48mm roof armour of the M4A3 Sherman.

The AP warhead of the 3-inch rocket, developed for attacking tanks, was stable under water, and so an excellent anti-submarine weapon; it was however inferior as a tank-killer to the 60lb HE warhead, originally developed for use against submarines.

The German SD4/HL anti-tank bomb is credited with being able to penetrate the roof armour of "any Soviet tank", but no penetration figure is given. There was also an SD9/HL, the designations indicating hollow-charge bombs of 4 and 9Kg respectively.

Two other snippets from the variety of fascinating information to be found in this book: PTAB attacks (a Shturmovik carried 220) against "small groups of tanks" showed a hit probability of 15%; US P-47 pilots found they could destroy German AFV with 0.5" API fire from the rear.

## von Senger und Etterlin, 1971

"German Tanks of World War II Tanks", Dr F. M. von Senger und Etterlin, A&AP, 1971, pp 209–210. Armour type unstated, ranges in metres.

| Weapon | Ammo | Slope | 0 | 457 | 915 | 1372 | 1829 | 2286 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sPzB 41 | AP 41 | 90º |  | 66 |  |  |  |  |
|  |  | 60º |  | 52 |  |  |  |  |
| KwK 35 |  | 90º |  | 48 |  |  |  |  |
| PaK 36 |  | 60º |  | 36 |  |  |  |  |
|  | AP40 | 90º |  | 51 |  |  |  |  |
|  |  | 60º |  | 43 |  |  |  |  |
| PaK 4.7t |  | 60º |  | 55 | 47 |  |  |  |
| KwK 42 |  | 60º |  | 56 |  |  |  |  |
| KwK 39 | AP39 | 90º | 99 | 78 | 61 | 47 |  |  |
| PaK 38 |  | 60º | 73 | 61 | 50 | 40 |  |  |
|  | AP40 | 90º | 165 | 120 | 84 |  |  |  |
|  |  | 60º | 143 | 86 | 55 |  |  |  |
| KwK 37 |  | 60º |  | 41 |  |  |  |  |
| 7.5 L43 |  | 60º |  | 89 |  |  |  |  |
| 7.5 L46 | AP39 | 90º | 149 | 135 | 121 | 109 | 98 |  |
| (PaK 40) |  | 60º | 121 | 106 | 94 | 83 | 73 |  |
|  | AP40 | 90º | 176 | 154 | 133 | 115 | 98 | 83 |
|  |  | 60º | 137 | 115 | 96 | 80 | 66 | 53 |
| 7.5 L48 |  | 60º |  | 90 | 80 |  |  |  |
| 7.5 L70 |  | 60º |  | 141 | 121 |  |  |  |
| PaK 36(r) | AP39 | 90º | 133 | 120 | 108 | 97 | 87 | 78 |
|  |  | 60º | 108 | 98 | 88 | 79 | 71 | 64 |
|  | AP40 | 90º | 190 | 158 | 130 | 106 | 84 | 65 |
|  |  | 60º | 152 | 118 | 92 | 71 | 55 | 43 |
| KwK 36 |  | 60º |  | 110 |  |  |  |  |
| KwK 43 | AP 39/43 | 90º | 225 | 207 | 190 | 174 | 159 | 145 |
|  |  | 60º | 198 | 182 | 167 | 153 | 139 | 127 |
| PaK 43 | AP 40/43 | 90º | 311 | 274 | 241 | 211 | 184 | 159 |
|  |  | 60º | 265 | 226 | 192 | 162 | 136 | 114 |
| PaK 44 | AP 43 |  | 215 (500m) | 202 (1000m) |  |  |  |  |

Comments and corrections

It is unlikely that the 7.5 cm L46 is really superior to the L48 by the amount indicated.

The author indicates that hollow-charge rounds were available for most 7.5 and 8.8cm guns.

## Weeks, 197?

"Men against Tanks, A History of Anti-Tank Warfare" (Book Club edition), John Weeks, Purnell, undated, but after 1973.

Ammunition type and armour type not specified, ranges in units stated.

| Weapon | Range | Slope | Pen (ins) | Pen (mm) |
| --- | --- | --- | --- | --- |
| Various 20mm | 400 yds |  | 0.6–1 | 15–25 |
| Various 37mm | 1000 yds |  | just over 1 | 26 |
| Armstrong 37mm | 300 yds |  | just over 1 | 26 |
| Marosczek ATR | 300 yds |  | ¾ | 19 |
| Boys ATR | 300 yds |  | just over ½ | 14 |
| No. 74 "Sticky bomb" | 0 |  | 1 | 25 |
| PAK 36 stick bomb | 200 yds |  | 5 | 127 |
| 20mm PzB41 | 250 yds |  | 1¼ | 30 |
| PAK 40 | 1000 metres | 30º | 3.7 | 94 |
| Panzerfaust |  |  | 7–8 | 178–203 |

Comments and corrections

Penetration figures in the original are given in inches.

PAK 41 performance quoted as being "about 50 percent better than the PAK 40".

The Japanese 75mm anti-tank gun M90 is quoted as being able to knock out a Sherman at 1000 yards.

## Wiener, 1978

"The Armies of the Warsaw Pact Nations" (second edition), Friedrich Wiener, Carl Uebereuter, 1978, pages 197–199 and 210. Armour at 0º, type unspecified, ranges in metres.

| Weapon | Ammunition | Range | Penetration |
| --- | --- | --- | --- |
| 57mm M-43 ZiS-2 | HVAP | 500 | 140 |
| 85mm M-44 (D-45) | HVAP | 500 | 130 |
| 152mm M-3 (ML-20) | AP | 1000 | 124 |
| 122mm M-31/37 | AP | 1000 | 160 |

Comments and corrections

This book is a translation of the German-language original, "Die Armeen der Warschauer-Pakt-Staaten".

## Williams, 1999

"The Vickers 40mm S gun with Littlejohn Adpator", A G Williams, article published in "The Cartridge Researcher", 1999. Ranges in metres, armour type unspecified, angle of impact 30º.

| Weapon | Ammo | Range | Penetration |
| --- | --- | --- | --- |
| 2-pdr | APCBC | 450 | 53 |
|  | APSV Mk 1 | 450 | 88 |
| Vickers S gun |  | 360 | 47 |

Comments and corrections

Thanks again to Tony Williams for providing this information, which is from his own original research. It appears that in May 1944, an air test was conducted at Boscombe Down using a Hurricane IID mounting S guns fitted with the LittleJohn Adaptor. The equipment was found to be insufficiently reliable.

Tony points out that the Vickers 'S' figures given above do not include any allowance for the velocity of the aircraft, and estimates that at close range the Littlejohn-equipped gun might have achieved a penetration performance of some 80mm. He also mentions that some armoured car crews fired APSV from 2-pounders without the Littlejohn adaptor fitted, so that they could also fire HE rounds.

## Woodman, 1991

"Tank Armament in World War Two", Paul Woodman, in: Airfix Magazine, Vol. 3 No. 5, Alan W. Hall (publications) Ltd, August 1991, page 174.

All penetration figures come from official archive material and are from tests performed against Homogenous type armour plate". Slope is stated as 30 degrees. Ranges are in metres.

| Weapon | Ammo | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- |
| 2 Pounder | AP | 52 | 40 |  |  |
|  | APCBC | 57 | 45 | 40 |  |
|  | APCNR | 88 | 72 |  |  |
| 6 Pounder | APCBC | 81 | 74 | 68 | 63 |
|  | APDS | 131 | 117 | 103 |  |
| 17 Pounder | APCBC | 140 | 130 | 120 | 111 |
|  | APDS | 208 | 192 | 176 | 161 |
| 77mm | APCBC | 120 | 110 | 100 | 90 |
| 3.7cm KwK36 | APC | 29 | 22 | 19 |  |
|  | HVAP | 34 |  |  |  |
| 5cm KwK38 | APCBC | 46 | 36 | 28 |  |
|  | HVAP | 58 |  |  |  |
| 7.5cm KwK40 | APCBC | 96 | 85 | 74 | 64 |
|  | HVAP | 120 | 97 |  |  |
| 7.5cm KwK42 | APCBC | 124 | 111 | 99 | 88 |
|  | HVAP | 174 | 149 | 127 |  |
| 8.8cm KwK36 | APCBC | 110 | 100 | 94 | 83 |
|  | HVAP | 156 | 138 | 123 |  |
| 8.8cm KwK43 | APCBC | 185 | 165 | 148 | 132 |
|  | HVAP | 217 | 193 | 171 |  |
| Sov 45mm | AP | 51 | 36 |  |  |
| Sov 76mm | APC | 60 | 53 | 47 | 41 |
|  | HVAP | 80 | 52 |  |  |
| Sov 85mm | APC | 96 | 88 | 76 | 68 |
|  | HVAP | 121 | 80 |  |  |
| Sov 100mm | APCBC | 170 | 160 | 151 | 139 |
| Sov 122mm | APC | 138 | 132 | 117 | 98 |
| Sov 152mm | APC | 125 | 110 |  |  |
| 37mm M5/6 | APC | 46 | 42 | 37 |  |
| 75mm M2 | AP | 60 | 53 | 46 |  |
|  | APC | 65 | 55 | 49 |  |
| 75mm M2 | AP | 76 | 63 | 51 |  |
|  | APC | 66 | 60 | 55 |  |
| 76mm M1 | APCBC | 94 | 89 | 81 | 76 |
|  | HVAP | 158 | 134 | 116 |  |
| 90mm M3 | APCBC | 126 | 120 | 114 | 105 |
| It 47mm | APC | 55 | 43 |  |  |
| Jap 75mm? | AP | 59 | 51 | 42 |  |

Comments and corrections

In the original table, the British 77mm is mislabeled 76mm, and the APCNR ammunition for the 2 pounder is mislabeled as HVAP. These errors are corrected here.

It is not stated which version of the Soviet 76mm is intended, but the L41 version is assumed. Unless the performance of this gun increases with range, the entries for 500 and 1000 metres have been transposed: The error is corrected here.

The designation of the Japanese weapon is missing in the original. It is noted as being the armament of the Type 97 and Type 1, which is not as helpful as it might be, as there are Type 97 light and medium tanks, and Type 1 medium and gun tanks. It is assumed from the performance figures that the 75mm is intended, as they agree quite closely with those given in the Bovington "Fire and Movement" booklet.

## WO 171/336, 30 Corps "G" War Diary

Quoting 21 A Gp Int Summary No. 131, Penetration of homogenous armour by 2cm KwK 38, ranges in yards.

| Slope | Normal | Normal | 30º | 30º |
| --- | --- | --- | --- | --- |
| Range | 100 | 400 | 100 | 400 |
| AP-T | 48 | 38 | 31 | 25 |
| AP 40 | 57 | 43 | 49 | 37 |

Comments and corrections

No comments.

## WO 185/15, Anti-tank grenades.

Trials comparing the British No. 68 Mk III grenade with the American "trombone" (presumably M9) grenade.

A 1-inch (25mm) deal screen 1 inch in front of 1¼-inch (32mm) FH plate gave immunity to the US grenade at normal impact; the No. 68 grenade penetrated at 20º. US Ordnance stated that the limit of effective regular penetration for the American grenade was 1¼-inch (32mm) FH at angles between normal and 20º. Against this, the US grenade was easier to aim.

Another report says that "the greatest thickness of tank armour that can be perforated by these grenades at 30 degrees to the normal is approximately 30mm. At this limiting thickness, the fragment effect behind the palte is poor."

## WO 185/118, DDG/FV(D) Armour plate experiments.

"...the following figures for penetration are given for attack on MQ plate":

| Range | 3" M62 |  |  | 75mm M61 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Normal | 20º | 30º | Normal | 20º | 30º |
| 500 yards | 128 | 112 | 94 | 95 | 75 | 65 |
| 1500 yards | 106 | 91 | 83 | 76 | 60 | 55 |

Annoyingly, the first digit of the first range bracket has been removed by a hole punched in the paper of the original, but comparing these figures with those given by Hunnicut I take it to be a 5. The figures at normal and 20º are taken from graphs issued by the American Ordnance Office; the figures at 30º are estimates.

The following "armour basis curve" is taken from a US publication, and it is suggested that it be adopted, as it does not differ too greatly from that previously used in Britain:

| Slope (degrees) | Multiplier | Divisor |
| --- | --- | --- |
| 10 | 1.01 | 0.99 |
| 15 | 1.03 | 0.97 |
| 20 | 1.07 | 0.935 |
| 25 | 1.15 | 0.87 |
| 30 | 1.25 | 0.8 |
| 35 | 1.37 | 0.73 |
| 40 | 1.52 | 0.658 |
| 45 | 1.69 | 0.59 |
| 50 | 1.89 | 0.53 |
| 55 | 2.13 | 0.47 |
| 60 | 2.5 | 0.4 |

"It is considered, however, that the facts are too complex to be represented even approximately by any single armour basis curve, and, as illustrated in figures I to V, the armour basis curve varies widely according to the type of projectile and plate attacked."

"...in the case of the 6pdr the armour basis curve is wrong by 7% and in the case of the 2pdr wrong by 28%."

It is noted that the Panther's front plate is 75mm thick (in fact it is 80mm) and sloped at 55º. This is calculated to be equivalent to 155mm of vertical armour (my arithmetic makes it more like 160mm, and 170mm if the correct thickness is used). It is stated that the 17 pdr APC round will perforate 155mm at normal at 650 yards, and APCBC at 1200 yards. "...but recent trials have shown that a 75mm plate at 53º can only be holed at point blank range by 17 pdr APC and that the 17 pdr APCBC will not defeat the plate at any range."

Commenting on the interlocking plates on the Tiger, it is said:

"Attention had previously been drawn (Minute 177) to the poor quality of German welding from the point of view of holding the hull and turret structures together under attack."

On armour hardness:

"A survey of the plates on the Pz.Kw.VI with the portable hardness tester had shown that all the plates were of machineable quality. The hardness of the 26mm plates was in the range 298–343 Brinnel which was harder than British plates of the same thickness namely 262–311 Brinnel. The hardness of the plates in the range 60–102mm was 257–310 Brinnel which was essentially the same as that of British plates of the same thickness."

A test shoot on a captive Tiger in Tunisia gave the following penetration ranges:

| #### Plate | Thickness | Tilt | Angle | Yds | Gun & ammo |
| --- | --- | --- | --- | --- | --- |
| Side superstructure | 82mm | N | N | 1250 | 6pr Mk V+ HV AP |
|  |  |  | N | 900 | 6pr Mk III + LV AP |
|  |  |  | 30º | shatters |  |
|  |  |  | N | 850 | 6pr Mk V + HV APC |
|  |  |  | 30º | shatters |  |
|  |  |  | N | 800 | 75mm M3 + APC M61 |
| Rear plate | 82mm | 10º | 20º | 1000 | 6pr Mk V+ HV AP |
|  |  |  |  | 650 | 6pr Mk III + LV AP |
| Driver's visor plate | 102mm | 10º | 10º | 700 | 6pr Mk V+ HV AP |
|  |  |  |  | 350 | 6pr Mk III + LV AP |
| Front lower plate | 102mm | 25º | 25º | shatters | 6pr AP |
|  |  |  |  | 1600 | 17pr Mk I AP |

"An occasional bad plate may be found in German tanks, such as the port side superstructure. This should not be considered typical nor predictions based on its performance." (The above figures are for the starboard superstructure plate.)

A memo dated 27 Oct 1943 gives the following vulnerability ranges, in yards, for Churchill and "Heavy Churchill", which will have 95mm of side armour, which I take to mean the Churchill Mks VII and VIII:

|  | 88mm Flak 36 |  | 50mm APHE |  |
| --- | --- | --- | --- | --- |
|  | Normal | 30º | Normal | 30º |
| Standard Churchill | over 3000 | over 3000 | 900 | 200 |
| Heavy Churchill | 2250 | 1500 | Immune | Immune |

The following vulnerability table is given for the proposed Valiant tank:

| Plate | 88mm APCBCHE |  | 75mm APCBCHE |  |
| --- | --- | --- | --- | --- |
|  | Normal | 30º | Normal | 30º |
| 4" rear, vertical | 2020 | 860 | 950 | Immune |
| 4½" side of hull, vertical | 1650 | 350 | 520 | Immune |
| 5" side of turret, vertical | 1160 | Immune | 120 | Immune |
| 6" hull front 20º | Immune | Immune | Immune | Immune |
| Turret front vertical | 270 | Immune | Immune | Immune |

## WO 185/122, Churchill tank with 17 pdr gun.

Commenting on the advantage of sloped armour, a memo in this file says "Two examples indicate the advantage of this based on results of firing trials". The first example is for the assumed thickness and slope of the Panther lower nose plate and the equivalent weight of vertical armour; the second is for the assumed thickness and slope of the Panther upper nose plate and equivalent weight of vertival armour.

| Thickness (mm) | Slope | Range immune to 17 pdr AP |
| --- | --- | --- |
| 75 | 53º | 800 yds |
| 125 | 0º | 1600 yds |
| 85 | 57º | 0 yds |
| 156 | 0º | 900 yds |

Mention is made of the possibility of mounting the 3.7" Mk VI as a turret gun. "The penetration performance of this weapons is likely to be considerably in excess of the 17 pdr." A pencilled note in the margin says "(+25%)".

## WO 185/133, Tungsten Carbide Production.

| Weapon | #### Ammo | Range | Angle | Penetration (mm) |
| --- | --- | --- | --- | --- |
| 2 pdr | AP | "limit of perforation" | 30º | 55mm |
| 2 pdr Littlejohn | APCNR | "limit of perforation" | 30º | 80-90mm |
|  |  | at least 1,000 yds | 30º | 55mm |
| 6 pdr | APCBC | 1,500 yds | 30º | 60mm |

A letter in this file dated 14th Nov 1942 contains an interesting postscript: "Since this subject was raised we have struck a snag in the attack of double skin frontal armour as used in the latest German Tanks." A 20mm thick front plate breaks up or distorts the core of 2 pdr Littlejohn shot sufficiently to prevent penetration of the 50mm plate behind. Mention is made of a "rumoured new tank" with 100mm front and 60mm side armour, a fairly good approximation to the Tiger, but erroneously believed to have double skin armour.

It is stated that AP steels "properly treated" can defeat plates 25% greater in thickness than the projectile diameter. It also says that 2 pdr service shot had to date defeated the side and rear armour of German AFV up to 1,000 yards at 30º.

Diameters of tungsten carbine penetrating cores for various weapons are given as (inches in original):

| Weapon | 37mm Littlejohn | 2 pr Littlejohn | 6 pr APCR or DS | 17 pr APDS |
| --- | --- | --- | --- | --- |
| Diameter (ins) | 0.66 | 0.79 | 1.1 | 1.5 |
| Diameter (mm) | 17 | 20 | 28 | 38 |

In each case, this is half the projectile calibre.

## WO 185/145, New infantry anti-tank weapon.

This is a post-war file, dealing only with requirements, but is of interest because it gives a clue as to the expected effectiveness of spaced armour against HEAT rounds.

The final requirement is stated as being penetration of 120mm of armour protected by a 10mm skirting plate positioned at 3 feet from the main armour (earlier statements mention figures of 100 and 160mm), or 200mm of armour unprotected by skirting plates.

It is stated that trials of the 3.45" RCL showed a blind rate of 6%.

Mention is made of an abandoned "improved PIAT" bomb, capable of penetrating 145mm of armour at normal or 125mm at 30º. The fighting range was to be 125 yards from the existing projector. Penetration of the 3.5" bazooka is stated, from American sources, to be 11", or 279mm.

Mention is also made of the low lethality of HEAT rounds, "follow-though" rounds, and the Misznay-Schardin effect, which seems to be the basis of an early form of self-forging fragment warhead.

## WO 185/171, Armour plate experiments.

This file states that 50 Brinnel points corresponds to 10 tons per square inch. Referring to armour on German light AFVs, it says "The hardness of the homo-hard plates was reduced in 1938 from a minimum of 495 Brinnel to the range of 415–461 Brinnel" in order to reduce cracking.

Information collected up to June 1944 shows the Brinnel hardness ranges of British and German plates of different thicknesses in mm:

|  | Thickness: | 40 | 60 | 80 | 100 | over 100 |
| --- | --- | --- | --- | --- | --- | --- |
| German | Upper | 280 | 270 | 250 | 220 | 215 |
|  | Lower | 335 | 320 | 300 | 290 | 250 |
| British | Upper | 262 | 255 | 248 | 240 | 240 |
|  | Lower | 302 | 293 | 277 | 269 | 269 |

## WO 185/178, Tank armament versus armour.

This file is dated 1943.

"Perforation of armour in millimetres":

| Gun | Ammo | #### Type | Range (yards) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #### At normal |  |  | At 30º |  |  |
|  |  |  | 500 | 1000 | 1500 | 500 | 1000 | 1500 |
| 6 pdr Mk II & III | AP | MQ | 100 | 87 | 62 | 82 | 68 | 46 |
|  | APC | MQ | 96 | 83 | 58 | 72 | 63 | 46 |
|  |  | FH |  |  |  | 90 | 74 | 47 |
|  | APCBC | MQ | 93 | 84 | 64 | 68 | 60 | 44 |
|  |  | FH | 104 | 95 | 76 | 80 | 75 | 57 |
| 6 pdr Mk IV & V | AP | MQ | 106 | 92 | 66 | 82 | 71 | 48 |
|  | APC | MQ | 102 | 88 | 62 | 78 | 67 | 44 |
|  |  | FH | 110 | 96 | 70 | 95 | 78 | 50 |
|  | APCBC | MQ | 97 | 88 | 70 | 72 | 64 | 49 |
|  |  | FH | 108 | 98 | 81 | 97 | 82 | 64 |
| 2 pdr Mk IX & X | AP | MQ | 67 | 48 | 26 | 51 | 41 | 24 |
|  | APCBC | MQ | 63 | 56 | 39 | 54 | 47 | 35 |
|  |  | FH | 67 | 59 | 42 | 58 | 51 | 38 |
|  | L/john | MQ | 111 | 87 |  | 90 | 71 |  |
|  |  | FH | 111 | 87 |  | 90 | 71 |  |
| 17 pdr | AP | MQ | 138 | 126 | 102 | 118 | 110 | 89 |
|  | APC | MQ | 138 | 126 | 102 | 118 | 110 | 89 |
|  |  | FH | 155 | 135 | 110 | 135 | 118 | 91 |
| 75mm M2 | AP | MQ | 79 | 66 | 44 | 62 | 54 | 39 |
|  | APCBC | MQ | 82 | 72 | 56 | 63 | 58 | 44 |
|  |  | FH | 80 | 70 | 54 | 67 | 61 | 47 |
| 75mm M3 | AP | MQ | 88 | 75 | 51 | 73 | 64 | 55 |
|  |  | FH | 80 | 65 | 45 |  |  |  |
|  | APCBC | MQ | 94 | 85 | 68 | 76 | 69 | 58 |
|  |  | FH | 92 | 83 | 66 | 75 | 67 | 52 |
| 8 pdr (short) | APCBC | MQ | 107 | 101 | 84 | 79 | 73 | 66 |
|  |  | FH | 112 | 103 | 87 | 97 | 87 | 69 |
| 8 pdr (short) | APCBC | MQ | 114 | 105 | 89 | 86 | 78 | 64 |
|  |  | FH | 118 | 110 | 92 | 104 | 94 | 75 |

The 75mm AP round is the M72, the APCBC is M61. The figure for 75mm M3 APCBC on homogenous armour at 500 yards and 30º is given as 67 in the original, but ringed; I assume this indicates a transposition error, and have corrected it to 76. In the original, the figures for the 8 pdr on face-hardened armour at normal were originally 7 to 10mm greater than shown here, but have been corrected by hand.

"Performance against Homogenous Plate at 30 degrees Angle of Attack":

|  | m.v. (fps) | weight | 500 yds | 1000 yds | 1500 yds |
| --- | --- | --- | --- | --- | --- |
| 88mm APCBCHE | 2600 | 21lb | 114 |  |  |
| 17 pdr APC | 2900 | 17lb | 135 121 | 118 109 | 103 98 |
| 6 pdr APCBC | 2725 | 7lb 3 oz | 86.5 | 79.8 | 73.5 |

It is stated that the 3.7" howitzer and 95mm tank howitzer penetrate a minimum of 90mm at all ranges.

The reason for the 17 pdr figures being crossed out and replaced is not clear, but the uncorrected figures are preserved here.

"Thickness of armour penetrated by 80% of projectiles striking the plate at an angle of 30º to the normal":

| Gun | Ammo | m.v. (fps) | Armour | 500 yds | 1000 yds | 1500 yds |
| --- | --- | --- | --- | --- | --- | --- |
| 6 pr Mk III | AP | 2650 | MQ | 73 | 62 | 41 |
|  |  |  | FH | 64 | 51 | 31 |
|  | AP | 2800 | MQ | 79 | 66 | 44 |
|  |  |  | FH | 69 | 55 | 34 |
|  | APC | 2800 | MQ | 72 | 61 | 40 |
|  |  |  | FH | 88 | 72 | 45 |
| 6 pr Mk V | APC | 2800 | MQ | 78 | 67 | 44 |
|  |  |  | FH | 93 | 76 | 48 |
| 17 pr | APC | 2900 | MQ | 118 | 110 | 84 |
|  |  |  | FH | 133 | 116 | 89 |
| 75mm M2 | APCBC | 1850 | MQ | 63 | 55 | 42 |
|  |  |  | FH | 65 | 56 | 44 |
| 75mm M3 | APCBC | 2050 | MQ | 69 | 62 | 47 |
|  |  |  | FH | 74 | 65 | 50 |
| 5cm KWK 39 | AP | 2700 | MQ | 65 | 56 | 38 |
| 7.5 cm KWK 40 | APCBC | 2400 | MQ | 89 | 79 | 62 |

This table contains numerous corrections by hand; the final figures are given here.

A memo from the Ministry of Supply dated 1st April 1943 gives the following figures for "Single homo plate penetration at 30º in mm.":

|  | 500 yds | 1000 yds | 1500 yds |
| --- | --- | --- | --- |
| 6-pdr APCBC | 86.5 | 79.8 | 73.5 |
| 6-pdr Littlejohn | 123 | 111 | 101 |
| 6-pdr composite rigid | 109 | 90 | 75 |
| 17-pdr APC | 121 | 109 | 98 |
| 3" (17-pdr) composite rigid | 155 | 138 | 122 |
| 75mm APCBC (MV 2030 fps) | 69.5 | 63 | 57 |

"Penetration in mm against homogenous plate":

| Range (yds) | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- |
| 7.5cm KwK40 APCBC | 89 | 79 | 70 | 62 |
| 7.62/36(r) APCBC | 79 | 70 | 61 | 53 |
| 6 pdr AP | 75.9 | 67.1 | 55.2 | 44.7 |
| 17 pdr APC | 112.7 | 111.2 | 99.8 | 88.7 |
| 3" 20 cwt AP | 88.5 | 76.5 | 65.5 | 55 |

The usual cautions about the spurious precision of fractional millimetres apply.

On 95mm shaped charge ammunition:

"For the reasons given in paragraph 4 above, the characteristics of the weapon greatly reduce the chances of direct hits with all types of projectile – at a range of 1,000 yards, the chances are about one-third those of the 6 pdr and at longer ranges the falling-off increases rapidly. For effective anti-tank engagement the maximum opening range of the 95mm is say 600/700 yards, preferably less."

"The German 28/20 Gerlich projectile penetrated a 76mm MQ plate at 100 yards and nearly succeeded against 87mm F.H. plate at the same range."

The following table shows "Performance against Homogenous Plate at 30 degrees Angle of Attack", and includes estimates of the "Minimum weight of metal shot into tank by perforation of 75mm plate".

| Gun | 500 yds | 1000 yds | 1500 yds | pen dia | Wt of metal (lbs) |
| --- | --- | --- | --- | --- | --- |
| 75mm KWK 40 | 90 | 80 | 71 | 75 | 6 + 12 = 18 |
| 75mm long Pak | 114 | 105 | 94 | 75 | 6 + 12 = 18 |
| 75mm Pak 41 | 142 | 121 | 101 | 30 | ¾ + 2 = 2¾ |
| 88mm | 114 | 105.5 | 96.5 | 88 | 8¼ + 18 = 26¼ |
| 88mm Flak 41 | 159 | 149 | 139 | 88 | 8¼ + 18 = 26¼ |
| 75mm M3 | 70 | 63 | 57 | 75 | 6 + 12 = 18 |
| 6-pdr | 86.5 | 79.8 | 73.5 | 57 | 3¼ + 6¼ = 9½ |
| 17-pdr | 121 | 109 | 98 | 76 | 6 + 15 = 21 |
| 3.7" or 95mm how | Minimum of 90 at all ranges |  |  | 25 | 2/3 + 1/3 = 1 |

American and German guns fire APCBC/HE, except the Pak 41 (the paper also mentions a KWK 41), the 6-pdr APCBC, the 17-pdr APC and the howitzers hollow charge.

The original penetration diameters were given in inches, but have been converted to millimetres here.

The figures in the last column show the expected weight of armour plus the weight of projectile entering the tank, added to give a total weight.

A note on the howitzer entry says "Perforation of Hollow Charge projectiles is not controlled by striking velocity but by chances of hitting. The maximum range at which a sufficiency of hits is likely is probably 700 yds. owing to low velocity. The nature of the damage is small, usually about a 1" hole.

"The 6 pdr hitherto in use perforated with AP ammunition, the 80mm homogenous side armour and turret of the Tiger tank in Tunisia at ranges between 600 and 700 yards at various angles of strike estimated at between 15 and 35º. The higher velocity long barrelled 6 pdr gun is now being issued. APCBC ammunition, which will be used by both types of gun and should begin to reach UK depots about May, should improve on the performance of the present AP shot by being effective at longer ranges."

"6 pdr AP shot successfully engaged the Pz.Kw. VI tank in Tunisia, perforating 84mm plate at 550 yards at 20º attack."

"17 pdr APC ammunition will be produced this year which will perforate 114mm homogenous armour at 750 yards 30º attack."

The following table of fighting ranges is based on hits on the heaviest front armour at 30º. Armour thickness of the Pz IV is taken as 50mm, of the Pz. III as 50+20mm.

|  |  | Pz III | Pz IV |
| --- | --- | --- | --- |
| Crusader 50mm | Penetrates | 300 | 1000 |
|  | Is penetrated | 1300 | 2000 |
| Grant | Penetrates | 150 | 1500 |
|  | Is penetrated | Immune | 1200 |
| Sherman | Penetrates | 700 | 2000 |
|  | Is penetrated | Immune | 1200 |

It is unfortunate that the armament of the enemy tanks was not specified, but we might guess at the 50/42 for the Pz III and the 75/43 for the Pz IV.

## WO 185/182, Recoilless weapons.

This says that the 4.7" Burney "Wallbuster" shell knocked 20–30lbs of scabs off a 6" plate at 30º.

## WO 185/194, Tank and anti-tank armament.

A table of "important German SP equipments" gives the following figures for penetration of homogenous armour at 30º with APCBC ammunition:

|  | 500 yds | 1000 yds |
| --- | --- | --- |
| 7.5 cm PaK 39 (L/48) | 84 | 72 |
| 7.5 cm StuK42 (L/70) | 141 | 121 |
| 8.8 cm Pak 43/1 (L/71) | 184 | 169 |
| 12.8 cm Pak 44 (L/55) | 172 (APC) | 148 (APC) |
|  | 212 (APCBC) | 200 (APCBC) |

Comments and corrections

No comments.

## WO 190/706, German Army infantry weapons.

This document is dated 1938. Range in units stated, penetration in mm.

| Weapon | Angle | Range | Penetration |
| --- | --- | --- | --- |
| 7.92mm s.m.k bullet | 90º | 400m | 8.5 |
|  | 90º | 100m | 10 |
| 20mm "Tankbusche" | 90º | 100m | 35 |
|  | 90º | 300m | 30 |
|  | 90º | 600m | 22 |
|  | 60º | 100m | 23 |
|  | 60º | 300m | 18 |
|  | 60º | 600m | 13 |
| MG30 2cm Mk 65/80 | 90º | 1000m | 25 |
|  | 60º | 500m | 15 |
| A.T. gun 3.7 | 90º | 650 yds | 33 |
|  | 60º | 650 yds | 26 |
|  | 60º | 1090 yds | 22 |
|  | 45º | 765 yds | 14 |

Comments and corrections

The performance of the "Tankbusche" is quoted against armour of hardness 170 Kg/mm2.

## WO 194/598, Interim damage assessment report on 20mm, 25mm and 30mm calibre cannon attacks against APC targets.

This file is dated 1967, and details trials of different cannon against a box target made armour plates to represent a Soviet APC and against an FV432. Against the box target:

| Ammo | Plate | Angle | Range (m) | Results |
| --- | --- | --- | --- | --- |
| 30mm Hispano AP (RINT) | 25mm | 60º | 500 | 2 attacks, 2 failures |
|  | 40mm | 45º | 500 | 5 attacks 4 failures |
| 30mm Hispano APHE (RIA) | 25mm | 45º | 500 | 4 attacks 2 failures |
|  | 15mm | 60º | 1000 | 2 attacks 1 failure |
| 25mm Oerlikon APHE | 25mm | 45º | 500 | 2 attacks 2 failures |
|  | 15mm | 60º | 500 | assumed to fail |
|  | 15mm | 45º | 1000 | successful |
| 25mm Oerlikon APDS | 25mm | 60º | 1000 | 2 attacks 2 failures |
|  | 25mm | 60º | 500 | 2 attacks 0 failures |
|  | 40mm | 40º | 500 | 5 attacks 5 failures |
| 20mm Hispano APHE (RIA) | 15mm | 45º | 500 | 2 attacks 0 failures |
|  | 15mm | 45º | 1000 | assumed to fail |
| 20mm Hispano AP (RINT) | 15mm | 45º | 500 or 1000 | successful |

Against the FV432 the 30mm RINT and RIA rounds could penetrate the frontal armour (12.5mm at 41º and 16mm at 39º) and cause serious internal damage at 1000 metres. The 25mm was also successful, bit caused less internal damage.

## WO 219/2785, SHAEF equipment and supply returns.

Against armour sloped at 20º, at any range, the 57mm recoilless rifle penetrates about 51mm of armour and the 75mm recoilless rifle about 76mm .

Comments and corrections

The penetrations are given in the original as 2 and 3 inches. The specific memo containing these figures is from the New Developments Division to the Chief of Staff, dated 13th April 1945. It states that 20 57mm and 2 75mm recoilless rifles were dropped during the airborne operation around Wesel. One 57mm knocked out three tanks, and one or two 57mms knocked out two tanks, but the type of tank is not identified.

## WO 219/2801, SHAEF documents on tanks.

Appendix A to 21 AGp/1420/12/G(SD), dated 11 Mar 1945, on the vulnerability of the King Tiger, states that the King Tiger's glacis is invulnerable to 17-pdr DS. It is stated that the nose can be penetrated up to 1,200 yards, and the turret front at 1,100 yards.

For the performance of the 77mm gun, subtract 1,100 yards from the APCBC performance and 650 yards from the DS performance of the 17-pdr.

To penetrate a King Tiger, 75mm M61 must strike the lower hull side at 750 yards or the turret side at 400 yards.

A number called the "critical azimuth at 1,000x" is mentioned, but not explained.

## WO 219/2806, Appendix G to SHAEF/16652/GCT/Arty

Dated 11 July 1944. "Perforation of homo at 30º Strike", ranges in yards.

| Weapon | Ammo | 600 | 1000 | 1600 | 2000 |
| --- | --- | --- | --- | --- | --- |
| US 57mm | AP M70 | 78 | 64 | 55(a) | 50(a) |
| Br 6-Pr | APCBC | 81 | 74 | 64 | 58 |
|  | SABOT(b) | 130 | 118 | 101 | 90 |
| US & Br 75mm | APCBC M61 | 100 | 93 | 82 | 77 |
| US 76mm | APC M62A | 97 | 90 | 80 | 74 |
| Br 77mm(c) | APCBC | 110 | 100 | 91 | 85 |
|  | SABOT | 178 | 149 | 131 | 120 |
| Ger 75mm PAK 40 | APCBC | 110 | 102 | 92 | 86 |
| (d) PAK 41 | AP | 137 | 100 | ? | ? |
| (e) KWK 42 | APCBC | 119 | 100 | 95(a) | 89 |
| US & Br 3" M-10 | APC M62 | 97 | 90 | 80 | 74 |
| US 90mm | APC M77 | 111 | 103 | 93 | 86 |
| Br 17-Pr | APCBC | 127 | 120 | 112 | 107 |
|  | SABOT | 183 | 172 | 155 | 145 |
| German 88mm KWK 36 (f) | APCBC | 108 | 102 | 94(a) | 87 |
| KWK 43 (f) | APCBC | 178 | 140 | 131(a) | 121 |
| Br 32-Pr (g) | APCBC | 138 | 131 | 121 | 115 |
|  | SABOT | 215 | 205 | 189 | 180 |

Notes:	(a) Approximate figure			(d) Tapered bore		(g) A 1945 project

(b) Fits US 57mm			(e) PANTHER gun

(c) In production but NOT in service	(f) TIGER gun

Comments and corrections

The original table shows note (b) where note (f) is plainly intended. This has been corrected here.

The high figures for the 75mm gun match those given for APCBC in the Bovington "Fire and Movement" booklet, but are much more generous than other sources. A memo in this document says there seems "little to choose" between the 75mm and 76mm.

The US 90mm round given here seems to be a very weak load — a memo in this document says that 90mm ammunition is being loaded for a higher velocity.

In the memo this table accompanies, it is claimed that "the U.S. 76mm is ineffective against TIGER and only effective against PANTHER flanks", but, by an ordnance officer in the USA that "75mm gun will penetrate Panther tank turret, sides and rear and lower hull to ranges of 2,000 yards. 76mm gun will penetrate Panther tank at all points except chassis front plate to ranges of 5,000 yards and turret front at 2,500 yards, hull front nose plate 1,600 yards".

A very sensible comment by a Colonel Burlton, originator of the memo to which this table forms an appendix, says "Probably, there is a conflict between theory and practice".

## WO 279/200, The Development of Artillery Tactics and Equipment

Homogenous armour, ranges in yards.

| Weapon | Ammo | #### Angle | 500 | 1000 | 2000 |
| --- | --- | --- | --- | --- | --- |
| 5 cm |  | 30º | 65 | 53 |  |
| 8.8 cm |  | 30º | 112 | 103 | 86 |
| 6 pdr |  | 30º | 75 | 63 |  |
|  | APDS | 0º |  | 146 |  |
| 17 pdr |  | 30º | 123 | 113 | 93 |
|  | APDS | 0º |  | 231 |  |
| 25 pdr |  | 30º | 62 | 54 |  |

Comments and corrections

Thanks to Ernest Ashworth for providing these figures; only later did I find the book in the PRO. It was published by the War Office in 1951, compiled by Brigadier A L Pemberton MC.

This source says that APDS ammunition was introduced in Summer 1944. It also states that "...where APCBC could penetrate, it did greater damage inside the tank and was less likely to pass straight through a lightly armoured target. In consequence both kinds of shot had to be retained in the service."

Notice that these figures seem to have been copied, with some inaccuracy, in Bidwell's "Artillery Tactics" and "Artillery at War".

## WO 291/385, "The Performance of the Infantry Sledge under Fire."

This paper mentions trials of a low-profile armoured sledge for the transport of infantry, towed by tanks. The armour protection od 7mm thick IT100 plate. This is credited with giving immunity from AP small arms fire at about 700 yards at normal impact, and at all ranges at 35º.

## WO 291/741, "Comparison of the performance of 75mm and 76mm tank gun ammunition."

"Thickness of homogenous armour plate penetrated at 30angle of attack by APCBC/HE shell." Ranges in yards.

| #### Range | 75mm | 76mm |
| --- | --- | --- |
| Point blank | 79.5 | 108.2 |
| 200 | 75.3 | 104.2 |
| 400 | 72 | 100.2 |
| 600 | 68.5 | 96.7 |
| 800 | 65.5 | 93 |
| 1000 | 63 | 89.7 |
| 1200 | 60.3 | 86.3 |
| 1400 | 57.8 | 83.1 |
| 1600 | 55 | 80 |
| 1800 | 52.6 | 77 |
| 2000 | 50 | 74 |

Comments and corrections

Precision in fractional millimetres seems excessive, and suggests results by interpolation rather than measurement. This document also compares the HE performance of the two guns, concluding that in this respect the 75mm is noticeably superior. It also points out that inferior HE performance can be compensated for by using more of the less effective shell, whereas it is impossible to remedy a deficiency in penetrative performance.

## WO 291/1143, "Effectiveness of British and Russian tanks."

Penetration in millimetres, ranges in yards:

| Weapon | #### Range | 0º | 30º | 60º |
| --- | --- | --- | --- | --- |
| 77mm APCBC | 600 | 145 | 115 | 52 |
|  | 1000 | 136 | 108 | 48 |
|  | 1500 | 124 | 100 | 45 |
| 17 pdr APCBC | 600 | 172 | 136 | 62 |
|  | 1000 | 162 | 128 | 58 |
|  | 1500 | 150 | 119 | 55 |
| 17 pdr APDS | 600 | 248 | 200 | 70 |
|  | 1000 | 232 | 188 | 65 |
|  | 1500 | 213 | 172 | 60 |
| 20 pdr APDS Mk 1 | 600 | 300 | 240 | 80 |
|  | 1000 | 280 | 225 | 75 |
|  | 1500 | 260 | 210 | 67 |
| 85mm APCBC | 600 | 137 | 110 | 49 |
|  | 1000 | 128 | 103 | 46 |
|  | 1500 | 118 | 95 | 43 |
| {from Russian | 600 | 110 | 88 | 39 |
| range tables) | 1500 | 96 | 76 | 35 |
| 122mm APCBC | 600 | 203 | 162 | 74 |
|  | 1000 | 193 | 153 | 70 |
|  | 1500 | 182 | 145 | 66 |
| (from Russian | 600 | 149 | 118 | 53 |
| range tables) | 1500 | 130 | 104 | 47 |

Comments and corrections

This document is dated August 1951. The figures for Russian guns are derived from 17-pdr APCBC performance; the figures quoted from Russian range tables are much lower. The report concludes that the Centurion 3 is slightly superior to the JS-3, and the Comet roughly equivalent to the T-34/85.

## Zaloga, 1979

"Modern Soviet Armour", Steven J. Zaloga, A&AP, 1979, page 57. Penetration at 1000 metres and 0º, armour type not stated.

| Weapon | with APHE | with APCR |
| --- | --- | --- |
| 57mm Ch-51M | 85 | 100 |
| 85mm D-5S85 | 102 | 130 |

Comments and corrections

Both weapons are developments of WW2 weapons, so performance is assumed to be similar. Still, it seems strange that the figure for the 57mm should be so far inferior to the 140mm normally quoted.

## Zaloga & Grandsen, 1984

"Soviet Tanks and Combat Vehicles of World War Two", Steven J. Zaloga and James Grandsen, A&AP 1984, page 225. Ranges in metres, "against vertical steel armour plate".

| Weapon | #### Ammunition | 500 | 1000 |
| --- | --- | --- | --- |
| 20mm TNSh L/107 | APHE | 22 |  |
| 45mm mod 32, 34, 38 L/45 | BR-240 APHE | 42 | 38 |
|  | BR-240P APCR | 80 | 50 |
| 76.2mm mod 27/32 L/16.5 | BR-350 APHE | 35 | 30 |
| 76.2mm L-11 L/30.5 | BR-350 APHE | 62 | 56 |
| 76.2mm F-34 L/42.5 | BR-350A APHE | 69 | 61 |
|  | BR-350P APCR | 92 | 60 |
| 85mm ZiS-S-53, D-5 L/54.6 | BR-365 APHE | 111 | 102 |
|  | BR-365P APCR | 138 | 100 |
| 57mm ZiS-2 | BP-571P APCR | 140 |  |
| 100mm D-10 L/56 | BR-412 APHE | 195 | 185 |
| 122mm A-19, D-25 L/46.5 | BR-471B APHE | 145 | 145 |
| 152mm ML-20 L/28.8 | BR-540 APHE | 124 | 124 |

Hollow charge rounds, same at all ranges:

| Weapon | Ammunition | Penetration |
| --- | --- | --- |
| 76.2mm F-34 L/42.5 | BR-353A HEAT | 75 |
| 122mm M-30 L/22.7, and A-19 or D-25 L/46.5 | BR-460A HEAT | 200 |

Comments and corrections

In the original, APCR rounds are noted as APDS. This has been corrected here.

It seems extremely odd that the 122mm BR-471B and 152mm BR-540 APHE should both have the same penetration at 500 as at 1000 metres; this is probably a typographical error.

The author states that HEAT for the short 122mm in the original SU-122 "never lived up to expectations".

This book seems to be the source for the Soviet figures in the "World War II Databook", although Ellis has arbitrarily changed ranges into yards from the metres specified here.

Comments from Valeriy Potapov:

1. Neither BR-353A nor BR-460A are HEAT rounds. Designations for both rounds are refer to common armour-piercing projectile. Correct designations for HEAT round are BP-540 (for 152mm) and BP-460A (for 122mm M-30 and SU-122), while there were no HEAT ammunition for 122mm A-19/D-25.

2. BR-353 refers to short-barelled 76-mm Regimental Gun M1927 and was prohibited to use for 40-calibre artillery (F-34, ZiS-3, etc)

3. BP-271P is a wrong designation. If should be either BR-271 (APHE) or BR-271SP (AP) or BR-271P (APCR)

## Zaloga & Ness, 1998

Source: "Red Army Handbook 1939–45", Steve Zaloga and Leland Ness, Sutton Publishing, 1998, page 203. Ranges in metres, armour type unstated.

| #### Weapon | Ammunition | Slope | 500 | 1000 |
| --- | --- | --- | --- | --- |
| 45mm Model 37 | AP | 90º | 43 | 35 |
|  |  | 60º | 35 | 28 |
| 45mm Model 42 | AP | 90º | 61 | 51 |
|  |  | 60º | 50 | 41 |
|  | HVAP | 90º | 81 |  |
|  |  | 60º | 52 |  |
| 57mm ZiS-2 | AP | 90º |  |  |
|  |  | 60º | 84 |  |
|  | HVAP | 90º | 145 |  |
|  |  | 60º | 120 |  |
| 100mm BS-3 | AP | 90º | 200 | 185 |
|  |  | 60º | 150 | 140 |

Comments and corrections

The text of the source also attributes to the PTRS and PTRD ATRs a penetration performance of 35mm at 100 metres, which seems remarkably generous, and repeats the unbelievable assertion that seems to be gaining currency that German Schürtzen were intended to counter these.

This source also states that the HVAP round for the 45mm Model 37 became available in April 1942.

## Zaloga & Sarson, 1993

"Sherman Medium Tank 1942-1945", Steve Zaloga and Peter Sarson, Osprey 1993, pages 10 and 14. Armour type and slope unspecified, ranges in yards.

| Weapon | Ammunition | 500 | 1000 |
| --- | --- | --- | --- |
| 75mm M3 | M61 APC | 68 | 60 |
| 76mm M1A1C | M-62 APC-T | 98 | 90 |
|  | T-4 HVAP-T | 150 (approx) | 132 |

Comments and corrections

The authors say that, before 1945, seldom were more than 2 or 3 HVAP rounds available per tank, and maybe 5 thereafter, as preference was given to tank destroyer units.

# Hit probabilities

## Featherstone, 1973

"Tank Battles in Miniature: A wargamer's guide to the Western Desert Campaign 1940–1942", Donald Featherstone, Patrick Stevens Ltd, 1973, page 105. "Hit probabilities for Second World War tank guns", ranges in yards.

|  | 500 | 1000 | 1500 | 2000 | 2500 |
| --- | --- | --- | --- | --- | --- |
| 9-ft high target | 78% | 37% | 10% | 2% | 1% |
| 3-ft high target | 45% | 21% | 6% | 1% |  |
| 2nd shot on target | 95% | 90% | 80% | 20% | 15% |

Comments and corrections

No indication is given as to the kind of gun, ammunition or fire control system used. I assume the targets are static, though this is not stated.

## Isby, 1979

"Panzer Battles: The Evolution of Mechanized Warfare, 1939–1979", David C. Isby, in: Strategy & Tactics No. 73, Simulations Publications, Inc., March/April 1979, page 10. "Probability of a first round hit (not kill) of a tank gun firing at a tank-sized, standing target. Ammunition is assumed to be APDS — any other type would have somewhat less accuracy due to its lower velocity." Ranges in metres.

|  | 500 | 1000 | 1500 | 2000 | 2500 | 3000 |
| --- | --- | --- | --- | --- | --- | --- |
| 76mm | 87% | 21% | 5% | 3% | 2% | 2% |
| 90mm | 90% | 65% | 31% | 14% | 6% | 5% |

Comments and corrections

The original shows the 76mm gun as representative of the WW2 era, the 90mm as representative of Korea. It is claimed that "These figures show not only the calibre increase in the tank guns, but the increased effectiveness of their range-finders and fire-control systems as well". Tanks carrying the 90mm gun in Korea were the M-26 Pershing and M-46 Patton, neither of which has an advanced range-finder, so presumably the difference is to be attributed to improved quality of optics and linkages; nonetheless, a three-fold improvement at 1000 yards seems distinctly excessive.

Why APDS is assumed is unclear, as no such round existed for the 76mm, nor, I believe, for the 90mm in Korea.

## WO 291/171, "Effectiveness of British anti-tank guns"

Dated 1943. Ranges in yards, armour as detailed in notes.

| Exposure | Success % | 0º | 45º | 90º | 135º | 180º |
| --- | --- | --- | --- | --- | --- | --- |
| 6 pdr AP on Tiger I |  |  |  |  |  |  |
| Full view | 90 % | - | - | - | - | - |
|  | 50 % | - | - | 1200 | - | 1000 |
|  | 30 % | 650 | 650 | 1350 | 500 | (missing) |
| Hull down | 90 % | - | - | - | - | - |
|  | 50 % | 300 | - | 500 | - | - |
|  | 30 % | 650 | 200 | 850 | 550 | 650 |
| 6 pdr APCBC on Tiger I |  |  |  |  |  |  |
| Exposure | Success % | 0º | 45º | 90º | 135º | 180º |
| Full view | 90 % | - | - | 700 | - | - |
|  | 50 % | 500 | - | 1650 | 500 | 1650 |
|  | 30 % | 750 | 650 | 2100 | 350 | 1350 |
| Hull down | 90 % | - | - | - | - | - |
|  | 50 % | 300 | - | 500 | - | - |
|  | 30 % | 650 | 200 | 900 | 550 | 650 |
| 17 pdr AP on Tiger I |  |  |  |  |  |  |
| Full view | 90 % | - | - | 700 | - | 500 |
|  | 50 % | 1750 | 700 | 2050 | 600 | 1950 |
|  | 30 % | 1850 | 1450 | 2600 | 1600 | 2500 |
| Hull down | 90 % | - | - | - | - | - |
|  | 50 % | 750 | - | 800 | 600 | - |
|  | 30 % | 1300 | 550 | 1200 | 1000 | 800 |
| 17 pdr APCBC on Tiger I |  |  |  |  |  |  |
| Full view | 90 % | - | - | 900 | - | 550 |
|  | 50 % | 2000 | 600 | 2300 | 650 | 2050 |
|  | 30 % | 2700 | 1300 | 2850 | 1750 | 2850 |
| Hull down | 90 % | - | - | - | - | - |
|  | 50 % | 700 | 50 | 750 | 650 | - |
|  | 30 % | 1300 | 450 | 1250 | 1050 | 850 |
| 6 pdr AP on Panther |  |  |  |  |  |  |
| Full view | 90 % | - | - | 750 | - | 900 |
|  | 50 % | 50 | 800 | 1350 | 800 | 1200 |
|  | 30 % | 100 | 1200 | 1800 | 1200 | 1800 |
| Hull down | 90 % | - | 100 | 100 | 50 | 50 |
|  | 50 % | - | 300 | 850 | 300 | 800 |
|  | 30 % | 50 | 700 | 1150 | 800 | 850 |
| 6 pdr APCBC on Panther |  |  |  |  |  |  |
| Full view | 90 % | - | - | 850 | - | 1150 |
|  | 50 % | 50 | 750 | 1650 | 1000 | 1450 |
|  | 30 % | 100 | 1500 | 2200 | 1400 | 1950 |
| Hull down | 90 % | - | 50 | 50 | 50 | 50 |
|  | 50 % | - | 250 | 850 | 250 | 850 |
|  | 30 % | 50 | 800 | 1150 | 850 | 1150 |
| 17 pdr AP on Panther |  |  |  |  |  |  |
| Full view | 90 % | - | - | 1250 | 1200 | 1100 |
|  | 50 % | 50 | 1800 | 2350 | 2000 | 2100 |
|  | 30 % | 100 | 2150 | 2850 | 2550 | 2750 |
| Hull down | 90 % | - | - | 300 | 400 | 350 |
|  | 50 % | 450 | 550 | 1000 | 1150 | 1100 |
|  | 30 % | 650 | 1150 | 1750 | 1750 | 1800 |
| 17 pdr APCBC on Panther |  |  |  |  |  |  |
| Full view | 90 % | - | - | 1200 | 1250 | 1200 |
|  | 50 % | 50 | 1750 | 2350 | 2350 | 2250 |
|  | 30 % | 100 | 2550 | 2900 | 2850 | 2850 |
| Hull down | 90 % | - | - | 400 | 500 | 450 |
|  | 50 % | 300 | 550 | 1100 | 1200 | 1100 |
|  | 30 % | 700 | 1150 | 1850 | 1850 | 1850 |
| 6 pdr AP on Panzer IVG |  |  |  |  |  |  |
| Full view | 90 % | - | - | 650 | 650 | - |
|  | 50 % | 800 | 1000 | 1450 | 1450 | 1250 |
|  | 30 % | 1250 | 1550 | 1900 | 1500 | 1700 |
| Hull down | 90 % | 50 | 50 | 100 | 100 | 50 |
|  | 50 % | 450 | 300 | 600 | 500 | 450 |
|  | 30 % | 800 | 500 | 950 | 900 | 800 |
| 6 pdr APCBC on Panzer IVG |  |  |  |  |  |  |
| Full view | 90 % | 50 | 50 | 700 | 750 | 150 |
|  | 50 % | 450 | 1100 | 1450 | 1500 | 1300 |
|  | 30 % | 1400 | 1750 | 2100 | 2100 | 1850 |
| Hull down | 90 % | - | - | 100 | 150 | 50 |
|  | 50 % | 300 | 200 | 550 | 500 | 350 |
|  | 30 % | 800 | 850 | 1000 | 1000 | 800 |
| 17 pdr AP on Panzer IVG |  |  |  |  |  |  |
| Full view | 90 % | 50 | 550 | 800 | 750 | 650 |
|  | 50 % | 1350 | 1700 | 2050 | 2100 | 1700 |
|  | 30 % | 1900 | 2250 | 2600 | 2600 | 2250 |
| Hull down | 90 % | - | 200 | 250 | 300 | 300 |
|  | 50 % | 600 | 650 | 800 | 800 | 850 |
|  | 30 % | 1050 | 1150 | 1250 | 1250 | 1050 |
| 17 pdr APCBC on Panzer IVG |  |  |  |  |  |  |
| Full view | 90 % | 250 | 500 | 800 | 800 | 700 |
|  | 50 % | 1350 | 1850 | 2100 | 2250 | 1700 |
|  | 30 % | 2200 | 3000+ | 3000+ | 3000+ | 2550 |
| Hull down | 90 % | - | 100 | 250 | 250 | 300 |
|  | 50 % | 550 | 700 | 850 | 850 | 850 |
|  | 30 % | 1050 | 1150 | 1250 | 1250 | 1050 |
| 6 pdr AP on Panzer IIIL |  |  |  |  |  |  |
| Full view | 90 % | - | - | 600 | 200 | 100 |
|  | 50 % | 800 | 950 | 1450 | 1200 | 1100 |
|  | 30 % | 1000 | 1550 | 1900 | 1550 | 1250 |
| Hull down | 90 % | - | - | 100 | 100 | 100 |
|  | 50 % | 300 | 250 | 550 | 550 | 550 |
|  | 30 % | 800 | 400 | 850 | 900 | 950 |
| 6 pdr APCBC on Panzer IIIL |  |  |  |  |  |  |
| Full view | 90 % | - | - | 700 | 450 | 50 |
|  | 50 % | 1050 | 1000 | 1500 | 1200 | 1050 |
|  | 30 % | 1500 | 1600 | 2150 | 1900 | 1600 |
| Hull down | 90 % | - | 50 | 100 | 150 | 50 |
|  | 50 % | 450 | 250 | 550 | 550 | 500 |
|  | 30 % | 900 | 450 | 950 | 1000 | 900 |
| 17 pdr AP on Panzer IIIL |  |  |  |  |  |  |
| Full view | 90 % | 250 | 300 | 750 | 700 | 150 |
|  | 50 % | 1300 | 1450 | 2000 | 1650 | 1500 |
|  | 30 % | 2000 | 2150 | 2600 | 2400 | 2150 |
| Hull down | 90 % | 50 | 200 | 250 | 350 | 350 |
|  | 50 % | 650 | 700 | 800 | 900 | 750 |
|  | 30 % | 1050 | 1000 | 1200 | 1250 | 1150 |
| 17 pdr APCBC on Panzer IIIL |  |  |  |  |  |  |
| Full view | 90 % | 50 | 700 | 800 | 750 | 200 |
|  | 50 % | 700 | 1850 | 2050 | 2000 | 1500 |
|  | 30 % | 2500 | 3000 | 3000+ | 3000+ | 2300 |
| Hull down | 90 % | 50 | 150 | 50 | 250 | 300 |
|  | 50 % | 650 | 700 | 650 | 750 | 750 |
|  | 30 % | 1050 | 1150 | 1200 | 1200 | 1150 |

Comments and corrections

The tables above are extracted from a set of polar diagrams plotted to show the 30%, 50% and 90% contours for the chance of success from all angles around the target vehicle. The tables therefore omit much of the information in the original diagrams, which was calculated using 15º intervals from head-on (0º) to rear-on (180º). There is a noticeable tendency for many of the polar plots to form "clover-leaves", and these are especially clear on the Panther and Tiger.

The original document emphatically stresses the approximate nature of these results, and cautions that they should be treated as comparative rather than absolute figures. It details the definitions used, simplifying assumptions made, and in some cases assesses the distortion these may cause.

"Chance of success", as used here, means the probability of hitting and killing with a single shot.

There is assumed to be a systematic error of 35 yards in range and zero in line. The gun layer is assumed to lay with the same accuracy as the 90% zone of the gun. This will tend to overestimate effectiveness at long range. All targets are assumed to be static.

The damage criterion for a kill is taken to be complete perforation of the armoured box, through which the whole length of the projectile passes. All tank components outside the armoured box, such as running gear, are ignored. Behind-armour effects are ignored, although it is noted that tanks are very unlikely to survive penetrations by projectiles of 6-pdr size and greater.

Shots that strike the target are assumed to be uniformly, not normally, distributed over the target areas presented. This may tend to overestimate the amount of "invulnerable" side-armour presented in fine front quarter shots, and neglects the possibility of the gunner choosing to aim at a specific vulnerable point on the target tank. Hull-down targets have been treated as being bounded below by the turret ring.

Armour quality has been treated as falling into three categories. German machineable-quality (MQ) armour on the Panther and Tiger has been treated as equivalent to British MQ. The MQ armour on the Panzer III and Panzer IV has been treated as equivalent to a 10% greater thickness of British MQ. The face-hardened (FH) plate has been taken as having a critical impact velocity for penetration 500 feet per second greater than British MQ with respect to AP projectiles, 200 feet per second greater with respect to APCBC. The first assumption is regarded as reasonable; the second as rough, possibly tending to overestimate the German armour; and the final assumption is thought to be very doubtful.

The original document does not state whether the Panzer III and IV targets were considered to be using Schürzen.

It is hard to see how some diagrams, for example that for the 17 pounder vs Panther, can show a better performance from some aspects against a hull-down target than a fully exposed one.

## WO 291/180, "Accuracy of anti-tank gunnery."

Ranges in yards, target assumed to be Pz VI size.

Probability (%) of hitting static hull-up target with first round:

| Gun | OK for... | 500 | 1000 | 1500 | 2000 | 2500 |
| --- | --- | --- | --- | --- | --- | --- |
| 6 pdr | line | 100 | 100 | 96 | 87 |  |
|  | range | 87 | 33 | 13 | 3 |  |
|  | both (hit) | 87 | 33 | 12 | 3 |  |
| 17 pdr | line | 100 | 100 | 100 | 98 | 93 |
|  | range | 98 | 46 | 20 | 10 | 5 |
|  | both (hit) | 98 | 46 | 20 | 10 | 5 |

Probability (%) of hitting static hull-up target after first round:

| Gun | 500 | 1000 | 1500 | 2000 | 2500 |
| --- | --- | --- | --- | --- | --- |
| 6 pdr | 100 | 86 | 59 | 41 |  |
| 17 pdr | 100 | 94 | 71 | 50 | 36 |

Probability (%) of hitting moving target (direct-crossing at 15 mph) after first round:

| #### Gun | 500 | 1000 | 1500 | 2000 | 2500 |
| --- | --- | --- | --- | --- | --- |
| 17 pdr | 86 | 48 | 26(e) | 16 |  |

Probability (%) of hitting hull-down target with first round:

| Gun | 500 | 1000 |
| --- | --- | --- |
| 17 pdr | 59 | 18 |

Probability (%) of hitting static hull-down target after first round:

| Gun | 500 | 1000 | 1500 | 2000 | 2500 |
| --- | --- | --- | --- | --- | --- |
| 6 pdr | 85 | 43 | 22 | 14 |  |
| 17 pdr | 88 | 51 | 29 | 18 | 12 |

Comments and corrections

These values are those plotted on the graphs in the report; the value marked (e) is interpolated from other data points. The overall shape of the fitted curves in each case is sinusoidal.

This report recommends that the maximum range of engagement for 6-pdr and 17-pdr ATk guns be considered 800 and 1000 yards respectively. The criteria stated for maximum range of engagement for a statically-sited ATk gun are:

50% chance of first-round hit on a static hull-up target;
90% chance of subsequent rounds hitting a static hull-up target;
50% hits on a hull-up direct-crossing target moving at 15 mph after MPI roughly corrected;
50% hits on a static hull-down target after MPI roughly corrected.

The first table clearly shows that errors in range have a much more important effect on accuracy than errors in line.

Penetration ranges against Pz VI for each gun (ammunition not specified) are stated as being 800 yards for 6-pdr on the front, 1600 yards on the side, and 2000–2500 yards for 17-pdr. Hit probability is therefore regarded as a more important limitation on maximum engagement range than penetration.

## WO 291/1263, "Firing Trials, 17pdr Sherman."

"Table VI has been constructed which shows the probability of a hit on a target 5' wide by 2' high (representing a Panther turret) at various ranges using both types of round."

| Range (yards) | APC % | AP/DS % |
| --- | --- | --- |
| 400 | 90.5 | 56.6 |
| 600 | 73.0 | 34.2 |
| 800 | 57.3 | 21.9 |
| 1000 | 45.3 | 14.9 |
| 1500 | 25.4 | 7.1 |

Comments and corrections

These assume that the MPI is placed centrally on the target.

The trace from the AP/DS round was not seen in 73% of cases by a flank observer, and in no case from inside the tank.

## WO 291/2398, "The hit chance of WOMBAT against moving targets."

Hit chance on a moving target:

| Range (metres) | Calculated % | Observed % |
| --- | --- | --- |
| 360 | 75 | 70 |
| 630 | 72 | 55 |
| 840 | 60 | 34 |
| 1030 | 37 | 22 |

Comments and corrections

Although Wombat is a post-war weapon (this paper is dated July 1964), these figures are included as representing upper bounds for WW2 recoilless weapons. Wombat uses a ballistically-matched 0.5" spotting rifle, so these figures might be comparable to shooting with a WW2-vintage RCL once ranged on target. A system of "offset shooting" had to be devised, as the impact of even an inert 120mm round was sufficient to disable the radio-controlled Stuart tank used in the trials.

For some reason which is not clear, a second table is included for targets moving at up to 12 mph:

| Range (metres) | Hit chance % |
| --- | --- |
| 460 | 60 |
| 630 | 44 |
| 840 | 31 |
| 1040 | 15 |

It is stated that a good fit for hit probability is given by the formula P% = 97– 8R, where R is the range in hundreds of metres.