Circuit Analysis –number 1                             (2 marzo 2006)

by Reinaldo Baretti Machín

www.geocities.com/serienumerica

www.geocities.com/reibaretti2004

e-mail: reibaretti2004@yahoo.com

Reference : Jimmie J Cathey and Syed A. Nasar, Basic Electrical  Engineering, Schaum’s Outlines , page 92 (problem 6.10)

For the given circuit i1 and i2 are sought. Let i1=x1 , i2=x2

The governing ODE are

dx1/dt= vx1(i)= -(rs/(2.*L1))*(x1(i)+x2(i)-Vs/Rs)

dx2/dt=  vx2(i)=(-.5*Rs/L2)*x1(i)-(.5*rs/L2 +R2/L2)*x2(i) +(.5/L2)*Vs

and the initial conditionas are  i1=i2=0.

The code given below solves them by the  finite difference method,

where

x1(n) = x1(n-1)+ dt* vx1(n-1)

x2(n) = x2(n-1)+ dt* vx2(n-1)

FORTRAN CODE

c Circuit analysis problem 6.10   basic EE Scahum' Outlines

c     Cathey page 92

dimension x1(0:10000) , x2(0:10000)

real L1 , L2

data Vs,R1,R2, Rs , L1,L2/1.,2.,1.,2.,1.,2./

vx1(i)= -(rs/(2.*L1))*(x1(i)+x2(i)-Vs/Rs)

vx2(i)=(-.5*Rs/L2)*x1(i)-(.5*rs/L2 +R2/L2)*x2(i) +(.5/L2)*Vs

t1=Rs/l2

t2=r2/l2

t3=rs/l1

tscale=amin1(t1,t2,t3)

dt=tscale/20.

tf=3.5*amax1(t1,t2,t3)

nstep=int(tf/dt)

kount=int(float(nstep)/50.)

kprint=kount

x1(0)=0.

x2(0)=0.

print 100 ,0, x1(0),x2(0)

do 10 i=1,nstep

t=dt*float(i)

x1(i)=x1(i-1)+ dt*vx1(i-1)

x2(i)=x2(i-1)+ dt*vx2(i-1)

if(i-kount)10,17,17

17    kprint=kprint+kount

print 100,t , x1(i),x2(i)

10    continue

100   format(2x,'t,x1,x2=',3(3x,e11.4))

stop

end