Coupled Circuits                                        17 oct. 2006

by Reinaldo Baretti Machín

www.geocities.com/serienumerica

e-amil : reibaretti2004@yahoo.com

Reference :

1. Applied mathematics for engineers and physicists by L. A. Pipes  , page 174.

The Kirchoff  equations are

L1 * d I1/dt + R1*I1   + q1/C1  +  (q1-q2)/C12  = E

L2 * dI2/dt  + R2*I2    +q2/C1   + (q2-q1)/C12  =0

The finite difference solution is

i1one=i1zero+(dt/L1)*a1(i1zero,q1zero,q2zero)

i2one=i2zero + (dt/L2)*a2(i2zero,q1zero,q2zero)

q1one=q1zero+dt*i1one

q2one=q2zero+dt*i2one

where                 a1(i1,q1,q2)= -r1*i1-q1/c1 -(q1-q2)/c12 + V

a2(i2,q1,q2) = -r2*i2-q2/c2 - (q2 -q1)/c12                .

c surge prob 9 oct 2006 resuelto por dif finitas

c  the i0 , i1  represent values of the current on the LHS mesh

c  the amp0,amp1 are values of the current on the RHS mesh

real L1,L2 ,i1zero,i1one,i2zero,i2one ,i1,i2

data c1,c2,c12,r1,r2,V/1.E-6,1.E-6,1.E-6,100.,100.,12./

a1(i1,q1,q2)= -r1*i1-q1/c1 -(q1-q2)/c12 + V

a2(i2,q1,q2) = -r2*i2-q2/c2 - (q2 -q1)/c12

l1=r1*(r1*c12)

L2=r2*(r2*c12)

t1= L1/r1

t2= sqrt(L1*c1)

t3= sqrt(L1*c12)

t4=L2/r2

t5=sqrt(L2*c2)

t6=sqrt(L2*c12)

tscale=amin1(t1,t2,t3,t4,t5,t6)

print*,'l1,l2,tscale=',l1,l2,tscale

c ADJUST FINAL TIME ACCORDING TO THE GRAPH YOU WANT TO PLOT  COMPARE

c  TFINAL  with t1,t2,t3

tlarge=amax1(t1,t2,t3,t4,t5,t6)

timeratio=tlarge/tscale

tfinal=7.*tlarge

print*,'tscale,tlarge,timeratio,tfinal=',tscale,tlarge,timeratio,

$ tfinal

dt=tscale/100.

nstep=int(tfinal/dt)

i1zero=0.

i2zero=0.

q1zero=0.

q2zero=0.

print*,'dt,tscale,tfinal,nstep =', dt,tscale,tfinal,nstep

print*,'  '

kp=int(float(nstep)/80.)

kount=kp

print 100,0., i1zero, i2zero

do 10 i=1,nstep

t=dt*float(i)

i1one=i1zero+(dt/L1)*a1(i1zero,q1zero,q2zero)

i2one=i2zero + (dt/L2)*a2(i2zero,q1zero,q2zero)

q1one=q1zero+dt*i1one

q2one=q2zero+dt*i2one

if(i.eq.kount)then

print 100 ,t, i1one, i2one

kount=kount+kp

endif

i1zero=i1one

i2zero=i2one

q1zero=q1one

q2zero=q2one

10    continue

100   format(1x,'t,i1,i2=',3(3x,e10.3))

stop

end