Percent overshoot of a capacitor

by Reinaldo Baretti Machín

www.geocities.com/serienumerica

e-mail: reibaretti2004@yahoo.com

Reference:

1.  Schaum BEE prob 4.10

data for this exercise  V= 100volts , R =200.ohms, C=4.76E-6 F ,L=0.150 H

percent overshoot =  11.6802597

FORTRAN code

c percent overshoot  Schaum's BEE   prob 4.10

real L

data V,R,C,L/100.,200.,4.76E-6,0.150/

pi=2.*asin(1.)

q0=0.

q1=0.

period=2.*pi*sqrt(L*c)

nstep=2000

dt=2.*period/float(nstep)

percent=0.

kount=20

kp=kount

print 100,0., q0/C ,V

do 10 i=2,nstep

t=dt*float(i)

q2=2.*q1-q0 +dt**2*(V- r*(q1-q0)/dt -q1/C)/L

if(q1/c.gt.v)then

percenti= 100.*(q1/c -V)/V

c      print*,' percenti=',percenti

if(percenti.gt.percent)percent=percenti

endif

if(i.eq.kp)then

print 100, t , q2/C  ,V

kp=kp+ kount

endif

q0=q1

q1=q2

10    continue

100   format(1x,'t,Vc=',3(4x,e11.4))

print*,'   '

print*,'percent overshoot =',percent

stop

end