Scahum’s  BEE Problem 17.8

by Reinaldo Baretti Machín

www.geocities.com/serienumerica

e-mail: reibaretti2004@yahoo.com

The Routh-Hurwitz procedure is  employed in Control Theory to determine the number of RHSP roots and thus the stability of a system.

We propose here to solve the characteristic equation and plot the solution by means of a simple procedure .

Corresponding to the characteristic equation ( in the theory of control criteria)

D(s) = s5 + s4 + 4s +4                                             (1)

we postulate the differential equation

d5 x/dt5  + d4 x/dt4  +4 dx/dt +4 x  = u(t)                       (2)

with zero  intial conditions and u(t) is the unit step function.

We convert (2) into five first order differential equations that are solved by the finite difference method.

Define variables , v, a, p and q

v =dx/dt                                                  x n = xn-1 + vn-1 ∆t

a=dv/dt = d2 x/dt 2                                  v n = v n-1 + a n-1 ∆t

p=da/dt  = d3 x/dt 3                                 a n = a n-1 +p n-1 ∆t

q=dp/dt  = d4 x/dt 4                                  p n = p n-1 + q n-1 ∆t

and finally equation (2) is equivalent to

dq/dt =- ( q +4 v +4 x + u(t) )      and

qn = qn-1 +  ∆t   * ( qn-1 +4 vn-1 +4 xn-1 + u(tn-1 )     )

FORTRAN code

c Schaum BEE prob 17.8 control criteria and response

u(t)=1.

f(q,v,x,t)=-q-4.*v-4.*x+u(t)

x0=0.

v0=0.

a0=0.

p0=0.

q0=0.

tscale=sqrt(1./4.)

dt=tscale/50.

tfinal =10.

nstep= int(tfinal/dt)

kp=int(float(nstep)/60.)

kount=kp

print 100,0.,x0,v0

do 10 i=1,nstep

t=dt*float(i)

q1=q0+dt*f(q0,v0,x0,t-dt)

x1=x0+v0*dt

v1=v0+a0*dt

a1=a0+p0*dt

p1=p0+q0*dt

if(i.eq.kount)then

print 100,t,x1,v1

kount=kount+kp

endif

q0=q1

x0=x1

v0=v1

a0=a1

p0=p1

10    continue

100   format(1x,'t,x,v=',3(4x,e10.3))

stop

end

The response becomes unstable after  t>8 sec