Schaum BEE problem 3.7

by Reinaldo Baretti Machín

www.geocities.com/serienumerica

e-mail: reibaretti2004@yahoo.com

An RCL circuit, connected to a generator, has the following component values R=100 ohms, L =1.H  , C=5E-6 F. The driving voltage is V= 100 sin (2π f t)  , with   f= 79.6Hz.

The steady state analysis gives

XL= 500 ohms , XC = 400 ohms  , Z= 100 + j ( 100)

or Z = 141.4 at 45 deg.

The steady state analyzed using phasors  gives,

I =V/Z = 0.707 at (-45 deg)

VR= IZ =70.7 at(-45 deg)

VC=(400 at(-90) ) * (.707 at (-45deg)) = 282 at (-135 deg)

VL=(500 at (+90deg) ) * ( .708 at(-45) ) = 353 at (45 deg)

It is instructuive to solve the problem numerically ( see Fortran code below). The one sees the solution developing in time and has a sense of how much one has to wait for the transient effects to die out.

We use two Taylor series.

The differential equation is

di/dt=  (1./L)*(-R*i -q/C +V(t) ) ≡ g (i,q,t)    .               (1a)

or

d2q /dt2 = (1./L)*(-R*(dq/dt) -q/C +V(t) )                       (1b)

To solve numerically eq. 1b the initial conditions q(0) and (dq/dt)0

We will also need

d2 i/dt2 = dg/dt = (1./L)*(-R*di/dt -i/C +dV(t)/dt )                (2)

The series for the charge to second order is

qn=qn-1 + dt*(dq/dt)n-1 +.5*dt**2*(d2q/dt2)n-1                     (3)

and that for the current is

in=in-1 + dt*(di/dt)n-1 +.5*dt**2*(d2i/dt2)n-1                        (4)

Using ( 1-a) and (2) , (3) and (4) are rewritten as

qn=qn-1 + dt*in-1 +.5*dt**2*gn-1                                            (5)

in=in-1 + dt* g n-1 +.5*dt**2*(dg/dt)n-1                                  (6)

Let  τ be the period of the generator  = 2π/ω.The plot show the voltages for the first 3(360 degrees)=  1080 degrees.

The maximum voltages should de be compared to those of the steady state regime calculated above.

RCL first oscillations

-400

-300

-200

-100

0

100

200

300

400

0.00E+00

2.00E+02

4.00E+02

6.00E+02

8.00E+02

1.00E+03

theta (degrees)

volts

VR

VC

VL

Vgenerator

First oscillations for a time            0    ≤ time ≤ 3τ

RCL circuit with sinusoidal input

-400

-300

-200

-100

0

100

200

300

400

2.16E+03

2.36E+03

2.56E+03

2.76E+03

theta (degrees)

volts

VR

VC

VL

V generator

The steady state regime has been achieved  sfter 6 oscillations.

The plot is for        6τ    ≤ time ≤ 8τ .

FORTRAN code

c AC circuit steady state  Schaum's BEE prob 3.7

c        L di/dt= -RI -q/c  +V(t)

real L ,i ,i0,i1

data Vm,f,R,L,C/100.,79.6,100.,1.,5.E-6/

v(t)=Vm*sin(omega*t)

g(i,q,t)= (1./L)*(-R*i -q/C +V(t) )

dgdt(i,q,t)= (1./L)*(-R*g(i,q,t)-i/C+ omega*vm*cos(omega*t) )

pi=2.*asin(1.)

omega=2.*pi*f

t1=L/R

t2=R*C

tau=2.*pi/omega

tscale=amin1(t1,t2,tau)

dt=tscale/400.

tfinal=8.*tau

print*,'tfinal/tL ,tfinal/Tc=', tfinal/t1, tfinal/t2

nstep=int(tfinal/dt)

kprint=int(float(nstep)/200.)

kount=kprint

q0=0.

i0=0.

vr0=R*i0

vc0=q0/c

vl0=L* g(i0,q0,0.)

print*,'      theta (deg) ,     , Vr,     ,vc,    ,vL,

$ ,Vsource'

print*,'       '

print 100,0.,vr0,vc0,vl0,v(0.)

do 10 j=1,nstep

t=dt*float(j)

q1=q0 + dt*i0+.5*dt**2*g(i0,q0,t-dt)

i1=i0 + dt*g(i0,q0,t-dt) +.5*dt**2*dgdt(i0,q0,t-dt)

if(j.eq.kount)then

vr=R*i1

vc=q1/C

vL=L*g(i1,q1,t)

if(t.le.3.*tau)then

print 100, omega*t*180./pi,vr,vc,vl, v(t)

endif

kount=kount+kprint

endif

q0=q1

i0=i1

10    continue

100   format(5(3x,e11.4))

stop

end