THE CATENARY

by Reinaldo Baretti Machín

www.geocities.com/serienumerica

reibaretti2004@yahoo.com

Reference:

1. Electric Power Transmission System Engineering: Analysis and Design by Turan Gonen ,chapter 10.

If  the equation of the catenary ,

dy/dx =  (w / H) s   =  (w/H) ∫ ( 1+ (dy/dx)2 )1/2 dx  , (1)

is differentiated once more , one obtains

d2 y/dx2 =   (w/H) ( 1+ (dy/dx)2 )1/2                              (2)

The initial conditions are y(0) =0 , (dy/dx)x=0 =0.

The finite difference solution of (2) is

y2=2.*y1-y0 + dx**2* (w/H)*(1.+ ((y1-y0)/ ∆x)**2 )1/2 . (3)

FORTRAN CODE

c      The Catenary  see Ref .

c w/h ~ lb/ft/lb~1/ft   y~ ft , x~ ft

real length ,lscale

a(y0,y1)=woverh*sqrt(1.+ ((y1-y0)/dx)**2 )

length=250.

c=1.6E3

WoverH= 1./c

lscale=amin1(length,c)

dx=lscale/200.

nstep=int(length/dx)

dydx=0.

y0=0.

y1=y0+dx*dydx

kount=4

print 100,0.,y0

do 10 i=2,nstep

x=dx*float(i)

y2=2.*y1-y0 + dx**2*a(y0,y1)

if(i.eq.kount)then

print 100, x,y2

print 100, -x,y2

kount=kount+4

endif

y0=y1

y1=y2

10    continue

100   format(1x,'x,y=',2(4x,e10.3))

stop

end