Power at the load using  Complex Fortran

by Reinaldo Baretti Machín , Alfonso Baretti Huertas

www.geocities.com/serienumerica2

www.geocities.com/serienumerica

reibaretti2004@yahoo.com

The power at the load RL =100ohms on the diagram is sought.

In another page  (http://www.geocities.com/serienumerica2/Averagepoweroct6.doc

)

we applied the finite difference method to solve two coupled differential equations for the currents I1 and I2.

The results,I1,I2 and  I2**2*Rload , are plotted and the FORTRAN code employed is given.

In complex notation the results follow simply from the equations,

zpar=1./(1./z1+1./z2)

zt=r1+zpar

I=v/zt

i1=(v-I*r1)/z1

i2=I-I1

pload=i2**2*rload

In the example worked below C=3e-3 F , the inductance  L= 0.2H , and the two resistance are R=20ohms , Rload =100 ohms.

abs(i), anglei=  7.49074125  2.5413928

abs(i1), anglei1=  7.52259398  2.86566758

abs(i2), anglei2=  0.0530997179  55.8432732

abs(pload),Real(pload), aimag(pload),angle=  0.281958014 -0.104191564

0.262000829 -68.313446

c example power at the load ...

real L

complex V , I , I1, I2 , z1 , z2, zpar,  zt ,Pload

data r1,rload,c,L, omega/ 20.,100.,3.e-3, 0.2,377./

pi=2.*asin(1.)

rtodeg=180./pi

degtor=1./rtodeg

v=cmplx(150.,0.*degtor)

z1=cmplx(0.,-1./(omega*c))

z2=cmplx(rload,omega*L)

zpar=1./(1./z1+1./z2)

zt=r1+zpar

I=v/zt

i1=(v-I*r1)/z1

i2=I-I1

pload=i2**2*rload

print*,'abs(i), anglei=',abs(i),rtodeg*atan(aimag(i)/real(i))

print*,'  '

print*,'abs(i1), anglei1=',abs(i1),

$ rtodeg*atan(aimag(i1)/real(i1))

print*,'  '

print*,'abs(i2), anglei2=',abs(i2),

$  rtodeg*atan(aimag(i2)/real(i2))

print*,'  '

print*,'abs(pload),Rael(pload), aimag(pload),angle=',abs(pload),

$real(pload),aimag(pload),

$ rtodeg*atan(aimag(pload)/real(pload))

stop

end