Bulgarian Chamber of Commerce and Industry Registration Number: 1017244600 / 24.02.1992 www.desislava20granulators.com VAT No.: BG040002183

2 GRANULATORS KOMPASS STRENGTHEN DESIGN SUITABLE FOR

DRY GRINDING OF RUBBER / TYRE PIECES WITH SIZES:

ABOUT 35 mm X 35 mm X 20 mm

UP TO END MATERIAL WITH SIZES 4 mm

PRODUCTIVITY PER HOUR AT THE INPUT UP TO 2’000 kg

ZINC COATED BODIES OF THE GRANULATORS, CONVEYORS AND DPS

1 GRANULATOR KOMPASS 500/1200 SOLID ROTOR (CS) CARBON STEEL (First Grinding)

1 GRANULATOR KOMPASS 800/1600 OPEN ROTOR (CS) CARBON STEEL (Second Grinding)

GRANULATOR KOMPASS 500/1200 SOLID ROTOR (CS) CARBON

(First Grinding)

| Purpose | Dry Grinding of rubber pieces together with the metal admixtures (initial material) Capacity: ABOUT 2’000 kg per hour at the input Max. sizes of the initial material: about 25 mm X 25 mm X 25 mm thickness |
| --- | --- |
| Motors | 110 kW / 1500 rpm / IP54 / 50 Hz / 3 phases / 400 V 1.5 kW for the hydraulic unit suitable for the opening of the machine 1.5 kW for the hydraulic unit suitable for the opening of the screen cradle |
| Rotor | Staggered / 12 sections X 3 rows of fly knives on each section Cutting Cylinder of the rotor 500 mm X 1200 mm |
| Knives Steel | 36 pcs. Fly knives X 100 mm 4 pcs. Bed knives X 600 mm (slant) – two rows of the bed knives X150CRVMO121 = D2 = 1.2379 |
| Screens holes | 16 mm |
| Bearings | SKF |
| Motor control | Will be manufactured with units from "Telemechanique"- France |
| Note | The granulator will be equipped with cooling system of the rotor and the cutting chamber. |
| Warranty | 12 months from the date of delivery (the warranty doesn’t include the cutting instruments, screen and the servicing). If it is needed to be sent spare parts the warranty terms cover the cost of the spare parts only Ex Works (the cost of the transport of the spare parts is not included in the warranty terms). |

GRANULATORS KOMPASS 800/1600 OPEN ROTOR (CS) CARBON STEEL

(Second grinding)

| Purpose | Dry Grinding of the rubber pieces received from the granulator Kompass 500/1200 Solid rotor (First Grinding). Capacity: Up to 2’000 kg per hour at the input Max. sizes of the initial material: about 16 mm Note: All metal admixtures should be separated from the ground material after the granulator Kompass 500/1200 Solid rotor (First Grinding) and after that the clear rubber will feed the granulator Kompass 800/1600 (Fine grinding). |
| --- | --- |
| Motors | 160 kW / 1500 rpm / IP54 / 50 Hz / 3 phases / 400 V 1.5 kW for the hydraulic unit suitable for the opening of the machine 1.5 kW for the hydraulic unit suitable for the opening of the screen cradle |
| Rotor | Straight/ 9 rows of fly knives Cutting Cylinder of the rotor 800 mm X 1600 mm |
| Knives Steel | 27 pcs. Fly knives X 533 mm 9 pcs. Bed knives X 533 mm (slant) – three rows of the bed knives X150CRVMO121 = D2 = 1.2379 |
| Screens holes | 4 mm |
| Bearings | SKF |
| Motor control | Will be manufactured with units from "Telemechanique"- France |
| Note | The granulator will be equipped with cooling system of the rotor and the cutting chamber. |
| Warranty | 12 months from the date of delivery (the warranty doesn’t include the cutting instruments, screen and the servicing). If it is needed to be sent spare parts the warranty terms cover the cost of the spare parts only Ex Works (the cost of the transport of the spare parts is not included in the warranty terms). |

Notes:

| 1. | The motors 110 kW and 160 KW will be delivered from Germany or Italy The motors 1.5 kW and the other small motors are made in Bulgaria |
| --- | --- |
| 2. | We will supply the needed Metal-separators and Chillers |

Feeding Belt Conveyor 1000 х 9000

a) Sizes:

- effective width – 1'000 mm - Height of the axis of the charge shaft – 1'000 mm - Height of the axis of the discharge shaft– 3'100 mm - distance between the shafts – 9'000 mm

b) Leading:

- side of assembly – left in the direction of the belt motion - Ø pushing drum – 320 mm - Ø pulling drum – 320 mm - el. motor – 2,2 kW/400 V/50 Hz/1'500 rpm /4 pol.

c) Rubber belt:

- with increased performance for hardiness of cutting and strokes - thickness – 8 mm - vulcanizing way of connection - cleats – height 20 mm, width 2 x 350 mm, distance 500 mm

d) Legs:

- regulation +/- 150 mm - 4 pieces. / 2 pieces with interlock/, Ø wheels 150 mm - paint: body blue - Blu Ral 5002 guard - yellow Ral 1003

Feeding Belt Conveyor 1000 х 7000

a) Sizes:

- effective width – 1000 mm - Height of the axis of the charge shaft – 1000 mm - Height of the axis of the discharge shaft– 3100 mm - distance between the shafts – 7000 mm

b) Leading:

- side of assembly – left in the direction of the belt motion - Ø pushing drum – 320 mm - Ø pulling drum – 320 mm - el. motor – 2,2 kW/400 V/50 Hz/1500 rpm /4 pol.

c) Rubber belt:

- with increased performance for hardiness of cutting and strokes - thickness – 8 mm - vulcanizing way of connection - cleats – height 20 mm, width 2 x 350 mm, distance 500 mm

d) Legs:

- regulation +/- 150 mm - 4 pieces. / 2 pieces with interlock/, Ø wheels 150 mm - paint: body blue - Blu Ral 5002 guard - yellow Ral 1003

Possibility for assembly of metal detector

Possibility for assembly of metal separator for Fe.

Belt Conveyor 600/3000

(Placed under the granulator Kompass 500/1200)

a)  Sizes:

- effective width – 600 mm - height of the axis of the charge shaft – 200 mm - height of the axis of the discharge shaft – 200 mm - distance between the shafts – 3000 mm

b) Leading

- side of assembly – right by the direction of the belt motion. - Ø pushing drum – 320 mm - Ø pulling drum – 320 mm - motor – 1,5 kW/400 V/50 Hz/1500 rpm /4 pol. - reducer – 86

c) Rubber belt:

- with increased performance for hardiness of cutting and strokes - thickness – 8 mm - vulcanizing way of connection - cleats – height 20 mm, width 450 mm, distance 500 mm

d) Legs – static

DISCHARGE PNEUMATIC SYSTEM (DPS)

| Purpose | The purpose of the DPS is to discharge the granulator and make the separation of the dust from the ground material. |
| --- | --- |
| Motor | 22.5 kW / 1’500 rpm / 50 Hz / 400 V |
| Warranty | 12 months from the date of delivery (the warranty doesn’t include the servicing). If it is needed to be sent spare parts the warranty terms cover the cost of the spare parts only Ex Works. The cost of the transport of the spare parts is not included in the warranty terms. |
| Note | The DISCHARGE PNEUMATIC SYSTEM (DPS) contain: fan - working pipes - cyclone - cyclone stand - dust collection system and the stand for this system. Textile (WOOL) separation plant. |

Sincerely Yours, 
Konstantin Tomov,

President of Desislava 20 Granulators Company

ADDRESS:

9 Zlatovrah Str.

1164 Sofia, Bulgaria

tel./fax: 359 2 870 3295

mobile: 359 896 872 859

URL:

www.desislava20granulators.com

http://www.geocities.com/desislava20/demolition_recycling.html

http://www.geocities.com/desislava20_/Movie800.wmv

http://www.geocities.com/desislava20_/500all800.wmv

E-mails:

info@desislava20granulators.com

support@desislava20granulators.com

desislava20@fastbg.net

ktomov@usa.com

Skype:

konstantint9047

Desislava 20 Granulators Company wishes you successful business!