TMWACC2 decoder ver 2.0

Operates four Tortoise Slow Motion Turnout Motors.

The outputs are paired and can not be operated individually.

The following NMRA defined CV's and ranges are supported...

CV513	Lower 6 address bit  00h - 3Fh

CV514	Output enable.           00h - FFh  Bits 0 to 7.Each bit sets an output.

CV515	F1   Should be set to 0 = continuous ON

CV516	F2   Should be set to 0 = continuous ON

CV517	F3   Should be set to 0 = continuous ON

CV518	F4   Should be set to 0 = continuous ON

CV521 	Upper 3 address bits	00h -07h

Read only CVs.

CV519	Version Number	= 20 (ver 2.0)	Cannot be changed

CV520	Manufacturers ID	= 13  (DIY decoder) 	Cannot be changed

CV541	Configuration       =08  (Accessory decoder)	Cannot be changed

No other CVs are supported.