MiniDVD – the new format for the “financially challenged” 

How To Make MiniDVD disc (updated Sept. 10, 1999).

(thanks goes to the friends who mailed me for test results, Blight, b27, Dracore, tom^jerry, n0mad, dubnium, mdx, Kakz-youknowwhoyouare, Max10, PowerKiller and all of the nice dwellers of #pcdvd – the source of all things PCDVD)

Guys, now that we have a working ripper that rips from A to Z, and allow us to make ANY video/media format of our choosing from it, I will try to push this a little further.

For those who ripped dvd to mpeg1 or .asf, don't bother reading this writing. This writing is purely intended to those who want to maintain the high quality mpeg2 files that they rip with DVDRip.

In short, with a 650MB capacity of a CDR/CDRW, we HAVE the possibility to make our OWN DVD, or to put it more eye-catching, MiniDVD disc.

1. What the hell is miniDVD?

MiniDVD refers to a "supposedly" DVD-VIDEO content written on CDR/CDRW. This MiniDVD is readable AS DVD-VIDEO DISC by your PC-DVD PLAYER (hardware/software) AND certain Standalone DVD player(s).

So, now, you CAN make your own kinky DVD MENU. Now this would push your artistic creativity, don’t you think?

2. Is this only Theoretical? Are there any documents backing this theory up?

Yes, here are the links to get more info:

http://www.heuris.com/MPEG/MPEG%20Information/dvd%20faq.html

#Can I use a CD-ROM as a short-length DVD-VIDEO disk?

I quote:

What files and folder structure define a DVD-VIDEO disk?

The basic element that distinguishes as DVD-VIDEO file structure is the presence of the VIDEO_TS folder (similarly, the distinguishing element of those yet-to-be-released DVD-AUDIO disks will be the presence of the AUDIO_TS folder). The VIDEO_TS folder is the first thing that a DVD-VIDEO set-top box will look for when the disk is inserted. The VIDEO_TS folder contains the .IFO files that describe the functionality of the DVD-VIDEO title and the .VOB files that contain the actual video content.

Can I use a CD-ROM as a short-length DVD-VIDEO disk?

Theoretically - but we have not tested this, yet. BUT NOW WE HAVE 

Many second-generation DVD-VIDEO set-top boxes are able to read a CD-ROM disk. That means that you can create a DVD disk image using a DVD authoring system. You can then use a product like Toast DVD from Adaptec to burn a CD-R disk that uses the UDF disk format that is used by DVDs. A second-generation DVD-VIDEO player should be able to read the UDF-formatted CD-R and will recognize the disk as a valid DVD-VIDEO disk. One thing to remember, though, is that a CD-R can hold only 650 MB, which translates to perhaps 15 minutes of content. This makes this method suitable only for short clips.

Robshot’s note: Unfortunately, this is not 100% true. NOT MANY second generations of DVD-VIDEO set-top boxes can play miniDVD. This is confirmed by the Great Jim Taylor’s DVD FAQ.

http://www.videodiscovery.com/vdyweb/dvd/dvdfaq.html

#5.7

I quote:

[5.7] Can I put DVD-Video content on a CD-R or CD-RW?

[Note: This section refers to creating original DVD-Video content, not copying from DVD to CD. The latter is impractical, since it takes 7 to 14 CDs to hold one side of a DVD. Also, most DVD movies are encrypted so that the files can't be copied.]

Obviously there are advantages to recording a DVD-Video volume on inexpensive recordable CD rather than expensive recordable DVD. It would be perfect for testing and for short video programs. Unfortunately, you can put DVD-Video files on CD-R or CD-RW media but you'll have a hard time finding a player that can play the disc. The Philips DVD170 industrial player seems to be the only player that can play DVD-Video content from CD-R and CD-RW discs. The Pioneer DVD-V7200 industrial player can play DVD-Video from CD-RW, but not CD-R. In all cases, the disc must be formatted with the UDF file system in accordance with the DVD-Video spec.

Computers are more forgiving. DVD-Video files on CD-R or CD-RW, with or without UDF, will play back on any DVD-ROM PC as long as the drive can read recordable media (all but early models can).

An alternative is to put Video CD content on CD-R or CD-RW media for playback in a DVD player. Players that are Video CD capable and can read recordable media will be able to play such discs. The limitations of Video CD apply (MPEG-1 video and audio, 1.152 Mbps, 74 minutes of playing time). All DVD-ROM PCs able to read recordable CD media can play recorded Video CD discs.

Robshot’s note: Now, with all due respect to Jim Taylor, I say that there IS another Alternative. It is to make a ¼ D1 mpeg2 video stream. THIS WOULD ALLOW storing from 40 – 50 minutes of VOB with AC3 sound to the CDR/CDRW. A detail for a creation of such ¼ D1 vob has been written by b27. I put the article below, and added some “additional details” by his permission.

So, the possibilities are endless:

- Your Family Video Recording, made as half D1 DVD specs of 352x480 or 576, would fit approx. 25-30 mins on your MiniDVD, or at ¼ D1, at approx. 40 – 50 mins.

- Your TRAILER COLLECTION DVD :))

- Mebbe 7 to 8 CDROM of the whole movie, but who wants to do this anyway :))))

- Any other use you can think of.

3. So, HOW?

First, the requirements

- AN UDF CAPABLE CDROM WRITER or CDRWOM REWRITER. This is a MUST. Forget it if you don't have it. You can check compatibility or capability of your CDR/CDRW with UDF writing, at www.cequadrat.com and www.adaptec.com

- AN UDF CAPABLE CDR/CDRW BURNING/WRITING SOFTWARE. Nero, WinOnCD PowerEdition, both offers UDF format writing.

- A DVD AUTHORING SOFTARE: Scenarist NT, Streamweaver and later revised as CDMOTION6 are some of the options.

- A DVD DISC IMAGE CREATOR, such as ForDVD

- THE MATERIAL VIDEO/AUDIO along with their accompanying SOFTWARE UTILITIES (this is the widest range of utils you might or might not need).

THE PROCESS:

- You have 2 options for starting this:

- Your original home video

You will need a capture card to capture your video and audio footage into an .avi format. You can capture at 720x480/576, 640x480, or 352x240/288. Make sure no frame drop. Then, you will need to convert the VIDEO ONLY of the avi to an mpeg2 format (m2v/mpv) using available avi->mpeg2 video converter. My Encoding article covers this subject. You will also need to convert your .wav to .ac3 OR mpa/mp2 audio file to later multiplex them with your video only files (at the later stage of AUTHORING).

- A ripping session with either VideoMatrix (getting .avi from your dvd) or DVDRip (and get a vob).

You either get an .avi file, and get the audio file, and continue as the steps for original home video, OR, with DVDRip, you get a .vob WHICH YOU NEED TO DEMUX/SEPARATE to video only and audio only.

When you have a .m2v and .ac3 files separately, you have the option of WHICH AUTHORING software you want to make your miniDVD. If you want to go with CDMOTION6, you need their accompanying STREAMWEAVER to remultiplex your separated files into a “CDMOTION6 READY VOB”. YOU CANNOT use your freshly-ripped .vob straight into CDMOTION6.

If you want to go with Scenarist NT, you are not needed to remultiplex your separate files. Scenarist NT will do the job while you author  your miniDVD title.

- The Authoring Process:

Create a DVD-Video TITLE DISC. This process is for you to find out. No other document can help you, only the MANUALS of the Authoring software you use.

Scenarist, Streamweaver, CDMOTION6, they have the manuals. READ! Do some testing.. You can open a channel for it on EFNET. This is the most difficult task. You are on your own on this subject. Basically you want to make a menu, and a subpicture menu to make it available for button navigation assignment and interactivity. Confusing? Read the Manual 

3. Burning the title

There are mainly 2 ways of going further on this stage... Either use ProDVD to make the DISC image of your Video_ts directory created by The DVD Authoring program, OR JUST WRITE OFF THE VIDEOTS DIRECTORY USING either ISO, UDF/ISO or pure UDF FORMAT with NERO.

A note from a successful author:

I used the HP 8100 CD Writer Plus. Contents created with Scenarist NT v1.2 (ifo/bup/vob and disk image).

Masterization programs:

- Adaptec Easy CD deluxe write the DVD disk image using Mode 2 XA

- Nero write the IFO, BUP, VOB files and correct directory structures (Video_ts and Audio_ts as root) utilizing the built-in pure UDF formatting support of Nero

- Nero write the DVD disk image using Mode 2 XA (2048 bytes per sector). CDRs created in either way do play on a DVD-ROM as a video DVD, but not on a standalone.

I will add more tips when some tests have been done again

As I have mentioned earlier, to push this process further, you can make the ¼ D1 miniDVD and have a 40-50mins of mpeg2 ¼ D1 quality WITH .AC3 files from your ripped DVDs. Here I present the “b27 document”

The new vcd’s – Mini DVD

- Note that the steps below are for NTSC. PAL would need a slightly changed fps than the explanation below – 25fps.

- First step is to rip the dvd – using dvdrip you should rip around 45-50 minutes of video and audio in 1 vob file (no separate files).

- Then you use vobtool (by our own Dracore) to fix the vob and split it into files if needed

- Use vobsnoopy by cdmotion to separate the m2v and ac3 files from the vob creaed by vobtool

- Using mpeg2avi command line – mpeg2avi –b d:\*.m2v f –2 –1 352 240 –o7 *.avi. Choosing 23.976 as frame rate and picvideo mjpg codec at maximum quality – you make a great avi from the mpeg2 file

- using lsx mpeg encoder you make a new mpeg2 file from the avi you just made changing the fps to 29.97 and choosing in advance option to keep the same time. You can also try Heuris mpeg2 encoder, digigami, or even the freely distributed BBMPEG (a nice result with this one ).

- using streamweaver multiplex by cdmotion you multiplex the new m2v file and the ac3 file to 1 vob with ifo created also by the program

- after doing the same for all the files you want you go to the next step which is dvd Authoring

- using cdmotion6 pro you create a new project

- than if you want a menu you can create one but it’s not really necessary

- the you insert the vob inside the dvd video track also giving it a pic to show

- than in the edit applets you can choose what will each button do and what to do in the end of the track (jump to the next one , back to the menu …)

- after doing so for all the tracks you go to the grey area and pushing the right mouse button you choose to compile it

- if everything is done well it will create you a video_ts directory with a dvd inside

- you can test if it’s working properly with xingdvd moving the video_ts folder to the root of the drive and the playing it with xing

- if everything gone well till now you are ready to burn

- the first way is to burn the VIDEO_TS directory with nero using udf format

- second choice is making a dvd image with for-dvd by cdmotion and burning the image with nero choosing iso format

If you’ve done everything correctly, you should now have your MiniDVD with video better than any vcd and dolby digital sound – what more can u ask for?

thanx to robshot for all the help

b27

Well, that sums up the miniDVD document. Do some tests, and expect failures .

Regards, Robshot