## 3. Narratives describing career episode 4

## 4.3 Career Episode 4

### 4.3.1 Introduction

This narrative describing second career episode on my professional engineering work experience with the leaders in Chemical Manufacturing Complex Atul Limited, India.

Duration of Career Episode: July `91 - April `97

Location: Atul, India

Organisation: Atul Limited, Atul, India

Web site: http://www.atul.co.in

Title of Position: Assistant Manager-Mechanical

### 4.1.2 Background

This is the real place where I have started to build my professional career after my engineering course. The journey from Management Trainee to Assistant Manager in this short period was the result of my contribution to the company's success.

Nature of the overall engineering project

Overall engineering project was to reengineering of the total engineering practices and maintenance practices which were in use. The main objective was to improve plant efficiency and reliability by implementing preventive/proactive maintenance practices, condition monitoring, failure analysis, total quality management, improving plant safety and debottlenecking by capital projects and process improvements.

Objectives of the project

Overall objective was to improve the plant reliability by effective maintenance techniques and implementation of ISO 9000 for group of plants whose products were in high export demand. Due the old age of plants the reliability was the main concern and improvement of that could only possible by effective maintenance practices.

Nature of my particular work area

My particular work area was to implement the innovative maintenance practices, implementation of ISO 9000, debottlenecking by process improvement and capacity expansion of existing plants and all of these with prime safety in to the main consideration. As a management trainee I was trained in different departments of company like maintenance, manufacturing, operations, marketing, finance, materials, logistics, Import/export, IT and others.

After completion of my training period I was placed in mechanical maintenance department was responsible for maintenance of my zone of process plants. I have implemented preventive and proactive maintenance practices and prepared SOP and SWP for operation and maintenance.

Organisational structure

Atul Limited

Atul Chemical Complex, India

(Shown up to 4 levels for main function areas only)

MANAGING DIRECTOR

S Lalbhai

SHE & QA MANAGER

S Nagia

ADMIN MANAGER

Linda Murphy

VICE PRESIDENT

N S Daga

GM MANUFACTURING

P Krishna

GM MARKETING

S L Nayak

GM PROJECT/MAINTENANCE

MN Desai

GM MATERIALS

K P Naik

SR.MGR PROCESS

B S Patel

SR. MGR MKTG

P C Shah

SR.MGR PROJ/MAINTENANCE

H Bomi

SR. MGR PROCUREMENT

P Rana

ELEC ENGG OFFICER

R Raju

ASST MGR PROJ/MAINT

Sanjiv Parekh

ASST MGR MATERIALS

S Mohanan

ASST MGR PROCESS

AH Gandhi

MANAGEMENT TRAINEE

T Raman

MAINTENANCE ENGR

P T Panchal

ELEC DRAFTER

B Modi

Statement of your duties

Please refer to my Curriculum Vitae for statement of duties under "Responsibility".

### 4.1.3 Personal Workplace Activity

During the course of employment I came across different technical problems and I have resolved them in innovative way. I have suggested process and maintenance improvement programs and have implemented them successfully.

A heat exchanger in distillation unit in beta-naphthol plant was the heart of process for manufacturing the beta-naphthol. Frequent breakdown in the heat exchanger had not only created down time but also resulted in production loss along with the variation in desired quality of product. The leakage from the shell and tube type heat exchanger resulted in mixing of thermic fluid (Hytherm 500) in to distilled product of beta-naphthol. In another unit where we were using (Santotherm 66) for heating fluid , it has created loss of valuable imported thermic fluids.

I have studied the problem in detail and gathered all past historical data about the failure of heat exchanger and plotted a logarithmic curve for failure analysis. The meantime between failure (MTBF) was more. As a result I have found that it was because of the wrong selection of material for the tube of the heat exchanger.

I have collected the fluid properties, which are flowing through tubes and shells and from the corrosion guide available for ASTM better material selection I have selected SS 304(stainless steel) for material of existing MS- (mild steel) tubes. The reason behind the selection was to improve life of the tubes with better corrosion and erosion properties.

I have made corrosion test samples for both material and have carried out laboratory as well as filed test and found SS 304 very much suitable. Even SS 316 was better but I had to think about the cost factor also and that was the reason, why I have kept the shell as it is with MS construction. There was no significant corrosion from shell side due to non-corrosive properties of Hytherm 500 thermic fluid.

Next step was to design the heat exchanger with the combination of the material SS tubes with MS shell. I have selected the correct welding electrodes for MS to SS welding for Eutectic welding electrodes manual and have prepared a procedure for heat treatment from ASME section VIII. I monitored the whole job and as outcome a long life heat exchanger was developed for uninterrupted continuous service to process plant.

Another special assignment was given to me for fusion pots used at Fusion process of SBS (Sodium Beta Sulphonic Acid) fusion in to alpha and beta-naphthol. Failure of the cast Iron(CI) pots  in fusion process has created major losses in production and heavy loses due to immature failures with cracking in casting.

Rather then focusing on the CI fusion pots I have concentrated to the complete system to analyze the failure and to develop permanent solution. I have studied the frequency of failure and observed the operating parameters of the total system.

My findings for the reasons of the failures were

- Poor operation and system control

Due to poor the operation and control of process parameters, fusion pots are getting thermal shocks and which lead immature cracking of CI pots.

- Quality of casting for fusion pot

As the quality of casting was not good to cope up the unexpected changes in process parameters, it had tendency to crack frequently with out having any indications for failure.

- Incomplete combustion in furnaces to heat the CI fusion pot

Operation of furnace was not controlled and monitored which made the fusion pot failure as routine

I have taken following engineering steps to resolve the issue.

- As the operation parameters included the feeding rate of SBS, moisture content in SBS, temperature of furnace, temperature of pot, heating and cooling cycle, batch size etc. which have greatest effect on the fusion pot life. I have made provision with modified feeder for uniform SBS feeding to pots. I have included one more stage in the process for SBS drying with electric ovens and increasing centrifuging cycle. The heating rate, temperatures of pots as well as furnaces was reset so that the thermal shocks would be minimum. Batch size of the fusion batch is increased and that way I have reduced the uneven corrosion rate for CI pot. More thermal probes installed in furnaces and CI pots for accurate temperature monitoring

Schematic Diagram of Fusion Pot System- Beta Naphthol Plant

(only part of system shown in diagram)

Chimney

Vent

Gearbox & Drive

SBS Feeder

Burner Block

Burner

Air Blower

- Quality of the casting was improved with my suggestion of added Cr- Chromium and Ni- Nickel % in casting. Internals are re-casted with more Si- Silicon to harden the casting and increase corrosion erosion resistant to avoid frequent failures. Even I have changed practice of throwing the fusion pots in scarp after cracking in the bottom. I have develop a unique repairing practices for CI welding with proper heat treatment, which was rewarded me promotion to Assistant Manager position in organisation. I have applied the innovative repair practices to numbers of fusion pots and have recovered more than 100 bathes for each pot.

- Incomplete combustion in furnaces had created the uneven heating of fusion pots from bottom, which was the other reason for failure. I have studied the furnaces and developed a new contour for the burner block as well as for flue gas path within the furnace. The new casting with more silicon in burner has given more life of burner and the redirection of the flue gas with modified path has given uniform heating of CI fusion pots.

- I have installed a stack monitoring system, which was measuring the Sox, Nox as well as carbon, sulfur and oxygen % in the flue gas at outlet of the furnace. This has help to control the complete combustion process. I have also installed the PLC base control panel to monitor the total process, which was finally merged with the newly installed DCS panel for the total pant controls.

I have developed a Cyclone Separator System to avoid spray of alkaline mass through vent of the acidifier reaction vessels. I have received an award from safety division- Atul Limited for that innovative development.

During the acidification process, alkaline mass was coming out of the vent of the acidifier reaction vessel, which has created a hazardous environment and unsafe working condition at process plant. I have designed a cyclone separator to separate out the liquid alkaline mass from the process fumes vent. I have made provision of the heated steam tube to avoid solidification of mass and have provided sprayers, which can dissolve the alkaline fumes and create clear vent gas, which is safe to dispose off.

I was also responsible for the preparation of documentation and implementation of ISO 9000 for our maintenance department. I have worked with the expert from TUV and BVQI for ISO 9000 procedure preparation for each of the maintenance activities like for breakdown maintenance, preventive and proactive maintenance, condition monitoring, shutdown planning, inspection and report, safety in operation and maintenance and other issues related to maintenance practices.

As I was sponsored for Safety Diploma course I have conducted the HAZOP (HAZard and OPerability) Study for the new expansion of BON (Beta Oxy Naphthoic Acid) acid plant. I was the HAZOP leader as safety coordinator/capital project engineer and in coordination with Plant operation, maintenance, quality controls, engineering and project department, I have successfully completed the HAZOP study for the capacity expansion project.

I was project engineer for the revamping of ETP (Effluent Treatment Plant) and that time I was in interaction with local people, government and statutory authority for final approval of ETP proposal. I have explained them for the potential risks, hazards and consequences, if we would not take the project as serious matter.

I have published outcome of this analysis in report for heat exchangers, fusion pots, cyclone separator and corrosion controls, ISO9000 maintenance procedures and HAZOP study report in process industries and submitted to our central technical library for future use and reference to anybody in our organisation.

Cyclone Separator System

Vent to Atmosphere

Acidifier Reaction Vessel

Sprayer

Cyclone Separator

Heating Coil

Drain

Catch Pot

### 4.1.4 Summary

Overall I have gain experience in Maintenance, Expansion Projects and Safety in Process Plants. Major knowledge gained in process plants, systems, equipments. I have contributed by improved process plant reliability and plant efficiency achieved by selection of better material of construction, implementing preventive/proactive maintenance practices, condition monitoring, failure analysis and improving plant safety.

As a result of my active participation to achieve the common goal of company, I was rewarded with frequent grade changes from Management Trainee to Assistant Manager with generous remuneration rises. I was sponsored for the Post Diploma in Industrial Safety (PDIS) course and ranked second in centre.