ANEXO 1: Parâmetros SPICE dos Transistores Bipolares

(Modelo de Gummel-Poon Modificado)

|  | Símbolo |  |  | Significado do parâmetro |  | Unidade |  | Default |  | Exemplo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | IS | transport saturation current |  | A |  | 1.0E-1 A |  | 1.0E-15 |  |
| 2 |  | BF | ideal maximum forward beta |  | - |  | 100 |  | 100 |  |
| 3 |  | NF | forward current emission coefficient |  | - |  | 1. |  | 1 |  |
| 4 |  | VAF | forward Early voltage |  | V |  | infinite |  | 200 |  |
| 5 |  | IKF | corner for forward beta high-current roll-off |  | A |  | infinite |  | 0.01 |  |
| 6 |  | ISE | Base-Emitter leakage saturation current |  | A |  | 0 |  | 1.0E-13 |  |
| 7 |  | NE | Base-Emitter leakage emission coefficient |  | - |  | 1.5 |  | 2 |  |
| 8 |  | BR | ideal maximum reverse beta |  | - |  | 1 |  | 0.1 |  |
| 9 |  | NR | reverse current emission coefficient |  | - |  | 1 |  | 1 |  |
| 10 |  | VAR | reverse Early voltage |  | V |  | infinite |  | 200 |  |
| 11 |  | IKR | corner for reverse beta high-current roll-off |  | A |  | infinite |  | 0.01 |  |
| 12 |  | ISC | Base-Collector leakage saturation current |  | A |  | 0 |  | 1.0E-13 |  |
| 13 |  | NC | Base-Collector leakage emission coefficient |  | - |  | 2 |  | 1.5 |  |
| 14 |  | RB | zero bias base resistance (maximum) |  | W |  | 0 |  | 100 |  |
| 15 |  | IRB | current where base resistance falls halfway to its minimum value |  | A |  | infinite |  | 0.1 |  |
| 16 |  | RBM | minimum base resistance at high currents |  | W |  | RB |  | 10 |  |
| 17 |  | RE | emitter ohmic resistance |  | W |  | 0 |  | 1 |  |
| 18 |  | RC | collector ohmic resistance |  | W |  | 0 |  | 10 |  |
| 19 |  | CJE | Base-Emitter zero-bias depletion capacitance |  | F |  | 0 |  | 2 p |  |
| 20 |  | VJE | Base-Emitter built-in potential |  | V |  | 0.75 V |  | 0.6 |  |
| 21 |  | MJE | Base-Emitter junction exponential factor |  | - |  | 0.33 |  | 0.33 |  |
| 22 |  | TF | ideal forward transit time |  | s |  | 0 |  | 0.1 n |  |
| 23 |  | XTF | coefficient for bias dependance of TF |  | - |  | 0 |  |  |  |
| 24 |  | VTF | voltage describing VBC dependence of TF |  | V |  | infinite |  |  |  |
| 25 |  | ITF | high-current parameter for effect on TF |  | A |  | 0 |  |  |  |
| 26 |  | PTF | excess phase at freq=1.0/(TF*2p) Hz |  | deg |  | 0 |  |  |  |
| 27 |  | CJC | Base-Collector zero-bias depletion capacitance |  | F |  | 0 |  | 2 p |  |
| 28 |  | VJC | Base-Collector built-in potential |  | V |  | 0.75 V |  | 0.5 |  |
| 29 |  | MJC | Base-Collector junction exponential factor |  | - |  | 0.33 |  | 0.5 |  |
| 30 |  | XCJC | fraction of Base-Collector depletion capacitance connected to internal base node |  | - |  | 1 |  |  |  |
| 31 |  | TR | ideal reverse transit time |  | s |  | 0 |  | 10 n |  |
| 32 |  | CJS | zero-bias collector-substrate capacitance |  | F |  | 0 |  | 2 p |  |
| 33 |  | VJS | substrate junction built-in potential |  | V |  | 0.75 |  |  |  |
| 34 |  | MJS | substrate junction exponential factor |  | - |  | 0 |  | 0.5 |  |
| 35 |  | XTB | forward and reverse beta temperature exponent |  | - |  | 0 |  |  |  |
| 36 |  | EG | energy gap for temperature effect on IS |  | eV |  | 1.11 |  |  |  |
| 37 |  | XTI | temperature exponent or effect on IS |  | - |  | 3 |  |  |  |
| 38 |  | KF | flicker-noise coefficient |  | - |  | 0 |  |  |  |
| 39 |  | AF | flicker-noise exponent |  | - |  | 1 |  |  |  |
| 40 |  | FC | coefficient for forward-bias depletion capacitance formula |  | - |  | 0.5 |  |  |  |