TRANSEUROPA F-27 "RIO DUERO"

Designers:  Tom Gibson/Harry Follas

Textures: Harry Follas

FDE: Brian Horsey

Repainted in Transeuropa livery by: Antonio Morales

ame@sinix.net

http://www.smartec.es/aeronaves/

El Fokker 27 EC-BOA llega a España y se incorpora a la flota de Iberia en Enero de 1.968. Este avión que rpresentamos fué bautizado como "Rio Duero", ya que todos los F-27 de Iberia tenían nombres los nombres de los principales rios españoles, el EC-BOA conservó este nombre durante toda su vida en España.

En 1.982 es vendido a Aviaco, donde continua prestando servicio hasta su retirada  y posterior venta a Cuba en 1.992.

*****************************************************************

Flying the F27:(Brian Magby First Officer F27 Eagle Jet Charter.)

(From a Sigfrid Roig File)

*****************************************************************

F27 Speed limitations:

Max Gear Extension Speed-168kts.

Max Flap Extension Speed-144kts.

Never Exceed Speed-254kts.

Take Off:

1)Apply full power and accelerate to 109kts.

2)Rotate to 10 degrees pitch up.

3)Maintain 10 degrees pitch, and climb through 400 feet AGL.

4)Once through 400 feet AGL, reduce power to 14,000 rpm, and 700 tgt.

5)Maintain 10 degrees pitch and climb to level off.

6)Let the plane accelerate, and leave power set where it is for cruise.

(In the real aircraft, once through 400 ft AGL, the power is set to 14,200 rpm and 755 tgt. Once leveled off for cruise, the power is left at 14,200 rpm and the tgts are reduced to 730 degrees. There is no way that I know to do this in FS98. So just use the above mentioned settings of 14,000 rpm and 700 tgt.)

The F27 should climb at about 1,100 ft/min at 150kts and 10 degree's pitch up.

It should cruise at about 190kts indicated airspeed at the above mentioned power setting.

this is extremely close to the actual performance of the F27 loaded to about 40,000 lbs.

Landing: (ILS)

1)Five miles from the final approach fix (outer marker) you should slow to 140kts.

2)One mile from the final approach fix you should set flaps 16, and slow to 130kts.

3)One and one half dot's below glide slope select gear down.

4)One half dot below glide slope select flaps 26 and adjust power to fly the glide slope at 120kts.

5)Once airport is in site, and you are below 400ft. select flaps 40 reduce power and land.

************************************

LEGAL STUFF

************************************

THIS AIRCRAFT IS FREEWARE- DO NOT MAKE ANY MONEY FROM THIS PROJECT. I´LL NOT ACCEPT ANY RESPONSIBILITY FOR CONSEQUENTIAL INCIDENTAL OR ANY SYSTEM DAMAGE DUE TO THE FAILURE OR IMPROPER OPERATION OF THE SOFTWARE. NOR IS ANY WARRANTY EXPRESSED OR IMPLIED FOR ANY CIRCUMSTANCE. ALSO, THE ABOVE MENTIONED CAN IN NO WAY BE LIABLE FOR ANY PROBLEMS THAT THE FILES MAY CAUSE.

Comments about this aircraft are welcome.

ANTONIO MORALES

Madrid (Spain)

ame@sinix.net

http://www.smartec.es/aeronaves/