# Flow Chart

Main(void)

| Define the register address of the display Define 1S and counter Initialize PIT1 Mode 0, submode2 PA1,PA0, PB0…PB7 as output, PA7 as input Initialize Port A, Port B as 0x00 Initialize PIT2 Mode 0, submode 2 Initialize graphics display Initialize interrupt handler Set 1S timer in PIT2 Set downcounter in PIT1 |  |  |
| --- | --- | --- |
|  | Clear graphics display |  |
|  | For (j=0;j<=3839;j++) |  |
| Switch off the display Draw a circle of analog part on the display Switch on the display |  |  |
|  | Reset timer of PIT1 Timer enable Intflag=0 |  |
|  |  | While intflag=0 |
|  | Timer halt Read the time from the counter of PIT1 timer Caculate the speed and get the direction of wheel circumference reset the timer status register Display the direction and speed |  |
|  | Repeat forever |  |

Interrupt void inthndl(void)

Set intflag=1 Reset the timer status register

Unsigned int get_cntr(void)

Get counter number Return counter number

Unsigned int direction(void)

| Initialize handshake H12 H34 enable, active high, rising edge H2 is output, low H2 is output, low |  |  |  |
| --- | --- | --- | --- |
| Repeat until both(H4S and H1S) signals low |  |  |  |
| Clear H4S and H1S |  |  |  |
|  | Get the information of H4S and H1S |  |  |
| Repeat if both of H4S and H1S are 0 |  |  |  |
| Repeat if both of H4S and H1S are not 1 |  |  |  |
| If bit H1S euqal 0 |  |  |  |
| Y |  |  | N |
| Set H4 Clear h2 Direction is forward |  |  | Set H2 Clear H4 Direction is backward |

unsigned short convertXYtoAdr(unsigned char x, unsigned char y,unsigned char* value)

Calculate the address by the given coordinate X, Y Get the precise position of known address Return address and position

drawCircle(unsigned char x,unsigned char y)

|  | Calcuate every precise coordinate of the circle Transfer it into address mode Write address into the memory of the display |
| --- | --- |
|  | For(i=0;I<360;I++) |

void doClear(int speedBefore,unsigned char x,unsigned char y)

| Calculate the angle of the hand by the value of last speed |  |
| --- | --- |
|  | Calculate the coordinate of every point of the hand by the angle Write them into the memory of the display |
|  | Repeat 20 times |

void setHand(int speed,unsigned char x,unsigned char y)

| Calculate the angle of the hand by the value of the current speed |  |
| --- | --- |
|  | Calculate the coordinate of every point of the hand by the angle Write them into the memory of the display |
|  | Repeat 20 times |

unsigned short setStroke_H(unsigned short adr)

Write address into the memory of the display Draw 6 points every row, and 2 rows as horizontal line

unsigned short setStroke_V(unsigned short adr,unsigned char side)

| Get side from parameter |  |  |  |
| --- | --- | --- | --- |
| If side equal “l” |  | If side equal “l” | Else |
| Draw the vertical line in the left side |  | Draw the vertical line in the right side | Draw the vertical line in the middle |
|  | Write the address into the memory of the display Turn to next line |  |  |
| Repeat 4 times |  |  |  |
| Return adress |  |  |  |

void setNine(unsigned char x, unsigned char y)

void setEight(unsigned char x, unsigned char y)

void setSeven(unsigned char x, unsigned char y)

void setSix(unsigned char x, unsigned char y)

void setFive(unsigned char x, unsigned char y)

void setFour(unsigned char x, unsigned char y)

void setThree(unsigned char x, unsigned char y)

void setTwo(unsigned char x, unsigned char y)

void setOne(unsigned char x, unsigned char y)

void setZero(unsigned char x, unsigned char y)

Convert coordinate into address Draw the digital by setStroke_H(adr) and setStroke_V(adr)

void display(int speed) /*converse the result and display*/

| Switch off the display |  |
| --- | --- |
| If the direction is forward |  |
| Y | N |
| Set forward into memory of the display | Set backward into the memory of the display |
| Calculate every bit of speed Write the decimal dot bit into the memory of the display Write the hundred bit into the memory of display Write the ten bit into the memory of display Write the one bit into the memory of display Write the decimal ten bit into the memory of display Write the decimal hundred bit into the memory of display Set the hand of analog part into the memory of the display Switch on the display |  |

Void lcd_datcmd(unsigned char adr_code, unsigned char value)

|  | Output of RS=1 and R/┐W=1 Output of E=1c Read busy flag Output of E=0 |
| --- | --- |
| Repeat until busy flag not equal 0 |  |
| Output of RS=1 and R/┐W=0 Output of E=1 Write address or code “adr_code” into command register Output of E=0 |  |
| Output of RS=0 and R/┐W=0 Output of E=1 Write “value” Output of E=0 |  |

# Project plan

The project should be finished in 4 labs(5 weeks).

1st lab: The project was designed, and was divided into several parts, every member of the group got the different part of the project, and begin to implement some fundmental parts.

2nd lab: Most source code of the project was implemented, and in the labortory it was tested in the first step. At first it could get the speed and the direction of the wheel circle, it was proved by its print on the minitor. The display part of the project began to be tested and be debug.

3rd lab: The complete project was tested, and it could run steadily without bug, and the group discussed the bias of the project.

4th lab: The report was finished with project plan, concept description, source code and anlysis of the results.

# Assignment of work

Our group members are Rong Zhang, Joseph Man-kit Chung and Bin Fu.

Responsibility of Rong Zhang:

Get the speed and direction of the wheel by interrupt handler method, and transfer them as parameter to the display part. She is responsible for the main() and intialization parts.

Responsibility of Bin Fu:

Write the functions for accessing the memory registers of the display, and display function, which determines the layout of every part on the display.

Responsibility of Joseph Man-kit Chung:

Most of graphical design, and implement them into several functions which can draw the analog circle, digits, decimal dot, hand of the analog meter, and so on.

## Concept description

- Get the speed and the direction of the revolving wheel

there are two hole on the revolving disc, and the disc is scanned optoelectronically and provides two short active high impulses of the signals S1 and S2 at each rotation. Then in the program the PIT2’s timer was used, which can call a interrupt handler every 1 second, and the PIT1’s timer was used as a down counter, which can record the time of the rotation in this 1 second. The circumference of the wheel was assumed as 1 meter, then the speed of the wheel can be calculated by the times of the rotation in 1 second. The direction can also be determined by two times’ short active high impulses of the holes.

- Access the registers of the graphics display

The following actions habe to be repeated on and on for control of the graphics display:

- Read accesses as long as the busy flag is not zero

- One Write access for storing the address of a data register or a command code into the command register

- One further write access with the contents of the data register or the command parameter.

Therefor the function lcd_datcmd(unsigned char adr_code, unsigned char value) was implemented, and every time when it needs write the order into the register of the display, just call this function.

- The basic thinking of the graphics design

The graphics display mainly consists of a display unit with 240*128 pixels, a LC display controller and a video RAM. All the pixels are divided into address code by every 8 pixels as following figure 1:

8 pixels

| 0 | 1 | 2 |  | ……. |  |  | 28 | 29 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 31 | 32 |  | ……. |  |  |  | 59 |  |
| .. | .. | .. |  | ……. |  |  | .. | .. |  |
| .. | .. | .. |  | ……. |  |  |  |  |  |
| : : : : |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ……. |  |  |  |  |
|  |  |  |  |  | ……. | 3808 |  |  | 3809 |

Figure 1.

The result of the program is shown in two ways, one is digital type, the other is analogic type(something like a clock).

For the digital part every digit can be seen as the combination of many point. It has two possibility, one is horizontal, the other is vertical, and in the program they were implemented as function setStroke_H and setStroke_V correspondingly. All the digits can be combined by both function. Function setOne(), setTwo() and so on are implemented fundmentally by these both function.

For the analogic part, the circle with scale is printed on the display once, and it is done by a circle drawing function called drawCircle(). In the program only the hand move, that is implemented by a function called setHand(), so every time it needs a clear function called doClear() in the program to clear the last state of the  hand. All these functions need the formular of the circle, which can calculate the exactly coordinate of every dot of the circle.

To display them, the program needs a display function called display(), which determine the layout of every part. Every time before the program accesses the register of the display, it switch off the display, then after that switch it on.

The layout on the display is something like following figure 2:

| Analog Part (like a clock) | Direction (F / B) |
| --- | --- |
|  | Speed (Km/H) |

Figure 2.

### Assessment and discussion of the results

1. Result analysis

A minitor was used to show the results got from the program, and compared it to the results   shown on the graphics display, we found that both results were completely same. It means that the graphics display can display the results, which are transferred by the VME bus system from the rotating wheel correctly.  Every second the program deals with an interrupt request, and it calls the display function to refresh the current rusults on the graphics display, in the same word every current results are stored and displayed for 1 second.

2. The flaw of the project

Our group’s thinking on getting the speed and the direction from the running wheel is that, every second, which is ensured by the timer counter of PIT2, it calls an interrupt request, so the results are refreshed every second. And the results are always changed even although the speed of the running wheel isn’t modified, because every second the wheel sensor can’t record the precise number of the rotation, and it depends on how far the wheel rotates, it is impossible every second the number of the rotation is exactly an integer number. For a better way, if any high impulse of S1 or S2 is detected, an interrupt handler can be called, then records the duration time of every rotation of the wheel. If so, the results on the display will be refreshed about every 1/40 seconds (it depends on the speed of the wheel), the display should be more practical and precise.

3. The experiences of this project

i. Considerate the structure of the complete project. At first it should be designed on the paper(not to be implemented at once), define all the functions and their relationships, in order to prevent the redundant source code.

ii. Pay attention enough to the correct data type. Like in this project, the address code was defined as “unsigned short” but not “unsigned char”, because “unsigned char” can only hold 0-256, but the range of the address code of the display is 0-3809.

iii. Cooperation of the group member. The group members should always communicate well in order to make it easy to intergrate the project at last.