Grundig Satellit 700 RDS test mode

PTY codes used in North America

No      Prog type

0           No programme type/undefined

1           News

2           Information

3           Sports

4           Talk

5           Rock

6           Classic Rock

7           Adult Hits

8           Soft Rock

9           Top 40

10         Country

11         Oldies

12         Soft

13         Nostalgia

14         Jazz

15         Classical

16         Rhythm & Blues

17         Soft Rhythm & Blues

18         Language

19         Religious Music

20         Religious Talk

21         Personality

22         Public

23         College

24         Jazz Music

25        Unassigned

26        Unassigned

27        Unassigned

28        Unassigned

29         Weather

30         Emergency Test

31         Emergency

To activate, select FM, & press “SYNCH” key for 3 seconds. To deactivate, repeat the above. Also automatically deactivates when set is turned off. In test mode, the tuning meter becomes an RDS error bar graph in all pages, function as described in instruction book.

When tuned to an RDS station, select required page, in any order, as follows:

Page 1: Press”SYNCH” briefly. Displays PI code, XNnn format,

X = country code, (eg, UK=C, Eire=2)

N = area code, 0 = Local (single transmitter), 1 = International, (Same programme may be carried in other countries), 2 = National (programme should be available over the whole country), 3 = Supra-regional (covering more than one transmitter site), 4 to F = regional (This “N” list may not be true for America)

nn = unique station identifier.

&

TA/TP flag, n n format:

1st position = traffic announcement, 1 = active. 2nd position = traffic programme, (part of EON TA group) 1 = yes.

Page 2: Press “BANDWIDTH” key.

% Error rates will be displayed. 1st position is instantaneous, 2nd position is max error since last complete block received. If bargraph is at full deflection, both numbers should be zero.

Page 3: Press “USB” which gives PS-name, as in normal mode. Press again for PTY code, number in nn format, as list on right,

& M/S indicator, (Music or Speech). Should be M if broadcaster is not using this indicator actively.

Page 4: Full non-discriminatory AF function, press “LSB”, short press for higher frequencies, longer press for lower frequencies. Unlike normal AF function, this will hop sequentially to the next possible AF frequency stored for the tuned frequency at each push of button. Note that some of these frequencies may be silent or have other stations on them, as they may be frequencies for a distant part of the coverage area. Using the FM-RDS AF button, as in normal mode, will only pick frequencies above a certain signal strength.

Note: this list applies in North America, where the RDS PTY codes are regulated by the National Association of Broadcasters / National Radio Systems Committee.

Published by www.geocities.com/grundigradioboy/