TYPES OF MULTIPLEXING SYSTEMS

PREPARED BY

HEMAKUMAR.B

MULTIPLEXING

INTRODUCTION:

To transmit a number of signals over the same channel the signals must be kept apart so that they do not interfere with each other and thus they can be separated at receiving end. This technique is called multiplexing.

Multiplexing is the sending of number of separate signals over the same channel simultaneously without interference. This is accomplished by separating the signal either in frequency or in time.

The technique separating the signal in frequency is referred Frequency Division Multiplexing (FDM), whereas technique separating the signal in time is Time Division Multiplexing (TDM).

TIME DIVISION MULTIPLEXING:

The block diagram shown below illustrates the concept of TDM.

Each input message (MI) signal is first restricted in bandwidth by a low pass filter, to remove the frequencies that are not essential. The LPF outputs are applied to a commutator, which is usually implemented using electronic switching circuitry. The function of the commutator is two-fold:

- To take a narrow sample of each of the n input messages at a rate fs i.e. slightly higher than 2fa (sampling theorem), where fa is the maximum frequency component or cutoff frequency of the filter.

- To sequentially interleave this ‘n’ samples in the sampling interval Ts.

Following the commutator process multiplexed output is applied to a pulse

modulator. The purpose of which is to transform the multiplexed signal into a form suitable for transmission over the communication channel. At the receiving end of the system the received signal is applied to a pulse demodulator, which performs the reverse operation of a pulse modulator. The narrow samples produced at pulse demodulator output are distributed through the appropriate low pass reconstruction filter by means of decommutator, which operates in synchronization to commutator & transmitter. This synchronization is essential for satisfactory operation of the system.

The TDM system is highly sensitive to dispersion in the common channel i.e. to

variation of amplitude with frequency or lack of proportionality of space with frequency.

FREQUENCY DIVISION MULTIPLEXING:

The block diagram shown below illustrates the concept of FDM.

The incoming message signals are assumed to be of low pass filter type, which are applied to LPF. The low pass filters are used to remove the high frequencies components, that are not essential. The filtered signal are applied to modulators (MOD) that shift the frequency range of the signals so as to occupy mutually exclusive frequency intervals. The necessary carrier frequencies needed to perform this frequency translation are obtained from a carrier supply.

The most widely used method of modulation in FDM is Single Sideband Modulation (SSM). The band pass filters (BPF) following the modulators are used to restrict the band of each modulated wave to its prescribed range. The resulting band pass filter outputs are next combined in parallel to form the input to the common channel. At the receiving end, a band of BPF’s with their inputs connected in parallel is used to separate the message signals on a frequency basis. Finally the originally message signal are recovered by individual demodulators (DEM).

Examples of data usually sent by FDM systems are vibration, acceleration and dynamic strain.

REFERENCE:

“COMMUNICATION SYSTEMS” by Simon Haykin.