Writing  Neuron C Source Code

You will develop applications with the NodeBuilder tool using the Neuron C programming language.

Double click the “Example1.nc” on the left pane to create a new source file.

Paste the following program to the Nodebuilder.

#pragma num_alias_table_entries 	14

IO_0 output bit ioLED1;

IO_1 output frequency clock(5) ioSpeaker = 0;

IO_2 output bit ioLED3;

IO_4 input bit ioButton1;

IO_5 input bit ioButton2;

IO_6 input bit ioButton3;

when (io_changes(ioButton1))

{

io_out(ioLED1,!input_value);

}

when (io_changes(ioButton2))

{

if (!input_value)

{

io_out(ioSpeaker,200);

}

else

{

io_out(ioSpeaker,0);

}

}

when (io_changes(ioButton3))

{

io_out(ioLED3,!input_value);

}

Open the File menu then click Save All to save all your changes to the source files.

Program explanation

#pragma num_alias_table_entries 	14

Use to control the number of alias table entries. Normally set to 14.

IO_0 output bit ioLED1;

IO_1 output frequency clock(5) ioSpeaker = 0;

IO_2 output bit ioLED3;

IO_4 input bit ioButton1;

IO_5 input bit ioButton2;

IO_6 input bit ioButton3;

Use to define the function of IO pins of the device.

You can refer to Chapter “I/O objects” (page 191) of Neuron C Reference manual. There are more than 30 I/O type for neuron chip, which is very useful.

We have used:

Bit Input/Output object (p.193 of NC Ref.) for LED and Button.

Frequency Output object (page 205 of NC Ref.) for the speacker.

when (io_changes(ioButton1))

{

io_out(ioLED1,!input_value);

}

This procedure will be executed when the input “ioButton1” change to 0 or 1.

The “io_out” function will send the data to the “ioLED1” to control the LED on or off. “input_value” is the current value of the “ioButton1”. The “!” at the beginning means NOT. You can try to delete the “!” later to see what happen.

when (io_changes(ioButton2))

{

if (!input_value)

{

io_out(ioSpeaker,200);

}

else

{

io_out(ioSpeaker,0);

}

}

This procedure will generate the frequency to the speaker. You can refer to the Frequency Output object (page 205 of NC Ref.) for detail.

io_out(ioSpeaker,200); will generate a signal with period of 200*12.8us = 2.56ms. You can find the table in the NC Ref. for the clock is 5.

io_out(ioSpeaker,0); will stop to generate a frequency.