Name: Tam Kin Cheung

Student ID#: 51075869

Lab Section: L04

Lab07: Transient Response of First-order RC and RL Circuit

Introduction:

This experiment is about the transient response of first-order RC and RL subject. Firstly, the response to an applied voltage pulse can be observed on the oscilloscope. Secondly, the charging and discharging cycle of the circuit is studied. By comparing experimental and theoretical results, it should be very similar to each other.

Procedure and Data:

Experiment 1: To study the transient response of RC circuits

a. Procedure

At the beginning, a circuit should be connected as Figure 1 using resistor and capacitor. The inputVinand outputVoutwould be connected to oscilloscope and can be observed. 1kHz frequency square wave is applied on the circuit and record the response of the input and output. Then, frequency is changed from 1kHz tofmax. The setting of oscilloscope should be adjusted to show a cycle of the response, and record the output under 1RC, 2RC, 3RC, 4RC and 5RC times. A comparison can be concluded between the experimental results and theoretical values.

b. Data and Result

| Nominal value | Measured value |
| --- | --- |
| 1 kΩ | 0.974 kHz |
| 0.01μF | 10.57 nF |
| 10 mH | 10.15 mH |

Table 1  Measured values of components used in circuit

Time constant (τ) = RC = 0.974k × 10.57 nF = 10.295μ

Charging time = 5RC = 5 × 10.295μ = 51.475μs

Maximum square-wave frequency (fmax) =12T=110RC= 9.713 kHz

Frequency = 1 kHz, amplitude = 1V

Timebase = 20μs, Volt/Div = 0.5V

input

output

Figure 2 Waveform of input

Figure 3 Waveform of output

Frequency =fmax=9.713 kHz, amplitude = 1V

Timebase = 20μs, Volt/Div = 0.5V

output

input

Figure 4 Waveform of input

Figure 5 Waveform of output

Frequency =12τ=12×10.295μ= 48.567 kHz, amplitude = 1V

Timebase = 10μs, Volt/Div = 0.5V

output

Figure 7 Waveform of output

input

Figure 6 Waveform of input

Pulse width input = 9μs, Pulse width output = 9.5μs

＝> Pulse width ≈ Time constant

Frequency = 200 kHz, amplitude = 1V

Timebase = 2μs, Volt/Div = 0.2V

input

Figure 8 Waveform of input

output

Figure 9 Waveform of output

f↑=> output changed to saw tooth-form

| Time | Calculated values |  | Measured values |  |
| --- | --- | --- | --- | --- |
|  | Charging | Discharging | Charging | Discharging |
|  | Vout (V) |  |  |  |
| 1RC | 0.632 | 0.558 | 0.5 | 0.4 |
| 2RC | 0.865 | 0.205 | 0.6 | 0.15 |
| 3RC | 0.950 | 0.075 | 0.8 | 0.04 |
| 4RC | 0.982 | 0.028 | 0.9 | 0.02 |
| 5RC | 0.993 | 0.010 | 0.95 | ≈ 0 |

Table 2  Data of output at different time constants

Frequency =fmax=9.713 kHz, amplitude = 1V

Timebase = 10μs, Volt/Div = 0.5V

Figure 11 Outputs at different time constant

Experiment 2: To study the transient response of RL circuits

a. Procedure

At the beginning, a circuit should be connected as Figure 12 using resistor and inductor. The inputVinand outputVoutwould be connected to oscilloscope and can be observed. 1kHz frequency square wave is applied on the circuit and record the response of the input and output. Then, frequency is changed from 1kHz tofmax. The setting of oscilloscope should be adjusted to show a cycle of the response, and record the output under 1RC, 2RC, 3RC, 4RC and 5RC times. A comparison can be concluded between the experimental results and theoretical values.

b. Data and Result

Time constant (τ) =LR=10.15m0.947k= 10.718μ

Charging time = 5LR= 5 × 10.718μ = 53.59μs

Maximum square-wave frequency (fmax) =12T=R10L= 9.33 kHz

Frequency = 1 kHz, amplitude = 1V

Timebase = 0.2μs, Volt/Div = 0.5V

input

Figure 13 Waveform of input

output

Figure 14 Waveform of output

Frequency =fmax= 9.33 kHz, amplitude = 1V

input

Figure 15 Waveform of input

output

Figure 16 Waveform of output

Timebase = 5μs, Volt/Div = 0.5V

Frequency =12τ=12×10.718μ= 46.65 kHz, amplitude = 1V

Timebase = 0.2μs, Volt/Div = 0.5V

input

Figure 17 Waveform of input

output

Figure 18 Waveform of output

Pulse width input = 8μs, Pulse width output = 8.5μs

Frequency = 200 kHz, amplitude = 1V

Timebase = 1μs, Volt/Div = 0.5V

input

Figure 19 Waveform of input

output

Figure 20 Waveform of output

| Time | Calculated values |  | Measured values |  |
| --- | --- | --- | --- | --- |
|  | Charging | Discharging | Charging | Discharging |
|  | Vout (V) |  |  |  |
| 1RC | 0.3679 | -0.408 | 0.34 | -0.36 |
| 2RC | 0.1353 | -0.15 | 0.1 | -0.14 |
| 3RC | 0.0498 | -0.026 | 0.04 | -0.06 |
| 4RC | 0.0183 | -0.0203 | ≈ 0 | -0.04 |
| 5RC | 0.0067 | -0.007 | ≈ 0 | -0.02 |

Table 3  Data of output at different time constants

Frequency =fmax=9.33 kHz, amplitude = 1V

Timebase = 20μs, Volt/Div = 0.5V

Figure 21 Outputs at different time constant

Discussion:

- It is because oscilloscope can’t show exact value of voltage output. However, it would be similar to each other. Another reason is that the resistance of oscilloscope does not include in the calculation, so the value also would be affected.

- The problem is also found in the experiment 2, because the design of the circuit is basically same, except the capacitor replaced by inductor.

- If resistor interchanges the position of capacitor, the waveform of resistance will be different.

In charging time,

VS=VR+VC

VS=VR+VS(1−e−tτ)

VS=VR+VS−VSe−tτ

VR=VSe−tτ

In discharging time,

VS=VR+VC

VS=VR+VS(1−e−Tτ)e−(t−T)τ

VS=VR+VSe−(t−T)τ−VSe−tτ

VS=VR+VSe−tτ−VSeTτ−VSe−tτ

VR=VS−VSeTτ

VR=VS(1−eTτ)

Therefore, the waveform should be like this,

output

Figure 23 Waveform of output

input

Figure 22 Waveform of input

- If resistance interchanges the position of inductor, the waveform of resistance will be different.

In charging time,

VS=VR+VL

VS=VR+VSe−tτ

VR=VS(1−e−tτ)

In discharging time,

VS=VR+VL

VS=VR−VS(1−e−Tτ)e−(t−T)τ

VS=VR−VSe−(t−T)τ+VSe−tτ

VS=VR−VSe−tτ−VSeTτ+VSe−tτ

VR=VS(1−eTτ)

input

Figure 24 Waveform of input

output

Figure 25 Waveform of output

Therefore, the waveform should be like this,