Name: Tam Kin Cheung

Student ID#: 51075869

Lab Section: L04

Lab8: Phase and Impedance of Simple RC Filiters

Introduction:

This experiment is about properties of RC circuit. From voltage measurement, the reactance of a capacitor can be calculated in a series RC circuit. Using the data in the experiment also can draw impedance and voltage phasors. To study how frequency affect the output of series RC circuit.

Procedure and Data:

Experiment 1:

- Procedure

First, resistor and capacitor are measured by DMM before connected in series RC circuit. Second, a circuit is connected as Figure 1. A 500Hz sine wave is applied in the circuit, and the output of resistor and capacitor in oscilloscope can be observed. The current, the capacitor reactance and the total impedance of circuit also can be calculated by using data.	Third, different frequencies 1.0 kHz, 1.5kHz, 2.0kHz, 4.0kHz and 8.0kHz are applied in the same circuit and do the same steps as above. Forth, using DMM generate outputs of resistor and capacitor. Comparing two results, impedance and voltage phasors can be drawn.

- Data and Result

| Nominal value | Measured value |
| --- | --- |
| 6.8 kΩ | 6.76 kΩ |
| 0.01μF | 10.66 nF |

Table 1 Value of components using in circuit

f = 500 Hz

Volt/Div = 2V

Timebase = 0.5ms

VR

Figure 3 Output of resistor

VC

Figure 4 Output of capacitor

|  | DMM | CRO |
| --- | --- | --- |
| VR (r.m.s.) | 0.442 V | 0.424 V |
| VC (r.m.s.) | 1.950 V | 2.121 V |
| IR | 65μA | 62.39μA |
| Xc | 30 kΩ | 34 kΩ |
| Z | 30.77 kΩ | 34 kΩ |

Table 2 Value of components using in circuit

f = 1.0 Hz

Volt/Div = 2V

Timebase = 0.2ms

VR

Figure 5 Output of resistor

VC

Figure 6 Output of capacitor

|  | DMM | CRO |
| --- | --- | --- |
| VR (r.m.s.) | 0.849 V | 0.807 V |
| VC (r.m.s.) | 1.697 V | 1.738 V |
| IR | 128.85μA | 118.68μA |
| Xc | 6.8 kΩ | 6.8 kΩ |
| Z | 16.99 kΩ | 14.64 kΩ |

Table 3 Value of components using in circuit

f = 1.5 Hz

Volt/Div = 2V

Timebase = 0.2ms

VR

Figure 7 Output of resistor

VC

Figure 8 Output of capacitor

|  | DMM | CRO |
| --- | --- | --- |
| VR (r.m.s.) | 1.272 V | 1 V |
| VC (r.m.s.) | 1.697 V | 1.58 V |
| IR | 187.06μA | 147.06μA |
| Xc | 9.07 kΩ | 10.75 kΩ |
| Z | 11.34 kΩ | 14.42 kΩ |

Table 4 Value of components using in circuit

f = 2.0 Hz

Volt/Div = 2V

Timebase = 0.1ms

VR

Figure 9 Output of resistor

VC

Figure 10 Output of capacitor

|  | DMM | CRO |
| --- | --- | --- |
| VR (r.m.s.) | 1.202 V | 1.238 V |
| VC (r.m.s.) | 1.697 V | 1.395 V |
| IR | 176.76μA | 182.06μA |
| Xc | 9.6 kΩ | 7.66 kΩ |
| Z | 12.8 kΩ | 12.43 kΩ |

Table 5 Value of components using in circuit

f = 4.0 Hz

Volt/Div = 2V

Timebase = 50μs

VR

Figure 11 Output of resistor

VC

Figure 12 Output of capacitor

|  | DMM | CRO |
| --- | --- | --- |
| VR (r.m.s.) | 1.697 V | 1.774 V |
| VC (r.m.s.) | 0.849 V | 0.914 V |
| IR | 249.56μA | 260.88μA |
| Xc | 3.4 kΩ | 3.5 kΩ |
| Z | 9.634 kΩ | 9.216 kΩ |

Table 6 Value of components using in circuit

f = 8.0 Hz

Volt/Div = 2V

Timebase = 20μs

VR

Figure 13 Output of resistor

VC

Figure 14 Output of capacitor

|  | DMM | CRO |
| --- | --- | --- |
| VR (r.m.s.) | 1.838 V | 1.894 V |
| VC (r.m.s.) | 0.424 V | 0.367 V |
| IR | 270.29μA | 278.53μA |
| Xc | 1.567 kΩ | 1.318 kΩ |
| Z | 8.89 kΩ | 8.63 kΩ |

Table 7 Value of components using in circuit

| Phase angle | Frequency |  |
| --- | --- | --- |
|  | 1 kHz | 4 kHz |
| Impedance phase angle | 45° | 26.56° |
| Voltage phase angle | 63.42° | 26.58° |

Table 8 Phase angles of outputs at 1 kHz and 4 kHz

Impedance phasor

1 kHz

R = 6.76 kΩ

XC = 6.8 kΩ

θ=45°

Z = 16.99 kΩ

θ=26.56°

4 kHz

XC = 3.4 kΩ

R = 6.76 kΩ

Z = 9.643 kΩ

Voltage phasor

θ=26.58°

4 kHz

VC = 0.849 V

VR = 1.697 V

V = 2.404 V

θ=63.42°

1 kHz

VC = 1.697 V

VR = 0.849 V

V = 2.121 V

Voltage across capacitor: VC=ISXC → VC=IS2πC(1f)

# High  freq

# Low  freq

f -1/ Hz

VC / V

Figure 15 Voltage across capacitor

Voltage across resistor: VR=VS−VC

VC / V

f -1/ Hz

# High  freq

# Low  freq

Figure 15 Voltage across resistor

Discussion:

1. 	Z=R2+XC2	VS=VR2+VC2

| Frequency |  | Impedance (kΩ) | Supplied voltage (V) |
| --- | --- | --- | --- |
| 1 k Hz | CRO | 16.99 | 2.121 |
|  | Calculated | 9.59 | 1.898 |
| 4 k Hz | CRO | 9.643 | 2.404 |
|  | Calculated | 7.567 | 1.898 |

2. If low frequency is filtered out, the output is connected to resistor. It is shown on the figure 15.

3. When frequency is increased, the total impedance is decreased at the same time. The frequency is increased, so the reactance of capacitor is decreased. It provides that voltage across capacitor also be decreased. The phase angle is ∠θ=tan−1(VCVR)

It is resulted that the phase angle is decreased.

4. The capacitance is decreased ten times, it can cause the value of voltage output is increased. However, it should be no effect to frequency response.

5. DMM can measure a pure sine wave signal, but it usually can work under 10 kHz. However, if DMM give out a more accuracy value, the frequency should be under 500 Hz. Therefore, DMM also can be used in this experiment. If frequency is over 500 Hz, the output (r.m.s.) may be not true value, it consists some errors properly.