HONG KONG INSTITUTE OF VOCATIONAL EDUCATION

(HAKING WONG)

Department: Electrical Engineering

Module Title: Engineering Science

Module Code: MMI6205

Assignment 2

Name: Ko Siu Tai

Class: 01601T-G

Student No: 020058936

- A 8 kg block is suspended by two strings as shown
in the diagram. The strings are assumed inextensible. 
 
( a ) Calculate the weight of the block.
8 x 10 = 80N

( b ) Find the tensions along the strings.

| Four co-polar forces act at a point O, the values and directions of the forces being as shown in figure. Calculate the value and direction of resultant force. | F x | F y |
| --- | --- | --- |
| F 1 | 50 | 0 |
| F 2 | F 2cos60∘= 0.5 | F 2sin60∘= 0.87 |
| F 3 | F 3cos30∘= 0.87 | F 3sin30∘= 0.5 |
| F 4 | 0 | -10 |
|  | ΣFx = 51.37 | ΣFy = - 8.69 |

R2 = 51.372 + (- 8.69)2
R = 52.09

- According to question no. 2, check the answer by means of a force polygon drawn
to scale.

- A uniform lever is pivoted at is its mid-point C. A body having a mass 5kg is suspended a point E, 180MM to the right of C. Calculate the mass to be suspended at point D, 400MM to the left of C to maintain the lever in balance.

400mm X mkg = 180MM X 5kg
 	   mkg = (180 X 5) / 400
 	   mkg = 2. 25

- A uniform horizontal beam, 6m ling, rests on two supports A and B, 4m apart, A being
at one end of the beam. The mass of the beam is 20kg. Calculate the reactions of the supports on the beam.

( a ) Take moment at B
 	FA X 4 = 20 X 1
 	   FA = 20 / 4
 	   FA = 5kg
( b ) Take moment at A
 	FB X 4 = 20 X 3
 	   FB = 60 / 4
 	   FB = 15kgf

- If a mass of 8kg be hung from the beam of Question (5) at a distance of 1m from A, calculate the reaction the reaction of the supports.

( a )Take moment at B
 	FA X 4 = (8 X 3) + (20 X 1)
 	   FA = (24 + 20) / 4
 	   FA = 44 / 4
 	   FA = 11
( b )Take moment at A
 	FB X 4 = (8 X 1) + (20 X 3)
 	   FB = (8 + 60) / 4
 	   FB = 17kgf
 	17 X 10 = 170N