Basic Geometrical Constructions and Regular Polygons

Pencils

|  | To achieve the best results, pencils are sharpened in two forms: |  |  |
| --- | --- | --- | --- |
|  | (i) | chisel-edged | use to draw straight lines |
|  | (ii) | conical pointed | use to draw continuous irregular lines, arrowheads, and lettering |
|  | When drawing with cartridge paper, use the following grade of pencils: |  |  |
|  | (i) | HB pencils | for lettering and arrowheads |
|  | (ii) | H pencils | for outlines and all thin lines |
|  | (iii) | 2H, 3H pencils | for construction lines |

| General rule pencil work | The more abrasive the drawing paper the harder the pencils should be used. |
| --- | --- |
|  | Sharpen the pencils frequently to produce bold, black and dense lines. |
| Compasses |  |

| 1. | To achieve the best results, compass leads should be sharpened by rubbing one side (inside for small compasses and outside for large compasses) to produce a sharp, curved edge. |
| --- | --- |
| 2. | Use a softer lead to compensate less pressure being applied when drawing arcs or circles, e.g. F pencil lead. |
| 3. | Compasses should be fitted with a shouldered needle point which should project a little way beyond the lead when the compasses are closed. |

| General rule on compass work | use two sizes of compass for drawing small and large circles or arcs separately |  |
| --- | --- | --- |
|  | draw circles or arcs in a clockwise direction with the instrument inclined slightly forward. |  |
| Tee-square |  |  |

Tee-square is mainly used for drawing horizontal lines. It is also used as a guide for set-squares when drawing vertical and inclined lines.

| General rule on the use of tee-square | lines are drawn along the top of the straight edge |
| --- | --- |
|  | hold the tee-square head against the left edge of the drawing board to slide the tee-square |
|  | apply little pressure toward the right to slightly adjust the tee-square position |

Set-squares

| Set-squares are used, in combination with a tee-square, for drawing vertical lines. It is also used to draw: |  |  |
| --- | --- | --- |
|  | (i) | inclined lines of 15 0, 30 0, 45 0, 60 0, or 75 0 |
|  | (ii) | parallel lines |
|  | (iii) | perpendicular lines |

General rule on the use of set-squares: draw vertical lines upward along the edge of the set-square which is supported by the tee-square place the set-square to the right of the line being drawn use set-squares singly or in combination to draw inclined lines, parallel lines or perpendicular lines

Geometrical constructions

(i)(ii)

(iii)(iv)(v)(vi)

LINE PERFERRED

| i) | visible outlines |
| --- | --- |
| ii) | hidden outlines |
| iii) | cutting planes |
| iv) | centre lines |
| v) | outlines and edges of adjacent parts |
| vi) | projection lines |