Tutorial 10

- The following class B address 150.100.0.0 has a subnet mask of 255.255.255.128. State the following information for the first 4 valid subnets : (a) subnet network address, (b) host IP address range and (c) subnet broadcast address.

- Discuss the advantages and disadvantages that the re-assembling process of a fragmented Internet datagram to be taken place at its destination versus at intermediate points on its path to the destination.

- In IP datagram, the checksum covers only the header and not the data. What do you think why this design is being implemented ?

- 2 clients X and Y are going to access a Web server W. Suggest the possible port numbers (both source and destination) for :

  - Segments from X to W.

  - Segments from Y to W.

  - Segments from W to X.

  - Segments from W to Y.

  - If X and Y are on different machines, can their source port numbers be the same ? Explain.

  - If X and Y are on the same machines, can their source port numbers be the same ? Explain

Tutorial 10

- The following class B address 150.100.0.0 has a subnet mask of 255.255.255.128. State the following information for the first 4 valid subnets : (a) subnet network address, (b) host IP address range and (c) subnet broadcast address.

- Discuss the advantages and disadvantages that the re-assembling process of a fragmented Internet datagram to be taken place at its destination versus at intermediate points on its path to the destination.

- In IP datagram, the checksum covers only the header and not the data. What do you think why this design is being implemented ?

- 2 clients X and Y are going to access a Web server W. Suggest the possible port numbers (both source and destination) for :

  - Segments from X to W.

  - Segments from Y to W.

  - Segments from W to X.

  - Segments from W to Y.

  - If X and Y are on different machines, can their source port numbers be the same ? Explain.

  - If X and Y are on the same machines, can their source port numbers be the same ? Explain