Tutorial 7 Answer

- 11000000.10101000.00001010.00000000

- Network Octet : 1,2,3     Host Octet : 4

- 3 bits

- 11111111.11111111.11111111.11100000

- 255.255.255.224

| Usable Yes/No | No | Yes | Yes | Yes | Yes | Yes | Yes | No |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Broadcast Address | 192.168.10.31 | 192.168.10.63 | 192.168.10.95 | 192.168.10.127 | 192.168.10.159 | 192.168.10.191 | 192.168.10.223 | 192.168.10.255 |
| Possible Host IP Address Range | 192.168.10.1- 192.168.10.30 | 192.168.10.33- 192.168.10.62 | 192.168.10.65- 192.168.10.94 | 192.168.10.97- 192.168.10.126 | 192.168.10.129- 192.168.10.158 | 192.168.10.161- 192.168.10.190 | 192.168.10.193- 192.168.10.222 | 192.168.10.225- 192.168.10.254 |
| Subnet Network Address | 192.168.10.0 | 192.168.10.32 | 192.168.10.64 | 192.168.10.96 | 192.168.10.128 | 192.168.10.160 | 192.168.10.192 | 192.168.10.224 |
| Subnet Mask | 255.255.255.224 | 255.255.255.224 | 255.255.255.224 | 255.255.255.224 | 255.255.255.224 | 255.255.255.224 | 255.255.255.224 | 255.255.255.224 |
| Network Address | 192.168.10.0 | 192.168.10.0 | 192.168.10.0 | 192.168.10.0 | 192.168.10.0 | 192.168.10.0 | 192.168.10.0 | 192.168.10.0 |
| Subnet Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

- Interface A : 192.168.10.33

Interface B : 192.168.10.65

Interface C : 192.168.10.97

Interface D : 192.168.10.129

- PC1 : 192.168.10.34

PC2 : 192.168.10.98

For PC1 :

IP address 		:	192.168.10.34

IP address (binary)	:	11000000.10101000.00001010.00100010

Subnet Mask		:	11111111.11111111.11111111.11100000

Binary AND		:	11000000.10101000.00001010.00100000

Subnet			:	192.168.10.32

For PC2 :

IP address 		:	192.168.10.98

IP address (binary)	:	11000000.10101000.00001010.01100010

Subnet Mask		:	11111111.11111111.11111111.11100000

Binary AND		:	11000000.10101000.00001010.01100000

Subnet			:	192.168.10.96

- They are on the same network but in different subnets.

- PC1 identify the IP address of PC2 and learn that it is in other network. Thus, it will send the packet to interface A of the router. However, firstly it need to get the MAC address of interface A by looking at its ARP table or broadcast an ARP packet to get this MAC address. As the router get this packet, it learns that the destination IP address is on the same network of its interface C. Similarly, it will pass it to its link layer, get the MAC address of PC2 and send it through interface C. Ultimately, this packet will reach PC2.