The Hong Kong Polytechnic University

Department of Electronic and Information Engineering

EIE371

CONTROL SYSTEM

(Laboratory report)

CT023

COMPUTER AIDED ANALYSIS AND DESIGN OF CONTROL SYSTEM

| Student name: | Wong Tang Paai |
| --- | --- |
| Student no: | 01111911D |
| Course no.: | 42075- Yr 2 |

# CONTENT

- TILE

- OBJECTIVE

- APPARATUS

- Introduction

- MATLAB
<1> Poles and zeros
<2>Root Locus Plot 
<3>Step Response based on design considertion using root locus analysis 
<4> Bode Plot
<5> Nyquist Plot
<6>Design based on frequency domain analysis

- Observation and comments

- CONCLUSION

1. Title

COMPUTER AIDED ANALYSIS AND DESIGN OF CONTROL SYSTEM

2. Objective

System Analysis: To use MATLAB to perform:

- Root locus calculation and plot

- Bode Plot

- Nyquist Plot

- Step response

- Impulse response

Control System Design:

- Based on the Root Locus plot and time-domain performance specification, determine the settings of the controller gains with which the system should operate. Verify the suitability of these settings by observing the system closed-loop step and impulse responses.

- Based on the frequency response analysis, determine the controller gain for good system performance. Verify the suitability of design by applying a step function to the plant input.

3. Apparatus

PC with Windows 3.1x/95/98/NT and MATLAB software

4. Introdution

The availability of PC and user-friendly control system analysis packages at low cost make the control system analysis and design possible and easy to do. MATLAB is originally a matrix manipulation package. With the support of various toolboxes, the MATLAB environment, which is user-friendly code interpreter, becomes a very powerful CAD platform for control systems.

A feedback control system now under consideration comprises functional blocks of transfer functions. The block diagram is shown as follows:

Controller, Gc(s)=2Kcs(1+sτ);τ=2

Control gain, Gv(s)=1

Plant, Gp(s)=1s3+5s2+5.25s+5

Output Sensor and Feedback Amplifier, 	H(s) =1

Therefore, loop transfer function = Gc(s)Gv(s)Gp(s)H(s)=Kc(s+2)s(s3+5s2+5.25s+5)

5. <1>poles and zeros

| MATLAB | Description |
| --- | --- |
| >> num = [0,0,0,1,2]; >> den=[1,5,5.25,5,0]; >> [z,p,k]=tf2zp(num,den) z = -2 p = 0 -4.0000 -0.5000 + 1.0000i -0.5000 - 1.0000i k = 1 | numerator denominator finds the zeros, poles, and gains |

Questions:

| 1 | What does num and den contain? |
| --- | --- |
| Ans: | Vector DEN specifies the coefficients of the denominator in descending powers of s. Matrix NUM indicates the numerator coefficients with as many rows as there are outputs. |
| 2. | Is there any difference if you type ‘num = [1,2]’ instead? |
| Ans: | No, the coefficients of matrix are in descending powers of s |
| 3. | What does the function tf2zp do? |
| Ans: | TF2ZP Transfer function to zero-pole conversion. [Z,P,K] = TF2ZP(NUM,DEN) finds the zeros, poles, and gains: H(s)=K(s−z1)(s−z2)...(s−zn)(s−p1)(s−p2)...(s−pn) |

| 4. | What is the difference between adding a semicolon and not adding a semicolon at the end of each statement? |  |
| --- | --- | --- |
| Ans: | When no semicolon follows the statement, the calculated result will be display. With a semicolon adding at the end of each statement, the calculated result will be hidden |  |
| 5. | What are the open-loop poles and zero values? |  |
| Ans: | Zero: Pole: open-loop gain: | -2 0 -4.0000 -0.5000 + 1.0000i -0.5000 - 1.0000i 1 |

5. <2> root locus plot

| MATLAB | Description |
| --- | --- |
| >> 'Root Locus Plot, k=0.1 to 10'; >> n = logspace(-1,1,200); >> r=rlocus(num,den,n); >> plot(r,'.') >> title('ROOT LOCUS,0.1<=K<=10') >> xlabel('REAL') >> ylabel('IMAG') >> grid | Generate 200 point between 10^-1 to 10^1 |

Questions:

| 1. | What do the functions logspace, rlocus and plot do? |
| --- | --- |
| Ans: | [logspace] LOGSPACE Logarithmically spaced vector. LOGSPACE(X1, X2) generates a row vector of 50 logarithmically equally spaced points between decades 10^X1 and 10^X2. If X2 is pi, then the points are between 10^X1 and pi. LOGSPACE(X1, X2, N) generates N points. For N < 2, LOGSPACE returns 10^X2. [rlocus] RLOCUS Evans root locus. RLOCUS(SYS) computes and plots the root locus of the single-input, single-output LTI model SYS. The root locus plot is used to analyze the negative feedback loop and shows the trajectories of the closed-loop poles when the feedback gain K varies from 0 to Inf. RLOCUS automatically generates a set of positive gain values that produce a smooth plot. RLOCUS(SYS,K) uses a user-specified vector K of gain values. R = RLOCUS(SYS,K) returns the matrix R of complex root locations for the gains K. R has LENGTH(K) columns and its j-th column lists the closed-loop roots for the gain K(j). [Plot] PLOT Linear plot. PLOT(X,Y) plots vector Y versus vector X. If X or Y is a matrix, then the vector is plotted versus the rows or columns of the matrix, whichever line up. If X is a scalar and Y is a vector, length(Y) disconnected points are plotted. PLOT(Y) plots the columns of Y versus their index. If Y is complex, PLOT(Y) is equivalent to PLOT(real(Y),imag(Y)). In all other uses of PLOT, the imaginary part is ignored. Various line types, plot symbols and colors may be obtained with PLOT(X,Y,S) where S is a character string made from one element |
| 2 | What is the meaning of stating the second argument of the plot function as ‘.’. |
| Ans: | Plot(r,’.’), plot symbols with ‘.’ Character for the complex r |
| 3. | A quick method to determine Kc is by observing the values of the matrix r. Can you suggest the procedure of this? |
| Ans | The column is the locus; row is the values of k stage. By observing the matrix r, we find that when the locus 2 and locus 3 cut the real axis at point 0, the corresponding values of k is 146 stage. ie. k =0.1 + 146*(10/200) ≒ 7, for a stable system Kc should be less than 7 |
| 4. | What is the difference if you type ‘rlocus(num,den)’ instead of ‘r = rlocus(num,den,n)’? |
| Ans: | Plots the root locus with gain K varies from 0 to Inf, instead of K restrict varies from 0.1-10 |

5. <3> step response based on design consideration using root locus analysis

Design a value of K such that K<Kc.

| MATLAB |  | Description |
| --- | --- | --- |
| >> t=0:0.5:20 |  |  |
| >> k=5; >> num1=num1*k; >> den1=num1+den; >> y=step(num1,den1,t); >> plot(t,y) | ％OV=1.5−11×100 =50％ TP=3s |  |
| >> k=2; >> num1=num1*k; >> den1=num1+den; >> y=step(num1,den1,t); >> plot(t,y) | ％OV＞100％ as the system is unstable and does not have an steady state TP is uncertain |  |
| >> k=1; >> num1=num*k; >> den1=num1+den; >> y=step(num1,den1,t); >> plot(t,y) | ％OV=0 TP=4.5s |  |
| >> k=0.5; >> num1=num*k; >> den1=num1+den; >> y=step(num1,den1,t); >> plot(t,y) | ％OV=0 TP=20s |  |

Question:

| 1. | Why is that neither K > Kc nor K = Kc is acceptable? |
| --- | --- |
| Ans. | If K > Kc or K = Kc, the system may be unstable |
| 2. | What are num1 and den1? |
| Ans: | Vector DEN1 specifies the coefficients of the denominator in descending powers of s. Matrix NUM1 indicates the numerator coefficients with as many rows as there are outputs. |

5. <4> Bode plot

| MATLAB | Description |
| --- | --- |
| >> 'Bode Plot'; >> w=logspace(-1,1); >> [mag,phase]=bode(num,den,w); >> loglog(w,mag); >> title('MAGNITUDE RESPONSE, K=1'); >> xlabel('rad./sec') >> ylabel('magnitude') >> grid |  |
| >> semilogx(w,phase) >> title('PHASE RESPONSE, K=1') >> xlabel('rad/sec') >> ylabel('rad/sec') >> grid |  |

Question:

| 1. | What is the difference between ‘w=logspace(-1,1)’ and ‘w = logspace(-1,1,200)’? |
| --- | --- |
| Ans: | [logspace] LOGSPACE Logarithmically spaced vector. LOGSPACE(X1, X2) generates a row vector of 50 logarithmically equally spaced points between decades 10^X1 and 10^X2. If X2 is pi, then the points are between 10^X1 and pi. LOGSPACE(X1, X2, N) generates N points. For N < 2, LOGSPACE returns 10^X2. |

| 2. | What does the function loglog do? |
| --- | --- |
| Ans: | LOGLOG Log-log scale plot. LOGLOG(...) is the same as PLOT(...), except logarithmic scales are used for both the X- and Y- axes. |
| 3. | Why are that num and den used instead of num1 and den1? |
| Ans: | Num and den with unity gain, num1 and den1 with modified gain and poles |
| 4. | How can we display the magnitude plot in dB by employing the function log10? |
| Ans: | DB(xx) |
| 5. | What does the function semilogx do? |
| Ans: | SEMILOGX Semi-log scale plot. SEMILOGX(...) is the same as PLOT(...), except a logarithmic (base 10) scale is used for the X-axis. |

| 6. | How can we display both the magnitude response and the phase response simultaneously on the same screen using the funtion subplot? |
| --- | --- |
| Ans: | [subplot] SUBPLOT(m,n,p), breaks the Figure window into an m-by-n matrix of small axes, selects the p-th axes for for the current plot >> subplot(2,1,1) >> loglog(w,mag); >> subplot(2,1,2) >> semilogx(w,phase) |

| 7. | Estimate from the graphs the gain margin, phase margin, gain crossover frequency, and phase crossover frequency. |  |
| --- | --- | --- |
| Ans: |  | >> bode(num,den) Gain margin = 9.43 Gain crossover frequency = 1.26 rad/sec Phase margin = 180-107 =73° Phase crossover frequency = 0.45 rad/sec |

| 8 | Verfiy the results obtained by typing the following command: |  |
| --- | --- | --- |
|  | >> 'To obtain Gain and Phase Margins, etc.'; >> [Gm,Pm,Wcg,Wcp]=margin(mag,phase,w) Gm = 2.9039 Pm = 73.2050 Wcg = 1.2572 Wcp = 0.4459 | Gain margin: Gm¦dB=20log2.9039=9.26dB |

5. <5>Nyquist plot

| MATLAB | Description |
| --- | --- |
| >> 'Nyquist Plot'; >> theta=phase*pi/180; >> polar(theta,mag) >> title('NYQUIST PLOT') >> grid | To obtain the Nyquist Plot, convert phas in degree into theta in radian. |

Question:

| 1. | What does the function polar do? |
| --- | --- |
| Ans: | [Polar] POLAR Polar coordinate plot. POLAR(THETA, RHO) makes a plot using polar coordinates of the angle THETA, in radians, versus the radius RHO. POLAR(THETA,RHO,S) uses the linestyle specified in string S. See PLOT for a description of legal linestyles. |
| 2. | From the Nyquist Plot, find the values of gain margin and phase margin. |
| Ans: | Gm=2.9 , PM=73° |
| 3 | Compare your results with those obtained by typing ‘nyquist(num,den)’ |
| Ans: |  |

5. <6>Design based on frequency domain analysis

Determine an appropriate K setting based on the frequency response characteristics given by the bode plot. Verify the suitability of your choice of K (such that K<Kc and gives certain values of gain and phase margins to the system) by running a step response for the closed-loop system.

| MATLAB |  | Description |
| --- | --- | --- |
| >> t=0:0.5:20 |  |  |
| >> k=5; >> num2=num2*k; >> den2=num2+den; >> y=step(num2,den2,t); >> plot(t,y) | ％OV=1.5−11×100 =50％ TP=3s |  |
| >> k=2; >> num2=num2*k; >> den2=num2+den; >> y=step(num2,den2,t); >> plot(t,y) | ％OV＞100％ as the system is unstable and does not have an steady state TP is uncertain |  |
| >> k=1; >> num2=num2*k; >> den2=num2+den; >> y=step(num2,den2,t); >> plot(t,y) | ％OV=0 TP=4.5s |  |
| >> k=0.5; >> num2=num2*k; >> den2=num2+den; >> y=step(num2,den2,t); >> plot(t,y) | ％OV=0 TP=20s |  |

6. Observation and Comments

MATLAB is an user-friendly system analysis software packages. It make the control system analysis and design more easier without the complex calculation. Because it is simply to used, many user may eventually not understand the procedure of control system, as they no need to memoryiz or even understand the mathametic process. Hence, the disadvantage of using MATLAB possibly is the degrade of reasoning of control system. However, as the development of science, the complex thing should trend to be more easy for human how to user it!

The design of linear control systems can be carried out in either the time domain or the frequency domain. For instance, steady-state accuracy is often specified with respect to a step input, a ramp input, or a parabolic input, and the design to meet a certain requirement is more conveniently carried out in the time domain. Other specifications such as maximum overshoot, rise time, and settling time, are all defined for a unit-step input, and therefore are used specifically for time-domain design. The relative stability is also measured in terms of gain margin and phase margin. These are typical frequency-domain specifications, which should be used in conjunction with such tools as Bode Plot, Polar plot and gain-phase Plot.

7. conclusion

The design of linear control systems can be carried out in either the time domain or the frequency domain. For instance, steady-state accuracy is often specified with respect to a step input, a ramp input, or a parabolic input, and the design to meet a certain requirement is more conveniently carried out in the time domain. Other specifications such as maximum overshoot, rise time, and settling time, are all defined for a unit-step input, and therefore are used specifically for time-domain design. The relative stability is also measured in terms of gain margin and phase margin. These are typical frequency-domain specifications, which should be used in conjunction with such tools as Bode Plot, Polar plot and gain-phase Plot.

[Time domain]

A compromise in the value ofζ should be made when all these criteria are to be satisfactorily met in a design problem. Together with the consideration on max. overshoot, a generally accepted ranged of ζ for satisfactory all normal performance is between 0.5 and 0.8

[Frequency domain]

The Phase margin indicated the amount of additional phase allowable in a stable open-loop plant frequency response before the point of marginal stability for the closed-loop system is reached.

Gain Margin = (Max. allowable open-loop gain ) / (Actual open-loop gain)

A Gain Margin of 4 or greater is an arbitrary “rule of- thumb” in many design applications. Larger phase margin and gain margin will improve damping.