Interrupt Service Routines

void function_name (void) interrupt interrupt_number _using register_bank_

The interrupt_number determines the interrupt vector address of the interrupt

function. Use the following table to determine the interrupt number.

# Interrupt number       Address         Interrupt number     Address

0 (EXTERNAL INT 0) 		0003h 			16 					0083h

1 (TIMER/COUNTER 0) 	000Bh 			17 					008Bh

2 (EXTERNAL INT 1) 		0013h 			18 					0093h

3 (TIMER/COUNTER 1) 	001Bh 			19 					009Bh

4 (SERIAL PORT) 		0023h 			20 					00A3h

5 (TIMER/COUNTER 2) 	002Bh 			21 					00ABh

6 (PCA) 					0033h 			22 					00B3h

7 						003Bh 			23 					00BBh

8 						0043h 			24 					00C3h

9 						004Bh 			25 					00CBh

10 						0053h 			26 					00D3h

11 						005Bh 			27 					00DBh

12 						0063h 			28 					00E3h

13 						006Bh 			29 					00EBh

14 						0073h 			30 					00F3h

15 						007Bh 			31					00FBh

The using attribute lets you specify a register_bank that is selected during the

execution of the interrupt function. Small interrupt routines might be more

efficient without the using attribute, since they use the default register bank 0.