JEDEC

Jointed Electronic Device Engineering Council

In this chapter I will discuss the structure of the GAL16V8, illustrate how the GAL is work for the logic gates. The usage of the software CUPL will also implemented, take few simple examples to introduce the CUPL programming. The CUPL output file .jed will be investigated illustrating how to decode the file to I/O logic expression in Boolean form.

Generic Array Logic (GAL)

The GAL, like the PAL, has a programmable AND array and a fixed OR array with programmable output logic, this structure allows any sum of products (SOP) logic expression. The two main differences between GAL and PAL devices are the GAL is reprogrammable and has programmable output configuration.

The GAL can be reprogrammed again and again because it uses E2CMOS (electrically erasable CMOS) technology instead of bipolar technology and fusible links.

Each row is connected to the input of an AND gate, and each column is connected to an input variable or its complement. By programming each E2CMOS cell to be either on or off, any combination of input variables or complements can be applied to an AND gate to from any desired product term. A cell that is on effectively connects its corresponding row and column, and a cell that is off disconnects the row and column,

Electrical Erasable Technology

EEMOS use a technology called “floating-gate MOS”. A floating gate transistor has two gates, control gate and floating gate. The floating gate is unconnected and is surrounded by extremely high-impedance insulating material. In the original-manufactured state, erased state, the floating gate has no charge, the transistor functions just as a transistor without floating gate; that is, there is a connection at the corresponding cross-point of the AND/ OR plane. In programmed state, the programmer applies a very high voltage (say,12V) pulse to hammer the insulator around the neutral floating gate at each cross-point where a connection is not wanted. The pulse temporarily breaks down the insulating material and causes some negative charge accumulating on the floating gate, preventing the transistor turn on. In this case, the cross-point fuse is effectively blown/ removed.

CUPL Example

# CUPL file logic1.pld

Name logic1; Partno xxxx; Revision xxxx; Date xxxx; Designer xxxx; Company xxxx.; Location xxxx; Assembly xxxx; Device g16v8; /****************************************************************/ /* */ /* This is a example to demonstrate how CUPL */ /* compiles simple gates. */ /* */ /****************************************************************/ /* * Inputs: define inputs to build simple gates from */ Pin 1 = a; Pin 2 = b; /* * Outputs: define outputs as active HI levels * */ Pin 19 = logicOut; /* * Logic: examples of simple gates expressed in CUPL */ logicOut = a&b;

# CUPL file logic1.jed

CUPL(WM)       4.7b  Serial# MW-65999997

Device          g16v8s  Library DLIB-h-36-9

Created         Sun Aug 17 14:12:10 2003

Name           logic1

Partno          xxxx

Revision        xxxx

Date            xxxx

Designer        xxxx

Company       xxxx.

Assembly       xxxx

Location        xxxx

pin19 = pin2 & pin1

*QP20

*QF2194

*G0

*F0

*L00000 01011111111111111111111111111111

*L02048 10000000011110000111100001111000

*L02080 01111000000000000000000000000000

*L02112 00000000011111111111111111111111

*L02144 11111111111111111111111111111111

*L02176 111111111111111110

*C0D67

*7FDD