# Table of Contents

1. Introduction	2

2. System Analysis	5

2.1 Identification of Needs	6

2.2 Preliminary investigation	7

2.3 Feasibility Study	8

3. Software Requirement Specification	9

3.1 Data Flow Diagram	10

3.2 User Requirements	11

3.3 System Requirements	12

4. Design	14

4.1 The Process	14

4.2 Modules and their Description	19

5. User Manual	27

6. Conclusion	29

7. Bibliography	30

Introduction

# Chapter 1

# INTRODUCTION

The Trivial File Transfer Protocol (TFTP) proposes a simple method of transferring files between two computer systems. It is much smaller than the Internet File Transfer Protocol (FTP) and does not provide many of the features such as, directory listing and user authentication. The main service provided by TFTP is the ability to send and receive files between a client process and a server process.

Network API

The TFTP server is a concurrent server and has been implemented using a connectionless protocol – User Datagram Protocol (UDP). Also implementation of the UDP protocol gave us a better perspective of how data is transferred and routed over a network in the lower layers.

Our TFTP implementation makes use of Berkley sockets, which provides several Application Program Interfaces (APIs) to the communication protocols. The API is the interface available to the programmer. The availability of an API depends on both the OS and the programming language.

The two most prevalent communication APIs for UNIX Systems are Berkeley Sockets and System V Transport Layer Interface (TLI). Both these interfaces have been developed for C language. The Berkeley Sockets were chosen over System V TLI, because of it’s popularity and wide availability.

Network Connection

A network connection can be connection-oriented or connectionless. The former is more like file I/O than the latter, since once we open a connection a connection with another process, the network I/O on that connection is always the same peer process. With a connectionless protocol, every network I/O operation could be with a different process on a different host.

Connection Parameters

There are five parameters that must be specified for a network connection

- Protocol to be used

- Local address

- Local process

- Foreign address

- Foreign process

Concurrent Server

Our TFTP implementation uses a concurrent server. A concurrent server is one that listens for client requests on a particular port. When a client request is received, the server then spawns a child server process, which handles further interaction with the client process on a newly assigned unique port.

Fig 1.1

System Analysis

# Chapter 2

# SYSTEM ANALYSIS

## 2.1 Identification of Needs

The Internet uses many protocols for it’s proper operation. The Trivial File Transfer Protocol is one among them. It uses the client – server mechanism to transfer the files between machines.

The File Transfer Protocol is much bigger and provides many options, whereas the Trivial File Transfer Protocol is much smaller, and provides only the basic file transfer operations. It does not provide some of the features provided by File Transfer Protocol, such as directory listing, user authentication, etc.,

Because of it’s wide usage, it is important to understand its mechanism and simulate a simple project that works similar to the actual Trivial File Transfer Protocol.

The software should provide some basic features such as

- Ability to transfer a text file from one machine to another remote machine connected in a network.

- Ability to download a file from a machine connected in the network.

## 2.2 Preliminary investigation

The project can be broadly divided into two stages. Firstly, to develop a working model, which could be run from the command line, i.e., the back end. Secondly to develop the graphical user interface which will be done at the client side, the process known as the front end.

To develop the back end, various network APIs are used. There are two options for developing the back end, viz.,

    - Connection oriented

    - Connection less

Since the Internet uses Connection less mode of file transfer, it would be better to simulate a similar file transfer mode. So all the basic network APIs provided by Berkley were studied. Many programs involving these APIs were developed for this purpose.

Finally to develop the front end, LINUX provides ‘Kdevelop’, which has many front-end tools like,

    - Qt

    - GIMP

    - GTK+

Qt has a simpler developing mechanism, having the ‘drag and drop’ interface, which is simple to develop the forms. Also the software is capable of automatically generating the code, which is in C++.

Initially many simple forms were designed, with the basic functionalities, for the purpose of studying the usage of Qt.

## 2.3 Feasibility Study

The back end involves developing a program that involves transferring files between two machines. The language selected is C++. Before starting the project many programs were developed using the network APIs in C++. So developing the back end of the project was feasible.

Now, as mentioned earlier, to develop the front end, many types of software are available. But the drag and drop interface of Qt was easier to develop the forms. Also Qt is able to automatically generate the code in C++. So some simple applications were developed to see how feasible it is to develop the front end using Qt. And since Qt passed the feasibility study, Qt was chosen for developing the front end.

Thus it was decided that it would be possible to develop the software with the available tools and software.

Software Requirement Specification

# Chapter 3

# SOFTWARE REQUIREMENT SPECIFICATION

## 3.1 Data Flow Diagram

0 level DFD

Process input

File Name

Output

Option

Fig 3.1

Filename

Filename

Filename

Filename
me

Filename

Option

Option

Option

Option

Select operation

Upload

Download

1st level DFD

Fig 3.2

## 3.2 User Requirements

The requirements by the end user can be stated as follows

- Ability to transfer any number of Text files one at a time between two machines connected in a server.

- To be able to connect more than one client to the server concurrently.

- A graphical user interface for the client to allow easier usage of the software.

- A user manual with details about how to use the software.

## 3.3 System Requirements

The system requirements can be stated as follows,

### Hardware Requirements

- Standard Specifications.

- To use the software to transfer files between two different computers, a network is needed, although the software can be used in a single machine.

- A LAN server should be connected to the network.

### Software requirements

- The machines should have LINUX installed in them, preferably Red Hat LINUX 7.2 onwards.

Design

# Chapter 4

# DESIGN

## 4.1 The Process

The process of developing the software began with requirement analysis, where the requirements of the end user were listed. After the requirement analysis phase, the next stage was the feasibility study. In this phase the project was thoroughly analyzed and many programs involving the network APIs were executed to understand the concepts of network programming in LINUX and to find out whether it is possible to complete the project within the specified time. After performing the feasibility study, the next phase was the design, where the project was split into modules, and functionality and the relationship between these modules were decided. And finally the implementation and testing phase involved the actual coding process and simultaneous testing. And after the entire process was completed, the project was again put to final testing where the project was tested for all possible inputs and conditions.

### Connectionless Protocol Process

Fig 4.1

- In a connectionless protocol the client does not establish a connection.

- Instead a client side sends a datagram to the server using sendto which addresses the destination as a parameter.

- Similarly server does not establish a connection from a client.

- Instead server issues a readfrom system call that waits until data arrives from some client.

- The recvfrom returns the network address to the client or the server process, along with the datagram so the server can send it’s response to the correct process

The contents of the 14 bytes of protocol specific addresses are interpreted according to the type of addresses, here the Internet family.

### Packet Formats

The TFTP specification specifies 5 types of packets / headers that can be used for the implementation of the protocol.

#### RRQ and WRQ Packets

Fig 4.2

RRQ and WRQ packets (opcodes 1 and 2 respectively) have the format shown above. The file name is a sequence of bytes in netascii terminated by a zero byte. The mode field contains the string "netascii", "octet", or "mail" (or any combination of upper and lower case, such as "NETASCII", NetAscii", etc.) in netascii indicating the three modes defined in the protocol. A host which receives netascii-mode data must translate the data to its own format. Octet mode is used to transfer a file that is in the 8-bit format of the machine from which the file is being transferred. It is assumed that each type of machine has a single 8-bit format that is more common, and that format is chosen.

#### DATA Packet

Fig 4.3

Data is transferred in DATA packets. DATA packets (Opcode 3) have a block number and data field. The block numbers on data packets begin with one and increase by one for each new block of data. This restriction allows the program to use a single number to discriminate between new packets and duplicates. The data field is from zero to 512 bytes long. If it is 512 bytes long, the block is not the last block of data; if it is from zero to 511 bytes long, it signals the end of the transfer.

#### ACK Packet

Fig 4.4

All packets other than duplicate ACK’s and those used for termination are acknowledged unless a timeout occurs. Sending a DATA packet is an acknowledgment for the first ACK packet of the previous DATA packet. The WRQ and DATA packets are acknowledged by ACK or ERROR packets, while RRQ and ACK packets are acknowledged by DATA or ERROR packets. An ACK packet has opcode 4. The block number in an ACK echoes the block number of the DATA packet being acknowledged. A WRQ is acknowledged with an ACK packet having a block number of zero..

#### ERR Packet

Fig 4.5

An ERROR packet (Opcode 5) can be the acknowledgment of any other type of packet. The error code is an integer indicating the nature of the error.  The error message is intended for human consumption, and should be in netascii. Like all other strings, it is terminated with a zero byte.

#### Error Codes

| Error Code | Description |
| --- | --- |
| 0 | Not defined |
| 1 | File not found |
| 2 | Access violation |
| 3 | Disk full or allocation exceeded |
| 4 | Illegal TFTP operation |
| 5 | Unknown port number |
| 6 | File already exists |
| 7 | No such user |

### Read from Server

To read a file from the server the interaction is carried out in the following manner.

Fig 4.6

- The client sends a RRQ packet to the server specifying the filename and the transfer mode.

- If the file exists and can be sent, then the server responds with the first DATA packet for the client with block number 1.

- When the client receives a DATA packet it responds with a corresponding ACK packet.

- This protocol between the client and server continues until the last packet from the server is received and the client responds with ACK packet.

- If any error occurs ERR packets are sent.

### Write to server

To write a file to the server the client and the server interact in the following manner –

Fig 4.7

- The client sends a WRQ packet to server with the filename and transfer mode.

- The server responds with a ACK packet containing block number 0. This is the only case when block number 0 is used.

- The client then starts sending DAT packets each containing block numbers and 512 byte data.

- The server with ACK packets responds to each DATA packet sent.

## 4.2 Modules and their Description

### Inbuilt Modules

socket

To do network I/O the first thing the process must do is call the socket system call, specifying the type of communication protocol desired.

#include <sys/types.h>

#include <sys/socket.h>

int socket(int family, int type, int protocol );

The argument ‘family’ is AF_INET for Internet protocols. The prefix ‘AF_’ stands for address family.

The socket type is ‘SOCK_DGRAM’ for datagram socket. Not all combinations of family and type are valid. The above combination stands for the UDP.

The protocol argument to the socket system call is typically set to 0 for the most user applications. There are specialized applications, however, that specify a protocol value, to use a specific protocol.

For our implementation the protocol IPPROTO_UDP is to be specified. It is defined in <netinet/in.h>

The socket system call returns a small integer value, similar to a file descriptor. This is called ‘socket descriptor’.

bind

The bind system call assigns a name to an unknown socket.

#include <sys/socket.h>

#include <sys/types.h>

int bind(int sockfd, struct sockaddr *myaddr, int addrlen);

The last argument is the size of this address structure. There are 3 uses of bind.

- Server registers their well known with the system. It tells the system its address. Both connection-oriented and connectionless servers need to do this before accepting client requests.

- A client can register a specific address for itself.

- A connection client needs to assure that the system assigns it some unique address, so that the other end has a valid return address to send its responses to.

sendto and recvfrom

These system calls are similar to the standard read and write system calls, but additional arguments are required.

#include <sys/types.h>

#include <sys/socket.h>

int sendto ( int sockfd, char* buff, int nbytes, int flags, struct sockaddr* to, int addrlen );

int recvfrom ( int sockfd, char* buff, int nbytes, int flags, struct sockaddr* from, int* addrlen );

The first three arguments, sockfd, buff, and nbytes, are similar to the first three arguments for read and write.

The flags arguments is either 0, or is formed by or’ing one of the following constants:

MSG_OOB			send or receive out-of-band data

MSG_PEEK			peek at incoming message ( recvfrom )

MSG_DONTROUTE		bypass routing ( sendto )

The last argument to recvfrom is a pointer to an integer value, while the final argument to recvfrom is a pointer to an integer value.

inet_addr

An Internet address is usually written in the dotted-decimal format, for eg. 192.168.1.246. The following functions convert between the dotted-decimal format and an in_addr structure.

#include <sys/socket.h>

#include <netinet/in.h>

#include <arpa/inet.h>

unsigned long inet_addr ( char *ptr );

The above system call converts a character string in dotted-decimal notation to a 32-bit Internet address.

### User defined modules

Some of the most important user defined modules are as follows.

Void get()

This function is used when a file has to be downloaded from the server.

Void put()

This function is used when a file has to be downloaded from the server.

init_client()

This function is used to initialize the sockets that will be used for interprocess communication.

showAbout()

This function is called when the user clicks the About button.

Put_warning()

This function displays any warning or error conditions detected during the file transfer.

Put_info()

This function displays any information to the user.

Put_critical()

This displays critical messages like  such as critical error in syscall .

Put_status()

This  fuction is used to display the file transfer status in a stsus box.

int make_rrq(char *fname, int mode)

Prepares the buffer to hold the RRQ packet.

int make_wrq(char *fname, int mode)

Prepares the buffer to hold the WRQ packet.

int make_data(int blkno, char *buf, int len)

Make the block with the given data.

int make_ack(int blkno)

Make the acknowledgement with the given block number.

int write_buf(struct packet pkt)

Writes the packet to the buffer.

void print_buf(char *buf)

Print the contents of buffer to screen.

Screen Shots

User Manual

# Chapter 6

# USER MANUAL

## Software Requirements

The software requires LINUX operating system, preferably Red Hat LINUX 7.2 onwards.

## Hardware Requirements

1. A network of computers with UDP support.

2. A PC with standard configuration.

3. A minimum disc space of 100 KB

## Installation

1. Install the application in the folder from or to where the files have to be transferred

2. Run the server by executing the output file ‘tftpd’.

3. Run the client by executing the output file ‘tftp’.

## The Graphical User Interface

The Graphical user interface of TFTP is very simple to use. The GUI consists of a single Document Interface, with the following components:

### Text boxes

It consists of two text boxes at the top. One of them is to enter the server address and the other one is to enter the port number.

### Tabbed Pane

The GUI consists of a tabbed pane that has two tabs. One of them is for uploading files and the other one is for downloading the file.

### Status Window

To the right side of this is a status box that displays the file transfer status. Block Display

There is a LCD display that indicates the number of blocks being transferred. In addition to this there are ‘About’ and ‘Help’ buttons.

Conclusion

# Chapter 7

# CONCLUSION

The Trivial File Transfer Protocol is used to transfer files from client to the server connected over the network. Files of any size can be transferred using this product. By completing this project we aimed at understanding the working of the computer network protocols. We learnt and understood many details and nuances of networking during the implementation of this project.

On completing this project we are now able to look further at the infinite horizon of the domain of network programming.

# BIBLIOGRAPHY

- W. Richard Stevens, 2002, UNIX Network Programming, Pentice Hall.

- W. Richard Stevens, 1993, Advanced Programming in the UNIX Environment, Addison Wesley.

- Terrance Chan, 1999, Unix System Programming using C++, Pentice Hall