## Preflight Inspection Checklist for Pilatus B4

- Remove rudder lock and pitot tube cover

- If glider is tied down do not untie it

- Front Seat

- Canopy, latches & sliding window

- Instruments

- Stick & rudder movement

- Rudder cable connections & anchor bolts

- Release cable connections & bolts

- Seat belts, straps & anchors

- Open spoilers & test brakes

- Close canopy

- Behind The Seat

- Battery charged, secured & connected

- Main wing bolts

- Right Wing

- Dive break bolts, pins, hinges & linkage

- Wing surface for damage & loose rivets

- Aileron movement, hinges, pushrod bolt & pin

- Wing tape

- Right Fuselage

- Tire inflation

- Fuselage skin

- Landing gear doors

- CG hook

- Rudder

- Free movement from stop to stop

- Cable connections

- Attachment point bolts

- Skin metal

- Tail wheel

- Elevator

- Pushrod bolts & pins

- Attachment point bolts & pins

- Horizontal stabilizer attachment bolts

- Skin metal

- Left Fuselage

- Fuselage skin

- Left Wing

- Dive break bolts, pins, hinges & linkage

- Wing surface for damage & loose rivets

- Aileron movement, hinges, pushrod bolt & pin

- Wing tape

- Nose

- Pitot tube

Notes:

When inspecting linkage, bolts, pins & hinges check for free movement, rust and security. Make sure that all castellated nuts have safety pins and Nylock* nuts are secure.  Do not leave canopy open when performing inspection. Always do inspection in sequence without interruption.

Tools needed are flashlight and common screwdriver.

Do not push down on the nose of the glider to pivot it around. Use the handle on the tail to lift it and walk around.

* If a Nylock nut is removed it must be replaced with a new one. Do not reuse it.