ADVANCE PROCESS DYNAMICS

## Instructor:  Prof. R.Shinnar

Submitted by

Hrishikesh Patel

M.S (Chemical Engg)

ID: 934003131

Department of Chemical Engineering

City College of New York

Executive Summary

- Production of Acetone ( CH3COCH3 )

- Process

- Cumene Oxidation process

- Dehydrogenation of 2-Propanol

- Specifications

- Product Specs – 99.5% Acetone

- Process Specs -  Concentration of Cumene ( >99.9%) and Reactor temperature maintenance to avoid byproduct formation

- Dominant Variables

- Purity of Cumene

- Cumene Hydroperoxide concentration in cleavage reactor ( 25 – 40 wt% )

- Pressure & Temperature in Reactor

- Constraints

1. Pure oxygen can’t be used for both economic and safety reasons.

- Primary Control

Cumene concentration maintenance by appropriate recycle rate. And Temperature control by using heat exchanger.

Introduction
Acetone, 2-propanone, dimethyl ketone, CH3COCH3, is the first and most important member of the homologous series of aliphatic ketones. It is a colorless, mobile liquid widely used as a solvent for various polymers. Its largest application, however, is as an intermediate in the synthesis of methyl methacrylate, bisphenol A, diacetone alcohol, and other products.

Acetone was first manufactured by the dry distillation of calcium acetate. Calcium acetate was originally a product of wood distillation, and later was obtained by fermentation of ethanol. Carbohydrate fermentation directly to acetone and butyl and ethyl alcohols displaced these processes in the 1920s. The carbohydrate route, in turn, was replaced in the 1950s and 1960s by the 2-propanol dehydrogenation process and by the oxidation of cumene to phenol plus acetone. Together with direct propene oxidation, these methods account for over 95 % of the acetone produced worldwide.

Production

Approximately 83 % of the acetone produced worldwide is manufactured from cumene as a coproduct with phenol. In the United States and Western Europe dehydrogenation of 2-propanol is also important, whereas in Japan catalytic oxidation of propene is used as a second process. Cumene, 2-propanol, and propene together as starting materials account for over 95 % of the acetone produced worldwide. Because propene is used in the manufacture of both cumene and 2-propanol, propene is the ultimate raw material for the production of Acetone.  Small amounts of acetone are made by oxidation of p-diisopropyl benzene and of p-cymene. Coproducts from these reactions are hydroquinone and p-cresol, respectively. Acetone is also produced by propene oxidation and as a byproduct of acetic acid manufacture.  Fermentation of cornstarch and molasses to acetone and 1-butanol was important in the past. It is believed to be practiced today to a limited extent in several countries.

Cumene Oxidation (Hock Process)

Propene is added to benzene to form cumene, which is then oxidized by air to cumene hydroperoxide, and cleaved in the presence of an acid catalyst. Phenol and acetone produced in the process are recovered by distillation.

The alkylation of benzene by propene proceeds under typical Friedel –Crafts conditions. The cumene produced is purified by chemical means and refined by distillation to 99.9 % minimum purity.

Cumene Oxidation. The oxidation of cumene is a free-radical chain reaction. The chain initiator is cumene hydroperoxide, the main product of the reaction. The rate of oxygen consumption can be approximated by the following expression:

cRH and cROOH are the concentrations of cumene and cumene hydroperoxide, respectively; ki , kp , and kt are the rate constants for chain initiation, propagation, and termination.  The expression shows that the rate of oxidation is zero in the absence of cumene hydroperoxide. This is not exactly true, because the expression is only an approximation; but the oxidation of cumene does require long induction periods when starting with pure cumene. Consequently, the industrial oxidation always is carried out in a series of continuous reactors; the concentration of cumene hydroperoxide is at least 8 wt % in the first reactor. The maximum rate is achieved at approximately 35 wt % cumene hydroperoxide. Besides cumene hydroperoxide, both dimethylphenylmethanol and acetophenone are also formed as byproducts during this oxidation.

The oxygen needed for cumene oxidation is supplied by air. Use of pure oxygen has been suggested but is disfavored by both economic and safety considerations. At low initiation rates, the rate of the reaction is essentially independent of the oxygen concentration at a partial pressure of oxygen over 33kPa ( 0.33 bar).

Fresh and recycled cumene are fed to the first reactor, which may operate at 8 – 12 wt % cumene hydroperoxide. The concentration increases by 4 –8 wt % in each successive reactor; the last reactor may operate at 25 –40 wt % cumene hydroperoxide. Fresh air is pumped in parallel to each reactor and vented at the top after removal of organic vapors.

Cumene Hydorperoxide Cleavage

Cumene Hydorperoxide is cleaved to phenol and acetone in the presence of catalytic amounts of a strong acid.  The acid most commonly used is sulfuric acid.  Sulfur dioxide is used as catalyst in the allied/UOP process.

Product Separation

In the Hercules process the cleavage mixture is neutralized with base and the fed to a separation column.  The overhead from this column contain Acetone.  In the Allied/UOP process the cleavage mixture is treated with an ion-exchange resin to remove the acid catalyst and then is distilled. Acetone is removed firs in a crude acetone column and purified by distillation with steam.

Dehydrogenation of 2-Propanol

The hydration of propene gives 2-propanol, which is then dehydrogenated to acetone. In the United States a C3 stream containing 40 – 60 % propene is used for the manufacture of 2-propanol.

The main side reaction is the dehydration of 2-propanol to propene. Other competing reactions are the self-condensation of acetone to diacetone alcohol, which leads to further condensation products.

Production Of Acetone By Dehydrogenation Process

Product Specifications

For 99.5% Acetone

Standard specifications for acetone, ASTM D329–90

Property; ASTM test; Specification. Relative density; D268. 20/20 °C; 0.7910 – 0.7930. 25/25 °C; 0.7865 – 0.7885. Color; D1209;  5 on platinum-cobalt scale. Distillation range; D1078; 1.0 °C, including 56.1 °C. Nonvolatile matter; D1353;  5 mg/100 mL. Odor; D1296; characteristic, nonresidual. Water; D1364; 0.5 wt % *. Acidity (as acetic acid); D1613; 0.002 wt %. Water miscibility; D1722; passes test. Alkalinity (as ammonia); D1614; 0.001 wt %. Permanganate time; D1363; 30 min at 25 °C.

- This water limit ensures that the material is miscible without turbidity with 19 volumes of 99 % heptane at 20 °C (ASTM D1476).

Process Specifications

  - Concentration of cumene hydroperoxide should be at least 8 wt% in first reactor. And maximum concentration in last reactor is approximately 35 wt%.

  - In Hercules process Cumene oxidation temp. is 90 – 120 0C, and 620 kPa pressure, and in Allied/UOP process atmospheric pressure and 80 - 1000C temp.

  - cleavage reactor temp must be below 70 0C.

  - For the dehydrogenation reactor temp. requirement is 327 0C

Feed Specification

Specifications for oxidation-grade cumene

Property; ASTM test; Specification. Appearance; Clear, colorless liquid. Color, Pt – Co scale; D1209–79; 15 max.. D891–59; 0.864 – 0.867. Acid wash color, W scale; D848–62; 2 max.. Sulfur compounds; D853–47; Free from H2S and SO2. Copper corrosion; D849–47; No iridescence, gray or black. Distillation range; D950–56; 1.0 °C max.. Cumene content; 99.93 % min.. Phenolics content; 5 ppm max.. Cumene hydroperoxide content; 200 ppm max.. Sulfur content; 0.1 ppm max..

Process Constraints

- High purity cumene requirement ( > 99.9 %)

- Pure oxygen can’t be used for oxidation of cumene because of both economic and safety considerations.

- Cumene hydroperoxide cleavage reaction rate accelerates rapidly with increasing temperature.

- Concentration of cumene must be lower than 40 wt% to reduce the byproduct formation.

Dominant Variables

- Concentration of Cumene: It affects the amount of Cumene Hydroperoxide produces in the reactor as well as by production formation rate.

- Temperature in the reactor must be maintained for higher selectivity.  The oxidation of cumene generates approximately 116 kJ of heat per mole of cumene oxidized. Part of this heat is carried to the condensers by organic vapors.

- Rate of reaction is also govern by Oxygen concentration. but its independent when partial pressure of oxygen over 33kPa.

Primary Control

- The concentration of cumene in the reactor is controlled by the amount recycled and by the feed control of high purity comene.

- Temperature in the reactor is controlled by the use of heat exchanger.

- In Hercules process of Cumene Hydroperoxide decomposition reactor temperature is maintained below 95 0C by refluxing approximately 2.8kg Acetone per kilogram Cumene Hydorperoxide.

- Dehydrogenation process can sustain the variable Feed of 40 – 60% C3 Stream. And temperature in the reactor is changed by the use of heat exchangers according to the Feed variation.