- Introduction

Acetone, 2-propanone, dimethyl ketone, CH3COCH3, [67-64-1], is the first and most important member of the homologous series of aliphatic ketones. It is a colorless, mobile liquid widely used as a solvent for various polymers. Its largest application, however, is as an intermediate in the synthesis of methyl methacrylate, bisphenol A, diacetone alcohol, and other products.

Acetone was first manufactured by the dry distillation of calcium acetate [62-54-4]. Calcium acetate was originally a product of wood distillation, and later was obtained by fermentation of ethanol. Carbohydrate fermentation directly to acetone and butyl and ethyl alcohols displaced these processes in the 1920s. The carbohydrate route, in turn, was replaced in the 1950s and 1960s by the 2-propanol dehydrogenation process and by the oxidation of cumene to phenol [108-95-2] plus acetone. Together with direct propene oxidation, these methods account for over 95 % of the acetone produced worldwide.

2. Physical Properties

Acetone has the following physical properties: Mr  58.081; bp at 101.3 kPa, 56.2 °C; mp – 94.7 °C; relative density, ; relative vapor density (air = 1) 2.0025; refractive index ; critical temperature 235.0 °C, critical pressure 4.6 MPa (46 bar), critical density 0.278 g/cm3; cubic expansion coefficient (18 °C) 1.43 × 10–3 K–1; compressibility coefficient (18 °C) 1.286 × 10 –6 kPa–1 (1.286×10–4 bar–1).

Viscosity in mPa · s: 1.53 (– 80 °C), 0.71 (– 40 °C), 0.40 (0 °C), 0.32 (20 °C), 0.27 (40 °C). Surface tension in mN · m–1: 38.1 (– 91.09 °C), 23.9 (15 °C), 23.3 (20 °C), 23.0 (24.8 °C), 22.0 (30 °C), 21.6 (42 °C).

Thermal properties: Specific heat capacity, cp (20 °C) 2.135 kJ kg–1 K–1; heat of fusion (– 95 °C) 98.47 kJ kg–1; heat of vaporization (30 °C) 545.2 kJ kg–1, (0 °C) 588.2 kJ kg–1; molar entropy 0.2001 kJ mol–1 K–1; heat of combustion 1804 kJ mol–1; heat of formation (20 °C) 235.3 kJ/mol; thermal conductivity of the liquid 1.976 W m–1 K–1.

Vapor Pressure in kPa: 24 (20 °C), 37.3 (30 °C), 56.0 (40 °C), 82.8 (50 °C), 114.8 (60 °C), 214.8 (80 °C), 372.8 (100 °C), 929.6 (140 °C).

Electrical properties: Electric conductivity (20 °C) 5.5 × 10–8  –1 cm–1; dipole moment (20 °C) 2.69 Debye; dielectric constant of the liquid 21.58 (0 °C), 22.64 (10 °C), 20.70 (25 °C), 19.38 (40 °C); dielectric constant of the vapor 1.0235 (24.8 °C), 1.0277 (29.8 °C).

At ambient temperature acetone is a clear, colorless liquid with a characteristic odor. It is miscible in all proportions with water and polar organic solvents, such as the lower molecular mass alcohols, carboxylic acids, and ethers. It is miscible in limited proportions with nonpolar solvents, such as hydrocarbons. Some azeotropic mixtures are shown in Tables 1 and 2 [3], [4].

- Chemical Properties

Pure acetone is essentially inert to air oxidation and to diffuse sunlight under ambient conditions. Its chemical stability diminishes significantly in the presence of water. Acetone may react violently and sometimes explosively, especially in a confined vessel [5]. For example it is particularly sensitive to oxidizing agents, such as nitrosyl chloride [6] [2696-92-6], chromium trioxide [7] [1333-82-0], and hydrogen peroxide [8] [7722-84-1], or organic peroxides [9]. Mixtures of acetone with chloroform [67-66-3] may react violently in the presence of alkali [10]. Reaction even may be initiated by surface alkali on new glassware [11]. Acetone has a flash point of –17 °C (closed cup). Flammability limits in air are: lower 2.13 vol %, upper 13 vol %; autoignition temperature 465 °C. The flammability of acetone can be reduced by mixing it with less flammable and/or less volatile solvents [12]. Fires have been started during recovery of acetone from air by adsorption on activated carbon when air flow was too low to effectively remove the heat generated by surface oxidation [13].

Acetone undergoes typical carbonyl reactions with particular ease. Acid- or base-catalyzed self-condensation produces the dimers diacetone alcohol and mesityl oxide and the cyclic trimer isophorone.

Under strongly basic conditions hydrogen cyanide adds to acetone to form 2-cyano-2-propanol (acetone cyanohydrin), an important intermediate in the manufacture of methyl methacrylate and other methacrylate esters (Section Methyl Methacrylate).

In liquid ammonia solution acetone condenses with acetylene [74-86-2] in the presence of catalytic amounts of alkali metals to form 2-methyl-3-butyn-2-ol [115-19-5], an intermediate in the synthesis of isoprene [14] [563-46-2]. Catalytic hydrogenation of acetone yields 2-propanol [67-63-0]. Pyrolysis produces methane [74-82-8] and ketene [463-51-4], a powerful acetylating agent. A more economical source of ketene, however, is the pyrolysis of acetic acid, which produces ketene and water.

Reductive ammonolysis of acetone yields isopropylamine [75-31-0]. Condensation with 2 mol phenol in the presence of an acidic catalyst yields bisphenol A (Section Bisphenol A ), an important monomer used in the manufacture of polycarbonate resins.

Perchlorination yields hexachloroacetone [116-16-5], which is cleaved into chloroform [67-66-3] and sodium trichloroacetate [650-51-1] upon treatment with sodium hydroxide.

4. Production

Approximately 83 % of the acetone produced worldwide is manufactured from cumene as a coproduct with phenol. In the United States and Western Europe dehydrogenation of 2-propanol is also important, whereas in Japan catalytic oxidation of propene is used as a second process. Cumene, 2-propanol, and propene together as starting materials account for over 95 % of the acetone produced worldwide. Because propene is used in the manufacture of both cumene and 2-propanol, propene is the ultimate raw material for the production of acetone.

Small amounts of acetone are made by oxidation of p-diisopropyl benzene and of p-cymene. Coproducts from these reactions are hydroquinone and p-cresol, respectively. Acetone is also produced by propene oxidation and as a byproduct of acetic acid manufacture.

Fermentation of cornstarch and molasses to acetone and 1-butanol was important in the past. It is believed to be practiced today to a limited extent in several countries.

4.1. Cumene Oxidation (Hock Process) (Fig. 1)

Propene [115-07-1] is added to benzene [71-43-2] to form cumene [98-82-8], which is then oxidized by air to cumene hydroperoxide (1), and cleaved in the presence of an acid catalyst. Phenol [108-95-2] and acetone produced in the process are recovered by distillation.

|  | Figure 1. Cumene phenol – acetone process (Allied) a) Oxidizers; b) Flash column; c) Carbon adsorber; d) Alkaline extraction and wash; e) Cumene hydroperoxide decomposer; f) Dicumyl peroxide decomposer; g) Ion exchange; h) Crude acetone column; i) Acetone-refining column; j) Cumene column; k) -Methylstyrene column; l) Phenol column; m) Phenol residue topping column AMS = -methylstyrene [Full View] |
| --- | --- |

The alkylation of benzene by propene proceeds under typical Friedel – Crafts conditions. In 1996, a number of processes using zeolite catalysts came on-stream. The cumene produced is purified by chemical means and refined by distillation to 99.9 % minimum purity. Oxidation-grade cumene must meet strict quality standards. The newer zeolite-based processes have led to slightly tighter specifications (Table 3) [15].

Table 3. Specifications for oxidation-grade cumene (zeolite process) [15]

Property; ASTM test; Specification. Appearance; Clear, colorless liquid. Color, Pt – Co scale; D1209–79; 15 max.. D891–59; 0.864 – 0.867. Acid wash color, W scale; D848–62; 2 max.. Sulfur compounds; D853–47; Free from H2S and SO2. Copper corrosion; D849–47; No iridescence, gray or black. Distillation range; D950–56; 1.0 °C max.. Cumene content; 99.93 % min.. Phenolics content; 5 ppm max.. Cumene hydroperoxide content; 200 ppm max.. Sulfur content; 0.1 ppm max..

Cumene Oxidation. The oxidation of cumene is a free-radical chain reaction [16]. The chain initiator is cumene hydroperoxide, the main product of the reaction. The rate of oxygen consumption can be approximated by the following expression:

cRH and cROOH are the concentrations of cumene and cumene hydroperoxide, respectively; ki , kp , and kt are the rate constants for chain initiation, propagation, and termination.

The expression shows that the rate of oxidation is zero in the absence of cumene hydroperoxide. This is not exactly true, because the expression is only an approximation; but the oxidation of cumene does require long induction periods when starting with pure cumene. Consequently, the industrial oxidation always is carried out in a series of continuous reactors; the concentration of cumene hydroperoxide is at least 8 wt % in the first reactor. Because the sum of cRH and cROOH remains roughly constant during the reaction, the rate of reaction cannot increase indefinitely as cROOH increases. The maximum rate is achieved at approximately 35 wt % cumene hydroperoxide.

Besides cumene hydroperoxide, both dimethylphenylmethanol and acetophenone are also formed as byproducts during this oxidation. These arise from a secondary chain reaction that proceeds in parallel with the main chain. Byproduct formation is accelerated as the concentration of cumene hydroperoxide increases. For these reasons, most plants operate between 25 and 40 wt % in the last oxidation reactor.

A minor but significant byproduct of the oxidation is dicumyl peroxide [80-43-3]. This arises during the termination of the chain reaction. Dicumyl peroxide also contributes to chain initiation [17], but to a much lesser degree than cumene hydroperoxide. Other minor byproducts are formaldehyde and formic acid, which are produced along with acetophenone by methyl group degradation.

The oxygen needed for cumene oxidation is supplied by air. Use of pure oxygen has been suggested [18] but is disfavored by both economic and safety considerations [19]. At low initiation rates, the rate of the reaction is essentially independent of the oxygen concentration at a partial pressure of oxygen over 33 kPa (0.33 bar) [20]. A detailed study of the rate of oxygen uptake in a bubble column as a function of temperature and partial oxygen pressure has been made [21]. The study served as a basis for a mathematical model of the oxidation [22], [23].

Two cumene oxidation processes are used in the United States, which with minor variations are practiced also in the rest of the world [19], [22]. One process was developed by Hercules and licensed by Kellogg (previously BP/Hercules) and GE/Lummus [24]. The other process was developed by Allied and licensed by Allied/UOP [25-28].

In both processes several reactors are employed in series. Fresh and recycled cumene are fed to the first reactor, which may operate at 8 – 12 wt % cumene hydroperoxide. The concentration increases by 4 – 8 wt % in each successive reactor; the last reactor may operate at 25 – 40 wt % cumene hydroperoxide. Fresh air is pumped in parallel to each reactor and vented at the top after removal of organic vapors.

In the Hercules process, the oxidation of cumene is carried out at approximately 620 kPa (6 bar)/90 – 120 °C, in the presence of a sodium carbonate buffer [29]. Under these conditions the residence time in the oxidizer train is 4 – 8 h and the hydroperoxide molar selectivity 90 – 94 %. The spent air is first passed through water cooled and refrigerated condensers in series to remove organic vapors, and is finally vented. The condensate is returned to the oxidizers after treatment [29].

In the Allied/UOP process (Fig. 1) the oxidation is carried out at atmospheric pressure. No buffer or promoter is added, but great care is taken to wash all streams recycled to the oxidizer with alkali and water [27]. Temperature is maintained at 80 – 100 °C. Residence time in the oxidizer train is 10 – 20 h and hydroperoxide molar selectivity is 92 – 96 %. Spent air is vented after organic vapors are removed by condensation followed by activated carbon adsorption. The recovered materials are washed with aqueous sodium hydroxide and water, then returned to the oxidizers.

The oxidation of cumene generates approximately 116 kJ of heat per mole of cumene oxidized [30]. Part of this heat is carried to the condensers by organic vapors (this part is larger in the Allied/UOP process because of the lower operating pressure). The rest is removed by heat exchangers.

In both processes cumene hydroperoxide is concentrated to over 80 wt % by evaporation of excess cumene. In the Hercules process the oxidate is washed with water prior to distillation in order to remove the buffer added during oxidation.

Cumene Hydroperoxide Cleavage. Cumene hydroperoxide [80-15-9] is cleaved to phenol and acetone in the presence of catalytic amounts of a strong acid. The acid most commonly used is sulfuric acid. Sulfur dioxide is used as catalyst in the Allied/UOP process. Several patents claim the use of solid acids as catalysts for the decomposition [31]. Strongly acidic resins have been used to that effect in the Soviet Union [32]. However, all commercial units use strong mineral acids or SO2, which generates sulfuric acid in situ, as catalysts.

The cleavage proceeds through an ionic mechanism and releases approximately 252 kJ/ mol of cumene hydroperoxide decomposed [30]. The reaction rate accelerates rapidly with increasing temperature. Consequently, decomposition of cumene hydroperoxide commonly is carried out in a continuously stirred reactor in which the steady-state concentration of cumene hydroperoxide is maintained at a low level. The heat released by the reaction can be used to estimate the concentration of hydroperoxide present in the reactor at any time [33].

The molar selectivity of the cleavage to phenol and acetone is higher than 99.5 % at temperatures below 70 °C, but it decreases at higher temperatures as increasing amounts of dimethylphenylmethanol and acetophenone (in addition to those present in the cumene oxidate) are formed (Table 4) [28].

Table 4. Formation of byproducts during cumene hydroperoxide decomposition *

Temp. °C; Molar ratio (byproducts/phenol)×100. DMPM equivalents**; Acetophenone. 70; 0.36; 0.06. 90; 0.61; 0.06. 110; 1.24; 0.15. 122; 2.19; 0.25. 146; 5.04; 0.69.

*  Pure cumene hydroperoxide added to phenol – acetone – cumene solution containing initially 0.5 wt % water and 100 ppm sulfuric acid; data from [28].

**  Sum of dimethylphenylmethanol, -methylstyrene, and their condensation products.

Acetone produced during the cleavage of cumene hydroperoxide can react further. Oxidation by cumene hydroperoxide forms hydroxyacetone [34] to the extent of 0.2 – 0.5 % of acetone present. Self-condensation catalyzed by acid results in diacetone alcohol and mesityl oxide. Conversion of acetone to these condensates is normally below 0.1 % but may increase upon protracted exposure to strong acid. For example, when the cumene hydroperoxide cleavage was carried out with refluxing acetone using a sulfonic acid resin as catalyst, approximately 1.7 % of acetone was transformed to diacetone alcohol and mesityl oxide [32].

Under the conditions of the cumene hydroperoxide cleavage, dimethylphenylmethanol is dehydrated to -methylstyrene (2) and also forms undesirable condensates.

Compound (2) may be either hydrogenated [35] to cumene and recycled, or recovered and sold.

In the Hercules process, the cumene hydroperoxide decomposition is carried out in a constant-flow, stirred tank reactor in the presence of sulfuric acid or another strong mineral acid [19], [28]. The acid is added to the reactor as an acetone solution. The reactor temperature is maintained below 95 °C by refluxing approximately 2.8 kg acetone per kilogram cumene hydroperoxide. The ratio of the quantity of reflux to the quantitiy of hydroperoxide fed is used as a monitor of the cleavage reaction [19].

In the Allied/UOP process the cumene hydroperoxide cleavage is carried out at 60 – 80°C in a pressurized, constant-flow, back-mixed reactor. Temperature is controlled by means of heat exchangers in the loop. The catalyst is either sulfuric acid or sulfur dioxide. Up to 5 wt % cumene hydroperoxide remains unreacted. Under these conditions, dimethylphenylmethanol combines with cumene hydroperoxide to form dicumyl peroxide (3) [ 80-43-3], which upon subsequent heating to 110 – 140 °C in a short-residence-time plug-flow reactor is cleaved into phenol, acetone, and -methylstyrene.

This sequence suppresses the formation of condensates by approximately 50 %. Variants of this two-stage process have been patented [36].

Product Separation. In the Hercules process the cleavage mixture is neutralized with base and then fed to a separation column. The overheads from this column contain acetone, -methylstyrene, and cumene; acetone is recovered by distillation, and -methylstyrene is hydrogenated without prior separation from cumene. This cumene stream is then recycled to the oxidizers. Phenol from the bottoms of the separation column is recovered by distillation.

In the Allied/UOP process the cleavage mixture is treated with an ion-exchange resin to remove the acid catalyst and then is distilled. Acetone is removed first in a crude acetone column and purified by distillation with steam in an acetone-refining column. Cumene, -methylstyrene, and phenol are recovered by sequential distillation of the bottoms from the crude acetone column. Cumene is recycled to the oxidizers after it has been washed with alkali, and -methylstyrene is marketed.

4.2. Dehydrogenation of 2-Propanol

The hydration of propene [115-07-1] gives 2-propanol [67-63-0], which is then dehydrogenated to acetone. In the United States a C3 stream containing 40 – 60 % propene is used for the manufacture of 2-propanol (4).

The dehydrogenation of (4) is endothermic by 66.6 kJ/mol at 327 °C. The equilibrium constant, Kp (bar), obeys the following equation [37]:

The main side reaction is the dehydration of 2-propanol to propene. Other competing reactions are the self-condensation of acetone to diacetone alcohol, which leads to further condensation products.

A large number of catalysts for 2-propanol dehydrogenation have been studied, including copper, zinc, and lead metals, as well as metal oxides, e.g., zinc oxide, copper oxide, chromium-activated copper oxide, manganese oxide, and magnesium oxide. Inert supports, such as pumice, may be used.

Highly active catalysts are the precious metals platinum and ruthenium [39] or 0.25 % platinum on sodium-activated alumina [40]. These catalysts are particularly effective for the dehydrogenation of aqueous 2-propanol, which is obtained by hydration of propene.

All catalysts gradually lose activity because of a buildup of carbon deposits, so the operating temperature is increased as the catalyst ages. The catalyst is regenerated periodically by burning out the deposits. A good catalyst lasts for several months.

In a typical process, the azeotropic mixture of water and 2-propanol (87.8 wt % 2-propanol) is evaporated (sometimes using steam as carrier) and fed to a catalyst bed in a reactor specially designed for effective heat transfer. Hydrogen, produced downstream, may be mixed with the feed to prevent catalyst fouling. The reactor consists of a multitude of 2.5-mm steel tubes heated by oil, high-pressure steam, hot gases, or molten salts. The reaction produces hydrogen (> 99 % purity) as a valuable byproduct. This is separated by condensing all other components. Acetone is separated by distillation. The process is illustrated in Figure 2. Typical operating conditions are shown in Table 5.

Table 5. Gas-phase dehydrogenation of 2-propanol

Company; Catalyst; Temperature, °C; Pressure, kPa; Conversion, %; Selectivity, %; Yield, %; Reference. Standard Oil; ZnO/ZnO2; 400; 201 – 304; 98.2; 90.2; 88.6; [38]. Knapsack-Griesheim; CuO/Cr2O3/Na2O pumice; 300; 89.5; 99.0; 88.6; [39]. Toyo-Rayon; CuO/NaF/SiO2; 300; 93.4; 100; 93.4; [40]. Engelhard Industries; 5 % Pt/C; 310; 92.4; [41]. Usines de Melle; CuO/Cr2O3 /SiO2; 220; 151; 75; 98.2; 73.7; [42].

|  | Figure 2. Acetone production via 2-propanol dehydrogenation a) Reactor; b) Heating loop; c) Refrigeration; d) Distillation columns [Full View] |
| --- | --- |

4.3. Propene Oxidation [43], [44]

Direct oxidation of propene (Wacker – Hoechst process) currently is practiced only in Japan. A mixture of acetone (92 % selectivity) and propionaldehyde (2 – 4 % selectivity) is produced.

The process is analogous to the oxidation of ethylene to acetaldehyde by the Wacker process. The catalyst solution typically contains 0.045 M palladium(II) chloride, 1.8 M copper(II) chloride, and acetic acid [45]. The reaction usually is carried out in two alternating stages. In the first stage, air is used to oxidize the metal ions to the +2 oxidation state. In the second, air is removed and propene added. Palladium(II) oxidizes propene, and the resulting palladium(I) is reoxidized by the pool of copper(II). Reaction conditions are 1 – 1.4 MPa (10 – 14 bar) and 110 – 120 °C. Propene conversion is higher than 99 %.

Besides propionaldehyde, chlorinated carbonyl compounds and carbon dioxide also are formed. Acetone and the byproducts are removed from the catalyst solution by flash evaporation with steam and separated by fractional distillation. Propionaldehyde (bp 49 °C) distills in one column and acetone (bp 56 °C) distills in the other.

4.4. Oxidation of 2-Propanol [46]

In the absence of catalysts 2-propanol reacts with oxygen via a free-radical reaction to form acetone and hydrogen peroxide.

Until the mid-1980s the Shell process used hydrogen peroxide for the manufacture of glycerol from propene. The theoretical yield of acetone based on glycerol produced is 1.26 kg/kg. Acetone yields of about 90 % of theoretical were obtained.

4.5. Oxidation of p-Diisopropyl Benzene

Acetone is coproduced with hydroquinone [123-31-9] from p-diisopropylbenzene [100-18-5] in a process analogous to the phenol – acetone production from cumene.

In the Goodyear process [47] p-diisopropylbenzene is oxidized by oxygen in the presence of caustic. The p-diisopropylbenzene dihydroperoxide (5) [3159-98-6] formed is crystallized and washed with benzene. It is then dissolved in acetone and cleaved to hydroquinone and acetone in the presence of sulfuric acid. Next the acid is neutralized with ammonia and the ammonium sulfate formed is filtered. Acetone is recovered by distillation from the reaction mixture. Some of this acetone is recycled to the cleavage section while the rest passes through a finishing column for purification to at least 99.5 %. Eastman Chemical and Goodyear Tire & Rubber Company use this process in the United States. Annual US capacity is estimated to be 18 – 20 t/a. Sumitomo Chemical Company and Mitsui Petrochemical Industries of Japan use a similar process to produce p-cresol from cymene. Their annual capacity of acetone byproduct is 48 000 t.

4.6. Fermentation of Biomass

The fermentation of cornmeal or molasses by various members of the Clostridium genus yields a mixture of 1-butanol, acetone, and ethanol in 2 % overall concentration. The products are recovered by steam distillation and then fractionated.

The process was started during World War II to provide acetone needed for the manufacture of cordite. The last operating plant in the United States (Publicker Industries) closed in 1977.

The mixture of butanol, acetone and ethanol produced has been considered for use as a gasoline substitute in France [48]. Research aimed at increasing the concentration of useful products obtained in the process was carried out in the United States in the early 1980s [49]. The future of the fermentation process is tied to the availability of petrochemical feedstocks. High oil prices during the oil crises of the mid to late 1970s led to renewed interest in the process. Given the low oil prices of the 1990s and the ready availability of feedstocks at reasonable prices, it does not appear that these processes can compete under current conditions.

5. Environmental Protection

Because approximately 70 % of acetone is produced from cumene, a close examination of this process is warranted. Potential pollution sources in a phenol – acetone plant are emissions to the atmosphere and liquid discharge. Atmospheric emissions from the phenol – acetone process in the late 1970s have been estimated [29]. However over the past 20 years, and particularly in the 1990s with the renewal of the Clean Air Act, these emissions have been reduced significantly.

Aqueous streams containing significant amounts of organic substances arise from the various wash operations and sumps at the plant. Insoluble material is recovered by decantation. Phenol and acetone (0.5 – 3 wt % each) are the most abundant organic compounds remaining in the water after decantation. There are also minor quantities (0.001 – 0.1 wt %) of cumene, -methylstyrene, dimethylphenylmethanol, acetophenone, formaldehyde, formic acid, and various condensates. Of these compounds, phenol, formaldehyde, and formic acid are listed as hazardous substances in the U.S. Federal Water Pollution Control Act [50], but only phenol is present in sufficient quantities to require removal. Phenol is removed from the aqueous solution by solvent extraction, steam stripping, or adsorption on carbon or resins [51] and subsequently is recovered. The recovered phenol is valuable enough to pay for the capital and operating expenses of phenol abatement. Residual phenol in the water (10 – 500 ppm) is destroyed by biological degradation.

The federal regulatory status of acetone has changed. Acetone was granted VOC-exempt status by EPA on June 16, 1995 [53]. As of August, 1997, forty-four states had promulgated similar state rules. In states that have not yet promulgated state exemptions, acetone may technically still be regulated as a VOC. Acetone is not listed as a hazardous air pollutant (HAP) under section 112(b) of the Clean Air Act (CAA), or as an extremely hazardous substance under EPCRA Section 302. Acetone is also not listed as a priority pollutant under the Clean Water Act. It has been approved under the CAA as a substitute for ozone-depleting substances. Acetone was removed from the Federal Emergency Planning and Community Right-to-Know Act (EPCRA Section 313) list in June of 1995. Acetone is not regulated as a known or suspected carcinogen, and the National Toxicology Program (NTP) has recommended against testing for carcinogenicity because of its low toxicity and absence of any evidence supporting the carcinogenic potential of acetone.

Acetone is listed as a “U” waste under the Resource Conservation and Recovery Act (RCRA) based on its ignitability. “U” wastes are commercial chemicals that must be treated as hazardous wastes when discarded. Because of its RCRA listing it is included in the list of hazardous substances in the Superfund statute (Comprehensive Environmental Response, Compensation, and Liability Act).

6. Quality Specifications and Analysis

Acetone is produced industrially in relatively high purity, the main impurity being water. Table 6 summarizes the quality requirements for commercial 99.5 % acetone. Methods for preparing very high purity acetone from the commercial material are given in reference [54].

Table 6. Standard specifications for acetone, ASTM D329–90

Property; ASTM test; Specification. Relative density; D268. 20/20 °C; 0.7910 – 0.7930. 25/25 °C; 0.7865 – 0.7885. Color; D1209;  5 on platinum-cobalt scale. Distillation range; D1078; 1.0 °C, including 56.1 °C. Nonvolatile matter; D1353;  5 mg/100 mL. Odor; D1296; characteristic, nonresidual. Water; D1364; 0.5 wt % *. Acidity (as acetic acid); D1613; 0.002 wt %. Water miscibility; D1722; passes test. Alkalinity (as ammonia); D1614; 0.001 wt %. Permanganate time; D1363; 30 min at 25 °C.

*  This water limit ensures that the material is miscible without turbidity with 19 volumes of 99 % heptane at 20 °C (ASTM D1476).

Gas chromatography is the most widely used method for the quantitative analysis of acetone. For example, good separation of acetone from other low-boiling organic compounds can be obtained on a 30 m × 0.32 mm Carbowax capillary column. Extensive data on packed column separations are compiled in [55]. Infrared (carbonyl absorption, 1711 cm–1) and 1H NMR (singlet at ca. 1.05 ppm) spectroscopy may be used for both qualitative and quantitative analysis.

7. Storage and Transportation

Acetone has a low flash point; therefore, all shipping and storage containers must carry a red, diamond shaped “flammable liquid” label. Strict precautions should be taken to guard against fire hazards whenever acetone is handled. All wiring should be installed as described in Article 500 of the U.S. National Electrical Code or corresponding regulations in other countries. Explosion-proof motors, switches, etc., should be used. Accumulation of static electricity should be prevented by grounding and humidity control. Use of spark-resistant tools is recommended. Small fires may be controlled by use of carbon dioxide or dry chemical extinguishers. “Alcohol”-type foam should be used on larger fires; water spray will reduce the intensity of the flame.

Contact of acetone with oxidants should be avoided because it may lead to explosion [5]. Contamination with chlorinating agents may lead to the formation of toxic chloroketones. Prolonged exposure to direct sunlight may result in the formation of carbon monoxide. Packaging requirements for acetone are described in paragraph 49 CFR 173.242 (bulk), Bulk Packaging with Packaging for Certain Medium Hazard Liquids and Solids, Including Solids with Dual Hazards [56]. Transportation of acetone is covered in paragraph 49 CFR 172.101, Table of Hazardous Materials, of the Department of Transport Regulations [57]. The international transportation codes are IMDG Code D 3102; UN no. 1090; CFR 49, 172.101; RID (ADR, ADNR): Class 3, IATA: flammable liquid. The quantity of acetone in one package may not exceed 5 L in plastic, metal or aluminum, 1 L in glass, or 0.5 L in a glass ampoule in a passenger aircraft. The quantity of acetone in one package may not exceed 60 L in a cargo plane.

8. Uses

The main uses of acetone are as a chemical intermediate and as a solvent. The estimated 1995 acetone consumption by area of application in the USA is shown in Table 7 [59].

Table 7. Estimated 1995 US acetone consumption by area of application [59]

Use; Acetone used, 103 t. Acetone cyanohydrin/methacrylates; 500. Bisphenol A; 203. Aldol chemicals; 140 (total). Methyl isobutyl carbinol.; 35. Methyl isobutyl ketone.; 76. Others.; 22. Solvent use; 191. Other uses; 90. Total; 1124.

8.1. Methyl Methacrylate

Acetone is condensed with hydrogen cyanide to form acetone cyanohydrin (6) (see Section Acetone Cyanohydrin), which is next hydrolyzed with sulfuric acid to methacrylamide sulfate (7).

Further reaction with methanol yields methyl methacrylate (8) [80-62-6]. Approximately 0.70 kg of acetone is required per kilogram of methyl methacrylate produced.

Higher methacrylate esters may be produced either by transesterification of methyl methacrylate or by esterification of methacrylic acid (9) [79-39-0]; the latter is made by hydrolysis of methacrylamide sulfate:

At the end of 1995 there were 22 plants manufacturing MMA in the United States, Western Europe, and Japan. Five basic process routes have been commercialized: The acetone cyanohydrin route; two-stage oxidation of isobutylene to methacrylic acid followed by esterification; two-stage oxidation of tert-butyl alcohol to methacrylic acid followed by esterification; hydroformylation of ethylene to propionaldehyde, condensation with formaldehyde to methacrolein, oxidation, and esterification (BASF); and ammoxidation of tert-butyl alcohol to methacrylonitrile, which is hydrolyzed to methacrylamide sulfate and then esterified to MMA (Asahi). One new route has been announced by Mitsubishi Gas Chemicals, which is a recycle version of the acetone cyanohydrin route. A 41 × 103 t/a plant to make MMA and MAA started up in 1997. Worldwide production of MMA in 1996 was about 1682 × 103 t [60].

8.2. Bisphenol A (  Phenol Derivatives)

Bisphenol A (10), 4,4-isopropylidenediphenol [80-05-7] is manufactured by condensation of 2 mol phenol with 1 mol acetone in the presence of an acid catalyst:

Approximately 0.28 kg of acetone is required per kilogram of bisphenol A. In the 1990s, bisphenol A has had the fastest growing demand of the phenol derivatives. Four US. companies produce bisphenol A: Shell (Deer Park, Texas), General Electric (Mount Vernon, Indiana), Dow (Freeport, Texas), and Aristech (Haverill, Ohio). Estimated worldwide usage in 1995 was 1600 × 103 t [61].

8.3. Aldol Chemicals (see Section Diacetone Alcohol, also   Ketones)

These chemicals are produced by condensation of acetone. Two moles of acetone form 1 mol of diacetone alcohol, 4-hydroxy-4-methyl-2-pentanone (11) [123-42-2]. Subsequent dehydration yields mesityl oxide, 4-methyl-3-penten-2-one (12) [141-79-7]. Hydrogenation of (11) yields 2-methyl-2,4-pentanediol (13) [107-41-5]. By hydrogenation of (12) methyl isobutyl ketone (14) [108-10-1] is available; further hydrogenation produces 4-methyl-2-pentanol (15) [108-11-2]. Three moles of acetone are condensed to 1 mol of isophorone, 3,5,5-trimethyl-2-cyclohexen-1-one (16) [78-59-1].

Approximately 1.25 kg of acetone is used per kilogram of methylisobutyl ketone produced. The US 1996 production of ca. 100 × 103 t of MIBK consumed ca 75 × 103 t of acetone. The US manufacturers are: Eastman (Kingsport, Tennessee), Shell (Deer Park, Texas), and Union Carbide (Institute, West Virginia) [62]. Methyl isobutyl ketone is used as a solvent for nitrocellulose lacquers, vinyl polymers, and acrylic resins [62].

Diacetone alcohol, mesityl oxide, and isophorone are used mainly as solvents. Their use has diminished in the USA because of their status as photochemically reactive solvents under Rule 66 of Los Angeles County [52]. The primary use of 4-methyl-2-pentanol is for ore flotation, and 2-methyl-2,4-pentanediol is used in hydraulic fluids and printing inks.

8.4. Solvent Uses

Acetone is used as a solvent for paints, varnishes, and lacquers. It is also used as a wash solvent for these materials and as a spinning solvent in the manufacture of cellulose acetate. A small amount of acetone is used as a solvent for acetylene. Approximately 191 × 103 t of acetone was consumed in direct solvent applications. The major solvent market for acetone is in paints and coatings. Consumption of acetone in these applications increased by 9 × 103 t in 1995 because of its delisting as a VOC.

The pharmaceutical industry is also a large consumer of acetone for the manufacture of pharmaceuticals, vitamins, and cosmetics. In 1995 acetone consumption in pharmaceutical and cosmetic applications was (36 – 43) × 103 t.

The removal of acetone from the VOC list has made it more attractive as a solvent, particularly for replacing other chemicals on the VOC list. Following the EPA's August 1995 action, eight states automatically delisted acetone as a volatile organic compound. As of August, 1997, forty-four states had promulgated similar state rules.

. Economic Aspects

The United States acetone capacity by manufacturer and production process can be found in [63]. World capacity data are given in Table 8 [59]. The United States acetone production for the last two decades is summarized in Table 9 [59]. Worldwide production in 1994 was: USA 1281 × 103 t/a; Western Europe 1200 × 103 t/a; Asia 746 × 103 t/a.

Table 8. World acetone capacity, 1995 [59], [64]

Location; Capacity, 103 t. United States; 1281. Mexico; 22.3. Western Europe; 1200. Japan; 475. Other Asia; 271.4. Others; 592. World total; 3842.

Table 9. United States acetone production, 103 t [59]

Year; From 2-propanol; From cumene; Other; Total. 1970; 379; 329; 26; 734. 1975; 312; 433; 745. 1980; 250; 693; 943. 1985; 41; 768; 3; 812. 1990; 70; 972; 17; 1059. 1994; 73; 1110; 20; 1203.

In 1995 the phenol process accounted for 83 % of all acetone made and 9 % was derived from 2-propanol; only 8 % was produced by all other processes (66 × 103  t/a from propene oxidation in Japan).

The economics of acetone are unusual. The bulk of acetone is made as a coproduct with phenol. Consequently, phenol demand determines to a large extent the availability of acetone. Fortunately acetone serves to some extent the same markets as phenol does. These are mainly the automotive and housing markets. As a result, when economic conditions place a demand on phenol, acetone demand also increases. However, an unusually steep demand for phenol may render acetone an overabundant byproduct [63].

Because the single largest use of acetone is as an intermediate in the manufacture of methacrylates, alternate routes to methacrylates, such as the oxidation of C4 hydrocarbons, are a potential threat to acetone. When C4 hydrocarbon stocks are in demand for the manufacture of gasoline additives, they are not likely to be used for acrylate manufacture. However, production of methacrylate by oxidation of C4 hydrocarbons started in Japan in 1982 [65].

Dehydrogenation of 2-propanol accounted for approximately 80 % of US acetone produced in 1960. As the cumene-based process expanded, the 2-propanol contribution shrank to 52 % in 1970, 27 % in 1980, and 6 % in 1994. If acetone supply does outstrip demand, it is likely that production based on 2-propanol will be further curtailed.

Phenol manufacturing processes that do not coproduce acetone have been developed partly because of fears that there will be a supply/demand imbalance between phenol and acetone. DSM and its licensees produce phenol by the air oxidation of toluene via benzaldehyde and benzoic acid. Subsequently, benzoic acid is decomposed to phenol with a copper catalyst. This route provides phenol, benzaldehyde, and benzoic acid. Mitsui Petrochemical has developed a recycle scheme for converting the acetone byproduct from the cumene hydroperoxide rearrangement back to propylene for feed to the front end of the cumene process. Solutia (formerly Monsanto) and the Boreskov Institute of Catalysis have developed a catalyst capable of oxidizing benzene in high yield with nitrous oxide to give directly phenol. The key to this process is the inexpensive nitrous oxide available as a byproduct from the manufacture of adipic acid. Asahi Chemical has patented a process in which benzene is partly hydrogenated to cyclohexene. The cyclohexene is hydrolyzed to cyclohexanol or oxidized to cyclohexanone; dehydrogenation then gives phenol [66].

10. Toxicology and Occupational Health

Acetone is one of the least toxic industrial solvents [67]. However, exposure to vapor at high concentration should be avoided because it can produce temporary narcosis and cause slight eye irritation. Repeated skin contact with the liquid defats the skin and may cause dermatitis. The liquid is also irritating to the eyes and may cause moderate corneal injury.

The ACGIH [68] has adopted a time-weighted average threshold limit value (TLV-TWA) of 750 ppm, 1.78 g/m3, and a short-term exposure limit (TLV-STEL) of 1000 ppm, 2.375 g/m3, for acetone. OSHA regulations [69] set a limit of 2.4 g/m3. However, the ACGIH has issued a notice of intended change to lower the TLV to 500 ppm (1.19 g/m3), and the STEL to 750 ppm (1.78 g/m3) [70]. Exposure limits (TLV-TWA) adopted by the main industrial countries are shown in Table 10 [71]. The odor threshold of acetone is 48 mg/m3 provided desensitization has not occurred. Animal studies have shown acetone to be relatively nontoxic [67], [71].

Table 10. Time-weighted average threshold limits for acetone [71]

Country; Limit, mg/m3. Australia; 1780. Belgium; 1780. Germany; 2400. Italy; 1000. Japan; 470. Netherlands; 1780. Former Soviet Union; 475. United States; 1780.

|  | LD50 (oral, mouse) 4 – 8 g/kg [67] |
| --- | --- |
|  | LD50 (oral, rabbit) 5.3 g/kg [67] |
|  | LD50 (intraperitoneal, mouse) 1.3 g/kg [67] |
|  | LD50(dermal, rabbit) 20 g/kg [71] |
|  | Nonteratogenic at 39 or 78 mg per chicken egg [67] |
|  | Nonmutagenic in the Salmonella/microsome (Ames) test [67] |
|  | Nononcogenic on skin of mice, three times a week for 1 year [67] |
|  | Moderate corneal injury on rabbit eye [67] |
|  | Environmental toxicity: LC50 (rainbow trout, 96 h) 5540 mg/L; LC50 (bluegill sunfish, 96 h) 8300 mg/L. |

Minimum lethal concentration in air (LC50) was 50.1 g/m3 (21 100 ppm) for rats exposed to acetone vapor for 8 h and 44 g/m3 (18 500 ppm) for mice exposed for 4 h. Human exposure to acetone has been studied [72-77]. Eye and nasal irritation were observed at 1.2 g/m3. Other effects are similar to those of ethanol, but the anesthetic potency is greater. Prolonged or repeated skin contact may defat the skin and can produce dermatitis. Direct contact of acetone with the eyes can produce corneal injury. Acetone is a solvent of comparatively low acute and chronic toxicity. However it does not have sufficient warning properties to prevent repeated exposures to vapors, which may have adverse effects. There have been no reports that prolonged inhalation of low vapor concentrations result in any serious chronic effects in humans.

Cases of acetone poisoning are rare [67]. In one case, a solvent mixture containing 90 % acetone and 9 % pentane was used to set a cast for a broken leg on a 10-year-old boy [78]. The boy became ill and collapsed 12 h later. After the cast was removed the boy became comatose but recovered completely in 4 days. In another case, a 42-year-old man ingested 200 mL of acetone and became comatose for 12 h [79]. Subsequently, hyperglycemia was diagnosed and attributed to acetone ingestion.

Acetone does not cause neurotoxicity, an occupational disorder caused by exposure to some higher aliphatic ketones and related compounds [80]. Acetone vapor is absorbed with 75 % efficiency by the lungs [76]. The half-life for the elimination of acetone by expired air is approximately 5 h. The metabolism of acetone may proceed through 1,2-propanediol [67], [81], [82].

11. Derivatives

11.1. Acetone Cyanohydrin

Acetone cyanohydrin, 2-hydroxy-2-methylpropanenitrile, CH3C(OH)(CN)CH3 [75-86-5] is an important chemical intermediate for the manufacture of methacrylates (  Methacrylic Acid and Derivatives). Small amounts of acetone cyanohydrin are used in insecticide manufacture.

Physical Properties. Acetone cyanohydrin is a colorless liquid. The pure compound is practically odorless but usually has an odor of bitter almonds because of traces of hydrogen cyanide. It is very soluble in water and polar solvents and sparingly soluble in hydrocarbons.

Mr  85.11, mp –19 °C, relative density , relative vapor density (air = 1) 2.96, refractive index , flash point 73 °C.

Vapor pressure 1.3 1.2p, kPa. t, °C 5.3 3.1; 95; 82; 74; 72.

Chemical Properties. Acetone cyanohydrin exhibits the combined characteristics of a nitrile and an alcohol. Under neutral and particularly under alkaline conditions it decomposes to acetone and hydrogen cyanide. The decomposition is inhibited by the addition of small amounts of sulfuric or phosphoric acid; consequently, technical-grade material is stabilized by addition of 0.01 wt % of either acid. Reaction with concentrated sulfuric acid converts acetone cyanohydrin to methacrylamide sulfate; subsequent neutralization with ammonia yields methacrylamide [79-39-0]; alcoholysis yields methacrylate esters; alternatively, hydrolysis gives methacrylic acid [79-41-4].The yield of methacrylic acid is improved if 3 – 10 % oleum is used instead of 100 % sulfuric acid in the reaction with acetone cyanohydrin [83].

Production. Acetone cyanohydrin is manufactured by the base-catalyzed condensation of acetone with hydrogen cyanide according to the following mechanism [84]:

The reaction is reversible but formation of the cyanohydrin is quite favorable; the equilibrium constant is 28 L/mol at 20 – 25 °C [85]. The reaction usually is carried out in the liquid phase. Representative catalysts used industrially are sodium hydroxide [86], potassium hydroxide [87], potassium carbonate [88], and anion-exchange resins. A schematic flowsheet of the Rohm & Haas [89] process is shown in Figure 3. Acetone and liquid hydrogen cyanide are fed continuously to a cooled reactor along with an alkaline catalyst. The catalyst is next neutralized with sulfuric acid and the resulting salt is removed by filtration. The crude product is then distilled in a two-stage process. The overheads from the first column consist mainly of acetone and hydrogen cyanide, which are recycled to the reactor. The second column removes water overhead and leaves 98 % pure acetone cyanohydrin at the bottom. Nitto Chemical claims a two-column distillation system that delivers acetone cyanohydrin of 99.1 % purity [90]. The manufacture of acetone cyanohydrin produces no byproducts other than small amounts of sulfate salts formed during catalyst neutralization. However, the conversion of acetone cyanohydrin to methacrylate in the classical process produces a large amount of ammonium sulfate byproduct, which is usually pyrolyzed to sulfuric acid. An alternative process developed by Mitsubishi Gas Chemical recycles the HCN via formamide. In this process HCN is not directly consumed and no ammonium sulfate is formed.

|  | Figure 3. Rohm and Haas acetone cyanohydrin process [89] a) Reactor; b) Cooling; c) Filter press; d) Concentrator; e) Concentrator; f) Condenser; g) Vacuum jet; h) Pump for acetone and HCN recycle [Full View] |
| --- | --- |

Uses. By far the largest use of acetone cyanohydrin is as an intermediate in the synthesis of methyl methacrylate [80-62-6], methacrylic acid, and higher methacrylate esters. A small amount is converted to methacrylamide. The estimated amount of acetone used in the manufacture of methacrylate esters in 1995 in the United States was 500 × 103 t (Table 7). Based on this estimate, the quantity of acetone cyanohydrin produced as an intermediate was approximately 714 × 103 t.

Certain esters of acetone cyanohydrin, such as 2-chloroethyl- -cyanoisopropylsulfite [91] and -cyanoisopropyl-2,6-dichlorobenzoate [92] have strong fungicidal, herbicidal, and insecticidal properties. Nitrilurethanes made from acetone cyanohydrin and substituted phenylisocyanates are intermediates for 4-iminooxazolidin-2-ones, which are plant growth inhibitors [93].

Transportation and Toxicology. Most acetone cyanohydrin is consumed on site for the manufacture of methacrylates. Because of its high toxicity, acetone cyanohydrin is classified as a poison B and all shipping containers must carry a “poison inhalation hazard” label [57]. Transport aboard a passenger-carrying or cargo aircraft is forbidden [58].

Acetone cyanohydrin has the following toxicologic properties [94]: LD50 (oral, rat) 18.6 mg/kg; LD50 (oral, rabbit) 14 mg/kg; LD50 (dermal, rabbit) 17 mg/kg; LD50 (dermal, guinea pig) 150 mg/kg; aquatic toxicity rating (TLm96) 10 to 1 ppm. Threshold limit values have been established as cyanide: TLV-STEL, 4.7 ppm ceiling limit (5 mg/m3) [68].

11.2. Diacetone Alcohol

Diacetone alcohol, 4-hydroxy-4-methyl-2-pentanone [123-42-2]

is a dimer of acetone that is used as a solvent and as an intermediate for the manufacture of mesityl oxide, methyl isobutyl ketone, and hexylene glycol. 

Physical Properties. Diacetone alcohol is a colorless liquid of mild odor. It is miscible with water and polar solvents and is an excellent solvent for cellulose acetate and various oils and resins. Mr 116.16, mp – 47 °C, relative density, , refractive index , heat of vaporization at the boiling point at 101.3 kPa 357.1 kJ/kg, specific heat capacity (20 °C) 18.84 kJ kg–1 K–1, thermal expansion coefficient (20 °C) 0.00099 K–1, viscosity at 20 °C 2.9 mPa · s, surface tension (20 °C) 31.0 mN/m, dielectric constant (25 °C) 18.2, heat of combustion 3544.5 kJ/mol, flash point 58 °C, lower explosion limit in air 2.6 vol %, auto ignition temperature 624 °C.

Vapor pressure 0.108p, kPa. t, °C101.3 1.7; 168.1; 61.7; 20.0.

Azeotropic mixture with water: bp 98.8 °C/ 101.3 kPa, 12.7 wt % diacetone alcohol.

Chemical Properties. Diacetone alcohol dehydrates readily in the presence of acids to form mesityl oxide. Catalytic hydrogenation [95] yields hexylene glycol. In the presence of bases, diacetone alcohol reverts to acetone. The reaction is first order in diacetone alcohol and first order in base. The dissociation is accompanied by volume increase; consequently, the reaction is inhibited by pressure [96]. The second-order rate constant is 7.33×10–4  L mol–1 s–1 at 101.3 kPa and decreases to 2.38×10 –4  L mol–1 s–1 at 4.05×105  kPa. In neutral aqueous solution, dissociation to acetone is very slow at room temperature; it reaches approximately 0.1 % in 1 year.

Diacetone alcohol may be acylated by acetic anhydride under mild conditions to form diacetone alcohol acetate [1637-25-8], 4-methyl-4-acetyloxy-2-pentanone, which is claimed as an octane-improving gasoline additive [97]. Condensation with urea in the presence of sulfuric acid yields diacetone-monourea, 3,4-dihydro-4,4,6-trimethyl-2(1H)-pyrimidone [4628-47-1] [98], [99].

The compound is claimed to improve the egg-laying capacity of hens [98]. 

Production. Diacetone alcohol is manufactured by self-condensation of acetone in the presence of a basic catalyst. The reaction is exothermic by 14.65 kJ/mol and is easily reversible. The equilibrium concentration of diacetone alcohol is 23.1 wt % at 0 °C and decreases with increasing temperature [100]. Kinetic considerations dictate, however, a higher temperature for the manufacture of diacetone alcohol. An optimum temperature range is 10 – 20 °C.

The self-condensation of acetone is carried out in continuous-flow reactors containing a solid alkaline catalyst, such as barium hydroxide or calcium hydroxide [101], [102]. Anion-exchange resins have been investigated [103], [104], but are not believed to be used commercially. Catalyst performance deteriorates with time, but may last up to 1 year. A patent [105] describes how addition of small amounts of methanol, ethanol, or 2-propanol to the reaction mixture retards catalyst deterioration. The selectivity for diacetone alcohol is 90 – 95 %. Mesityl oxide and higher condensates, such as triacetone alcohol, are the main products. The acetone solution of the crude product is neutralized, e.g., with phosphoric acid [106], prior to concentration under reduced pressure. The recovered acetone is recycled to the condensation reactor and the acidity adjusted by subsequent addition of a base, such as triethylamine. After such treatment, diacetone alcohol of 99.68 % purity was obtained by vacuum distillation [106].

Uses. Diacetone alcohol is an excellent solvent for many natural and synthetic resins. It is used in the coatings industry, especially for hot lacquers, and is also used as a solvent for nitrocellulose, cellulose acetate, and epoxy resins. However, its use has diminished because it is not exempt from restrictions under Rule 66 and related federal regulations [52]. Therefore, US sales have declined more than 50 % since 1978, reaching 10 000 t/a for the past several years and are expected to remain flat. A large portion of diacetone alcohol is used as an intermediate for the manufacture of mesityl oxide, methyl isobutyl ketone, and hexylene glycol.

Transportation and Toxicology. Diacetone alcohol has a flash point of 58 °C, and all transport containers must carry a “flammable liquid” label. Threshold limit values (TLV) for diacetone alcohol vapor at the workplace are: 50 ppm, 240 mg/m3 (TLV-TWA) [69]; MAK is 50 mL/m3, 240 mg/m3. These limits also apply to the other major industrial countries [71]. The toxicologic properties of diacetone alcohol are as follows [107]: LD50 (oral, rat) 4 g/kg; LD50 (intraperitoneal, mouse) 933 mg/kg; LD50 (dermal, rabbit) 13.5 g/kg; aquatic toxicity rating (TLm96) 1000 – 100 ppm.

11.3. Miscellaneous Derivatives

Acetone is used in the production of methyl amyl ketone (MAK, 2-heptanone). MAK is produced by the condensation of n-butyraldehyde and acetone. In 1995, US consumption of acetone for MAK was approximately 12 × 103 t, producing ca. 17.5 × 103 t of MAK. Eastman Chemical is the sole US producer. Acetone is also used to produce methyl isoamyl ketone (MIAK) by condensation of acetone with isobutyraldehyde. Eastman Chemical is the sole US producer of MIAK, which is primarily used in lacquers and surface coatings. Other minor uses of acetone include the manufacture of DuPont triazine herbicide Bladex [(3.5 – 3.6) × 103 t/a]; Ethoxyquin, a Monsanto antioxidant (2.7 × 103 t/a), as a raw material in the production of hexafluoroacetone, methylbutynol, and pseudoionone; and as an auxiliary blowing agent for the production of flexible polyurethane foam.

| Acetone |  |  |  |
| --- | --- | --- | --- |
| Figures |  |  |  |
|  | Figure 1. Cumene phenol – acetone process (Allied) a) Oxidizers; b) Flash column; c) Carbon adsorber; d) Alkaline extraction and wash; e) Cumene hydroperoxide decomposer; f) Dicumyl peroxide decomposer; g) Ion exchange; h) Crude acetone column; i) Acetone-refining column; j) Cumene column; k) -Methylstyrene column; l) Phenol column; m) Phenol residue topping column AMS = -methylstyrene [Full View] |  |  |
|  | Figure 2. Acetone production via 2-propanol dehydrogenation a) Reactor; b) Heating loop; c) Refrigeration; d) Distillation columns [Full View] |  |  |
|  | Figure 3. Rohm and Haas acetone cyanohydrin process [89] a) Reactor; b) Cooling; c) Filter press; d) Concentrator; e) Concentrator; f) Condenser; g) Vacuum jet; h) Pump for acetone and HCN recycle [Full View] |  |  |