Introduction

Acetone, 2-propanone, dimethyl ketone, CH3COCH3, [67-64-1], is the first and most important member of the homologous series of aliphatic ketones. It is a colorless, mobile liquid widely used as a solvent for various polymers. Its largest application, however, is as an intermediate in the synthesis of methyl methacrylate, bisphenol A, diacetone alcohol, and other products. Acetone was first manufactured by the dry distillation of calcium acetate [62-54-4]. Calcium acetate was originally a product of wood distillation, and later was obtained by fermentation of ethanol. Carbohydrate fermentation directly to acetone and butyl and ethyl alcohols displaced these processes in the 1920s. The carbohydrate route, in turn, was replaced in the 1950s and 1960s by the 2-propanol dehydrogenation process and by the oxidation of cumene to phenol [108-95-2] plus acetone. Together with direct propene oxidation, these methods account for over 95 % of the acetone produced worldwide.

Production

Approximately 83 % of the acetone produced worldwide is manufactured from cumene as a coproduct with phenol. In the United States and Western Europe dehydrogenation of 2-propanol is also important, whereas in Japan catalytic oxidation of propene is used as a second process. Cumene, 2-propanol, and propene together as starting materials account for over 95 % of the acetone produced worldwide. Because propene is used in the manufacture of both cumene and 2-propanol, propene is the ultimate raw material, for the production of acetone.

Small amounts of acetone are made by oxidation of p-diisopropyl benzene and of p-cymene. Coproducts from these reactions are hydroquinone and p-cresol, respectively. Acetone is also produced by propene oxidation and as a by product of acetic acid manufacture.  Fermentation of cornstarch and molasses to acetone and 1-butanol was important in the past. It is believed to be practiced today to a limited extent in several countries.

Cumene Oxidation (Hock Process)

Propene [115-07-1] is added to benzene [71-43-2] to form cumene [98-82-8], which is then oxidized by air to cumene hydroperoxide (1), and cleaved in the presence of an acid catalyst. Phenol [108-95-2] and acetone produced in the process are recovered by distillation.

|  | Figure 1. Cumene phenol – acetone process (Allied) a) Oxidizers; b) Flash column; c) Carbon adsorber; d) Alkaline extraction and wash; e) Cumene hydroperoxide decomposer; f) Dicumyl peroxide decomposer; g) Ion exchange; h) Crude acetone column; i) Acetone-refining column; j) Cumene column; k) -Methylstyrene column; l) Phenol column; m) Phenol residue topping column AMS = -methylstyrene [Full View] |
| --- | --- |

The alkylation of benzene by propene proceeds under typical Friedel – Crafts conditions. In 1996, a number of processes using zeolite catalysts came on-stream. The cumene produced is purified by chemical means and refined by distillation to 99.9 % minimum purity. Oxidation-grade cumene must meet strict quality standards. The newer zeolite-based processes have led to slightly tighter specifications (Table 3) [15].

Specifications for oxidation-grade cumene (zeolite process) [15]

Property; ASTM test; Specification. Appearance; Clear, colorless liquid. Color, Pt – Co scale; D1209–79; 15 max.. D891–59; 0.864 – 0.867. Acid wash color, W scale; D848–62; 2 max.. Sulfur compounds; D853–47; Free from H2S and SO2. Copper corrosion; D849–47; No iridescence, gray or black. Distillation range; D950–56; 1.0 °C max.. Cumene content; 99.93 % min.. Phenolics content; 5 ppm max.. Cumene hydroperoxide content; 200 ppm max.. Sulfur content; 0.1 ppm max..

Cumene Oxidation. The oxidation of cumene is a free-radical chain reaction [16]. The chain initiator is cumene hydroperoxide, the main product of the reaction. The rate of oxygen consumption can be approximated by the following expression:

cRH and cROOH are the concentrations of cumene and cumene hydroperoxide, respectively; ki , kp , and kt are the rate constants for chain initiation, propagation, and termination.

The expression shows that the rate of oxidation is zero in the absence of cumene hydroperoxide. This is not exactly true, because the expression is only an approximation; but the oxidation of cumene does require long induction periods when starting with pure cumene. Consequently, the industrial oxidation always is carried out in a series of continuous reactors; the concentration of cumene hydroperoxide is at least 8 wt % in the first reactor. Because the sum of cRH and cROOH remains roughly constant during the reaction, the rate of reaction cannot increase indefinitely as cROOH increases. The maximum rate is achieved at approximately 35 wt % cumene hydroperoxide.

Besides cumene hydroperoxide, both dimethylphenylmethanol and acetophenone are also formed as byproducts during this oxidation. These arise from a secondary chain reaction that proceeds in parallel with the main chain. Byproduct formation is accelerated as the concentration of cumene hydroperoxide increases. For these reasons, most plants operate between 25 and 40 wt % in the last oxidation reactor.

A minor but significant byproduct of the oxidation is dicumyl peroxide [80-43-3]. This arises during the termination of the chain reaction. Dicumyl peroxide also contributes to chain initiation [17], but to a much lesser degree than cumene hydroperoxide. Other minor byproducts are formaldehyde and formic acid, which are produced along with acetophenone by methyl group degradation.

The oxygen needed for cumene oxidation is supplied by air. Use of pure oxygen has been suggested [18] but is disfavored by both economic and safety considerations [19]. At low initiation rates, the rate of the reaction is essentially independent of the oxygen concentration at a partial pressure of oxygen over 33 kPa (0.33 bar) [20]. A detailed study of the rate of oxygen uptake in a bubble column as a function of temperature and partial oxygen pressure has been made [21]. The study served as a basis for a mathematical model of the oxidation [22], [23].

Two cumene oxidation processes are used in the United States, which with minor variations are practiced also in the rest of the world [19], [22]. One process was developed by Hercules and licensed by Kellogg (previously BP/Hercules) and GE/Lummus [24]. The other process was developed by Allied and licensed by Allied/UOP [25-28].

In both processes several reactors are employed in series. Fresh and recycled cumene are fed to the first reactor, which may operate at 8 – 12 wt % cumene hydroperoxide. The concentration increases by 4 – 8 wt % in each successive reactor; the last reactor may operate at 25 – 40 wt % cumene hydroperoxide. Fresh air is pumped in parallel to each reactor and vented at the top after removal of organic vapors.

In the Hercules process, the oxidation of cumene is carried out at approximately 620 kPa (6 bar)/90 – 120 °C, in the presence of a sodium carbonate buffer [29]. Under these conditions the residence time in the oxidizer train is 4 – 8 h and the hydroperoxide molar selectivity 90 – 94 %. The spent air is first passed through water cooled and refrigerated condensers in series to remove organic vapors, and is finally vented. The condensate is returned to the oxidizers after treatment [29].

In the Allied/UOP process (Fig. 1) the oxidation is carried out at atmospheric pressure. No buffer or promoter is added, but great care is taken to wash all streams recycled to the oxidizer with alkali and water [27]. Temperature is maintained at 80 – 100 °C. Residence time in the oxidizer train is 10 – 20 h and hydroperoxide molar selectivity is 92 – 96 %. Spent air is vented after organic vapors are removed by condensation followed by activated carbon adsorption. The recovered materials are washed with aqueous sodium hydroxide and water, then returned to the oxidizers.

The oxidation of cumene generates approximately 116 kJ of heat per mole of cumene oxidized [30]. Part of this heat is carried to the condensers by organic vapors (this part is larger in the Allied/UOP process because of the lower operating pressure). The rest is removed by heat exchangers.

In both processes cumene hydroperoxide is concentrated to over 80 wt % by evaporation of excess cumene. In the Hercules process the oxidate is washed with water prior to distillation in order to remove the buffer added during oxidation.

Cumene Hydroperoxide Cleavage. Cumene hydroperoxide [80-15-9] is cleaved to phenol and acetone in the presence of catalytic amounts of a strong acid. The acid most commonly used is sulfuric acid. Sulfur dioxide is used as catalyst in the Allied/UOP process. Several patents claim the use of solid acids as catalysts for the decomposition [31]. Strongly acidic resins have been used to that effect in the Soviet Union [32]. However, all commercial units use strong mineral acids or SO2, which generates sulfuric acid in situ, as catalysts.

The cleavage proceeds through an ionic mechanism and releases approximately 252 kJ/ mol of cumene hydroperoxide decomposed [30]. The reaction rate accelerates rapidly with increasing temperature. Consequently, decomposition of cumene hydroperoxide commonly is carried out in a continuously stirred reactor in which the steady-state concentration of cumene hydroperoxide is maintained at a low level. The heat released by the reaction can be used to estimate the concentration of hydroperoxide present in the reactor at any time [33].

The molar selectivity of the cleavage to phenol and acetone is higher than 99.5 % at temperatures below 70 °C, but it decreases at higher temperatures as increasing amounts of dimethylphenylmethanol and acetophenone (in addition to those present in the cumene oxidate) are formed (Table 4) [28].

Table 4. Formation of byproducts during cumene hydroperoxide decomposition *

Temp. °C; Molar ratio (byproducts/phenol)×100. DMPM equivalents**; Acetophenone. 70; 0.36; 0.06. 90; 0.61; 0.06. 110; 1.24; 0.15. 122; 2.19; 0.25. 146; 5.04; 0.69.

*  Pure cumene hydroperoxide added to phenol – acetone – cumene solution containing initially 0.5 wt % water and 100 ppm sulfuric acid; data from [28].

**  Sum of dimethylphenylmethanol, -methylstyrene, and their condensation products.

Acetone produced during the cleavage of cumene hydroperoxide can react further. Oxidation by cumene hydroperoxide forms hydroxyacetone [34] to the extent of 0.2 – 0.5 % of acetone present. Self-condensation catalyzed by acid results in diacetone alcohol and mesityl oxide. Conversion of acetone to these condensates is normally below 0.1 % but may increase upon protracted exposure to strong acid. For example, when the cumene hydroperoxide cleavage was carried out with refluxing acetone using a sulfonic acid resin as catalyst, approximately 1.7 % of acetone was transformed to diacetone alcohol and mesityl oxide [32].

Under the conditions of the cumene hydroperoxide cleavage, dimethylphenylmethanol is dehydrated to -methylstyrene (2) and also forms undesirable condensates.

Compound (2) may be either hydrogenated [35] to cumene and recycled, or recovered and sold.

In the Hercules process, the cumene hydroperoxide decomposition is carried out in a constant-flow, stirred tank reactor in the presence of sulfuric acid or another strong mineral acid [19], [28]. The acid is added to the reactor as an acetone solution. The reactor temperature is maintained below 95 °C by refluxing approximately 2.8 kg acetone per kilogram cumene hydroperoxide. The ratio of the quantity of reflux to the quantitiy of hydroperoxide fed is used as a monitor of the cleavage reaction [19].

In the Allied/UOP process the cumene hydroperoxide cleavage is carried out at 60 – 80°C in a pressurized, constant-flow, back-mixed reactor. Temperature is controlled by means of heat exchangers in the loop. The catalyst is either sulfuric acid or sulfur dioxide. Up to 5 wt % cumene hydroperoxide remains unreacted. Under these conditions, dimethylphenylmethanol combines with cumene hydroperoxide to form dicumyl peroxide (3) [ 80-43-3], which upon subsequent heating to 110 – 140 °C in a short-residence-time plug-flow reactor is cleaved into phenol, acetone, and -methylstyrene.

This sequence suppresses the formation of condensates by approximately 50 %. Variants of this two-stage process have been patented [36].

Product Separation. In the Hercules process the cleavage mixture is neutralized with base and then fed to a separation column. The overheads from this column contain acetone, -methylstyrene, and cumene; acetone is recovered by distillation, and -methylstyrene is hydrogenated without prior separation from cumene. This cumene stream is then recycled to the oxidizers. Phenol from the bottoms of the separation column is recovered by distillation.

In the Allied/UOP process the cleavage mixture is treated with an ion-exchange resin to remove the acid catalyst and then is distilled. Acetone is removed first in a crude acetone column and purified by distillation with steam in an acetone-refining column. Cumene, -methylstyrene, and phenol are recovered by sequential distillation of the bottoms from the crude acetone column. Cumene is recycled to the oxidizers after it has been washed with alkali, and -methylstyrene is marketed.

6. Quality Specifications and Analysis

Acetone is produced industrially in relatively high purity, the main impurity being water. Table 6 summarizes the quality requirements for commercial 99.5 % acetone. Methods for preparing very high purity acetone from the commercial material are given in reference [54].

Table 6. Standard specifications for acetone, ASTM D329–90

Property; ASTM test; Specification. Relative density; D268. 20/20 °C; 0.7910 – 0.7930. 25/25 °C; 0.7865 – 0.7885. Color; D1209;  5 on platinum-cobalt scale. Distillation range; D1078; 1.0 °C, including 56.1 °C. Nonvolatile matter; D1353;  5 mg/100 mL. Odor; D1296; characteristic, nonresidual. Water; D1364; 0.5 wt % *. Acidity (as acetic acid); D1613; 0.002 wt %. Water miscibility; D1722; passes test. Alkalinity (as ammonia); D1614; 0.001 wt %. Permanganate time; D1363; 30 min at 25 °C.

*  This water limit ensures that the material is miscible without turbidity with 19 volumes of 99 % heptane at 20 °C (ASTM D1476).