ADVANCE PROCESS DYNAMICS

## Instructor:  Prof. R.Shinnar

Submitted by

Pratik Sheth

M.S (Chemical Engg)

ID:-610472843

Department of Chemical Engineering

City College of New York

Process Control for Production of Nitric Acid: -

Introduction:-

The chemical compound nitric acid (HNO3), is a colorless, corrosive liquid, a toxic acid which can cause severe burns. At room temperature it gives off red or yellow fumes. Commonly used as a laboratory reagent, it is used in the manufacture of explosives such as nitroglycerin and trinitrotoluene (TNT), and as well as of fertilizers such as ammonium nitrate. It has additional uses in metallurgy and refining as it reacts with most metals, and in organic syntheses. When combined with hydrochloric acid it forms aqua regia, one of the few reagents capable of dissolving gold and platinum. Nitric acid is also a component of acid rain.

Nitric acid is a strong acid: in aqueous solution, it completely dissociates into the nitrate ion NO3− and a hydrated proton, known as a hydronium ion, H3O+. The salts of nitric acid (which contain the nitrate ion) are also known as nitrates. The overwhelming majority of them are very soluble in water.

Nitric acid is made by mixing nitrogen dioxide (NO2), with water. Creating a very pure nitric acid usually involves distillation with sulfuric acid, as nitric acid forms an azeotrope with water with a composition of 68% nitric acid and 32% water. Commercial grade nitric acid solutions are usually between 52% and 68% nitric acid. If the nitric acid solution contains more than 86% nitric acid, it is referred to as fuming nitric acid, and can be separated into two kinds of fuming acids, white fuming nitric acid, and red fuming nitric acid.

Commercial production of nitric acid is via the Ostwald process after Wilhelm Ostwald.

Process Description: -

Nitric acid is produced by 2 methods. The first method utilizes oxidation, condensation, and absorption to produce a weak nitric acid. Weak nitric acid can have concentrations ranging from 30 to 70 % nitric acid. The second method combines dehydrating, bleaching, condensing, and absorption to produce a high-strength nitric acid from a weak nitric acid. High-strength nitric acid generally contains more than 90 % nitric acid.

Weak Nitric Acid Production:  -

Nearly all the nitric acid produced in the U. S. is manufactured by the high-temperature catalytic oxidation of ammonia. This process typically consists of 3 steps: (1) ammonia oxidation, (2) nitric oxide oxidation, and

(3) Absorption. Each step corresponds to a distinct chemical reaction.

Ammonia Oxidation -

Liquid ammonia is fed to the ammonia evaporator, which is under a pressure of 0.65 MPa. The entire stream then goes to ammonia evaporator I, where 80 % of the ammonia is evaporated against cold water at a constant temperature of 12 °C. Residual liquid ammonia is fed to ammonia evaporator II (not illustrated) and evaporated at varying temperatures against cooling water. The evaporation temperature increases from 12 °C to 20 °C as water content builds up in the evaporator. Ammonia evaporator II is designed so that the entire ammonia stream can be evaporated in it, if necessary, at a maximum temperature of 20 °C. If the water content or the evaporation temperature in ammonia evaporator II is too high, the ammonia – water mixture is blown down and fed to the ammonia stripper. Ammonia is stripped to a residual content of 2.0 % as the temperature is raised to 150 °C with low-pressure steam. The residue, greatly depleted in ammonia, is discharged. Gaseous ammonia from the evaporator system is passed through the ammonia gas filter to remove contaminants, and then heated to 150 °C in the ammonia preheater.

Air is sucked through the filter and compressed to 0.5 MPa in the compressor. About 86 % of the air is fed as primary air to the ammonia – air mixer. The mixed gas stream has a temperature of ca. 220 °C and contains ca. 10 vol % ammonia. It is transported through the gas-mixture filter (not illustrated) to the reactor. Here, ammonia reacts on the platinum – rhodium catalyst with atmospheric oxygen at 890 °C to produce a 96.5 % yield of nitrogen monoxide and water.

The reaction gases next pass through the waste-heat boiler and are cooled to 355 °C. The heat is used to raise high-pressure steam in the pre-evaporator, superheater, and re-evaporator of the boiler.

The boiler system is fed with condensate from the steam turbine condenser plus demineralized water from the battery limit. Both streams are led to a tank and then pass through the feed water preheater to the deaerator mounted on the feed water tank. The tank serves as a suction vessel for

the feed water pumps, which deliver the water through the economizer into the steam drum. The water – steam mixture in the evaporator is circulated by the boiler circulation pump. Saturated steam and water are separated in the steam drum.

4NH3 + 5O2  4NO + 6H2O

Nitric Oxide Oxidation -

After exiting the waste-heat boiler and passing through a run of piping for oxidation, the nitrous gas is cooled from 390 to 280 °C in tail-gas preheater III. After further oxidation, it enters the economizer, where it is cooled from 310 to 200 °C. The nitrous gas gives up further useful heat to the tail gas in tail-gas preheater II and its temperature falls to 170 °C.

The gas passes through the feed water preheater, mounted on the cooler – condenser inlet and thereby cools to 100 °C. In the water- cooled cooler – condenser I, the gas is cooled to < 50 °C so that the water formed during ammonia oxidation condenses. A 43 % acid results and is pumped to the appropriate tray of the absorption tower. The secondary air laden with NOx and recycled from the bleacher is now mixed with the nitrous gas stream. The mixed stream is then led into cooler – condenser II, where it is cooled to 45 °C against chilled water. More water condenses and a 52 % acid is formed, which is forwarded for absorption along with acid formed in cooler –condenser I. The gas is passed through a mist collector (not illustrated) upstream of the nitrous gas compressor.

The nitrous gas is compressed to 1.1 MPa in the compressor and has an outlet temperature of ca. 130 °C. It passes to tail-gas preheater I, where it is cooled to 75 °C. Downstream of the preheater is cooler – condenser III, which the nitrous gas leaves at 50 °C. The resulting acid condensate has a concentration of ca. 58 wt % and is mixed with the raw acid from the absorption column.

The nitric oxide formed during the ammonia oxidation must be oxidized. The process stream is passed through a cooler/condenser and cooled to 100F or less at pressures up to 116 pounds per square inch absolute (psia). The nitric oxide reacts noncatalytically with residual oxygen to form nitrogen dioxide (NO2) and its liquid dimer, nitrogen tetroxide:

(Side reactions)

This slow, homogeneous reaction is highly temperature and pressure dependent. Operating at low temperatures and high pressures promotes maximum production of NO within a minimum reaction time.

Absorption -

From cooler – condenser III the nitrous gas, now at least 90 % oxidized, enters the absorption tower. The gas first passes through oxidation trays that receive only a slight flow of liquid, to attain the degree of oxidation needed for equilibrium with 68 % acid. In the absorption section, gas and liquid move counter currently. Required process water is pumped to the tower head. The 68 % raw acid is withdrawn from the first absorption tray and then treated with secondary air in the bleacher. It is delivered to the battery limit with the aid of the system pressure.

Approximately 80 % of the heat generated by absorption is transferred to the cooling water and the remainder to the chilled water employed in the last part of the absorption step where very little heat of reaction is liberated.

The tail gas contains < 150 – 200 ppm NOx and exits the absorber at 20 °C. It is heated to 110, 200, and finally 350 °C in the three downstream tail-gas preheaters before being expanded in the tail-gas turbine. The turbine supplies 67 % of the power required by the air and the nitrous gas compressors. The expanded tail gas has a temperature of 100 °C and is discharged through a stack.

The remaining power required to drive the rotating machinery can be obtained from a pass-out condensing steam turbine. Superheated steam is fed to the turbine or exported. Part of the steam supplied to the turbine may be withdrawn as low-pressure steam before it enters the condensing section. This low-pressure steam may be used in the plant or exported. The remainder of the steam fed to the turbine passes through the condensing section and is condensed in the steam turbine condenser.

Process Flow Sheet: -

Simplified flow sheet of a dual-pressure process

a) Ammonia evaporator; b) Ammonia stripper; c) Ammonia gas filter; d) Ammonia preheater; e) Ammonia – air mixer; f ) Air filter; g) Air compressor; h) Reactor; i) Waste-heat boiler; j) Tail-gas preheater III; k) Economizer; l) Tail-gas preheater II; m) Feedwater preheater; n) Cooler – condenser I; o) Cooler – condenser II; p) Nitrous gas compressor; q ) Tail-gas preheater I; r) Cooler – condenser III; s) Absorption tower; t) Tail-gas expansion turbine; u) Feedwater tank with deaerator; v) Steam drum; w) Steam turbine; x) Steam turbine condenser; y) Bleacher

Specifications: -

Product Specifications:-

% by mass

Total Acidity (as HNO3)                      69-72 % (min)
 Nitrous Acid (as HNO2)                      0.1    (max)
 Chlorides (as Cl)                                0.002 (max)
 Sulphates (asH2SO4)                         0.005 (max)
 Residue on ignition                           0.05    (max)

Process Specifications: -

Values are given per tonne of 72 % HNO3; the tail gas contains < 200 ppm of NOx)

| Parameter | Mono pressure processes |  | Dual-pressure process |
| --- | --- | --- | --- |
|  | Medium pressure | High pressure |  |
| Operating pressure, MPa | 0.55 | 1.080 | 0.45/1.1 |
| Ammonia, kg | 282 | 283 | 279 |
| Electric power, kW · h | 8.5 | 8.0 | 9.0 |
| Platinum, g | 0.14 | 0.30 | 0.11 |
| Cooling water, t (t = 10 °C) ** | 120 | 125 | 130 |
| Process water, t | 0.3 | 0.3 | 0.3 |
| Low-pressure heating steam, t | 0.1 | 0.1 | 0.1 |
| High-pressure excess steam, t (2.5 MPa, 400 °C) | 0.87 | 0.78 | 0.81 |

Feed Specifications: -

The ammonia – air reaction mixture should contain 14.38 % ammonia.

Process Constraints: -

Dominant Variable: -

Reactor:

The Feed Rate: -

If the gas velocities are too high conversion to nitrogen monoxide decreases because of the slip (leakage) of ammonia which reacts with it. If the gas velocities are too low decomposition of nitrogen monoxide is promoted.

Feed Properties: -Air and Ammonia must be filtered so that it doesn’t have any impurities. All the particles above than 5 micron in air and 3 micron in ammonia should be removed.

Reactor Temperature: -

A high reaction temperature promotes ammonia combustion, but high temperature decreases the conversion. Maximum temperatures up to 950 °C have been realized, but catalyst losses, chiefly due to vaporization, increase greatly. If the temperature is too low, conversion to nitrogen monoxide decreases. The reaction temperature also depends on the temperature of the inlet gas mixture.

Pressure: -

The air for combustion is delivered at 0.5 MPa by an uncooled radial or axial air compressor

Absorption: -

Water Flow rate: -Depending on the flow rate of the gas we need to change the flow rate of water to meet the concentration of the Product.

Pressure: -

The 1.0 – 1.2 MPa pressure is needed for absorption.

Temperature: - As temperature changes rate of absorption changes so we need to control the inlet temp of the gas.

Stability: -

We can ensure stability of the Process by controlling the temperature of the reactor Trea as the reaction involved is an exothermic reaction and temperature in the Absorber which affects the absorption rate. Thus Large variation in temperature than its limits affects the conversion and activity of the catalyst which hinders in meeting the specs.

Independent Degree Of Freedom: -

The variables to control in the unit are Trea, Prea, Frea, Tabs, Pabs, Fwat, Fair, and Famm. Among these, variables which can be controlled independently are

Inventory Control: -

The Feed control of the raw materials Ammonia ,Air and Water comes under inventory control.

Primary Control: -

Supervisory Control: -

possess