Das CanBus-Projekt

2. Teilschritt: Einbindung in Embedded Webserver.

Wir haben uns erst etwas in die Programmierung mit C beschäftigt, damit wir den Assembler-Code in C-Code übersetzen können.

Dann haben wir uns dazu entschlossen eine  Schnittstelle can.h zu schreiben und diese in can.c zu implementieren. Die Schnittstelle ermöglicht es, nach Initialisierung Daten zu verschicken, die Zieladresse und die Datenlänge festzulegen und natürlich über eingetroffene Daten informiert zu werden (über interrupt 28, Receive-Buffer-full).

Dafür haben wir dann eine kleine Beispielanwendung geschrieben, bei der 2 Webserver beteiligt sind. Der eine empfängt Daten über die serielle Schnittstelle und sendet sie dann über den Can-Bus an den anderen weiter. Dieser schreibt dann die Daten auf das Display.

Die Anwendung ist im abgegebenen zip- Archiv und die wichtigen Informationen stehen in can.h.

Die entgültige Schnittstelle liegt dann im 2. Archiv, wobei dort halt nur die can.h und can.c Datei zu finden sind. In der zugehörigen readme- Datei und in ca.h befinden sich alle notwendigen Informationen.

1.Teilschritt: Implementierung im Assembler- Code

Ziel: Zunächst haben wir uns vorgenommen, zwei Mikroprozessoren über CAN miteinander kommunizieren zu lassen.

Vorgehensweise:

Dazu haben wir uns ausführlich mit der angegebenen Dokumentation auseinandergesetzt und uns das Beispielprogramm angesehen. Ausgehend von diesen Erkenntnissen haben wir ein Programm geschrieben, das folgendes ermöglicht :

Über die serielle Schnittstelle empfängt der erste CAN die gesendeten Zeichen und leitet sie dann weiter an den zweiten CAN, welcher wiederum dieses empfangende Zeichen über die serielle Schnittstelle an den PC zurücksendet  also quasi ein Echo-Signal

Wichtige Programmfragmente :

; The CAN-Bus address of the HC912D60 will be set to %10000001000 (11 bits, CAN2.0A).

; In order to achieve proper receiption of messages, the MSCAN-controller is configured to

; use four 16 bit acceptance filters and each of them must be set up in the same way.

Start

clr	COPCTL			;clear Watchdog

lds 	#RAM_END		;init SP, top or RAM.Application Note

movb	#$01,CMCR0		; configure flags, enable SOFT-RESET state

clr	CMCR1			; Loop Back Self Test Mode off, clock source is   EXTALi

movb	#$09,CBTR0		; synchron jump width 1, baudrate prescaler 10

movb	#%00111010,CBTR1	; 1 samplepoint, tSEG2: 4 cycles, tSEG1: 11 cycles

movb	#$10,CIDAC		; use four 16 bit acceptance filters

movb	#$81,CIDAR0		; set up filter 1 (first filter bank)

clr	CIDAR1

clr	CIDMR0			; don't mask identifiers of received messages

movb	#$07,CIDMR1		; last three bits don't care

movb	#$81,CIDAR2		; set up filter 2 (first filter bank)

clr	CIDAR3

clr	CIDMR2			; don't mask identifiers of received messages

movb	#$07,CIDMR3		; last three bits don't care

movb	#$81,CIDAR4		; set up filter 3 (second filter bank)

clr	CIDAR5

clr	CIDMR4			; don't mask identifiers of received messages

movb	#$07,CIDMR5		; last three bits don't care

movb	#$81,CIDAR6		; set up filter 4 (second filter bank)

clr	CIDAR7

clr	CIDMR6			; don't mask identifiers of received messages

movb	#$07,CIDMR7		; last three bits don't care

bclr	CMCR0,$01		; clear SFTRES flag and switch to normal operation

;*************************2 CANBUS**************************************+

movb	#$01,CMCR0_2		; configure flags, enable SOFT-RESET state

clr	CMCR1_2			; Loop Back Self Test Mode off, clock source is EXTALi

movb	#$09,CBTR0_2		; synchron jump width 1, baudrate prescaler 10

movb	#%00111010,CBTR1_2	; 1 samplepoint, tSEG2: 4 cycles, tSEG1: 11 cycles

movb	#$10,CIDAC_2		; use four 16 bit acceptance filters

movb	#$80,CIDAR0_2		; set up filter 1 (first filter bank)

clr	CIDAR1_2

clr	CIDMR0_2		; don't mask identifiers of received messages

movb	#$07,CIDMR1_2		; last three bits don't care

movb	#$80,CIDAR2_2		; set up filter 2 (first filter bank)

clr	CIDAR3_2

clr	CIDMR2_2			; don't mask identifiers of received messages

movb	#$07,CIDMR3_2		; last three bits don't care

movb	#$80,CIDAR4_2		; set up filter 3 (second filter bank)

clr	CIDAR5_2

clr	CIDMR4_2		; don't mask identifiers of received messages

movb	#$07,CIDMR5_2		; last three bits don't care

movb	#$80,CIDAR6_2		; set up filter 4 (second filter bank)

clr	CIDAR7_2

clr	CIDMR6_2		; don't mask identifiers of received messages

movb	#$07,CIDMR7_2		; last three bits don't care

bclr	CMCR0_2,$01		; clear SFTRES flag and switch to normal operation

***** Prepare the transmit buffer to send out one byte ****

movb	#$80,Tx0	; IDR0, send to CAN-address %10000000000

clr	Tx0+1		; IDR1 (the rest of address)

movb	#1,Tx0+$0C	; DLR, send out one byte

clr	Tx0+$0D		; set local priotity to 0 (highest)

bsr	initSC0		; set up the serial interface 0

;**************************2nd CAN-BUS**************************************+

movb	#$81,Tx0_2	; IDR0, send to CAN-address %10000001000

clr	Tx0_2+1		; IDR1 (the rest of address)

movb	#1,Tx0_2+$0C	; DLR, send out one byte

clr	Tx0_2+$0D		; set local priotity to 0 (highest)

; ***** Main program loop *****

;

; Checks if new serial data is avaiable and transfers them via CAN-bus

; with the identifier %10010000000 and redirect messages from the

; CAN-Bus with identifiers starting with %10000000 to the host PC.

; The programm can be terminated by sending a 27 (ASCII-ESC) to the

; serial interface of the HC12compact board.

Send	brclr	SC0SR1,$20,Rec		; test if serial receive buffer=full

bsr	getSC0			; fetch new data from SC0

cmpa	#27			; <ESC>-key pressed?

beq	End			; yes -> terminate program

;	bsr	putSC0			; local echo of serial data

Sloop	brclr	CTFLG,$01,Sloop		; wait until the Transmit Buffer is released

staa	Tx0+4			; write new data into the CAN-transmitt buffer

movb	#$01,CTFLG		; initiate the transfer by clearing TXE0-flag

Rec	brclr	CRFLG,$01,Rec_2		; if CAN rec. buffer=empty, check for serial data

ldaa	RxFG+4			; read new byte from receive buffer

bset	CRFLG,$01		; release Receive Buffer (clear RXF-Flag)

bsr	putSC0			; transmit the byte to host PC

bra	Send			; check for new serial data

Rec_2	brclr	CRFLG_2,$01,Send	; if CAN rec. buffer=empty, check for serial data

ldaa	RxFG_2+4		; read new byte from receive buffer

bset	CRFLG_2,$01		; release Receive Buffer (clear RXF-Flag)

bsr	putSC0			; transmit the byte to host PC

bra	Send			; check for new serial data

End	rts				; back to monitor

; Func:	Initialize serial communication interface SC0 (19200 baud, 8N1)

; Args:	-

; Retn: -

; Dest: -

initSC0

clr	SC0BDH

movb	#$1A,SC0BDL		; set baud rate to 19200

clr	SC0CR1			; 8 data bits, 1 stop bit, no parity

movb	#$0C,SC0CR2		; enable receiver and transmitter

rts

; Func:	Gets a byte from SC0

; Args:	-

; Retn: A = byte

; Dest: -

getSC0

brclr 	SC0SR1,$20,getSC0	; wait for receive data register full

ldaa  	SC0DRL			; read out data register

rts

; Func:	Sends out a byte via SC0

; Args:	A = byte

; Retn: -

; Dest: -

putSC0

brclr	SC0SR1,$80,putSC0	; wait until transmit data register=empty

staa	SC0DRL			; send new data

rts