The PGLinkbox is designed to communicate with your Nintendo Power glove and convert its raw data to a standard serial format that can be read by almost any PC.

Your Power Glove must already be hacked for a parallel port interface.  There are many resources on the web for this modification.  One such website is www.geocities.com/mellott124.

- Plug in the Power Glove keyboard port power cable.

- Plug the Power Glove DB-25 into the PGLinkbox.

- Connect the PGLinkbox DB-9 serial output to a suitable serial port on your PC.

- Turn the PGLinkbox power on.  The green LED should be lit.

- Press and hold the reset button, on the side of the PGLinkbox enclosure, for approx. 3 seconds.  Release the button.

- Make sure the Power Glove is facing its sensor array.

- Put the Power Glove on and make a fist.

- The Power Glove should now be communicating with the PGLinkbox.  Serial data is now streaming to your PC serial port at a baud rate of 4800 in 8,N,1 format.

- If you are having trouble with the serial data output stream of the PGLinkbox, repeat from step 4 and on.

If you have any additional questions or comments please contact TechSupport@VRImmersions.com.