It is recommended that only a TRAINED TECHNICAN perform the following modification!!!!

- Unplug all cables from the VFX1.  Be sure to ground yourself with a grounding strap before continuing with this modification.  Any ESD can possibly destroy your VFX1.

- Remove the foam liner of the VFX1.

- Remove the screws around the base of the HMD where the cable plugs in.

- Remove the two screws holding the plastic closest to the speaker wires.  These two screws are located in the corners just as the HMD starts to come over the top of your head.

- Place your fingers where the speaker wires are coming out of the HMD.

- Pull slightly until the tracking board pops out of the HMD.

- Carefully remove the tracking board.  Disconnect both speaker wires.

- Slowly remove the two ribbon cables located at the base of the board.  These are the ribbon cables to your LCD’s.  They pull out.

- You will notice at the top of the card a paper ribbon cable.  Slowly and carefully pull this out of the connector.  This is the cable to your tracking circuitry.

- Now you should have the tracker board assembly removed from the VFX1.

- Remove the four screws (MT1, MT2, MT3 & MT4) securing the board to the housing.

- Take note of the small rectangular black component above and slightly to the left of the white ribbon cable connector (the LCD connector).   This is the relay for the HMD backlights.

- Carefully remove the board from the housing.

- Take some time to figure out which pins are the relay pins.  There are four.  They are lined up in a straight line just above the set of multi-row pins.

- The left two pins (as looking from the non component side) are connected by a trace.  This trace ends slightly to the left of the fourth pin from the left.  You need to soldier the end of this trace to the fourth pin from the left.

- Just below the trace you just soldiered to is another smaller trace.  It connects to a VIA (thru board connection) below the third pin from the left.  You need to disconnect this trace.  Use a small knife or something sharp and cut the trace.  This trace needs to be disconnected.  It goes to the microprocessor, which controllers the backlights.  We will not be using this feature anymore.

- Repeat the above in reverse to put the VFX1 back together.

- Now your VFX1 is ready to be used with the Linkbox.  Enjoy!!!