| Standard Resistor Color Code and Resistors Used In Sparton Radio Receiving Sets and Sparton Ensembles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## STANDARD RESISTOR COLOR CODE |  |  |  |  |  |  |  |  | sistor, and the second figure of resistance value by the color of the tip of the resistor. The number of ciphers following the second figure is determined by the color of the dot or stripes in the center of the body of the resistor. For example, a 20,000 ohm resistor has a red body, black tip, with orange dot or orange stripe. A 2,200 ohm resistor would be red body, with red tip and red dot, or red stripe, and as all colors are the same, it would be a single color resistor. |  |  |  |  |  |
| 0 |  | -Black |  | 5 |  |  | -Green |  |  |  |  |  |  |  |
| 1 |  | -Brown |  | 6 |  |  | -Blue |  |  |  |  |  |  |  |
| 2 |  | -Red |  | 7 |  |  | -Violet |  |  |  |  |  |  |  |
| 3 |  | -Orange |  | 8 |  |  | -Grey |  |  |  |  |  |  |  |
| 4 |  | -Yellow |  | 9 |  |  | -White |  |  |  |  |  |  |  |
| To determine the value of a resistor, the first significant figure of resistance value is represented by the color of the body of the re- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sparton Part No. | Resistance Ohms |  | Size Watts |  |  | Old Color |  |  | # STANDARD RESISTOR COLOR CODE |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | BODY |  | TIP |  | DOT OR STRIPE |  |
| A-2934 | 20,000 |  | 2 |  |  | Green |  |  | Red |  | Black |  | Orange |  |
| A-3397 | 1,000 |  | 2 |  |  | Tan |  |  | Brown |  | Black |  | Red |  |
| A-3397-X | 1,000 |  | 0.5 |  |  | Tan |  |  | Brown |  | Black |  | Red |  |
| A-3423 | 50,000 |  | 2 |  |  | Red |  |  | Green |  | Black |  | Orange |  |
| *A-3750 | 1,250 |  | 3 |  |  | Black, Silver Ends |  |  | Brown |  | Orange |  | Red |  |
| A-4107 | 15,000 |  | 5 |  |  | Black, Silver Ends |  |  | Brown |  | Green |  | Orange |  |
| A-4234 | 250,000 |  | 5 |  |  | Brown, Blue Ends |  |  | Red |  | Green |  | Yellow |  |
| A-4261 | 20,000 |  | 1 |  |  | Green |  |  | Red |  | Black |  | Orange |  |
| A-4353 | 2,800 |  | 0.5 |  |  | Gray |  |  | Red |  | Gray |  | Red |  |
| A-4613 | 1,700 |  | 1 |  |  | Brown |  |  | Brown |  | Violet |  | Red |  |
| A-4614 | 10,000 |  | 1 |  |  | Blue |  |  | Brown |  | Black |  | Orange |  |
| A-5139 | 30,000 |  | 1 |  |  | Red |  |  | Orange |  | Black |  | Orange |  |
| A-5180 | 5,000 |  | 15 |  |  | Green |  |  | Green |  | Black |  | Red |  |
| A-5269 | 500,000 |  | 1 |  |  | Green |  |  | Black |  | Black |  | Yellow |  |
| A-5270 | 250,000 |  | 1 |  |  |  |  |  | Red |  | Green |  | Yellow |  |
| A-5354 | 100,000 |  | 1 |  |  |  |  |  | Brown |  | Black |  | Yellow |  |
| *1250 ohm resistors same color scheme as 1300 ohm resistors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # Standard Resistor Color Code Is Not Applied to Vitreous Enamel Resistors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPARTON PART NO. | RESISTANCE OHMS |  |  |  | SIZE WATTS |  |  | COLOR | SPARTON PART NO. | RESISTANCE OHMS |  | SIZE WATTS |  | COLOR |
| A-4363 | 7 |  |  |  | 20 |  |  | Blue | A-4365 | 15 |  | 50 |  | Blue |
| A-4364 | 12 |  |  |  | 30 |  |  | Blue | A-5177 | 160 |  | 1 |  | Blue |
| A-4365 | 63 |  |  |  | 10 |  |  | Blue | A-5426 | 2,400-1,800 |  | 8 |  | Blue |
| A-5889 | 54 |  |  |  | 175 |  |  | Blue | A-5990 | 14 |  | 6 |  | Blue |
| # Standard Color Code Is Not Applied to Wire Wound Resistors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPARTON PART NO. | RESISTANCE OHMS |  |  |  | SIZE WATTS |  |  | COLOR | SPARTON PART NO. | RESISTANCE OHMS |  | SIZE WATTS |  | ### COLOR |
| A-3383 | 3,000 |  |  |  | 10 |  |  | Black | A-4915 | 110 |  | 1 |  | Black |
| A-3535 | 7,000 |  |  |  | 10 |  |  | Black | A-4974 | 1,250 |  | 5 |  | Gray |
| A-3536 | 900 |  |  |  | 10 |  |  | Black | A-5137 | 330 |  | 1 |  | Gray |
| A-3311 | 30,000 |  |  |  | 0.5 |  |  | Black | A-5502 | 200 |  | 1 |  | Red |
| A-4260 | 7,000-2,000 |  |  |  | 20 |  |  | Black | A-5861 | 57 |  | 175 |  | Blue |
| A-4363 | 7 |  |  |  | 20 |  |  | Black | A-5862 | 12 |  | 10 |  | Blue |
| A-4583 | 7,000 |  |  |  | 10 |  |  | Black | A-5863 | 2 |  | 5 |  | Blue |
| A-4670 | 110 |  |  |  | A |  |  | Black |  |  |  |  |  |  |