# Assignment 3 (Fall 2003)

CIRCUIT THEORY (PHY301)

MARKS: 30

Due Date: 30/10/2003

### Q.1.

Use nodal analysis to find VO in the following circuit.

### Sol.

Consider the above fig. reference node is selected, and node voltages are labeled as V1 and V2. Directions of unknown currents are chosen according to the rule, ‘all unknown currents will be considered as flowing outwards from a particular node’. Writing the KCL equations for the two nodes,

At node 1

-4m + V1/3K + (V1-V2)/4K = 0

4V1 + 3V1 – 3V2 = 48

7V1 – 3V2 = 48	(a)

At node 2,

(V2-V1)/4K + 2m + V2/4K = 0

V1 – 2V2 = 8		(b)

Solving equations (a) and (b) simultaneously, yields

V1 = 6.54V

V2 = -0.73V

Required voltages VO is given by

VO = V2-V1 = -0.73 – 6.54 = -7.27V

### Q.2.

Find the voltages V1, V2 and VO in the following network.

### Sol.

Considering above fig. we write the KCL equations for the two nodes as

At node 1;

4m + V1/3K + (V1-V2)/6K = 2m

3V1 – V2 = -12	(a)

At node 2;

(V2-V1)/6K + V2/4K + V2/12K = -2m

V1 – 3V2 = 12	(b)

solving (a) and (b) simultaneously we get,

V1 = -6V

V2 = -6V

Since V2 is across the two resistors 2K and 2K in series, so using voltage divider rule,

Vo = (2/4).(-6) = -3V

### Q.3.

Find the voltage VO, in the following circuit.

### Sol.

At node 1;

(V1-V2)/1 + (V1-V3)/1 = 2

V1 - V2 + V1 - V3 = 2

2V1 - V2 - V3 = 2		(a)

At super node:

(V3-V1) + V3 + V2 + (V2-V1)= 0

-2V1 + 2V3 + 2V2 = 0

-V1 + V3 + V2 = 0	(b)

Now voltage source 12V between V2 and V3 has put a constrain on 	the values of V2 and V3, mathematically,

V2 – V3 = 12V		(c)

Solving (a), (b) and (c) simultaneously gives,

V1 = 2V

V2 = 7V

V3 = -5V

### Q.4.

Find the voltage across 8 resistor in the following circuit.

### Sol.

Consider the above fig. Voltage across resistor 8 resistor, considering the supposed direction of the current, is (v1 - 12), hence current passing through this resistor is (v1 - 12)/8. Now writing the KCL equations for the two nodes,

at node 1,

(V1 - 12)/8 + (V1 – V2)/1K = -2		(a)

at node 2,

(V2 – V1)/1K + V2/10 = 2			(b)

solving the two equations (a) and (b), yields,

V1 = -3.81V

V2 = 19.76V

Voltage across 8 resistor is (12 – V1) = 12 + 3.81 = 15.81V

### Q.5.Find IO in the following circuits.

### Sol.

A). Consider the following fig.

At node 1,

V1/6 + (V1-V2)/10 = -4m			(a)

AT supernode,

(V2-V1)/10 + V2/4 + V3/12 = 0		(b)

constrain due to voltage source 24V, gives

V2 – V3 = 24					(c)

Solving these three equations, we get

V1 = 1.91V

V2 = 5.06V

V3 = -18.94V

Required current Io passing through resistor 12 is

Io = -V3/12 = 18.94/12 = 1.58A

B).

Considering the above fig, at supernode,

V1/20 + 3 – 3 + V2/40 – 4 = 0

V1/20 + V2/40 – 4 = 0			(a)

Constraint due to the voltage source 16V,

V2 – V1 = 16				(b)

Solving (a) and (b) gives

V1 = 48V

V2 = 64V

required current Io is given by

Io = -V1/20 = -48/20 = -2.4A

----- Good Luck -----