# Assignment 5 (Fall 2003)

CIRCUIT THEORY (PHY301)

MARKS: 30

Due Date: 05/01/2004

### Q.1. Using superposition, determine the current through 6   	resistance in the following Circuit.

### Sol.

Considering the effect of the 44 voltage source in the figure below,

RT=R1+R2||R3 = 30 + 16 ||6 

= 96/22

= 34.36 

I = V1/RT

=44/34.36=1.28A

Using the current division rule,

I’3 = R2I/R2+R3

= (16)(1.28)/22

= 0.93 A

Considering the effect of the 38V source in the 	figure below,

RT=R3+R1||R2 = 6  + 30 ||16 

= 6 + 10.43

= 16.43 

I’’3 = V2/RT

= 38/16.43

= 2.31A

Total current through the 6 resistance,

I3  = I’’3-I’3 = 2.31A – 0.93 A

= 1.38 A (direction of I’’3)

### Q.2.   Find the current through 2 resistor of the network given below 	by using superposition theorem.

### Sol.

Considering the effect of the 12V source in the figure below,

### I’1 = V1/R1+R2

= 12V / 2 +4

=12/6 =2A

Considering the effect of the 6V source in the figure 	below,

### I’’1 = V2/R1+R2

= 6V / 2 +4

=6/6 =1A

Considering the effect of the 3-A source in the figure below,

### I’’’1 = R2(I)/R1+R2

= (4)(3A) / 2 +4

### = 12A/6 = 2A

### Total current through the 2W resistance

### I1 = I’’1 + I’’’1 - I’1

### = 1A + 2A - 2A

### = 1A

### Q.3. Find the Thevenin equivalent circuit for the network in the   	shaded area.

### Sol.

First step: Removing RL

Second step:  Calculating Vth

In this case since an open circuit exists between the two terminals, the current is zero between these terminals and through the 2ohm resistance. The voltage drop across R2 is therefore,

V2=I2R2=(0)R2 = 0V

and     Vth= V1=I1R1=IR1

=(12A)(4ohm)= 48V

Third step:  Calculating Rth

Rth=R1+R2 = 4 + 2 =6 ohm

Fourth step:

After calculating Vth and Rth, re-inserting the load 	resistance RL in the circuit in series with Rth 	and 	considering the Vth as a battery in series with 	these two resistances

Substituting the The’venin equivalent circuit in the 	network external to the resistor R3

------ Good Luck -----