# Assignment 1 (Fall 2003)

(Solution)

CIRCUIT THEORY(PHY301)

MARKS: 30

Due Date: 08/10/2003

*Note: Brain teasers are not a part of the assignment, they don’t carry any marks. Those who attempt these questions will be considered for the best students of the week for this course. So do the best of your efforts to answer those questions.

### Q.1.

If a current of 40 A exists for 1 min, how many coulombs of charge have passed through the wire. If the current consists of electrons only, calculate the number of electrons passed through the wire, during this period of time.

(electronic charge = 1.6 10-19 C)

### Sol.

Current is the total charge passing through a surface in one second. Hence for this problem, 40C charge will pass through the wire in 1 second. So in 1 min, 2400C charge will pass. From the electronic charge as give, 1C of charge contains 6.251018, thus 1.51022 electrons passed through the circuit.

### Q.2.

Point A is set at electric potential 2J, and point B has potential 10 J. Now if an electron is moved from point A to point B, It will release some energy or it will absorb?. Calculate that energy. If 5C charge is moved from point B to A, calculate the potential difference in Volts between points B and A.

### Sol.

Since point A is at low potential compared to point B, electron will absorb energy, which is equal to the difference of the potentials of two points, that is 8J. Mathematically volt is defined as the potential difference per unit charge. Thus if 5C charge is moved from point B to point A, hence there are 8J/5C = 1.6V  across points A and B.

### Q.3.

Find current Io in the following network.

(hint: In closed circuit, total power absorbed by the circuit elements must be equal to the total power delivered by circuit elements.)

### Sol.

First we have to find the power consumed or supplied by all elements. Now current entering the positive terminal of the element will be considered positive, while current leaving the positive terminal of the element will be considered negative. Energy conservation requires that the total power through a closed circuit must be zero, thus

24(-6) + 8(6) + 10(4) + 16(2) + 6(Io) + 6(1) + 8(3) + 8(-3) = 0

-144 + 48 + 40 + 32 + 6 Io + 6 + 24 – 24 = 0

-18 + 6 Io = 0

Io = 3A

Find the total resistance RT in the following circuits, please draw each step.

### Sol.

a). Since in this circuit, resistor R5 has no voltages across so it contains no current, hence R2 and R3 are in series. R1, R2, R3, and R4 are in series so total resistance will be 10.8k. as shown in fig below.

b). Same circuit can be drawn in the following way.

Now, R1 and R2 are parallel and R3, R4 are parallel. so solving for the parallel resistors.

resulting resistors are in series, hence final solution will be 2.1 k.

c). In this circuit, since resistant network is on both sides of the battery, so we start from both sides to solve for total resistance. As it is clear, on the left side R1, R2 and R3 are in series and on the right side R8 and R9 are in series, solving for resistors in series, we get

Resulting 6.4k is parallel to R3, and 14k is parallel to R7, hence

Now resistors on both sides of the battery are in series, so

if you look closely, we can rearrange this circuit in a bit different but simple way. as shown in the following fig.

Final solution is obvious now, that is 3.7k.

### Q.5. Fill in the following blanks.

1.	The valence of an element having 11 electrons is 1.

2.	The relative conductivity of silver is 105 percent and that of gold is 70.5 percent.

3.	Circuit is said to be short when there is no load resistance in the circuit.

4.	According to Ohm’s law voltages and currents have linear relationship in a  circuit.

5.	Two resistors are said to be in series if their common node is not connected to            a current carrying circuit element.

### *Brain Teasers

- If we look at the Ohmic relation (relation given by Ohm’s law), it assumes a constant resistance (no dependence on voltages) of circuit elements. Is it possible for any electronic component to work against the Ohm’s law. If so, explain in some detail.

- Insulators don’t conduct electric current at room temperature, does it mean that they are not of any use in electronic circuits?, if no, can you think of any possible use?

### Sol.

- Literally, resistance is actually the opposition offered by a certain material, in the path of current. Its one type is what you can call a permanent resistance, which arises when charges flow through the atomic structure of the material, that resistance should be independent of the applied voltages, it is actually a property of a material. Now it is possible that a bit different type of opposition may arise, which depends upon the voltages. One example is semiconductor PN junction (which we’ll discuss in detail in later lectures), current passing through this junction feels an opposition which depends upon the applied voltages, as the voltages increases opposition falls down and the current increases slowly with respect to the voltages, after certain applied voltages, current starts to increase linearly with voltages, satisfying Ohm’s law. But at low voltages, the relation between current and voltage doesn’t follow ohm’s law, as for that range of voltages the resistance is not independent but varies with the voltages. Following graph schematically shows the current voltage relation for a typical PN junction.

2.	Insulators are used as a dielectric in capacitors, they actually increases the capacity of the capacitor. This phenomenon is based upon state electricity, in which electronic region around the nucleus is deformed while still remain around it, forming a negative and positive regions. Technically this phenomenon is called polarization. Also read about the material used in inductors.