# Assignment 2 (Fall 2003)

Solution

CIRCUIT THEORY (PHY301)

MARKS: 30

Due Date: 17/10/2003

*Note: Brain teasers are not a part of the assignment, they don’t carry any marks. Those who attempt these questions will be considered for the best students of the week for this course. So do the best of your efforts to answer those questions.

### Q.1.

Find the voltage VAB in the following circuits.

(hint: use the voltage divider rule)

## Sol:

a). resistors R1.3 and R2.5 are in series so adding the two resistors gives 3.8K. Required voltages VAB are across this resistance. Using voltage divider rule,

VAB = (3.8/8.2).12 = 5.7 V

b). Resistors R1 and R1.5 are in series, so adding them we get 1.5 K. Required voltages VAB are across this resistance. Using voltage divider rule,

VAB = (1.5/3.5).6 = 2.6 V

### Q.2.

Calculate the indicated quantities in the following circuits.

## Sol.

a). To find voltages Vo we need to find the current passing through 6K. R6 and R6 are in series so their resultant will be 12K, now R8, R12 and R8 are parallel to each other, their resultant will be 3 K. Using current divider rule, current I passing through R12 will be

I = (3/12).24 mA = 6mA

This current is passing through 6K resistance, so voltages across it will be

Vo = 6K.6m = 36V

b). Resultant R of R3, R6 and R2 is 4K, which is parallel to 4K, thus using current divider rule we find the total current I` passing through R as

I` = (4/8).12mA =  6mA

This current divides further into two components, one passing through 3k and 6K. Thus current Io passing through 3K will be

Io = (6/9).6mA = 4mA

c). It’s a bridge network, no resistor is in series or parallel to any other resistor. Since R12, R12, and R12 is forming a delta network, we can change it into Wye network. Please check this topic Wye-Delta transformation in FAQs on your LMS. Corresponding resistors in Wye configuration will be 4, 4 and 4, as shown in the fig. Now its easy to find the total resistance RT of the network,

RT = (81/18) + 4 = 8.5

Thus source current I will be 12/8.5 = 1.4 A.

Since the two parallel resistors are equal, so the source current will divide equally to the two branches.

Hence the required current will be 0.7A.

d). Resultant of resistors R2K , R1K  and R5K is 1.5K .Voltages V9K across 9K, are divided into voltages V3K across 3K and V1.5K (=6V) across 1.5K resistance. Thus using voltage divider rule

6 = (1.5/4.5). V9K

This gives,

V9K = 18V

Thus voltages V3K across the resistor 3K will be 12V.

Now the source current IS is divided into a component I3K passing through 3K, and the other I3K passing through 9K,

I3K = V3K/3K = 4mA

I9K = V9K/9K = 2mA

Thus

IS = I9K + I3K = 6mA

### Q.3.

Find the total power absorbed by the following network.

## Sol.

Circuit can be rearranged as shown in the fig below

First we have to find the total current passing through the network.

Resistors R6K, R2K and R12K forms a delta network, its equivalent wye network is shown in the fig.

Corresponding resistors in wye network can be determined by using the transformation formulas (Check the FAQs on LMS for this topic).

Total resistance RT can be found easily now.

RT = [(18 + 1.2)(6 + 3.6)/(18+1.2+6+3.6)] + 0.6

RT = 7K

Since voltages across the network are 21V, so total current I passing through the network is given by,

I = 21/7K = 3mA

Total power delivered absorbed by the network is equal to the total power delivered to the system, which is given by

P = VI = 21.3m = 63.10-3 W = 0.063W

### Q.4.

The following network is the basic biasing arrangement for the field-effect transistor (FET). 16V (VDD) are the total voltages applied across the network with respect to the ground. Using these parameters

VGS = -1.75V

IG = 0 A

ID = IS

- Determine the voltages VG and VS.

- Find the currents I1, I2, ID, and IS.

- Determine VDS.

- Calculate VDG.

Hint:

Imagine some sort of resistance between the nodes G, D and S.

VG is the voltages at node G, with respect to the ground. If you look closely it is actually the voltages across the resistor R2, so use the voltage divider rule to find it.

VGS is given, hence its easy to find VS, which are the voltages at node S, with respect to the ground.

Voltages (VG) across R2 is now known, so current I2 can easily be found. Similarly voltages (VS) across RS is known so current IS can be found. Currents I1 can be found using KCL. Using same technique you can find IS and ID.

To find VDS and VDG, you first have to find VD, which is the voltages at node D, with respect to ground. Find voltages across RD first, and then find VD.

## Sol.

since IG = 0, so current I1 and I2 are equal, hence resistors R1 and R2 are in series, and the total voltages across them are 16V, thus current passing through them will be 16/(2.106 + 270.103)=7.10-6A = 7A. Now voltages VG are actually across the resistor R2, thus it is given by

VG = 270K . 7 = 1.89V

since VGS = -1.75V,

VGS = VG - VS = -1.75

VS = VG + VGS = 3.64V

VS is now known, hence IS is given by

IS = VS/RS = 3.64/1.5K = 2.43mA

also ID = 2.43mA

ID is now known, we can find Voltages VRD across RD as

VRD = RD.ID = 2.5K . 2.43m = 6.08V

Now the total voltages across this branch containing the resistors RD, RS and FET, are 16V, so voltages VD at terminal D of FET with respect to the ground, is given by 16 - VRD = 16 – 6.08 = 9.92V. Thus

VDS = VD – VS = 9.92 – 3.64 = 6.28V

VDS = VD – VG = 9.92 – 1.89 = 8.03V

### Q.5.

In the following network, calculate the value of source voltages VS given that

I4 = 0.5 mA.

(hint: use the KVL, KCL  and divider rules.)

## Sol.

Since I4 is 0.5mA, so Va = 6K . 0.5m = 3V. voltage across 3K is also Va so I3 is given by 3/3K = 1mA. Now at node y, I2 = I3 + I4, so I2 = 1.5mA, hence Vb = (1.5m)(2K)= 3V.

Total resistance RXZ between the nodes x and z is given by

RXZ = 2K

and total voltages across x and z,

VXZ = Va + Vb = 6V

Thus using voltages divider rule, consider the circuit consisting of one loop with series resistors 6K, 4K and RXZ = 2K, with total voltage VS across them.

VXZ = [2K/(2K + 6K + 4K)].VS = 6V

this gives

VS = 36V

### BRAIN TEASERS

### Q.1.

Here is a paradox. According to the ohm’s law voltages V across a resistor R is the product of its resistance and current I passing through it. Now we know that current can only pass between the two nodes, if there is some potential difference across them, that is, there must be some voltages across the nodes to have current between them. In short circuits, we know that current is maximum (practical infinite), this is because the path have zero resistance, now in that situation ohm’s law says that voltages across that path must be zero, as the resistance is zero (V = RI). Now if there is no voltages across that path, current must also be zero, but current is maximum in that situation.

How would you resolve this paradox.

## Sol.

For the short circuit, current passing through it, is actually controlled by the external voltage source (e.g. battery). Hence, although potential difference across nodes, between which the circuit is short, is zero, current flows through them due to the external circuit. Short circuit will act as a path with zero resistance. Ohm’s law will not be applicable between these nodes.

### Q.2.

Find the current passing through 1K and 1.5K in the following network.

## Sol.

Look closely at the circuit, there is a short circuit parallel to the resistors 1K and 1.5K, so no current will pass through them.

### Q.3.

Did you ever think about the basis of Kirchhoff’s Laws?, Can you think of any situation where you can’t use these laws. Try to give a mathematical argument in support of your point.

## Sol.

Kirchhoff’s voltage law is actually based upon energy conservation principle, that is total energy absorbed by the circuit must be equal to the energy delivered to it. Consider a single loop circuit with some independent voltage sources. Now energy conservation requires that total power delivered must be equal to the total power absorbed, so

Pabsorbed = Pdelivered

since same current I is flowing through all elements in the loop

IVdrop = IVsource

this gives

Vdrop = Vsource

This is the Kirchhoff’s voltage law.

Similarly, Kirchhoff’s current law is based upon the conservation of charge principle, that is, number of charges entering a node must be equal to the number of charges leaving it. Since these laws are based upon universal principles, so they must be satisfied in any circuit.

----Good luck----