# Assignment 7 (Fall 2003)

Solution

CIRCUIT THEORY(PHY301)

MARKS: 30

Due Date: 26/01/2004

### Q.1.

A silicon diode has a saturation current 1pA. Using the values of n given here and assuming the temperature is 25oC(“room temperature”) find  the current in the diode.

(a) It is reverse biased by 0.1 V (n = 2)

### (b) It is forward biased by 0.5 V (n=1)

### Sol.

We first calculate the thermal voltage at

T= 273 + 25oC= 298

VT =kT/q

= (1.38 x 10-23) (298) / 1.6 x 10-19

= 0.0257 V

(a) Since the diode is reverse biased, we substitute

V = -0.1 V in the following equation we have

I=IS(eV/nT-1)

= (1pA)( e(-0.1)/2(0.0257V)-1)

= (1pA)(0.143-1)= -0.857 pA

(b) Since the diode is forward biased, we substitute

V = 0.5 V

I=IS(eV/nT-1)              where 1pA10-12A

= (1pA)( e(0.5)/(0.0257V)-1)

= (1pA)( 2.814 x 108-1)

= (10-12A)( 2.814 x 108)= 0.2814 mA

Note that the value of the exponential term (2.814 x 108) in this case is so much large than 1 that for the practical purpose I=ISeV/nT

### Q.2.

Determine which diodes are forward biased and which are reverse biased in each of the configuration give arguments to verify your point of view.

### Sol.

- In (a) the anode is grounded and therefore at 0V. The cathode side is positive by virtue of 5V source connected to it through resistor R. The cathode is therefore positive w.r.t. the anode i.e. the anode is more negative than the cathode so the diode is reverse biased.

- In (b) the anode side is more positive than the cathode side (10V > +5V) so the diode is forward biased. Current flow from 10V source, through the diode and into the 5V source.

- In (c) the anode side is more negative than the cathode side, so the diode is reverse biased. No current flows in the circuit, so no drop across R.

### Q.3.

Determine the peak Load current for the circuit shown in the figure below.

Sol.

The input voltage is given an rms value. This value    	is converted to a peak value as follows:

V1(pk) = V1(rms)/0.707

= 150/0.707

V1(pk)  = 212.2 Vpk

Now, the load voltage and current are found, 	after fining peak voltage, as

V2(pk) = N2/N1  V1(pk)

= (1/10)(212.2 Vpk)

V2(pk) = 21.22Vpk

Finally, the load voltage and current and current 	values are found as:

VL(pk) = V2(pk) – VF

= 21.22 – 0.7

VL(pk)= 20.52Vpk

and the current will be

IL(pk) = VL(PK)/RL

= 20.52Vpk/5.1

IL(pk) = 4.02 mApk

------ Good Luck -----