# Assignment 6 (Fall 2003)

Solution

CIRCUIT THEORY(PHY301)

MARKS: 30

Due Date: 17/01/2004

### Q.1.

Assume that a silicon diode in the figure below requires a minimum of 1mA to be above the knee of its I-V characteristic.

(1)  What should be the value of R to establish 5mA in the circuit?

### (2) With the voltage value of R calculated in (1) what is the minimum value to     	   	    which the voltage E could be reduced and still maintain the diode current 	  	    above the knee?

### Sol.

(1)  If I is equal to 5mA we know that the voltage across the diode will be 0.7V.

Therefore

E = IR +0.7

R = E - 0.7/I = (5-0.7)/5 x 10-3= 860 

(2)  In order to maintain the diode current above knee .I must be at least 1mA.

I=E-0.7/R 10-3 A

Therefore, since R= 860

E-0.7/860 10-3 A

E (860 x10-3)+0.7

E 1.56 V

### Q.2.

Determine the values of VD, IT and VR for the circuits given below.

### (a)

(b)

### Sol. (a)

Because the arrow in the schematic symbol is pointing toward the positive terminal of the source, we know that the diode is reverse biased. Therefore,

- The full voltage is dropped across D1.

VD1=VS=5V

- D1 will not allow conduction. Therefore, IT=0A

- Since there is no current through R1, there is no voltage drop across the component (VR1=0V)

Sol. (b)

Because the arrow in the schematic symbol is pointing toward the negative terminal of the source, we know that the diode is forward biased. Therefore,

a. VD1 =0V, leaving the total applied voltage to be     	dropped across R1.

- VR1=VS=5V

- IT is determined by the source voltage and R1.

IT= VR1/R1

= 5V/1K

= 5mA

### Q.3.

Find the Norton equivalent circuit for the network in the shaded area of the figure:

First step:  Replacing RL with a short circuit to find IN.

Second step:  As it is shown in the above figure indicating that the short circuit connection between terminals a and b is in parallel with R2 and eliminates its effect. IN is therefore the same as through R1, and the full battery voltage appears across R1 since

V2=I2R2

=(0)(6)

= 0V

Therefore

IN = V/R1

= 9V/3 = 3A

### Third step:

### To calculate RN we will short circuit all voltage     	sources.

As shown in the above figure we have

RN=R1|| R2 = 3 ||6 = 2

Fourth Step:

After calculating IN and RN, re-inserting  the load resistance RL in the circuit in parallel RN and  considering the IN  current source parallel with these two resistances.

------ Good Luck -----