Bandwidth Management Ensemble

NMF 038

Issue 1.0

October 1995

ã Network Management Forum 1995.

Members of the Network Management Forum are granted limited copyright waiver, under the terms of the Agreement on Intellectual Property Rights, to reproduce this document.

NOTICE

The Network Management Forum 
has made every effort to assure that the contents of 
this document are accurate; however, due to the inherent 
complexity in the design and implementation of protocols, no 
liability is accepted for any errors or omissions or for 
consequences of any use made of this document.  Under no 
circumstances will the Network Management Forum be liable for 
direct or indirect damages or any costs or losses resulting from the 
use of this specification.  The risk of designing and 
implementing products in accordance with this specification is 
done solely by the user of this specification.  This document 
may involve a claim of patent rights by one or more of the 
contributors to this document, pursuant to the agreement on
Intellectual Property Rights between the 
Network Management Forum and its members.

Inquiries should be directed to the 
Network Management Forum office at:

1201 Mt. Kemble Avenue

Morristown, NJ  07960  USA

Tel. No.  +1 201-425-1900

ACKNOWLEDGMENTS

The Network Management Forum would like to acknowledge the contribution made to this document by the following people:

Michael Best 	Newbridge Networks

Vinod Modha	BT

Subhash Patel	Nortel

Richard Arthur	Newbridge Networks

Bruce Murrill	Network Management Forum

Marianne Jenkinson	Network Management Forum

NETWORK MANAGEMENT FORUM

BANDWIDTH MANAGEMENT ENSEMBLE (BME)

TABLE OF CONTENTS

1. INTRODUCTION	1

1.1 UNIQUE IDENTITY	1

1.2 GENERAL DESCRIPTION OF THE ENSEMBLE	2

1.3 SCOPE AND PURPOSE	5

1.3.1 Scope	5

1.3.2 Positioning	5

1.4 RELATIONSHIP WITH OTHER ENSEMBLES	6

1.5 RELATIONSHIP WITH OTHER STANDARDS	7

2. MANAGEMENT CONTEXT	9

2.1 GENERAL INTRODUCTION	9

2.2 MANAGEMENT VIEW AND LEVEL OF ABSTRACTION	12

2.3 RESOURCES	15

2.3.1 Discriminator	15

2.3.2 Network	15

2.3.3 ManagedElement	15

2.3.4 Equipment	15

2.3.5 GroomedCircuitTerminationPoint	15

2.3.6 GroomedCircuit	15

2.3.7 FacilityTerminationPoint	16

2.3.8 Facility	16

2.4 FUNCTIONS	17

2.4.1  Event Reporting Control	17

2.4.2  State Management	17

2.4.2.1 Administrative State	17

2.4.2.2 Operational State	17

2.4.2.3 Usage State	18

2.4.3  Alarm Management	18

2.4.3.1  Transmission Alarms	18

2.4.3.2  Equipment Alarms	18

2.4.4  Network Inventory	19

2.4.5  Resource Configuration	19

2.4.6  Creating a GroomedCircuitTerminationPoint	19

2.4.7  Creating GroomedCircuits	19

2.4.8  Deleting GroomedCircuits	19

2.4.9  Testing GroomedCircuitTerminationPoints	19

2.4.10  Tracing GroomedCircuits	19

2.4.11  Network Topology Monitoring	20

2.5 SYSTEM PRE-CONDITIONS	20

3A. OMNIPoint 0 MANAGEMENT INFORMATION MODEL	21

3A.1 GENERAL INTRODUCTION	21

3A.2 RELATIONSHIPS	21

3A.2.1 Entity Relationship Diagrams	21

3A.2.2. Object Inheritance Hierarchy	24

3A.3 SCENARIOS	25

3A.3.1 GroomedCircuit Creation	25

3A.3.2 GroomedCircuit Configuration	26

3A.3.3 Restore Groomed Circuit to Optimum Routing - Grooming	26

3A.3.4 Deleting a GroomedCircuit	27

3A.3.5 GroomedCircuitTerminationPoint Creation	27

3A.3.6 Resource Creation	27

3A.3.7 Resource Configuration	27

3A.3.8 Resource Alarm	28

3A.3.9 Resource Status Change	28

3A.3.10 Resource Attribute Change	28

3A.3.11 Perform Loop Back Tests	28

3A.3.12 Tracing a GroomedCircuit	29

3A.3.13 Retrieving Detailed Network Topology Information	29

3A.4 MANAGEMENT INFORMATION REFERENCES	29

3B. OMNIPoint 2 MANAGEMENT INFORMATION MODEL	31

3B.1 GENERAL INTRODUCTION	31

3B.2 RELATIONSHIPS	31

3B.2.1 Entity Relationship Diagrams	31

3B.2.2. Object Inheritance Hierarchy	34

3B.3 SCENARIOS	35

3B.3.1 GroomedCircuit Creation	35

3B.3.2 GroomedCircuit Configuration	36

3B.3.3 Restore Groomed Circuit to Optimum Routing - Grooming	36

3B.3.4 Deleting a GroomedCircuit	37

3B.3.5 GroomedCircuitTerminationPoint Creation	37

3B.3.6 Resource Creation	37

3B.3.7 Resource Configuration	37

3B.3.8 Resource Alarm	38

3B.3.9 Resource Status Change	38

3B.3.10 Resource Attribute Change	38

3B.3.11 Perform Loop Back Tests	38

3B.3.12 Tracing a GroomedCircuit	39

3B.3.13 Retrieving Detailed Network Topology Information	39

3B.4 MANAGEMENT INFORMATION REFERENCES	40

4A. ENSEMBLE CONFORMANCE REQUIREMENTS: OMNIPoint 0	41

4A.1 GENERAL CONFORMANCE REQUIREMENTS	41

4A.2 SPECIFIC CONFORMANCE REQUIREMENTS	41

4B. ENSEMBLE CONFORMANCE REQUIREMENTS:  OMNIPoint 2	42

4B.1 GENERAL CONFORMANCE REQUIREMENTS	42

4B.2 SPECIFIC CONFORMANCE REQUIREMENTS	42

ANNEX A - TERMS AND ACRONYMS	A-1

ANNEX B - MANAGEMENT CONFORMANCE SUMMARY (MCS) PROFORMA	B-1

B.1 INTRODUCTION	B-1

B.1.1 Purpose and structure	B-1

B.1.2 Instructions for completing the MCS proforma to produce an MCS	B-1

B.2 IDENTIFICATION OF THE IMPLEMENTATION	B-1

B.2.1 Date of statement	B-1

B.2.2 Identification of the implementation	B-1

B.2.3 Contact	B-1

B.3 IDENTIFICATION OF THE DOCUMENT IN WHICH THE MANAGEMENT INFORMATION IS DEFINED	B-2

B.3.1 Technical corrigenda implemented	B-2

B.3.2 Amendments implemented	B-2

B.4 MANAGEMENT CONFORMANCE SUMMARY	B-2

ANNEX C - MANAGED OBJECT CONFORMANCE STATEMENTS (MOCS) PROFORMAS	C-1

C.1  bMFacility	C-2

C.2  bMFacilityTP	C-12

C.3  bMGroomedCircuit	C-21

C.4  bMGroomedCircuitTP	C-31

ANNEX D - MANAGED OBJECT CLASSES (GDMO)	D-1

D.1 GDMO Definitions for OP0 Specification	D-1

D.2 GDMO Definitions for OP2 Specification	D-15

ANNEX E - MIGRATION INFORMATION	E-1

ANNEX F - REFERENCES	F-1

LIST OF TABLES

Table 2.4.3.1.1 : Facility state changes and corresponding Alarms	18

Table 2.4.3.1.2 : GroomedCircuit state changes and corresponding Alarms	18

Table 2.4.3.2.1 : Equipment state changes and corresponding Alarms	18

Table 3A.2.1: BME Resources and MO classes	22

Table 3B.2.1: BME Resources and MO classes	31

Table B.1 MOCS support summary	B-3

Table B.2 MRCS support summary	B-3

Table C.1.1  bMFacility Managed object class support	C-2

Table C.1.2  bMFacility Actual class support	C-2

Table C.1.3  bMFacility Package support	C-2

Table C.1.4  bMFacility Attribute support	C-3

Table C.1.5  bMFacility Attribute group support	C-5

Table C.1.6  bMFacility Action support	C-5

Table C.1.7  bMFacility Notification support	C-5

Table C.1.8  bMFacility Parameter support	C-11

Table C.1.9  bMFacility Conditions	C-11

Table C.1.10  bMFacility Name Binding support	C-11

Table C.2.1  bMFacilityTP Managed object class support	C-12

Table C.2.2  bMFacilityTP Actual class support	C-12

Table C.2.3  bMFacilityTP Package support	C-12

Table C.2.4  bMFacilityTP Attribute support	C-13

Table C.2.5  bMFacilityTP Attribute group support	C-14

Table C.2.6  bMFacilityTP Action support	C-14

Table C.2.7  bMFacilityTP Notification support	C-15

Table C.2.8  bMFacilityTP Parameter support	C-20

Table C.2.9  bMFacilityTP Conditions	C-20

Table C.2.10  bMFacilityTP Name Binding support	C-20

Table C.3.1  bMGroomedCircuit Managed object class support	C-21

Table C.3.2  bMGroomedCircuit Actual class support	C-21

Table C.3.3  bMGroomedCircuit Package support	C-21

Table C.3.4  bMGroomedCircuit Attribute support	C-22

Table C.3.5  bMGroomedCircuit Attribute group support	C-24

Table C.3.6  bMGroomedCircuit Action support	C-24

Table C.3.7  bMGroomedCircuit Notification support	C-24

Table C.3.8  bMGroomedCircuit Parameter support	C-29

Table C.3.9  bMGroomedCircuit Conditions	C-30

Table C.3.10  bMGroomedCircuit Name Binding support	C-30

Table C.4.1  bMGroomedCircuitTP Managed object class support	C-31

Table C.4.2  bMGroomedCircuitTP Actual class support	C-31

Table C.4.3  bMGroomedCircuitTP Package support	C-31

Table C.4.4  bMGroomedCircuitTP Attribute support	C-32

Table C.4.5  bMGroomedCircuitTP Attribute group support	C-33

Table C.4.6  bMGroomedCircuitTP Action support	C-34

Table C.4.7  bMGroomedCircuitTP Notification support	C-34

Table C.4.8  bMGroomedCircuitTP Parameter support	C-39

Table C.4.9  bMGroomedCircuitTP Conditions	C-39

Table C.4.10  bMGroomedCircuitTP Name Binding support	C-40

Table E.1  BME Resources and MO classes	E-1

LIST OF FIGURES

1.2.1 Positioning within the TMN architecture	2

1.2.2 Service Manager with Multiple Ensemble interfaces	2

1.2.3 Connection through a network cloud	3

1.4.1 Relationship with other Ensembles	6

2.1.1 Context of Ensemble	10

2.1.2 End to End Service within one Service Provider Jurisdiction	11

2.2.1 Network Level Bandwidth Service	12

2.2.2 Physical view of the network	13

2.2.3 Physical view of a network element	14

3A.2.1.1 Resource Entity Relationship Diagram	22

3A.2.1.2 Managed Object Entity Relationships	23

3A.2.2 Managed Object Inheritance Relationships	24

3B.2.1.1 Resource Entity Relationship Diagram	32

3B.2.1.2 Managed Object Entity Relationships	33

3B.2.2 Managed Object Inheritance Relationships	34

REVISION HISTORY

Draft 0.1	Mar 1995

Draft 0.2	April 1995

Draft 0.3	July 1995

Issue 1.0	Sept 1995

# 1. INTRODUCTION

Ensembles provide a top down view of a particular solution to a management problem. In order to focus on the solution to this management problem, specific restrictions are placed upon particular referenced definitions. The concepts and format of Ensembles are described in the "NM Forum Ensemble Concepts and Format"[1] specification document.

Each Ensemble contains general text in each section that is common to all Ensembles. By convention this common text is portrayed in bold italic characters.

This Ensemble, wherever possible, references documents which define the components of the Ensemble.

The management problem is identified as a set of requirements and constraints. In defining the solution to this management problem, the resources to be managed, the functions to be applied, and the scenarios describing the interactions are all identified. The Ensemble references base standards and International Standardized Profiles (ISPs). It also references libraries containing definitions expressed by GDMO (Guidelines for the Definition of Managed Objects [2]) templates.

The purpose of this document is to collect management information definitions and profiles, and show how they can be applied to manage the resources identified in this Ensemble.

This document is organized as follows:

Section 1	"General Information", provides a high level overview describing the Ensemble and the structure of the document.

Section 2	"Management Context", identifies the managed resources and

management capabilities of the Ensemble.

Section 3	"Information Model", specifies all management information

components of this Ensemble.

Section 4	"Ensemble Conformance Requirements", provides or references statements of conformance for this Ensemble. The Proformas, specific to the Ensemble are outlined in Annex B.

## 1.1 UNIQUE IDENTITY

The unique identity is a registered object identifier used to identify this Ensemble.

{iso(1) member-Body(2) canada(124) forum(360501) ensemble(11) bme( 3 ) }

## 1.2 GENERAL DESCRIPTION OF THE ENSEMBLE

This Ensemble specifies managed resources and application functions that define a management interface to the network management layer which would be used by the service management layer within one service provider domain in the TMN architecture (see Fig. 1.2.1).  Other interfaces are out of scope and therefore not considered by this Ensemble however they could be integrated with,  and supported by this interface;  an example is an X interface to another service provider domain or a service provider to customer interface.  Consult ITU-T Recommendation M.3010[16] for general TMN architectural concepts.

Fig 1.2.1 Positioning within the TMN architecture

Service level requirements demand a simplified model of network resources.  There are several reasons for this: first is the scale of managed networks at the service level (which are usually very large) and the second is the ability to represent different vendors equipment in the same uniform manner.  These requirements lead to scalable management solutions which will support services across multi-vendor platforms (see Fig 1.2.2).

Fig 1.2.2 Service Manager with multiple Ensemble interfaces

The interface enables the control and monitoring of various resources comprising intelligent networks;  which provide functions such as circuit provisioning,  bandwidth optimization, automatic reroute around network failures,  path tracing and testing capability.

The concepts of this Ensemble were first used to formulate a bandwidth model using OMNIPoint  0 (OP0) standards (circa 1991) as the base standards.  This model was agreed to and implemented by several companies,  but never formally documented.  This Ensemble provides several things:  the formal documentation of the OP0 bandwidth model, the specification of a new OMNIPoint  2 (OP2) bandwidth model and a discussion of migration issues between the two specifications.

A connection provided over a network (represented by a cloud) may be thought of as a bandwidth service.  At the edge of the cloud are points that a service user may access; they can be thought of as service access points. Typically the service user is unaware of the actual physical path that the service takes across the network, or indeed which network vendors equipment is used to provide that path.

The managed resources used to model network components provide functionality to fulfill bandwidth related service requirements.  The interface will incorporate many of the desirable characteristics of intelligent networks while focusing on bandwidth service management.

Fig 1.2.3 Connection through a network cloud

The Ensemble document is partitioned into sections.  Sections 1 and 2 define the problem space as well as provide an implementation-independent solution.  The solution is presented as a set of managed resources and functions performed on those resources.  Sections 3 and 4 map these generic concepts onto a specific set of standards to provide an implementable specification.

The first two sections of this Ensemble define the BME (Bandwidth Management Ensemble) problem space and provide a generic solution.  These ideas are then mapped onto two sets of standards producing two specifications.  The first maps onto OMNIPoint  0 standards;  it specifies the OP0 bandwidth model.  The second draws on the experience of implementing the OP0 model and uses more recent and recognized standards to specify the OP2 bandwidth model.  Having both OP0 and OP2 specifications provides a complete set of BME work to date;  as well it facilitates migration between the two specifications.

Sections 3 and 4 of this Ensemble map the high level solution presented in sections 1 and 2 onto these standards.  There are two versions of sections 3 and 4 in this Ensemble.  The first version provides a mapping onto OP0 while the second maps onto OP2.  In order to maintain the same look and feel of the Ensemble template the OP0 sections are named 3A  and 4A while the OP2 equivalents are named 3B and 4B.

## 1.3 SCOPE AND PURPOSE

Ensembles represent specific solutions to particular problems. Thus, an Ensemble is the complete description of the problem and the solution to that problem.	

This section describes the requirements that describe the problem. It includes the definition of the information model that represents the solution to a problem. These definitions comprise references to one or more management information libraries which contain definitions of managed object classes expressed in GDMO templates, packages, attributes, name bindings, etc. Also, included in the Ensemble definition are statements of conformance and suitable proformas.

This Ensemble addresses the management interface between the service management layer and the network management layer within a single service provider jurisdiction(see Fig. 1.2.1 and Fig. 1.2.2).  Its purpose is to provide services focused on bandwidth management of the network management layer.  This is based on an abstract view of the of the resources making up a network which shields the service management layer from a large amount of the detail of the network level.

### 1.3.1 Scope

The Ensemble encompasses the following functionality:

_	management of bandwidth services at the network level

_	alarm and event management

_	holding of a network inventory

_	configuration of network resources where applicable

### 1.3.2 Positioning

Systems implementing this Ensemble will not supplant or replace proprietary network management systems for the purpose of detailed management of the resources in the network. On the contrary,  it is designed to represent a network management jurisdiction such that distinct multi-vendor networks may be integrated by a service manager and end to end services which span multiple network management systems may be supported.

Though not in the scope of this Ensemble the service provider can use the information provided by this interface to support other management interfaces.

The document is intended to aid product developers in the design of management systems, and to help buyers in the specification of system requirements.

## 1.4 RELATIONSHIP WITH OTHER ENSEMBLES

There are a number of Ensembles which can be used by a service provider to tailor the service he offers to individual customers.  This Ensemble,  along with SIM[10],  CACM[11] and others are potential candidates.

The BME however is related to the RCS-CM [3] and RCS-AS [4] Ensembles which were part of the and OMNIPoint 1 specifications.  In fact the this Ensemble has evolved from some of the ideas in the RCS Ensembles.

Fig 1.4.1 Relationship with other Ensembles

This Ensemble provides two specifications based on OMNIPoint 0 and OMNIPoint 2 specifications (see Section 1.2).  Please consult Annex F for migration information.

## 1.5 RELATIONSHIP WITH OTHER STANDARDS

This Ensemble refers to a number of standards.  All the relevant standards are listed under reference specifications (ANNEX F) at the end of the document.

It is intended by OMNIPoint  program that wherever possible, International Standards from ISO (namely OSI Systems Management Standards) and ITU-T (TMN) standards are specified as first choice.  Where International Standards are not available or need supplementing, appropriate OMNIPoint  specifications, where they exist, are referenced as the next choice.  This Ensemble  follows that guide line.

# 2. MANAGEMENT CONTEXT

The "Management Context" describes why the Ensemble is required. The description of the "Management Context" includes the definition of the resources to be managed, the management functions to be performed, the scope of the problem to be solved, and the management view or level of abstraction from which the problem is to be approached.

## 2.1 GENERAL INTRODUCTION

Managing the introduction and ongoing operations of telecommunications services is a highly complex activity.  Telecommunications service providers build expensive and complex service management systems with which to carry out this activity.  These service management systems often have to interoperate with a number of lower level multi-vendor network management systems which control the underlying network resources.  Each vendor supplies unique capabilities and functions within their technologies and within the management systems that they provide.  In order to simplify the task of service managing a multi-vendor provided network the service provider needs to ensure that those vendors conform to a specific set of common management standards which support their service offerings.

Existing standards, such as those from the NMF, ISO, ITU-T, and the IETF, help by proposing network architectures,  providing objects to represent network resources and models to describe the way a function (such as an alarm) will operate between systems.  However, existing object models are usually very general, and have not been backed up by implementation experience.

A particular service provider is generally only interested in a small sub-set of the spectrum of standards to solve a particular management problem.  Ensembles are used to define and detail such a subset.

This Ensemble concentrates on a set of bandwidth related services to allow management of intelligent networks in a multi-vendor environment.  The equipment types targeted include channel banks, T1/E1 multiplexors, T3/E3 multiplexors, as well as Frame Relay,  ATM,  ISDN and SONET/SDH transmission equipment.

Within the context of this Ensemble, service providers can manage bandwidth services within individual networks and by correlating many such networks (see Fig. 2.1.1 and 2.1.2) can offer customers managed end-to-end bandwidth services with a wide variety of access interfaces.  Services such as automatic re-routing and connection optimization can also be offered.

Fig 2.1.1 Context of Ensemble

To enable the service provider to have end-to-end network visibility the service level manager interfaces with agents on each vendor's network management system who have implemented this specification; thus providing the following functions:

•	bandwidth services within individual network jurisdictions

•	end to end bandwidth services between one or more network domains

•	Alarm and event reporting for both physical resources and services (such as bandwidth)

•	Network resource inventory

•	Resource configuration

Therefore, through the use of this Ensemble service level managers can correlate information from multiple network management systems which act as delegates for their underlying network equipment.  The network management systems may be from the same or multiple vendors

(See Fig. 2.1.2 and Fig. 1.2.2).

Fig 2.1.2 End to End Service within one Service Provider Jurisdiction

## 2.2 MANAGEMENT VIEW AND LEVEL OF ABSTRACTION

This section indicates the management view of the Ensemble which includes information on the level of abstraction. For example, in an hierarchically organized system this section would indicate if the Ensemble deals with the management of equipment, the management of networks or the management of services. It may also indicate management perspectives and roles.

This Ensemble is concerned with the management of networks.  It focuses on management of bandwidth services at the network level.  Network level bandwidth services are modeled as connections through a network cloud while static bandwidth resources can be thought of as bandwidth 'pipes'.

Fig 2.2.1 Network Level Bandwidth Service

A high level equipment view of the underlying network resources is provided. The intent of this equipment modeling is not to provide detailed management capability for these resources;  it is meant to provide management correlation between the bandwidth model and the underlying network resources and inventory functionality.  The network view contains one or more networks.  A network contains managed elements.

Fig 2.2.2 Physical view of the network

At the managed element level, a hierarchy of contained equipment resources is used to represent a node and its underlying hardware components (i.e.  its shelves, cards and ports).

Fig 2.2.3 Physical view of a network element

## 2.3 RESOURCES

This section defines all the resources or components of resources that are to be the subject of the Ensemble. The definition of the resources contains all the resources and only those resources that are relevant to the Ensemble. The resources are defined by textual descriptions or by reference to other documents containing descriptions of the resources. When other documents are referenced statements are provided to indicate any restrictions and constraints on those source definitions.

### 2.3.1 Discriminator

A Discriminator resource is used to control the flow of alarms and events from the managed system to the managing system.  It specifies event and alarm criteria for the entire network.  The managed system must support at least one Discriminator.

### 2.3.2 Network

A Network represents a set of interconnected communications resources, both logical and physical.  These are high level resources that generally contain resources that have one or more characteristics in common (i.e. geographic location).  In the context of this Ensemble this resource will contain resources making up the network.

### 2.3.3 ManagedElement

A ManagedElement resource represents a network entity which provides containment for equipment resources in the network.  The ManagedElement allows the Equipment resources in the network to be organized and contained in a manner consistent with the real world entities they represent.

### 2.3.4 Equipment

The Equipment resource represents the physical and logical components of a ManagedElement down to a port (service access point) level (see Fig. 2.2.3).

### 2.3.5 GroomedCircuitTerminationPoint

A GroomedCircuitTerminationPoint represents a service access point in the network. This resource is used to terminate a GroomedCircuit resource.  In some circumstances (dependent upon the underlying technology) a GroomedCircuitTerminationPoint may be created and deleted across the interoperable interface.

### 2.3.6 GroomedCircuit

A GroomedCircuit represents a bandwidth service in the network.  It can automatically reroute around network failures, be restored to an optimal routing (groomed), tested (by the application of loopbacks on the end points of the connection) and have routing profile attributes. A GroomedCircuit may be created and deleted across the interoperable interface.

### 2.3.7 FacilityTerminationPoint

A FacilityTerminationPoint represents a static internal access point in the network.  This resource is used to terminate a Facility resource.  They are established through a provisioning process and therefore may not be created or deleted over the interoperable interface.

### 2.3.8 Facility

A Facility represents a static bandwidth resource, or 'pipe',  in the network;  this could be a physical entity such as a cable or a logical pipe within a physical entity.  Depending on the underlying technology they are either established through a provisioning process or are created/deleted through the interoperable interface.  In the case of provisioned Facilities they cannot be created or deleted over the interoperable interface.

## 2.4 FUNCTIONS

This section defines the management functions that can be performed on the resources described in Section 2.3. These functions may be primitive functions defined for OSI systems management (e.g. event management), higher level functions for general network management (e.g. alarm surveillance), or other functions unique to the problem the Ensemble addresses.

The following functions are provided:

•	Event reporting control

•	State Management

•	Alarm Management

•	Network Inventory

•	Resource Configuration

•	Creating GroomedCircuitTerminationPoints

•	Creating GroomedCircuits

•	Deleting GroomedCircuits

•	Testing GroomedCircuitTerminationPoints

•	Tracing a GroomedCircuit

•	Network Topology Monitoring

### 2.4.1  Event Reporting Control

The Managing system will be able to control the flow of events from the Managed System.  A Discriminator resource will be used to select event reports.  In this manner the Managing system will be able to control the flow of events resulting from Object Creation,  Object Deletion,  Status Change and Attribute Change notifications.  The managed system must support at least one  Discriminator resource,  it shall specify event reporting criteria for the entire managed network.

### 2.4.2  State Management

State management is an important management function.  The managing system will be notified through statusChange Event Reports of resource state changes (assuming the Discriminator resource has been appropriately configured - see section 2.4.1).

The overall state of a resource reflects a combination of Administrative State,  Operational State and Usage State.  A brief description of each of these states is as follows.

#### 2.4.2.1 Administrative State

This state reflects the managed state of a resource.  The Managing System is allowed to set the desired state of configurable resources such as GroomedCircuits and GroomedCircuitTerminationPoints.  Resources that are not configurable from the Managing System such as Equipment have a static Administrative state.  The administrative state can be unlocked,  locked or shuttingDown.  Within the context of this Ensemble only the unlocked and locked statuses are used.

#### 2.4.2.2 Operational State

This state reflects that actual status of the resource within the network.  This state is maintained by the Managed System the Managing System is not permitted to modify it.  It can have values of enabled or disabled.

#### 2.4.2.3 Usage State

Some resources have a Usage State which indicates the actual usage of a resource.  It can have values of idle,  active or busy.

### 2.4.3  Alarm Management

Alarms are generated because of resource state changes which are service affecting(i.e. GroomedCircuit has been re-routed,  a piece of equipment has failed etc.).  There are two types of Alarms which will be provided by the interface: Transmission Alarms and Equipment Alarms.

#### 2.4.3.1  Transmission Alarms

Transmission Alarms are generated by network events that affect Facility or GroomedCircuit resources.  The events in question involve state changes.  Table 2.4.3.1.1 describes the Facility state transitions that cause Transmission Alarms.

table 2.4.3.1.1: Facility state changes and corresponding Alarms

| Event | State Change | Alarm Generated |
| --- | --- | --- |
| Facility disrupted | Unlocked Enabled -> Unlocked Disabled | Transmission Alarm - active |
| Facility restored | Unlocked Disabled -> Unlocked Enabled | Transmission Alarm - clear |

GroomedCircuit resources can be created and subsequently configured by both the Managing System as well as the Managed System.   Transmission Alarms are produced against these resources in response to state transitions which can occur because of network events as well as actions performed by the Manager.  Table 2.4.3.1.2 documents these transitions and the corresponding Alarms generated.

table 2.4.3.1.2: GroomedCircuit state changes and corresponding Alarms

| Event | State Change | Alarm Generated |
| --- | --- | --- |
| service disrupted | Unlocked Enabled -> Unlocked Disabled | Transmission Alarm - active |
| service restored | Unlocked Disabled -> Unlocked Enabled | Transmission Alarm - clear |

#### 2.4.3.2  Equipment Alarms

Equipment Alarms are produced against resources in response to state transitions.  Table 2.4.3.2.1  shows the equipment state transitions which cause Equipment Alarms.

table 2.4.3.2.1: Equipment state changes and corresponding Alarms

| State Change | Alarm Generated |
| --- | --- |
| Unlocked Enabled -> Unlocked Disabled | Equipment Alarm - active |
| Unlocked Disabled -> Unlocked Enabled | Equipment Alarm - clear |

### 2.4.4  Network Inventory

The Managed System will maintain a set of objects representing an inventory of all the resources in the network.  The Managing System will be able to retrieve any or all of these resources depending on its own internal inventory requirements.  In addition, through the flow of event reports the Managing System will be notified about any changes to the network inventory (assuming the Discriminator has been configured appropriately - see section 2.4.1).  In a similar fashion any new resources provisioned on the network will result in event reports (i.e. Object Creation) arriving to the Managing System (assuming the discriminator has been appropriately configured - see section 2.4.1).

### 2.4.5  Resource Configuration

The configuration of static resources is a network provisioning/maintenance function and is beyond the scope of this Ensemble.  However,  functionality is provided for the configuration of GroomedCircuitTerminationPoints and GroomedCircuits.  These resources enable bandwidth services;  in order for the Manager to provide flexible bandwidth services these resources are configurable.

### 2.4.6  Creating a GroomedCircuitTerminationPoint

The Managing System may be able to create GroomedCircuitTerminationPoint resources across the interoperable interface under certain circumstances.  This functionality is provided in order to support virtual GroomedCircuitTerminationPoints (such as would be used with ATM,  Frame Relay and SONET/SDH equipment) without having to provision huge numbers of these statically.  The resource must be associated with an existing (previously provisioned) Managed Element resource.

### 2.4.7  Creating GroomedCircuits

The Managing System is able to create GroomedCircuit resources across the interoperable interface.  GroomedCircuits are bandwidth services.   The Managing System must provide a set of mandatory information,  i.e. A and Z endpoints (which are GroomedCircuitTerminationPoint resources).  Additionally the Managing System can supply optional configuration information such as bandwidth required,  routing profiles as well as others.

### 2.4.8  Deleting GroomedCircuits

GroomedCircuits can be deleted across the interoperable interface by the manager.  A GroomedCircuit can only be deleted when it is in a locked state (see Section 2.4.2).

### 2.4.9  Testing GroomedCircuitTerminationPoints

GroomedCircuitTerminationPoints can be tested by the application of loopbacks.  This type of testing can be used to ensure basic functionality.  Loopbacks can be initiated and terminated across the interoperable interface.

### 2.4.10  Tracing GroomedCircuits

This Ensemble focuses on bandwidth services represented by GroomedCircuits.  GroomedCircuits are considered to be connections through a network cloud however a managing system can request the detailed route taken by  a GroomedCircuit through a network.  This provides detailed information about the route through a network in order to support functions such as testing GroomedCircuitTerminationPoints.

### 2.4.11  Network Topology Monitoring

Facilities model the static connections between Managed Elements in the network.  These resources are statically provisioned and provide the managing system with a complete view of network topology.   In addition the managing system will be able to request detailed topology information from the managed system.  This information will correlate the Facility resources of the network with bandwidth services.  This will give the managing system a detailed view of the bandwidth resources in the network.

## 2.5 SYSTEM PRE-CONDITIONS

Existence of a number of pre-conditions was assumed during the development of this Ensemble and these pre-conditions are listed below for completion.

1)	Common functions such as alarm monitoring and reporting and communications path already exist between the manager role and agent role systems and are available for use for the purpose of this Ensemble.

2)	Where suitable object classes exist through implementations of, for instance, RCS-CM [3], RCS-AS [4] and objects aligned with M.3100, then these are available for re-use for the purpose of this Ensemble.

3)	The Ensemble may enhance an already existing service (based on OMNIPoint  [0 or 1]  specifications) or provide a completely new service (based on OMNIPoint 2 specifications).

Note: Section 3 consists of two parts;  section 3A specifies the OP0 			Information Model while section 3B specifies the OP2

Information model.  Please consult section 1.2 for more

information.

# 3A. OMNIPoint 0 MANAGEMENT INFORMATION MODEL

The Information Model focuses on the real world under study. It contains information about both the elements of the model and their interrelationships. The elements of management information are defined using GDMO templates and their interrelationships are graphically illustrated.

## 3A.1 GENERAL INTRODUCTION

This Ensemble defines an interface that enables the management of network layer resources in order to provide bandwidth management services.  The resources model physical and logical components which make up a network (i.e. nodes, cards) as well as bandwidth services.  In order to facilitate implementation and interoperability the resources (see Section 2.3) are represented as standardized managed objects.  The functions of the interface (see Section 2.4) are represented as operations on the objects.  The combination of this information model and operations on the model define a Q3 interface.

## 3A.2 RELATIONSHIPS

This section defines the relationships between the components of the model. These may be expressed in Entity Relationship (ER) diagrams or other similar graphical representations.	

Three types of diagrams are used:	
	- one for the relationships inherent in the underlying resources,
	- one for the relationships among the classes representing these

resources,
	- and one for the naming schema.

### 3A.2.1 Entity Relationship Diagrams

The relationships between resources and managed object classes is summarized in the following table:

Table 3A.2.1: BME Resources and MO classes

| Resource | Managed Object Class |
| --- | --- |
| Discriminator | "Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91": eventReportingSieve |
| Network | "Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91": network |
| Managed Element | "Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91": equipment |
| Equipment | "Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91": equipment |
| Facility | "Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91": facility |
| Groomed Circuit Termination Points | "NMF038, Issue 1": terminationPoint |
| Groomed Circuit | "NMF038, Issue 1": groomedCircuit |
| FacilityTerminationPoints | "NMF038, Issue 1": terminatingEquipment |

The following diagram illustrates the relationship between resources:

Fig 3A.2.1.1 Resource Entity Relationship Diagram

The following diagram illustrates the relationship between managed objects:

Fig 3A.2.1.2 Managed Object Entity Relationships

### 3A.2.2. Object Inheritance Hierarchy

The following diagram illustrates the inheritance hierarchy for the managed object classes.

Fig 3A.2.2 Managed Object Inheritance Relationships

## 3A.3 SCENARIOS

This section defines the Ensemble scenarios. Each of these definitions consists of a brief textual description and message flow diagrams.	

The scenarios are used to show how the managed objects in the information model can be used to accomplish the function listed in Section 2.4.

In the scenarios that follow, CMIP flows between (and corresponding CMIS primitives within) manager and agent systems are indicated by arrows with a three character abbreviation for request, indication, response and confirmation primitives shown at the head and tail of the arrow. For example:

o-- Req -------------------- Ind ->

CMIS request

<-- Cnf -------------------- Rsp -o

CMIS response

### 3A.3.1 GroomedCircuit Creation

A GroomedCircuit is modeled by an instance of groomedCircuit.  In order to create a GroomedCircuit the manager sends an M-CREATE request with an objectClass parameter of groomedCircuit.

Mandatory attributes must be supplied at create time (i.e. A and Z endpoints);  optional attributes (i.e.  administrativeState, circuitBandwidth) may also be set,  consult ANNEX B for full details.

The GroomedCircuit may be created in either the locked or the unlocked state.  When it is created in the locked state, it is not immediately available for use.  When it is created unlocked it may be used immediately.  After the connection is in place the user can use administrative state to control access to it.

A GroomedCircuit can be created by the network management system without a create request across the interoperable interface. In this case, the notifies the manager of a circuit creation via an objectCreation event report.

o-Req -------------------- Ind->

M-CREATE (...objectClass=groomedCircuit,

aEndpointName=endpoint1, zEndpointName=endpoint2,...) request

<-Cnf -------------------- Rsp-o

M-CREATE response

<-Ind --------------------- Req-o

M-EVENT-REPORT objectCreation

### 3A.3.2 GroomedCircuit Configuration

The user will have the ability to change the configuration of a GroomedCircuit by directing an M-SET operation to the instance of the groomedCircuit object representing it. Attributes which are capable of being set across the interoperable interface are shown in Annex B.

For example the service provider may ensure fibre routing of a GroomedCircuit resource by  issuing an M-SET operation for the trmFibreRouting attribute of the groomedCircuit object instance representing the GroomedCircuit.  This attribute takes the values of don't Care, required or notRequired.

o-Req -------------------- Ind->

M-SET (...administrativeState=Locked...)request

<-Cnf -------------------- Rsp-o

M-SET response

o-Req -------------------- Ind->

M-SET (trmFibreRouting=required) request

<-Cnf -------------------- Rsp-o

M-SET response

o-Req -------------------- Ind->

M-SET (...administrativeState=unlocked...) request

<-Cnf -------------------- Rsp-o

M-SET response

### 3A.3.3 Restore Groomed Circuit to Optimum Routing - Grooming

A manager will be able to reconfigure a GroomedCircuit to an optimum route following an automatic or a planned re-route.  This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmCircuitGrooming

<-Cnf -------------------- Rsp-o

M-ACTION response

<-Ind -------------------- Req-o

M-EVENT-REPORT CircuitGroomed

### 3A.3.4 Deleting a GroomedCircuit

A manager can request the deletion of a GroomedCircuit by directing an M-DELETE operation to the groomedCircuit object instance representing the GroomedCircuit.  The object must have an administrativeState of locked before it can be deleted.

o-Req --------------------	Ind->

M-DELETE request

<-Cnf --------------------	Rsp-o

M-DELETE response

<-Ind --------------------  Rsp-o

M-EVENT-REPORT objectDeletion

### 3A.3.5 GroomedCircuitTerminationPoint Creation

In some circumstances (see Section 2.4.5) a manager can create an instance of terminationPoint representing the GroomedCircuitTerminationPoint across the interoperable interface.  The object created must have a 'supported by' relationship with an existing terminatingEquipment object.

o-Req -------------------- Ind->

M-CREATE (...objectClass=terminationPoint...) request

<-Cnf -------------------- Rsp-o

M-CREATE response

<-Ind -------------------- Req-o

M-EVENT-REPORT objectCreation

### 3A.3.6 Resource Creation

All resources besides GroomedCircuits are statically provisioned.  The manager is notified of newly provisioned resources via objectCreation events.

<-Ind -------------------- Req-o

M-EVENT-REPORT objectCreation

### 3A.3.7 Resource Configuration

A manager can change the configuration of a resource by directing an M-SET operation to the object instance that represents it.  The attributes which are capable of being set across the interoperable interface are shown in the tables in Annex B.

For example,  the manager may wish to modify the attributes of a terminationPoint instance representing a GroomedCircuitTerminationPoint to change voice-related characteristics.

o-Req -------------------- Ind->

M-SET (...gender=dce...)request

<-Cnf -------------------- Rsp-o

M-SET response

o-Req -------------------- Ind->

M-SET (voiceMode=) request

<-Cnf -------------------- Rsp-o

M-SET response

### 3A.3.8 Resource Alarm

If a service affecting network event occurs,  and it is directly related to a network resource an Alarm is sent to the manager (assuming the Discriminator resource has been configured accordingly - see Section 2.4.1).

<-Ind -------------------- Req-o

### 3A.3.9 Resource Status Change

If the status of a resource changes it is made visible via a M-EVENT-REPORT stateChange notification.

<-Ind -------------------- Req-o

M-EVENT-REPORT stateChange

### 3A.3.10 Resource Attribute Change

If an attribute of an object representing a resource changes (besides state attributes), the change is made visible via a M-EVENT-REPORT attributeValueChange notification.

<-Ind -------------------- Req-o

M-EVENT-REPORT attributeValueChange

### 3A.3.11 Perform Loop Back Tests

A manager will be able to apply a loopback on an instance of terminationPoint representing a GroomedCircuitTerminationPoint .  This is done by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmLocalLoop

<-Cnf -------------------- Rsp-o

M-ACTION response

A manager will be able to remove a loopback applied on an instance of terminationPoint representing a GroomedCircuitTerminationPoint    This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmAbort

<-Cnf -------------------- Rsp-o

M-ACTION response

A manager will be able to identify whether a loopback is currently applied on a terminationPoint.  This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmGetStatus

<-Cnf -------------------- Rsp-o

M-ACTION response

### 3A.3.12 Tracing a GroomedCircuit

A manager will be able to trace the route taken by a GroomedCircuit through the network.  This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmCircuitTrace

<-Cnf -------------------- Rsp-o

M-ACTION response

### 3A.3.13 Retrieving Detailed Network Topology Information

The managing system will be able to request the correlation between the static bandwidth resources in the network and bandwidth services.  To do this the managing system issues an M-GET operation on an instance of the facility objectClass which represents a Facility resource.  The circuitNames attribute of the facility will contain all instances of groomedCircuit which traverse that facility instance.

o-Req -------------------- Ind->

M-GET (objectClass=facility) request

<-Cnf -------------------- Rsp-o

M-GET response

## 3A.4 MANAGEMENT INFORMATION REFERENCES

This section references all the definitions of management information relevant to the Ensemble. The definitions will be provided entirely by references to other documents which contain GDMO specifications.	

This section contains only references to definitions that are relevant to the Ensemble. Thus, this section also contains statements about any additional restrictions or constraints to those definitions.

Object classes from the following information models, and classes compatible with these, are included in this Ensemble:

Managed Object List:

Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91 Library

"Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91":eventReportingSieve

"Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91":facility

"Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91":network

"Forum 006 Issue 1.3, Volume 1 Supplement, Issue 1.1 Aug 91":equipment

Defined by this Ensemble

"NMF038, Issue 1": terminatingEquipment

"NMF038, Issue 1": terminationPoint

"NMF038, Issue 1": groomedCircuit

# 3B. OMNIPoint 2 MANAGEMENT INFORMATION MODEL

The Information Model focuses on the real world under study. It contains information about both the elements of the model and their interrelationships. The elements of management information are defined using GDMO templates and their interrelationships are graphically illustrated.

## 3B.1 GENERAL INTRODUCTION

This Ensemble defines an interface that enables the management of network layer resources in order to provide bandwidth management services.  The resources model physical and logical components which make up a network (i.e. nodes, cards) as well as bandwidth services.  In order to facilitate implementation and interoperability  the resources (see Section 2.3) are represented as standardized managed objects.  The functions of the interface (see Section 2.4) are represented as operations on the objects.  The combination of this information model and operations on the model define an Q3 interface.

## 3B.2 RELATIONSHIPS

This section defines the relationships between the components of the model. These may be expressed in Entity Relationship (ER) diagrams or other similar graphical representations.

Three types of diagrams are used:
	- one for the relationships inherent in the underlying resources,
	- one for the relationships among the classes representing these

resources,
	- and one for the naming schema.

### 3B.2.1 Entity Relationship Diagrams

The relationships between resources and managed object classes is summarized in the following table:

Table 3B.2.1: BME Resources and MO classes

| Resource | Managed Object Class |
| --- | --- |
| Discriminator | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": eventForwardingDiscriminator |
| Network | "Rec M.3100: 1995":network |
| Managed Element | "Rec M.3100: 1995":managedElement |
| Equipment | "Rec M.3100: 1995":equipment |
|  | "Rec M.3100: 1995":equipmentHolder |
|  | "Rec M.3100: 1995":circuitPack |
| Groomed Circuit Termination Points | "NMF038, Issue 1": bMGroomedCircuitTP |
| Groomed Circuit | "NMF038, Issue 1": bMGroomedCircuit |
| FacilityTerminationPoints | "NMF038, Issue 1": bMFacilityTP |
| Facility | "NMF038, Issue 1": bMFacility |

The following diagram illustrates the relationship between resources:

Fig 3B.2.1.1 Resource Entity Relationship Diagram

The following diagram illustrates the relationship between managed objects:

Fig 3B.2.1.2 Managed Object Entity Relationships

### 3B.2.2. Object Inheritance Hierarchy

The following diagram illustrates the inheritance hierarchy for the managed object classes.

Fig 3B.2.2 Managed Object Inheritance Relationships

## 3B.3 SCENARIOS

This section defines the Ensemble scenarios. Each of these definitions consists of a brief textual description and message flow diagrams.	

The scenarios are used to show how the managed objects in the information model can be used to accomplish the function listed in Section 2.4.

In the scenarios that follow, CMIP flows between (and corresponding CMIS primitives within) manager and agent systems are indicated by arrows with a three character abbreviation for request, indication, response and confirmation primitives shown at the head and tail of the arrow. For example:

o-- Req -------------------- Ind ->

CMIS request

<-- Cnf -------------------- Rsp -o

CMIS response

### 3B.3.1 GroomedCircuit Creation

A GroomedCircuit is modeled by an instance of bMGroomedCircuit.  In order to create a GroomedCircuit the manager sends an M-CREATE request with an objectClass parameter of bMGroomedCircuit.  Mandatory attributes must be supplied at create time (i.e. a-TPInstance, z-TPInstance);  optional attributes (i.e.  administrativeState, circuitBandwidth) may also be set,  consult ANNEX B for full details.

An instance of bMGroomedCircuit representing a GroomedCircuit may be created in either the locked or the unlocked state.  When it is created in the locked state, it is not immediately available for use.  When it is created unlocked it may be used immediately.  After the bMGroomedCircuit is created the user can use administrative state to control access to it.

A GroomedCircuit can be created by the element manager without a create request across the interoperable interface. In this case, the agent notifies the manager of a circuit creation via an objectCreation event.

o-Req -------------------- Ind->

M-CREATE (...objectClass=bMGroomedCircuit,

a-TPInstance=endpoint1, z-TPInstance=endpoint2,...) request

<-Cnf -------------------- Rsp-o

M-CREATE response

<-Ind --------------------- Req-o

M-EVENT-REPORT objectCreation

### 3B.3.2 GroomedCircuit Configuration

The user will have the ability to change the configuration of an instance of bMGroomedCircuit representing a GroomedCircuit by directing an M-SET operation to the instance. The attributes which are capable of being set across the interoperable interface are defined in the tables in Annex C and ANNEX D.

For example the service provider may ensure fibre routing of a GroomedCircuit resource by  issuing an M-SET operation for the trmFibreRouting attribute of the bMGroomedCircuit representing the GroomedCircuit.  This attribute takes the values of don't Care, required or notRequired.

o-Req -------------------- Ind->

M-SET (...administrativeState=Locked...)request

<-Cnf -------------------- Rsp-o

M-SET response

o-Req -------------------- Ind->

M-SET (trmFibreRouting=required) request

<-Cnf -------------------- Rsp-o

M-SET response

o-Req -------------------- Ind->

M-SET (...administrativeState=unlocked...) request

<-Cnf -------------------- Rsp-o

M-SET response

### 3B.3.3 Restore Groomed Circuit to Optimum Routing - Grooming

A manager will be able to reconfigure an instance of bMGroomedCircuit representing a GroomedCircuit to an optimum route following an automatic or planned re-route.  This is done by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmCircuitGroom

<-Cnf -------------------- Rsp-o

M-ACTION response

<-Ind -------------------- Req-o

M-EVENT-REPORT trmCircuitGroomed

### 3B.3.4 Deleting a GroomedCircuit

A manager can request the deletion of an instance of bMGroomedCircuit representing a GroomedCircuit by directing an M-DELETE operation to the instance.  The bMGroomedCircuit must be in the locked state before it can be deleted.

o-Req --------------------	Ind->

M-DELETE request

<-Cnf --------------------	Rsp-o

M-DELETE response

<-Ind --------------------  Rsp-o

M-EVENT-REPORT objectDeletion

### 3B.3.5 GroomedCircuitTerminationPoint Creation

In some circumstances (see Section 2.4.5) a manager can create an instance of bMGroomedCircuitTP representing the GroomedCircuitTerminationPoint across the interoperable interface.  The object created must have a 'supported by' relationship with an existing equipment object.

o-Req -------------------- Ind->

M-CREATE (...objectClass=

bMGroomedCircuitTP...) request

<-Cnf -------------------- Rsp-o

M-CREATE response

<-Ind -------------------- Req-o

M-EVENT-REPORT objectCreation

### 3B.3.6 Resource Creation

All resources which are statically provisioned will generate notifications via objectCreation events when they are provisioned.

<-Ind -------------------- Req-o

M-EVENT-REPORT objectCreation

### 3B.3.7 Resource Configuration

A manager can change the configuration of a resource by directing an M-SET operation to the instance that represents it.  The attributes which are capable of being set across the interoperable interface are defined in ANNEX C and ANNEX D.

For example,  the manager may wish to modify the attributes of an instance of bMGroomeCircuitTP representing a GroomedCircuitTerminationPoint to change voice-related characteristics.

o-Req -------------------- Ind->

M-SET (...trmGender=dce...)request

<-Cnf -------------------- Rsp-o

M-SET response

o-Req -------------------- Ind->

M-SET (trmVoiceMode=) request

<-Cnf -------------------- Rsp-o

M-SET response

### 3B.3.8 Resource Alarm

If a service affecting network event occurs,  and it is directly related to a network resource an Alarm is passed sent to the manager (assuming the Discriminator resource has been configured accordingly - see Section 2.4.1).

<-Ind -------------------- Req-o

### 3B.3.9 Resource Status Change

If the status of a resource changes it is made visible via a M-EVENT-REPORT stateChange notification.

<-Ind -------------------- Req-o

M-EVENT-REPORT stateChange

### 3B.3.10 Resource Attribute Change

If an attribute of a resource changes (besides status attribute), the change is made visible via a M-EVENT-REPORT attributeValueChange notification.

<-Ind -------------------- Req-o

M-EVENT-REPORT attributeValueChange

### 3B.3.11 Perform Loop Back Tests

Apply Loops / Conduct Test

A manager will be able to apply a loopback on an instance of bMGroomedCircuitTP representing a  GroomedCircuitTerminationPoint .  This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmLocalLoop

<-Cnf -------------------- Rsp-o

M-ACTION response

Removal

A manager will be able to remove a loopback applied on a bMGroomedCircuitTP representing a GroomedCircuitTerminationPoint.  This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmAbort

<-Cnf -------------------- Rsp-o

M-ACTION response

Get Status

A manager will be able to identify whether loop or loop test is currently applied on a bMGroomedCircuitTP representing a GroomedCircuitTerminationPoint.  This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmGetStatus

<-Cnf -------------------- Rsp-o

M-ACTION response

### 3B.3.12 Tracing a GroomedCircuit

A manager will be able to trace the route taken by a GroomedCircuit through the network.  This is achieved by invoking an action which is performed by the agent.

o-Req -------------------- Ind->

M-ACTION request trmCircuitTrace

<-Cnf -------------------- Rsp-o

M-ACTION response

### 3B.3.13 Retrieving Detailed Network Topology Information

The managing system will be able to request the correlation between the static bandwidth resources in the network and bandwidth services.  To do this the managing system issues an M-GET operation on an instance of bMFacility which represents a Facility resource.  The clientTrailList attribute of will contain all instances of bMGroomedCircuit which traverse that instance of bMFacility.

o-Req -------------------- Ind->

M-GET (objectClass=bMFacility) request

<-Cnf -------------------- Rsp-o

M-GET response

## 3B.4 MANAGEMENT INFORMATION REFERENCES

This section references all the definitions of management information relevant to the Ensemble. The definitions will be provided entirely by references to other documents which contain GDMO specifications.	

This section contains only references to definitions that are relevant to the Ensemble. Thus, this section also contains statements about any additional restrictions or constraints to those definitions.

Object classes from the following information models, and classes compatible with these, are included in this Ensemble:

Managed Object List:

Recommendation M.3100 Generic Network Information Model:

"Rec. X.721 | ISO/IEC 10165-2 : 1992":eventForwardingDiscriminator

"Rec. M.3100 : 1995":network

"Rec. M.3100 : 1995":managedElement

"Rec. M.3100 : 1995":equipment

"Rec. M.3100 : 1995":equipmentHolder

"Rec. M.3100 : 1995":circuitPack

Defined by this Ensemble

"NMF038, Issue 1": bMGroomedCircuitTP

"NMF038, Issue 1": bMGroomedCircuit

"NMF038, Issue 1": bMFacilityTP

"NMF038, Issue 1": bMFacility

Note: Section 4 consists of two parts;  section 4A specifies the OP0 			Conformance Requirements while section 4B specifies the OP2

Conformance Requirements.  Please consult section 1.2 for more

information.

# 4A. ENSEMBLE CONFORMANCE REQUIREMENTS: OMNIPoint 0

## 4A.1 GENERAL CONFORMANCE REQUIREMENTS

The OMNIPoint 2 specification of this Ensemble supersedes the OMNIPoint 0 specification.  In general, companies beginning a new development with a vendor that does not  already use the OMNIPoint 0 specification should use the OMNIPoint 2 specification.  If a procuring company or a vendor have already used all or part of the OMNIPoint 0 specification, then the OMNIPoint 0 specification  can be considered to minimize the cost, or the delivery time.

The OMNIPoint 0 set of specifications were released in 1991.  Network management standards and technologies have evolved considerably since then.  Many aspects of OMNIPoint 0 have been superseded.  The conformance requirements published along with OMNIPoint 0 would not be practical to apply to any new development;  however the object modeling techniques and actual object definitions are portable and can be easily implemented using more recent OMNIPoint 1 or OMNIPoint 2 technology.  Therefore in order for procurers and developers to pragmatically use the OMNIPoint 0 specification,   general conformance requires only full use of the OMNIPoint 0 object model.

## 4A.2 SPECIFIC CONFORMANCE REQUIREMENTS

Specific conformance is left up to local agreements between involved parties.

# 4B. ENSEMBLE CONFORMANCE REQUIREMENTS:  OMNIPoint 2

## 4B.1 GENERAL CONFORMANCE REQUIREMENTS

The General Conformance Requirements for OMNIPoint Solution Sets are identified as part of the “TMN Basic Management Platform Component Set” (Ref. 12), and are spelled out as part of the Product Descriptor that accompanies that Component Set. This platform supports the key functions and protocols required for most configuration and alarm oriented Solution Sets, plus other capabilities required to ensure interoperability between cooperating management systems.

The TMN Basic Management Platform references a core set of existing AOM2xx OSI Management Function ISP’s and other capabilities which can be specifically required to support the needs of particular Ensembles (Solution Sets).

## 4B.2 SPECIFIC CONFORMANCE REQUIREMENTS

This specification defers to ISPs,  AOM221[14] for general event report management capabilities,  AOM212[13] for alarm reporting and state management capabilities and AOM231[15] for general log control capabilities.  Specific conformance requirements for the OMNIPoint 2 specification are provided in ANNEX B (MCS Proforma) and ANNEX C (MOCS Proforma) of this document.

# ANNEX A - TERMS AND ACRONYMS

A.1			TERMS:

Bandwidth pipes	A static Bandwidth network resource used to make up the physical transmission path of specified bandwidth through the network.

A.2	ACRONYMS:

AS	Alarm Surveillance

ATM	Asynchronous Transfer Mode

BME	Bandwidth Management Ensemble

CM	Configuration Management

CNF	Confirmation

CTP	Connection Termination Point

CTPBD	Connection Termination Point BiDirectional

GDMO	Guidelines for the  Definition of Managed Object

H/W	Hardware

IETF	Internet Engineering Task Force

IND	Indication

ISO	International Standards Organization

ISP	International Standardized Profiles

ITU-T	International Telecommunications Union - Telecommunications Sector

MCS	Management Conformance Summary

MOCS	Managed Object Conformance Statements

N/A	Not Applicable

NM	Network Management

OP0	OMNIPoint 0

OP1	OMNIPoint 1

OP2	OMNIPoint 2

OSI	Open Systems Interconnection

Q3	Interface applied at the q3 reference point in the TMN Architecture - see

Recommendation M.3010[16]

RCS	Reconfigurable Circuit Service

REQ	Request

RSP	Response

SM	Service Management

TBD	To Be Defined

TMN	Telecommunications Management Network

TP	Termination Point

TTP	Trail  Termination Point

TTPBD	Trail  Termination Point BiDirectional

NMF	Network Management Forum

# ANNEX B - MANAGEMENT CONFORMANCE SUMMARY (MCS) PROFORMA

## B.1 INTRODUCTION

### B.1.1 Purpose and structure

The management conformance summary (MCS) is a statement by a supplier that identifies an implementation and provides information on whether the implementation claims conformance to any of the listed set of documents that specify conformance requirements to OSI management.

The MCS proforma is a document in the form of a questionnaire that when completed by the supplier of an implementation becomes the MCS.

### B.1.2 Instructions for completing the MCS proforma to produce an MCS

The supplier of the implementation shall enter an explicit statement in each of the boxes provided.  Specific instruction is provided in the text which precedes each table.

## B.2 IDENTIFICATION OF THE IMPLEMENTATION

### B.2.1 Date of statement

The supplier of the implementation shall enter the date of this statement in the box below.  Use the format DD-MM-YYYY.

### B.2.2 Identification of the implementation

The supplier of the implementation shall enter information necessary to uniquely identify the implementation and the system(s) in which it may reside, in the box below.

### B.2.3 Contact

The supplier of the implementation shall provide information on whom to contact if there are any queries concerning the content of the MCS, in the box below.

## B.3 IDENTIFICATION OF THE DOCUMENT IN WHICH THE MANAGEMENT INFORMATION IS DEFINED

The supplier of the implementation shall enter the title, reference number and date of the publication of the document which specifies the management information to which conformance is claimed in the box below.

### B.3.1 Technical corrigenda implemented

The supplier of the implementation shall enter the reference numbers of implemented technical corrigenda which modify the specification in the identified document, in the box below.

### B.3.2 Amendments implemented

The supplier of the implementation shall state the titles and reference numbers of implemented addenda to the identified document, in the box below.

## B.4 MANAGEMENT CONFORMANCE SUMMARY

The supplier of the implementation shall provide information on whether the implementation claims conformance to any of the set of documents globally representing the implementation under claim.  For each document the supplier of the implementation claims conformance to, the corresponding conformance statement(s) shall be filled in, or referenced by, the MCS.  Columns 7 (Support), 8 (Table numbers of PICS/MOCS/MRCS ) and 9 (Additional information) are to be filled in by the supplier of the implementation.

The following common notations, defined in CCITT Rec. X.291 | ISO/IEC 9646-2 and ITU-T Rec. X.296 | ISO/IEC 9646-7 are used for the status value column:

m	mandatory;

o	optional;

c	conditional;

x	prohibited;

-	not applicable or out of scope.

NOTES

1	The notations "c", "m", "o", and  "x" are prefixed by a "c:" when nested under a 	conditional or optional item of the same table.

2	The notation "o" may be suffixed by ".n" (where "n" is a unique number) for 	mutually exclusive or selectable options among a set of status values.  The 	requirement for 	this numbered set shall be explicitly stated, preferably in a 	footnote to the relevant 	table.

The following common notations, defined in CCITT Rec. X.291 | ISO/IEC 9646-2 and ITU-T Rec. X.296 ISO/IEC 9646-7 are used for the support answer column:

Y	implemented;

N	not implemented;

-	no answer required;

Ig	the item is ignored (i.e. processed syntactically but not semantically).

In the status column, the static requirements are stated.  The status will be as follows:

m	for characteristics contained in mandatory packages or in conditional packages if 	the GDMO condition is always true;

o	for characteristics of conditional packages with GDMO conditions that indicate 	static optionality, e.g. "if an instance supports it";

cn	for all other conditions, where "n" is a unique integer and  "cn"  is a reference to a 	conditional status expression.

x	for characteristics explicitly prohibited in the definition;

-	for characteristics that are not mentioned in the definition.

Table B.1 MOCS support summary

| Index | Identification of the document that includes the MOCS proforma | Table numbers of MOCS proforma | Description | Constraints and values | Status | Profile | Support | Table numbers of MOCS | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "NMF038, Issue 1" | C.1.1 - C.1.9 | bMFacility |  |  |  |  | C.1.1 - C.1.9 |  |
| 2 | "NMF038, Issue 1" | C.2.1 - C.2.9 | bMFacilityTP |  |  |  |  | C.2.1 - C.2.9 |  |
| 3 | "NMF038, Issue 1" | C.3.1 - C.3.9 | bMGroomedCircuit |  |  |  |  | C.3.1 - C.3.9 |  |
| 4 | "NMF038, Issue 1" | C.4.1 - C.4.9 | bMGroomedCircuitTP |  |  |  |  | C.4.1 - C.4.9 |  |

Table B.2 MRCS support summary

| Index | Identification of the document that includes the MRCS proforma | Table numbers of MRCS proforma | Description | Constraints and values | Status | Profile | Support | Table numbers of MRCS | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "NMF038, Issue 1" | C.1.10 | bMFacility-network |  |  |  |  | C.1.10 |  |
| 2 | "NMF038, Issue 1" | C.2.10 | bMFacilityTP-bMGroomedCircuitTP |  |  |  |  | C.2.10 |  |
| 3 | "NMF038, Issue 1" | C.3.10 | bMGroomedCircuit-network |  |  |  |  | C.3.10 |  |
| 4 | "NMF038, Issue 1" | C.4.10 | bMGroomedCircuitTP-managedElement |  |  |  |  | C.4.10 |  |
| 5 | "Recommendation M.3100" | C.4.10 | "Recommendation M.3100": trailTerminationPointSource-managedElement |  |  |  |  | C.4.10 |  |
| 6 | "Recommendation M.3100" | C.4.10 | "Recommendation M.3100": trailTerminationPointSink-managedElement |  |  |  |  | C.4.10 |  |

# ANNEX C - MANAGED OBJECT CONFORMANCE STATEMENTS (MOCS) PROFORMAS

These tables are formatted according to ITU-TS Rec. X.724 | ISO/IEC 10165-6 ("Requirements and Guidelines for Implementation Conformance Statement Proformas Associated with Management Information").

The Managed Object Conformance Statement (MOCS) proformas that follow should be used by an implementation to identify which features and properties of each managed object class are supported.  These tables have been prepared without regard to manager or agent role, so they can be used in either situation.  An implementation that supports both roles could either use one set of tables if all support details are the same, or a different set of tables for each role.

These tables were generated mechanically from the GDMO templates, with additions and clarifications added by hand.

These tables have columns for the "Base Status" and the "Profile Status", where the entries are identical, allowing the tables to be used directly.  Other profiles, such as an Ensemble, may modify these tables by replacing the "Profile Status" entries, using the normal rules for defining a profile.

Tables are numbered consistently for all classes, that is Table C.x.1 for Managed object class support, C.x.3 for Package support, etc.  To preserve this numbering scheme, table numbers are allocated, even if a particular class does not have any of the corresponding properties.

The following common notations, defined in CCITT Rec. X.291 | ISO/IEC 9646-2 and ITU-T Rec. X.296 | ISO/IEC 9646-7 are used for the status value column:

m	mandatory;

o	optional;

c	conditional;

x	prohibited;

-	not applicable or out of scope.

NOTES

1	The notations "c", "m", "o", and  "x" are prefixed by a "c:" when nested under a 	conditional or optional item of the same table.

2	The notation "o" may be suffixed by ".n" (where "n" is a unique number) for 	mutually exclusive or selectable options among a set of status values.  The 	requirement for 	this numbered set shall be explicitly stated, preferably in a 	footnote to the relevant 	table.

The following common notations, defined in CCITT Rec. X.291 | ISO/IEC 9646-2 and ITU-T Rec. X.296 ISO/IEC 9646-7 are used for the support answer column:

Y	implemented;

N	not implemented;

-	no answer required;

Ig	the item is ignored (i.e. processed syntactically but not semantically).

In the status column, the static requirements are stated.  The status will be as follows:

m	for characteristics contained in mandatory packages or in conditional packages if 	the GDMO condition is always true;

o	for characteristics of conditional packages with GDMO conditions that indicate 	static optionality, e.g. "if an instance supports it";

cn	for all other conditions, where "n" is a unique integer and  "cn"  is a reference to a 	conditional status expression.

x	for characteristics explicitly prohibited in the definition;

-	for characteristics that are not mentioned in the definition.

## C.1  bMFacility

Table C.1.1  bMFacility Managed object class support

| Index | Managed object class template label | Value of object identifier for class | Support of all mandatory features? (Y/N) | Is the actual class the same as the managed object class to which conformance is claimed? (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | bMFacility | {1 2 124 360501 3 78} | Y | Y |

Table C.1.2  bMFacility Actual class support

| Index | Managed object class template for actual class | Value of object identifier for managed object class definition of actual class | Additional information |
| --- | --- | --- | --- |
| N/A | N/A | N/A | N/A |

Table C.1.3  bMFacility Package support

| Index | Package template label | Value of object identifier for package | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "Recommendation M.3100": alarmSeverityAssignmentPointerPackage | {0 0 13 3100 0 4 3} | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" | c1 | c1 |  |  |
| 2 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphicPackage | {2 9 3 2 4 17} | "if an object supports allomorphism" | o | o |  |  |
| 3 | "Recommendation M.3100": attributeValueChangeNotificationPackage | {0 0 13 3100 0 4 4} | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c2 | c2 |  |  |
| 4 | bMFacilityPackage |  | Mandatory | m | m |  |  |
| 5 | "Recommendation M.3100": characteristicInformationPackage | {0 0 13 3100 0 4 7} | "an instance supports it." | o | o |  |  |
| 6 | "Recommendation M.3100": clientTrailPackage | {0 0 13 3100 0 4 9} | "an instance supports it" | o | o |  |  |
| 7 | "Recommendation M.3100": connectionPackage |  | Mandatory | m | m |  |  |
| 8 | "Recommendation M.3100": createDeleteNotificationsPackage | {0 0 13 3100 0 4 10} | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" | c3 | c3 |  |  |
| 9 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packagesPackage | {2 9 3 2 4 16} | "any registered package, other than this package has been instantiated" | c4 | c4 |  |  |
| 10 | "Recommendation M.3100": pipePackage |  | Mandatory | m | m |  |  |
| 11 | "Recommendation M.3100": protectedPackage | {0 0 13 3100 0 4 22} | "an instance supports it." | o | o |  |  |
| 12 | "Recommendation M.3100": serverTrailListPackage | {0 0 13 3100 0 4 25} | "an instance supports it" | o | o |  |  |
| 13 | "Recommendation M.3100": stateChangeNotificationPackage | {0 0 13 3100 0 4 28} | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c5 | c5 |  |  |
| 14 | "Recommendation M.3100": tmnCommunicationsAlarmInformationPackage | {0 0 13 3100 0 4 30} | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" | c6 | c6 |  |  |
| 15 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": topPackage |  | Mandatory | m | m |  |  |
| 16 | "Recommendation M.3100": userLabelPackage | {0 0 13 3100 0 4 32} | "an instance supports it" | o | o |  |  |

Table C.1.4  bMFacility Attribute support

|  |  |  |  |  | Set by create |  |  | Get |  |  |  | Replace |  |  |  | Add |  |  |  | Remove |  |  |  | Set to default |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Attribute template label | Value of object identifier for attribute | Constraints and values |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Additional information |
| 1 | "Recommendation M.3100": a-TPInstance | {0 0 13 3100 0 7 1} | CHOICE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 2 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": administrativeState | {2 9 3 2 7 31} | ENUMERATED |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 3 | "Recommendation M.3100": alarmSeverityAssignmentProfilePointer | {0 0 13 3100 0 7 5} | CHOICE |  | c1 | c1 |  |  | c1 | c1 |  |  | c1 | c1 |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 4 | "Recommendation M.3100": alarmStatus | {0 0 13 3100 0 7 6} | ENUMERATED |  | - | - |  |  | c6 | c6 |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 5 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphs | {2 9 3 2 7 50} | SET OF CHOICE |  | - | - |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 6 | "Recommendation M.3100": characteristicInformation | {0 0 13 3100 0 7 8} | OBJECT IDENTIFIER |  | o | o |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 7 | "OP1 Library Vol. 4": circuitBandwidth | {1 2 124 360501 1 10} | CHOICE |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 8 | "Recommendation M.3100": clientTrail | {0 0 13 3100 0 7 10} | CHOICE |  | o | o |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 9 | "Recommendation M.3100": connectionId | {0 0 13 3100 0 7 12} | CHOICE |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 10 | "Recommendation M.3100": currentProblemList | {0 0 13 3100 0 7 17} | SET OF SEQUENCE |  | - | - |  |  | c6 | c6 |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 11 | "Recommendation M.3100": directionality | {0 0 13 3100 0 7 18} | ENUMERATED |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 12 | "OP1 Library Vol. 4": facilityType | {1 2 124 360501 1 45} | SEQUENCE |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 13 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": nameBinding | {2 9 3 2 7 63} | OBJECT IDENTIFIER |  | o | o |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 14 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectClass | {2 9 3 2 7 65} | CHOICE |  | - | - |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 15 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": operationalState | {2 9 3 2 7 35} | ENUMERATED |  | - | - |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 16 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packages | {2 9 3 2 7 66} | SET OF OBJECT IDENTIFIER |  | o | o |  |  | c4 | c4 |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 17 | "Recommendation M.3100": protected | {0 0 13 3100 0 7 32} | BOOLEAN |  | o | o |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 18 | "Recommendation M.3100": serverTrailList | {0 0 13 3100 0 7 36} | SET OF CHOICE |  | o | o |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 19 | "OP1 Library Vol. 4": trmCircuitProfile | {1 2 124 360501 1 312} | SET OF CHOICE |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 20 | "OP1 Library Vol. 4": trmFunctionalInterface | {1 2 124 360501 1 317} | CHOICE |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 21 | "OP1 Library Vol. 4": trmPhysicalInterface | {1 2 124 360501 1 319} | INTEGER |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 22 | "OP1 Library Vol. 1": typeText | {1 3 14 2 2 4 59} | GraphicString |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 23 | "Recommendation M.3100": userLabel | {0 0 13 3100 0 7 50} | GraphicString |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 24 | "NMF038, Issue 1": userIdentifier | {1 2 124 360501 1 327} | GraphicString |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 25 | "Recommendation M.3100": z-TPInstance | {0 0 13 3100 0 7 55} | CHOICE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |

Table C.1.5  bMFacility Attribute group support

There are no attribute groups specified for this managed object class.

Table C.1.6  bMFacility Action support

There are no actions specified for this managed object class.

Table C1.7  bMFacility Notification support

|  |  |  |  |  |  | Support |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Notification type template label | Value of object identifier for notification type | Constraints and values | Status | Profile | Confirmed | Nonconfirmed | Additional information | Subindex | Notification field name label | Value of object identifier of attribute type associated with field | Constraints and values | Status | Profile | Support | Additional information |
| 1 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": attributeValueChange | {2 9 3 2 10 1} |  | c2 | c2 |  |  |  | 1.1 | AttributeValueChangeInfo |  | Information Syntax SEQUENCE | c2 | c2 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.2 | attributeIdentifierList | {2 9 3 2 7 8} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.2.1 | globalForm | - | OBJECT IDENTIFIER | c:o.1 | c:o.1 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.2.2 | localForm | - | INTEGER | c:o.1 | c:o.1 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3 | attributeValueChangeDefinition | {2 9 3 2 7 10} | SET OF SEQUENCE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.1 | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.2 | c:o.2 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.1.2 | localForm | - | INTEGER | c:o.2 | c:o.2 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.2 | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.3 | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.4 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.2 | nonSpecificForm | - | OCTET STRING | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.6 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 2 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": communicationsAlarm | {2 9 3 2 10 2} |  | c6 | c6 |  |  | "Recommendation Q.821:1992": correlatedRecordNameParameter "Recommendation Q.821:1992": logRecordIdParameter "Recommendation Q.821:1992": suspectObjectListParameter | 2.1 | AlarmInfo |  | Information Syntax SEQUENCE | c6 | c6 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.1 | probableCause | {2 9 3 2 7 18} | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.1.1 | globalValue | - | OBJECT IDENTIFIER | c:o.4 | c:o.4 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.1.2 | localValue | - | INTEGER | c:o.4 | c:o.4 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.2 | specificProblems | {2 9 3 2 7 27} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.2.1 | OBJECT IDENTIFIER | - | OBJECT IDENTIFIER | c:o.5 | c:o.5 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.2.2 | INTEGER | - | INTEGER | c:o.5 | c:o.5 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.3 | perceivedSeverity | {2 9 3 2 7 17} | ENUMERATED | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.4 | backedUpStatus | {2 9 3 2 7 11} | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5 | backUpObject | {2 9 3 2 7 40} | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.2 | nonSpecificForm | - | OCTET STRING | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.6 | trendIndication | {2 9 3 2 7 30} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7 | thresholdInfo | {2 9 3 2 7 29} | SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.1 | triggeredThreshold | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.7 | c:o.7 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.1.2 | localForm | - | INTEGER | c:o.7 | c:o.7 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.2 | observedValue | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.2.1 | integer | - | INTEGER | c:o.8 | c:o.8 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.2.2 | real | - | REAL | c:o.8 | c:o.8 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3 | thresholdLevel | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1 | up | - | SEQUENCE | c:o.9 | c:o.9 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1 | high | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.1 | integer | - | INTEGER | c:o.10 | c:o.10 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.2 | real | - | REAL | c:o.10 | c:o.10 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2 | low | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.1 | integer | - | INTEGER | c:o.11 | c:o.11 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.2 | real | - | REAL | c:o.11 | c:o.11 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2 | down | - | SEQUENCE | c:o.9 | c:o.9 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1 | high | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.1 | integer | - | INTEGER | c:o.12 | c:o.12 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.2 | real | - | REAL | c:o.12 | c:o.12 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2 | low | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.1 | integer | - | INTEGER | c:o.13 | c:o.13 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.2 | real | - | REAL | c:o.13 | c:o.13 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.4 | armTime | - | GeneralizedTime | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.8 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.2 | nonSpecificForm | - | OCTET STRING | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10 | stateChangeDefinition | {2 9 3 2 7 28} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.1 | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.15 | c:o.15 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.1.2 | localForm | - | INTEGER | c:o.15 | c:o.15 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.2 | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.3 | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11 | monitoredAttributes | {2 9 3 2 7 15} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.1 | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.16 | c:o.16 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.1.2 | localForm | - | INTEGER | c:o.16 | c:o.16 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.2 | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.12 | proposedRepairActions | {2 9 3 2 7 19} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.12.1 | OBJECT IDENTIFIER | - | OBJECT IDENTIFIER | c:o.17 | c:o.17 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.12.2 | INTEGER | - | INTEGER | c:o.17 | c:o.17 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.13 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 3 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectCreation | {2 9 3 2 10 6} |  | c3 | c3 |  |  |  | 3.1 | ObjectInfo |  | Information Syntax SEQUENCE | c3 | c3 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2 | attributeList | {2 9 3 2 7 9} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.1 | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.18 | c:o.18 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.1.2 | localForm | - | INTEGER | c:o.18 | c:o.18 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.2 | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.3 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.2 | nonSpecificForm | - | OCTET STRING | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.5 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 4 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectDeletion | {2 9 3 2 10 7} |  | c3 | c3 |  |  |  | 4.1 | ObjectInfo |  | Information Syntax SEQUENCE | c3 | c3 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2 | attributeList | {2 9 3 2 7 9} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.1 | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.20 | c:o.20 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.1.2 | localForm | - | INTEGER | c:o.20 | c:o.20 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.2 | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.3 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.2 | nonSpecificForm | - | OCTET STRING | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.5 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 5 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": stateChange | {2 9 3 2 10 14} |  | c5 | c5 |  |  |  | 5.1 | StateChangeInfo |  | Information Syntax SEQUENCE | c5 | c5 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.2 | attributeIdentifierList | {2 9 3 2 7 8} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.2.1 | globalForm | - | OBJECT IDENTIFIER | c:o.22 | c:o.22 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.2.2 | localForm | - | INTEGER | c:o.22 | c:o.22 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3 | stateChangeDefinition | {2 9 3 2 7 28} | SET OF SEQUENCE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.1 | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.23 | c:o.23 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.1.2 | localForm | - | INTEGER | c:o.23 | c:o.23 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.2 | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.3 | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.4 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.2 | nonSpecificForm | - | OCTET STRING | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.6 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |

Table C.1.8  bMFacility Parameter support

| Index | Parameter template label | Value of object identifier for parameter | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "Recommendation Q.821:1992": correlatedRecordNameParameter | {0 0 17 821 0 5 2} | EVENT-INFO communicationsAlarm | c6 | c6 |  |  |
| 2 | "Recommendation Q.821:1992": logRecordIdParameter | {0 0 17 821 0 5 1} | EVENT-INFO communicationsAlarm | c6 | c6 |  |  |
| 3 | "Recommendation Q.821:1992": suspectObjectListParameter | {0 0 17 821 0 5 3} | EVENT-INFO communicationsAlarm | c6 | c6 |  |  |

Table C1.9  bMFacility Conditions

| Condition number | Condition | Reference |
| --- | --- | --- |
| c1 | IF 1.1.3/1 THEN m ELSE - | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" |
| c2 | IF 1.1.3/3 THEN m ELSE - | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c3 | IF 1.1.3/8 THEN m ELSE - | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" |
| c4 | IF 1.1.3/9 THEN m ELSE - | "any registered package, other than this package has been instantiated" |
| c5 | IF 1.1.3/13 THEN m ELSE - | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c6 | IF 1.1.3/14 THEN m ELSE - | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" |

Table C.1.10  bMFacility Name Binding support

| Index | Name binding template label | Value of object identifier for name binding | Constraints and values | Status | Profile | Support | Additional information | Subindex | Operation | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | bMFacility-network | {1 2 124 360501 4 171} | Superior class: "Recommendation M.3100": network | o | o |  |  | 1.1 | Create support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.2 | Create with reference object |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.3 | Create with automatic instance naming |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.4 | Delete support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.5 | Delete only if no contained objects |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.6 | Delete contained objects |  | - | - |  |  |

## C.2  bMFacilityTP

Table C.2.1  bMFacilityTP Managed object class support

| Index | Managed object class template label | Value of object identifier for class | Support of all mandatory features? (Y/N) | Is the actual class the same as the managed object class to which conformance is claimed? (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | bMFacilityTP | {1 2 124 360501 3 76} | Y | Y |

Table C.2.2  bMFacilityTP Actual class support

| Index | Managed object class template for actual class | Value of object identifier for managed object class definition of actual class | Additional information |
| --- | --- | --- | --- |
| N/A | N/A | N/A | N/A |

Table C.2.3  bMFacilityTP Package support

| Index | Package template label | Value of object identifier for package | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "Recommendation M.3100": alarmSeverityAssignmentPointerPackage | {0 0 13 3100 0 4 3} | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" | c1 | c1 |  |  |
| 2 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphicPackage | {2 9 3 2 4 17} | "if an object supports allomorphism" | o | o |  |  |
| 3 | "Recommendation M.3100": attributeValueChangeNotificationPackage | {0 0 13 3100 0 4 4} | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c2 | c2 |  |  |
| 4 | bMFacilityTPPackage |  | Mandatory | m | m |  |  |
| 5 | "Recommendation M.3100": channelNumberPackage | {0 0 13 3100 0 4 6} | "an instance supports it" | o | o |  |  |
| 6 | "Recommendation M.3100": characteristicInformationPackage | {0 0 13 3100 0 4 7} | "an instance supports it." | o | o |  |  |
| 7 | "Recommendation M.3100": connectionTerminationPointSinkPackage |  | Mandatory | m | m |  |  |
| 8 | "Recommendation M.3100": connectionTerminationPointSourcePackage |  | Mandatory | m | m |  |  |
| 9 | "Recommendation M.3100": createDeleteNotificationsPackage | {0 0 13 3100 0 4 10} | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" | c3 | c3 |  |  |
| 10 | "Recommendation M.3100": crossConnectionPointerPackage | {0 0 13 3100 0 4 11} | "the termination point can be flexibly assigned, (i.e. cross connected)." | c4 | c4 |  |  |
| 11 | "Recommendation M.3100": ctpInstancePackage | {0 0 13 3100 0 4 12} | "the name binding used to create an instance of this object class requires this attribute." | c5 | c5 |  |  |
| 12 | "Recommendation M.3100": networkLevelPackage | {0 0 13 3100 0 4 18} | "an instance supports it" | o | o |  |  |
| 13 | "Recommendation M.3100": operationalStatePackage | {0 0 13 3100 0 4 19} | "the resource represented by this managed object is capable of assessing the ability to generate and/or receive a valid signal." | c6 | c6 |  |  |
| 14 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packagesPackage | {2 9 3 2 4 16} | "any registered package, other than this package has been instantiated" | c7 | c7 |  |  |
| 15 | "Recommendation M.3100": stateChangeNotificationPackage | {0 0 13 3100 0 4 28} | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c8 | c8 |  |  |
| 16 | "Recommendation M.3100": terminationPointPackage |  | Mandatory | m | m |  |  |
| 17 | "Recommendation M.3100": tmnCommunicationsAlarmInformationPackage | {0 0 13 3100 0 4 30} | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" | c9 | c9 |  |  |
| 18 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": topPackage |  | Mandatory | m | m |  |  |

Table C.2.4  bMFacilityTP Attribute support

|  |  |  |  |  | Set by create |  |  |  | Get |  |  |  | Replace |  |  |  | Add |  |  |  | Remove |  |  |  | Set to default |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Attribute template label | Value of object identifier for attribute | Constraints and values | Status |  | Profile | Support | Status |  | Profile | Support | Status |  | Profile | Support | Status |  | Profile | Support | Status |  | Profile | Support | Status |  | Profile | Support | Additional information |  |
| 1 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": administrativeState | {2 9 3 2 7 31} | ENUMERATED | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 2 | "Recommendation M.3100": alarmSeverityAssignmentProfilePointer | {0 0 13 3100 0 7 5} | CHOICE | c1 |  | c1 |  | c1 |  | c1 |  | c1 |  | c1 |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 3 | "Recommendation M.3100": alarmStatus | {0 0 13 3100 0 7 6} | ENUMERATED | - |  | - |  | c9 |  | c9 |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 4 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphs | {2 9 3 2 7 50} | SET OF CHOICE | - |  | - |  | o |  | o |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 5 | "Recommendation M.3100": cTPId | {0 0 13 3100 0 7 13} | CHOICE | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 6 | "Recommendation M.3100": channelNumber | {0 0 13 3100 0 7 7} | INTEGER | o |  | o |  | o |  | o |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 7 | "Recommendation M.3100": characteristicInformation | {0 0 13 3100 0 7 8} | OBJECT IDENTIFIER | o |  | o |  | o |  | o |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 8 | "Recommendation M.3100": crossConnectionObjectPointer | {0 0 13 3100 0 7 16} | CHOICE | c4 |  | c4 |  | c4 |  | c4 |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 9 | "Recommendation M.3100": currentProblemList | {0 0 13 3100 0 7 17} | SET OF SEQUENCE | - |  | - |  | c9 |  | c9 |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 10 | "Recommendation M.3100": downstreamConnectivityPointer | {0 0 13 3100 0 7 19} | CHOICE | m |  | m |  | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 11 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": nameBinding | {2 9 3 2 7 63} | OBJECT IDENTIFIER | o |  | o |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 12 | "Recommendation M.3100": networkLevelPointer | {0 0 13 3100 0 7 31} | CHOICE | o |  | o |  | o |  | o |  | o |  | o |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 13 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectClass | {2 9 3 2 7 65} | CHOICE | - |  | - |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 14 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": operationalState | {2 9 3 2 7 35} | ENUMERATED | - |  | - |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 15 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packages | {2 9 3 2 7 66} | SET OF OBJECT IDENTIFIER | o |  | o |  | c7 |  | c7 |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 16 | "Recommendation M.3100": supportedByObjectList | {0 0 13 3100 0 7 40} | SET OF CHOICE | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 17 | "OP1 Library Vol. 4": trmFunctionalInterface | {1 2 124 360501 1 317} | CHOICE | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 18 | "OP1 Library Vol. 4": trmPhysicalInterface | {1 2 124 360501 1 319} | INTEGER | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 19 | "OP1 Library Vol. 1": typeText | {1 3 14 2 2 4 59} | GraphicString | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 20 | "Recommendation M.3100": upstreamConnectivityPointer | {0 0 13 3100 0 7 49} | CHOICE | m |  | m |  | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 21 | "NMF038, Issue 1": userIdentifier | {1 2 124 360501 1 327} | GraphicString | m |  | m |  | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |
| 22 | "Recommendation M.3100": userLabel | {0 0 13 3100 0 7 50} | GraphicString | m |  | m |  | m |  | m |  | m |  | m |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  |

Table C.2.5  bMFacilityTP Attribute group support

There are no attribute groups specified for this managed object class.

Table C.2.6  bMFacilityTP Action support

There are no actions specified for this managed object class.

Table C.2.7  bMFacilityTP Notification support

|  |  |  |  |  |  |  |  |  | Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Notification type template label | Value of object identifier for notification type |  | Constraints and values |  | Status |  | Profile |  | Confirmed | Nonconfirmed |  | Additional information |  | Subindex |  | Notification field name label | Value of object identifier of attribute type associated with field | Constraints and values | Status | Profile | Support | Additional information |
| 1 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": attributeValueChange | {2 9 3 2 10 1} |  |  |  | c2 |  | c2 |  |  |  |  |  |  | 1.1 |  | AttributeValueChangeInfo |  | Information Syntax SEQUENCE | c2 | c2 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.1 |  | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.2 |  | attributeIdentifierList | {2 9 3 2 7 8} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.2.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.1 | c:o.1 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.2.2 |  | localForm | - | INTEGER | c:o.1 | c:o.1 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3 |  | attributeValueChangeDefinition | {2 9 3 2 7 10} | SET OF SEQUENCE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.1 |  | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.1.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.2 | c:o.2 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.1.2 |  | localForm | - | INTEGER | c:o.2 | c:o.2 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.2 |  | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.3 |  | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.4 |  | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.1 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2 |  | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.1 |  | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.1.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.1.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.2 |  | nonSpecificForm | - | OCTET STRING | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.3 |  | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.3.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.3.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.6 |  | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7 |  | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7.1 |  | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7.2 |  | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7.3 |  | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 2 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": communicationsAlarm | {2 9 3 2 10 2} |  |  |  | c9 |  | c9 |  |  |  |  | "Recommendation Q.821:1992": correlatedRecordNameParameter "Recommendation Q.821:1992": logRecordIdParameter "Recommendation Q.821:1992": suspectObjectListParameter |  | 2.1 |  | AlarmInfo |  | Information Syntax SEQUENCE | c9 | c9 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.1 |  | probableCause | {2 9 3 2 7 18} | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.1.1 |  | globalValue | - | OBJECT IDENTIFIER | c:o.4 | c:o.4 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.1.2 |  | localValue | - | INTEGER | c:o.4 | c:o.4 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.2 |  | specificProblems | {2 9 3 2 7 27} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.2.1 |  | OBJECT IDENTIFIER | - | OBJECT IDENTIFIER | c:o.5 | c:o.5 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.2.2 |  | INTEGER | - | INTEGER | c:o.5 | c:o.5 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.3 |  | perceivedSeverity | {2 9 3 2 7 17} | ENUMERATED | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.4 |  | backedUpStatus | {2 9 3 2 7 11} | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5 |  | backUpObject | {2 9 3 2 7 40} | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.1 |  | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.1.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.1.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.2 |  | nonSpecificForm | - | OCTET STRING | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.3 |  | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.3.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.3.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.6 |  | trendIndication | {2 9 3 2 7 30} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7 |  | thresholdInfo | {2 9 3 2 7 29} | SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.1 |  | triggeredThreshold | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.1.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.7 | c:o.7 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.1.2 |  | localForm | - | INTEGER | c:o.7 | c:o.7 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.2 |  | observedValue | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.2.1 |  | integer | - | INTEGER | c:o.8 | c:o.8 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.2.2 |  | real | - | REAL | c:o.8 | c:o.8 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3 |  | thresholdLevel | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1 |  | up | - | SEQUENCE | c:o.9 | c:o.9 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1 |  | high | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.1 |  | integer | - | INTEGER | c:o.10 | c:o.10 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.2 |  | real | - | REAL | c:o.10 | c:o.10 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2 |  | low | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.1 |  | integer | - | INTEGER | c:o.11 | c:o.11 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.2 |  | real | - | REAL | c:o.11 | c:o.11 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2 |  | down | - | SEQUENCE | c:o.9 | c:o.9 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1 |  | high | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.1 |  | integer | - | INTEGER | c:o.12 | c:o.12 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.2 |  | real | - | REAL | c:o.12 | c:o.12 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2 |  | low | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.1 |  | integer | - | INTEGER | c:o.13 | c:o.13 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.2 |  | real | - | REAL | c:o.13 | c:o.13 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.4 |  | armTime | - | GeneralizedTime | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.8 |  | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.1 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2 |  | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.1 |  | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.1.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.1.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.2 |  | nonSpecificForm | - | OCTET STRING | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.3 |  | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.3.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.3.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10 |  | stateChangeDefinition | {2 9 3 2 7 28} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.1 |  | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.1.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.15 | c:o.15 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.1.2 |  | localForm | - | INTEGER | c:o.15 | c:o.15 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.2 |  | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.3 |  | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11 |  | monitoredAttributes | {2 9 3 2 7 15} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.1 |  | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.1.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.16 | c:o.16 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.1.2 |  | localForm | - | INTEGER | c:o.16 | c:o.16 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.2 |  | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.12 |  | proposedRepairActions | {2 9 3 2 7 19} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.12.1 |  | OBJECT IDENTIFIER | - | OBJECT IDENTIFIER | c:o.17 | c:o.17 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.12.2 |  | INTEGER | - | INTEGER | c:o.17 | c:o.17 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.13 |  | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14 |  | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14.1 |  | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14.2 |  | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14.3 |  | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 3 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectCreation | {2 9 3 2 10 6} |  |  |  | c3 |  | c3 |  |  |  |  |  |  | 3.1 |  | ObjectInfo |  | Information Syntax SEQUENCE | c3 | c3 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.1 |  | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2 |  | attributeList | {2 9 3 2 7 9} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.1 |  | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.1.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.18 | c:o.18 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.1.2 |  | localForm | - | INTEGER | c:o.18 | c:o.18 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.2 |  | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.3 |  | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.1 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2 |  | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.1 |  | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.1.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.1.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.2 |  | nonSpecificForm | - | OCTET STRING | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.3 |  | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.3.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.3.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.5 |  | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6 |  | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6.1 |  | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6.2 |  | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6.3 |  | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 4 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectDeletion | {2 9 3 2 10 7} |  |  |  | c3 |  | c3 |  |  |  |  |  |  | 4.1 |  | ObjectInfo |  | Information Syntax SEQUENCE | c3 | c3 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.1 |  | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2 |  | attributeList | {2 9 3 2 7 9} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.1 |  | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.1.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.20 | c:o.20 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.1.2 |  | localForm | - | INTEGER | c:o.20 | c:o.20 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.2 |  | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.3 |  | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.1 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2 |  | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.1 |  | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.1.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.1.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.2 |  | nonSpecificForm | - | OCTET STRING | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.3 |  | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.3.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.3.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.5 |  | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6 |  | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6.1 |  | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6.2 |  | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6.3 |  | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 5 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": stateChange | {2 9 3 2 10 14} |  |  |  | c8 |  | c8 |  |  |  |  |  |  | 5.1 |  | StateChangeInfo |  | Information Syntax SEQUENCE | c8 | c8 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.1 |  | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.2 |  | attributeIdentifierList | {2 9 3 2 7 8} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.2.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.22 | c:o.22 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.2.2 |  | localForm | - | INTEGER | c:o.22 | c:o.22 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3 |  | stateChangeDefinition | {2 9 3 2 7 28} | SET OF SEQUENCE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.1 |  | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.1.1 |  | globalForm | - | OBJECT IDENTIFIER | c:o.23 | c:o.23 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.1.2 |  | localForm | - | INTEGER | c:o.23 | c:o.23 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.2 |  | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.3 |  | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.4 |  | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.1 |  | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2 |  | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.1 |  | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.1.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.1.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.2 |  | nonSpecificForm | - | OCTET STRING | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.3 |  | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.3.1 |  | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.3.2 |  | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.6 |  | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7 |  | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7.1 |  | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7.2 |  | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7.3 |  | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |

Table C.2.8  bMFacilityTP Parameter support

| Index | Parameter template label | Value of object identifier for parameter | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "Recommendation Q.821:1992": correlatedRecordNameParameter | {0 0 17 821 0 5 2} | EVENT-INFO communicationsAlarm | c9 | c9 |  |  |
| 2 | "Recommendation Q.821:1992": logRecordIdParameter | {0 0 17 821 0 5 1} | EVENT-INFO communicationsAlarm | c9 | c9 |  |  |
| 3 | "Recommendation Q.821:1992": suspectObjectListParameter | {0 0 17 821 0 5 3} | EVENT-INFO communicationsAlarm | c9 | c9 |  |  |

Table C.2.9  bMFacilityTP Conditions

| Condition number | Condition | Reference |
| --- | --- | --- |
| c1 | IF 1.2.3/1 THEN m ELSE - | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" |
| c2 | IF 1.2.3/3 THEN m ELSE - | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c3 | IF 1.2.3/9 THEN m ELSE - | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" |
| c4 | IF 1.2.3/10 THEN m ELSE - | "the termination point can be flexibly assigned, (i.e. cross connected)." |
| c5 | IF 1.2.3/11 THEN m ELSE - | "the name binding used to create an instance of this object class requires this attribute." |
| c6 | IF 1.2.3/13 THEN m ELSE - | "the resource represented by this managed object is capable of assessing the ability to generate and/or receive a valid signal." |
| c7 | IF 1.2.3/14 THEN m ELSE - | "any registered package, other than this package has been instantiated" |
| c8 | IF 1.2.3/15 THEN m ELSE - | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c9 | IF 1.2.3/17 THEN m ELSE - | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" |

Table C.2.10  bMFacilityTP Name Binding support

| Index | Name binding template label | Value of object identifier for name binding | Constraints and values | Status | Profile | Support | Additional information | Subindex | Operation | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | bMFacilityTP-bMGroomedCircuitTP | {1 2 124 360501 4 169} | Superior class: bMGroomedCircuitTP | o | o |  |  | 1.1 | Create support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.2 | Create with reference object |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.3 | Create with automatic instance naming |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.4 | Delete support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.5 | Delete only if no contained objects |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.6 | Delete contained objects |  | - | - |  |  |

## C.3  bMGroomedCircuit

Table C.3.1  bMGroomedCircuit Managed object class support

| Index | Managed object class template label | Value of object identifier for class | Support of all mandatory features? (Y/N) | Is the actual class the same as the managed object class to which conformance is claimed? (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | bMGroomedCircuit | {1 2 124 360501 3 77} | Y | Y |

Table C.3.2  bMGroomedCircuit Actual class support

| Index | Managed object class template for actual class | Value of object identifier for managed object class definition of actual class | Additional information |
| --- | --- | --- | --- |
| N/A | N/A | N/A | N/A |

Table C.3.3  bMGroomedCircuit Package support

| Index | Package template label | Value of object identifier for package | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | aTMProfilePackage | {1 2 124 360501 10 130} | Mandatory | m | m |  |  |
| 2 | "Recommendation M.3100": alarmSeverityAssignmentPointerPackage | {0 0 13 3100 0 4 3} | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" | c1 | c1 |  |  |
| 3 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphicPackage | {2 9 3 2 4 17} | "if an object supports allomorphism" | o | o |  |  |
| 4 | "Recommendation M.3100": attributeValueChangeNotificationPackage | {0 0 13 3100 0 4 4} | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c2 | c2 |  |  |
| 5 | bMGroomedCircuitPackage |  | Mandatory | m | m |  |  |
| 6 | "Recommendation M.3100": characteristicInformationPackage | {0 0 13 3100 0 4 7} | "an instance supports it." | o | o |  |  |
| 7 | "Recommendation M.3100": clientConnectionListPackage | {0 0 13 3100 0 4 35} | "an instance supports it" | o | o |  |  |
| 8 | commonProfilePackage | {1 2 124 360501 10 127} | Mandatory | m | m |  |  |
| 9 | "Recommendation M.3100": createDeleteNotificationsPackage | {0 0 13 3100 0 4 10} | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" | c3 | c3 |  |  |
| 10 | frameRelayPackage | {1 2 124 360501 10 129} | Mandatory | m | m |  |  |
| 11 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packagesPackage | {2 9 3 2 4 16} | "any registered package, other than this package has been instantiated" | c4 | c4 |  |  |
| 12 | "Recommendation M.3100": pipePackage |  | Mandatory | m | m |  |  |
| 13 | "Recommendation M.3100": protectedPackage | {0 0 13 3100 0 4 22} | "an instance supports it." | o | o |  |  |
| 14 | "Recommendation M.3100": serverConnectionListPackage | {0 0 13 3100 0 4 24} | "an instance supports it" | o | o |  |  |
| 15 | "Recommendation M.3100": stateChangeNotificationPackage | {0 0 13 3100 0 4 28} | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c5 | c5 |  |  |
| 16 | tDMProfilePackage | {1 2 124 360501 10 128} | Mandatory | m | m |  |  |
| 17 | "Recommendation M.3100": tmnCommunicationsAlarmInformationPackage | {0 0 13 3100 0 4 30} | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" | c6 | c6 |  |  |
| 18 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": topPackage |  | Mandatory | m | m |  |  |
| 19 | "Recommendation M.3100": trailR1Package |  | Mandatory | m | m |  |  |
| 20 | "Recommendation M.3100": userLabelPackage | {0 0 13 3100 0 4 32} | "an instance supports it" | o | o |  |  |

Table C.3.4  bMGroomedCircuit Attribute support

|  |  |  |  | Set by create |  |  |  | Get |  |  |  | Replace |  |  |  | Add |  |  |  | Remove |  |  |  | Set to default |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Attribute template label | Value of object identifier for attribute | Constraints and values |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Status | Profile | Support |  | Additional information |
| 1 | "Recommendation M.3100": a-TPInstance | {0 0 13 3100 0 7 1} | CHOICE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 2 | aTMProfile | {1 2 124 360501 1 341} | SEQUENCE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 3 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": administrativeState | {2 9 3 2 7 31} | ENUMERATED |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 4 | "Recommendation M.3100": alarmSeverityAssignmentProfilePointer | {0 0 13 3100 0 7 5} | CHOICE |  | c1 | c1 |  |  | c1 | c1 |  |  | c1 | c1 |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 5 | "Recommendation M.3100": alarmStatus | {0 0 13 3100 0 7 6} | ENUMERATED |  | - | - |  |  | c6 | c6 |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 6 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphs | {2 9 3 2 7 50} | SET OF CHOICE |  | - | - |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 7 | "Recommendation M.3100": characteristicInformation | {0 0 13 3100 0 7 8} | OBJECT IDENTIFIER |  | o | o |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 8 | "OP1 Library Vol. 4": circuitType | {1 2 124 360501 1 14} | SEQUENCE |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 9 | "Recommendation M.3100": clientConnectionList | {0 0 13 3100 0 7 53} | SET OF CHOICE |  | o | o |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 10 | commonProfile | {1 2 124 360501 1 328} | SEQUENCE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 11 | "Recommendation M.3100": currentProblemList | {0 0 13 3100 0 7 17} | SET OF SEQUENCE |  | c6 | c6 |  |  | c6 | c6 |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 12 | "Recommendation M.3100": directionality | {0 0 13 3100 0 7 18} | ENUMERATED |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 13 | frameRelayProfile | {1 2 124 360501 1 340} | SEQUENCE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 14 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": nameBinding | {2 9 3 2 7 63} | OBJECT IDENTIFIER |  | o | o |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 15 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectClass | {2 9 3 2 7 65} | CHOICE |  | - | - |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 16 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": operationalState | {2 9 3 2 7 35} | ENUMERATED |  | - | - |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 17 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packages | {2 9 3 2 7 66} | SET OF OBJECT IDENTIFIER |  | o | o |  |  | c4 | c4 |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 18 | "Recommendation M.3100": protected | {0 0 13 3100 0 7 32} | BOOLEAN |  | o | o |  |  | o | o |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 19 | "Recommendation M.3100": serverConnectionList | {0 0 13 3100 0 7 35} | SEQUENCE OF CHOICE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 20 | tDMProfile | {1 2 124 360501 1 329} | SEQUENCE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 21 | "Recommendation M.3100": trailId | {0 0 13 3100 0 7 47} | CHOICE |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 22 | "OP1 Library Vol. 4": trmEncryptionRouting | {1 2 124 360501 1 314} | ENUMERATED |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 23 | "OP1 Library Vol. 4": trmFibreRouting | {1 2 124 360501 1 315} | ENUMERATED |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 24 | "OP1 Library Vol. 4": trmTerrestrialRouting | {1 2 124 360501 1 321} | ENUMERATED |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 25 | "OP1 Library Vol. 1": typeText | {1 3 14 2 2 4 59} | GraphicString |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 26 | "Recommendation M.3100": userLabel | {0 0 13 3100 0 7 50} | GraphicString |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 27 | "NMF038, Issue 1": userIdentifier | {0 0 13 3100 0 7 50} | GraphicString |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |
| 28 | "Recommendation M.3100": z-TPInstance | {0 0 13 3100 0 7 55} | CHOICE |  | m | m |  |  | m | m |  |  | m | m |  |  | - | - |  |  | - | - |  |  | - | - |  |  |  |

Table C.3.5  bMGroomedCircuit Attribute group support

There are no attribute groups specified for this managed object class.

Table C.3.6  bMGroomedCircuit Action support

| Index | Action type template label | Value of object identifier for action type | Constraints and values | Status | Profile | Support | Additional information | Subindex | Action field name label | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "OP1 Library Vol. 4": trmCircuitGrooming | {1 2 124 360501 6 12} |  | m | m |  |  | 1.1 | GroomingTime | Information Syntax CHOICE | m | m |  |  |
|  |  |  |  |  |  |  |  | 1.1.1 | scheduled | GeneralizedTime | o.1 | o.1 |  |  |
|  |  |  |  |  |  |  |  | 1.1.2 | now | NULL | o.1 | o.1 |  |  |
| 2 | "OP1 Library Vol. 4": trmCircuitTrace | {1 2 124 360501 6 13} |  | m | m |  |  | 2.1 | Trace | Reply Syntax SEQUENCE OF CHOICE | m | m |  |  |
|  |  |  |  |  |  |  |  | 2.1.1 | distinguishedName | SEQUENCE OF SET OF SEQUENCE | o.2 | o.2 |  |  |
|  |  |  |  |  |  |  |  | 2.1.1.1 | AttributeType | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.1.1.2 | AttributeValue | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.1.2 | nonSpecificForm | OCTET STRING | o.2 | o.2 |  |  |
|  |  |  |  |  |  |  |  | 2.1.3 | localDistinguishedName | SEQUENCE OF SET OF SEQUENCE | o.2 | o.2 |  |  |
|  |  |  |  |  |  |  |  | 2.1.3.1 | AttributeType | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.1.3.2 | AttributeValue | ANY | c:m | c:m |  |  |
| 3 | "OP1 Library Vol. 4": trmGetStatus | {1 2 124 360501 6 14} |  | m | m |  |  | 3.1 | Status | Reply Syntax INTEGER | m | m |  |  |

Table C.3.7  bMGroomedCircuit Notification support

|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Notification type template label |  | Value of object identifier for notification type |  | Constraints and values |  |  | Status |  | Profile |  | Confirmed |  |  |  | Nonconfirmed |  | Additional information |  |  |  | Subindex |  |  | Notification field name label |  |  | Value of object identifier of attribute type associated with field |  |  | Constraints and values |  |  |  | Status |  |  | Profile |  |  | Support |  | Additional information |  |
| 1 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": attributeValueChange |  | {2 9 3 2 10 1} |  |  |  |  | c2 |  | c2 |  |  |  |  |  |  |  |  |  |  |  | 1.1 |  |  | AttributeValueChangeInfo |  |  |  |  |  | Information Syntax SEQUENCE |  |  |  | c2 |  |  | c2 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.1 |  |  | sourceIndicator |  |  | {2 9 3 2 7 26} |  |  | ENUMERATED |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.2 |  |  | attributeIdentifierList |  |  | {2 9 3 2 7 8} |  |  | SET OF CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.2.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.1 |  |  | c:o.1 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.2.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.1 |  |  | c:o.1 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3 |  |  | attributeValueChangeDefinition |  |  | {2 9 3 2 7 10} |  |  | SET OF SEQUENCE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.1 |  |  | attributeID |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.1.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.2 |  |  | c:o.2 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.1.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.2 |  |  | c:o.2 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.2 |  |  | oldAttributeValue |  |  | - |  |  | ANY DEFINED BY attributeID |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.3.3 |  |  | newAttributeValue |  |  | - |  |  | ANY DEFINED BY attributeID |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.4 |  |  | notificationIdentifier |  |  | {2 9 3 2 7 16} |  |  | INTEGER |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.1 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF INTEGER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2 |  |  | sourceObjectInst |  |  | - |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.1 |  |  | distinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.3 |  |  | c:o.3 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.1.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.1.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.2 |  |  | nonSpecificForm |  |  | - |  |  | OCTET STRING |  |  |  | c:o.3 |  |  | c:o.3 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.3 |  |  | localDistinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.3 |  |  | c:o.3 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.3.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.5.2.3.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.6 |  |  | additionalText |  |  | {2 9 3 2 7 7} |  |  | GraphicString |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7 |  |  | additionalInformation |  |  | {2 9 3 2 7 6} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7.1 |  |  | identifier |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7.2 |  |  | significance |  |  | - |  |  | BOOLEAN |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.1.7.3 |  |  | information |  |  | - |  |  | ANY DEFINED BY identifier |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
| 2 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": communicationsAlarm |  | {2 9 3 2 10 2} |  |  |  |  | c6 |  | c6 |  |  |  |  |  |  |  | "Recommendation Q.821:1992": correlatedRecordNameParameter "Recommendation Q.821:1992": logRecordIdParameter "Recommendation Q.821:1992": suspectObjectListParameter |  |  |  | 2.1 |  |  | AlarmInfo |  |  |  |  |  | Information Syntax SEQUENCE |  |  |  | c6 |  |  | c6 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.1 |  |  | probableCause |  |  | {2 9 3 2 7 18} |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.1.1 |  |  | globalValue |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.4 |  |  | c:o.4 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.1.2 |  |  | localValue |  |  | - |  |  | INTEGER |  |  |  | c:o.4 |  |  | c:o.4 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.2 |  |  | specificProblems |  |  | {2 9 3 2 7 27} |  |  | SET OF CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.2.1 |  |  | OBJECT IDENTIFIER |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.5 |  |  | c:o.5 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.2.2 |  |  | INTEGER |  |  | - |  |  | INTEGER |  |  |  | c:o.5 |  |  | c:o.5 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.3 |  |  | perceivedSeverity |  |  | {2 9 3 2 7 17} |  |  | ENUMERATED |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.4 |  |  | backedUpStatus |  |  | {2 9 3 2 7 11} |  |  | BOOLEAN |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5 |  |  | backUpObject |  |  | {2 9 3 2 7 40} |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.1 |  |  | distinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.6 |  |  | c:o.6 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.1.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.1.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.2 |  |  | nonSpecificForm |  |  | - |  |  | OCTET STRING |  |  |  | c:o.6 |  |  | c:o.6 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.3 |  |  | localDistinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.6 |  |  | c:o.6 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.3.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.5.3.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.6 |  |  | trendIndication |  |  | {2 9 3 2 7 30} |  |  | ENUMERATED |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7 |  |  | thresholdInfo |  |  | {2 9 3 2 7 29} |  |  | SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.1 |  |  | triggeredThreshold |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.1.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.7 |  |  | c:o.7 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.1.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.7 |  |  | c:o.7 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.2 |  |  | observedValue |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.2.1 |  |  | integer |  |  | - |  |  | INTEGER |  |  |  | c:o.8 |  |  | c:o.8 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.2.2 |  |  | real |  |  | - |  |  | REAL |  |  |  | c:o.8 |  |  | c:o.8 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3 |  |  | thresholdLevel |  |  | - |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1 |  |  | up |  |  | - |  |  | SEQUENCE |  |  |  | c:o.9 |  |  | c:o.9 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1 |  |  | high |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.1 |  |  | integer |  |  | - |  |  | INTEGER |  |  |  | c:o.10 |  |  | c:o.10 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.2 |  |  | real |  |  | - |  |  | REAL |  |  |  | c:o.10 |  |  | c:o.10 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2 |  |  | low |  |  | - |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.1 |  |  | integer |  |  | - |  |  | INTEGER |  |  |  | c:o.11 |  |  | c:o.11 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.2 |  |  | real |  |  | - |  |  | REAL |  |  |  | c:o.11 |  |  | c:o.11 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2 |  |  | down |  |  | - |  |  | SEQUENCE |  |  |  | c:o.9 |  |  | c:o.9 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1 |  |  | high |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.1 |  |  | integer |  |  | - |  |  | INTEGER |  |  |  | c:o.12 |  |  | c:o.12 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.2 |  |  | real |  |  | - |  |  | REAL |  |  |  | c:o.12 |  |  | c:o.12 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2 |  |  | low |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.1 |  |  | integer |  |  | - |  |  | INTEGER |  |  |  | c:o.13 |  |  | c:o.13 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.2 |  |  | real |  |  | - |  |  | REAL |  |  |  | c:o.13 |  |  | c:o.13 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.7.4 |  |  | armTime |  |  | - |  |  | GeneralizedTime |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.8 |  |  | notificationIdentifier |  |  | {2 9 3 2 7 16} |  |  | INTEGER |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.1 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF INTEGER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2 |  |  | sourceObjectInst |  |  | - |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.1 |  |  | distinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.14 |  |  | c:o.14 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.1.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.1.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.2 |  |  | nonSpecificForm |  |  | - |  |  | OCTET STRING |  |  |  | c:o.14 |  |  | c:o.14 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.3 |  |  | localDistinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.14 |  |  | c:o.14 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.3.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.9.2.3.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10 |  |  | stateChangeDefinition |  |  | {2 9 3 2 7 28} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.1 |  |  | attributeID |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.1.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.15 |  |  | c:o.15 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.1.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.15 |  |  | c:o.15 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.2 |  |  | oldAttributeValue |  |  | - |  |  | ANY DEFINED BY attributeID |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.10.3 |  |  | newAttributeValue |  |  | - |  |  | ANY DEFINED BY attributeID |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11 |  |  | monitoredAttributes |  |  | {2 9 3 2 7 15} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.1 |  |  | attributeId |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.1.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.16 |  |  | c:o.16 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.1.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.16 |  |  | c:o.16 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.11.2 |  |  | attributeValue |  |  | - |  |  | ANY DEFINED BY attributeId |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.12 |  |  | proposedRepairActions |  |  | {2 9 3 2 7 19} |  |  | SET OF CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.12.1 |  |  | OBJECT IDENTIFIER |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.17 |  |  | c:o.17 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.12.2 |  |  | INTEGER |  |  | - |  |  | INTEGER |  |  |  | c:o.17 |  |  | c:o.17 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.13 |  |  | additionalText |  |  | {2 9 3 2 7 7} |  |  | GraphicString |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14 |  |  | additionalInformation |  |  | {2 9 3 2 7 6} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14.1 |  |  | identifier |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14.2 |  |  | significance |  |  | - |  |  | BOOLEAN |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.1.14.3 |  |  | information |  |  | - |  |  | ANY DEFINED BY identifier |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
| 3 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectCreation |  | {2 9 3 2 10 6} |  |  |  |  | c3 |  | c3 |  |  |  |  |  |  |  |  |  |  |  | 3.1 |  |  | ObjectInfo |  |  |  |  |  | Information Syntax SEQUENCE |  |  |  | c3 |  |  | c3 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.1 |  |  | sourceIndicator |  |  | {2 9 3 2 7 26} |  |  | ENUMERATED |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2 |  |  | attributeList |  |  | {2 9 3 2 7 9} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.1 |  |  | attributeId |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.1.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.18 |  |  | c:o.18 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.1.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.18 |  |  | c:o.18 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.2.2 |  |  | attributeValue |  |  | - |  |  | ANY DEFINED BY attributeId |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.3 |  |  | notificationIdentifier |  |  | {2 9 3 2 7 16} |  |  | INTEGER |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.1 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF INTEGER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2 |  |  | sourceObjectInst |  |  | - |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.1 |  |  | distinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.19 |  |  | c:o.19 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.1.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.1.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.2 |  |  | nonSpecificForm |  |  | - |  |  | OCTET STRING |  |  |  | c:o.19 |  |  | c:o.19 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.3 |  |  | localDistinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.19 |  |  | c:o.19 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.3.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.4.2.3.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.5 |  |  | additionalText |  |  | {2 9 3 2 7 7} |  |  | GraphicString |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6 |  |  | additionalInformation |  |  | {2 9 3 2 7 6} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6.1 |  |  | identifier |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6.2 |  |  | significance |  |  | - |  |  | BOOLEAN |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.1.6.3 |  |  | information |  |  | - |  |  | ANY DEFINED BY identifier |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
| 4 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectDeletion |  | {2 9 3 2 10 7} |  |  |  |  | c3 |  | c3 |  |  |  |  |  |  |  |  |  |  |  | 4.1 |  |  | ObjectInfo |  |  |  |  |  | Information Syntax SEQUENCE |  |  |  | c3 |  |  | c3 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.1 |  |  | sourceIndicator |  |  | {2 9 3 2 7 26} |  |  | ENUMERATED |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2 |  |  | attributeList |  |  | {2 9 3 2 7 9} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.1 |  |  | attributeId |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.1.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.20 |  |  | c:o.20 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.1.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.20 |  |  | c:o.20 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.2.2 |  |  | attributeValue |  |  | - |  |  | ANY DEFINED BY attributeId |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.3 |  |  | notificationIdentifier |  |  | {2 9 3 2 7 16} |  |  | INTEGER |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.1 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF INTEGER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2 |  |  | sourceObjectInst |  |  | - |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.1 |  |  | distinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.21 |  |  | c:o.21 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.1.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.1.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.2 |  |  | nonSpecificForm |  |  | - |  |  | OCTET STRING |  |  |  | c:o.21 |  |  | c:o.21 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.3 |  |  | localDistinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.21 |  |  | c:o.21 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.3.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.4.2.3.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.5 |  |  | additionalText |  |  | {2 9 3 2 7 7} |  |  | GraphicString |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6 |  |  | additionalInformation |  |  | {2 9 3 2 7 6} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6.1 |  |  | identifier |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6.2 |  |  | significance |  |  | - |  |  | BOOLEAN |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.1.6.3 |  |  | information |  |  | - |  |  | ANY DEFINED BY identifier |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
| 5 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": stateChange |  | {2 9 3 2 10 14} |  |  |  |  | c5 |  | c5 |  |  |  |  |  |  |  |  |  |  |  | 5.1 |  |  | StateChangeInfo |  |  |  |  |  | Information Syntax SEQUENCE |  |  |  | c5 |  |  | c5 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.1 |  |  | sourceIndicator |  |  | {2 9 3 2 7 26} |  |  | ENUMERATED |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.2 |  |  | attributeIdentifierList |  |  | {2 9 3 2 7 8} |  |  | SET OF CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.2.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.22 |  |  | c:o.22 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.2.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.22 |  |  | c:o.22 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3 |  |  | stateChangeDefinition |  |  | {2 9 3 2 7 28} |  |  | SET OF SEQUENCE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.1 |  |  | attributeID |  |  | - |  |  | CHOICE |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.1.1 |  |  | globalForm |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:o.23 |  |  | c:o.23 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.1.2 |  |  | localForm |  |  | - |  |  | INTEGER |  |  |  | c:o.23 |  |  | c:o.23 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.2 |  |  | oldAttributeValue |  |  | - |  |  | ANY DEFINED BY attributeID |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.3.3 |  |  | newAttributeValue |  |  | - |  |  | ANY DEFINED BY attributeID |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.4 |  |  | notificationIdentifier |  |  | {2 9 3 2 7 16} |  |  | INTEGER |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.1 |  |  | correlatedNotifications |  |  | {2 9 3 2 7 12} |  |  | SET OF INTEGER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2 |  |  | sourceObjectInst |  |  | - |  |  | CHOICE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.1 |  |  | distinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.24 |  |  | c:o.24 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.1.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.1.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.2 |  |  | nonSpecificForm |  |  | - |  |  | OCTET STRING |  |  |  | c:o.24 |  |  | c:o.24 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.3 |  |  | localDistinguishedName |  |  | - |  |  | SEQUENCE OF SET OF SEQUENCE |  |  |  | c:o.24 |  |  | c:o.24 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.3.1 |  |  | AttributeType |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.5.2.3.2 |  |  | AttributeValue |  |  | - |  |  | ANY |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.6 |  |  | additionalText |  |  | {2 9 3 2 7 7} |  |  | GraphicString |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7 |  |  | additionalInformation |  |  | {2 9 3 2 7 6} |  |  | SET OF SEQUENCE |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7.1 |  |  | identifier |  |  | - |  |  | OBJECT IDENTIFIER |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7.2 |  |  | significance |  |  | - |  |  | BOOLEAN |  |  |  | c:o |  |  | c:o |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.1.7.3 |  |  | information |  |  | - |  |  | ANY DEFINED BY identifier |  |  |  | c:m |  |  | c:m |  |  |  |  |  |  |

Table C.3.8  bMGroomedCircuit Parameter support

| Index | Parameter template label | Value of object identifier for parameter | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "Recommendation Q.821:1992": correlatedRecordNameParameter | {0 0 17 821 0 5 2} | EVENT-INFO communicationsAlarm | c6 | c6 |  |  |
| 2 | "Recommendation Q.821:1992": logRecordIdParameter | {0 0 17 821 0 5 1} | EVENT-INFO communicationsAlarm | c6 | c6 |  |  |
| 3 | "Recommendation Q.821:1992": suspectObjectListParameter | {0 0 17 821 0 5 3} | EVENT-INFO communicationsAlarm | c6 | c6 |  |  |

Table C.3.9  bMGroomedCircuit Conditions

| Condition number | Condition | Reference |
| --- | --- | --- |
| c1 | IF 1.3.3/2 THEN m ELSE - | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" |
| c2 | IF 1.3.3/4 THEN m ELSE - | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c3 | IF 1.3.3/9 THEN m ELSE - | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" |
| c4 | IF 1.3.3/11 THEN m ELSE - | "any registered package, other than this package has been instantiated" |
| c5 | IF 1.3.3/15 THEN m ELSE - | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c6 | IF 1.3.3/17 THEN m ELSE - | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" |

Table C.3.10  bMGroomedCircuit Name Binding support

| Index | Name binding template label | Value of object identifier for name binding | Constraints and values | Status | Profile | Support | Additional information | Subindex | Operation | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | bMGroomedCircuit-network | {1 2 124 360501 4 170} | Superior class: "Recommendation M.3100": network | o | o |  |  | 1.1 | Create support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.2 | Create with reference object |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.3 | Create with automatic instance naming |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.4 | Delete support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.5 | Delete only if no contained objects |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.6 | Delete contained objects |  | - | - |  |  |

## C.4  bMGroomedCircuitTP

Table C.4.1  bMGroomedCircuitTP Managed object class support

| Index | Managed object class template label | Value of object identifier for class | Support of all mandatory features? (Y/N) | Is the actual class the same as the managed object class to which conformance is claimed? (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | bMGroomedCircuitTP | {1 2 124 360501 3 75} | Y | Y |

Table C.4.2  bMGroomedCircuitTP Actual class support

| Index | Managed object class template for actual class | Value of object identifier for managed object class definition of actual class | Additional information |
| --- | --- | --- | --- |
| N/A | N/A | N/A | N/A |

Table C4.3  bMGroomedCircuitTP Package support

| Index | Package template label | Value of object identifier for package | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": administrativeStatePackage | {2 9 3 2 4 14} | "the resource represented by the managed object is capable of being administratively placed in and out of service" | c1 | c1 |  |  |
| 2 | "Recommendation M.3100": alarmSeverityAssignmentPointerPackage | {0 0 13 3100 0 4 3} | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" | c2 | c2 |  |  |
| 3 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphicPackage | {2 9 3 2 4 17} | "if an object supports allomorphism" | o | o |  |  |
| 4 | "Recommendation M.3100": attributeValueChangeNotificationPackage | {0 0 13 3100 0 4 4} | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c3 | c3 |  |  |
| 5 | bMGroomedCircuitTPPackage |  | Mandatory | m | m |  |  |
| 6 | "Recommendation M.3100": characteristicInformationPackage | {0 0 13 3100 0 4 7} | "an instance supports it." | o | o |  |  |
| 7 | "Recommendation M.3100": createDeleteNotificationsPackage | {0 0 13 3100 0 4 10} | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" | c4 | c4 |  |  |
| 8 | "Recommendation M.3100": crossConnectionPointerPackage | {0 0 13 3100 0 4 11} | "the termination point can be flexibly assigned, (i.e. cross connected)." | c5 | c5 |  |  |
| 9 | "Recommendation M.3100": networkLevelPackage | {0 0 13 3100 0 4 18} | "an instance supports it" | o | o |  |  |
| 10 | "Recommendation M.3100": operationalStatePackage | {0 0 13 3100 0 4 19} | Mandatory | m | m |  |  |
| 11 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packagesPackage | {2 9 3 2 4 16} | "any registered package, other than this package has been instantiated" | c6 | c6 |  |  |
| 12 | "Recommendation M.3100": stateChangeNotificationPackage | {0 0 13 3100 0 4 28} | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" | c7 | c7 |  |  |
| 13 | "Recommendation M.3100": supportableClientListPackage | {0 0 13 3100 0 4 27} | "the object class can support more than one type of client" | c8 | c8 |  |  |
| 14 | "Recommendation M.3100": terminationPointPackage |  | Mandatory | m | m |  |  |
| 15 | "Recommendation M.3100": tmnCommunicationsAlarmInformationPackage | {0 0 13 3100 0 4 30} | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" | c9 | c9 |  |  |
| 16 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": topPackage |  | Mandatory | m | m |  |  |
| 17 | "Recommendation M.3100": trailTerminationPointBidirectionalPackage |  | Mandatory | m | m |  |  |
| 18 | "Recommendation M.3100": trailTerminationPointSinkPackage |  | Mandatory | m | m |  |  |
| 19 | "Recommendation M.3100": trailTerminationPointSourcePackage |  | Mandatory | m | m |  |  |
| 20 | "Recommendation M.3100": ttpInstancePackage | {0 0 13 3100 0 4 31} | "the name binding used to create an instance of this object class requires this attribute." | c10 | c10 |  |  |

Table C.4.4  bMGroomedCircuitTP Attribute support

|  |  |  |  | Set by create |  |  | Get |  |  | Replace |  |  | Add |  |  | Remove |  |  | Set to default |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Attribute template label | Value of object identifier for attribute | Constraints and values | Status | Profile | Support | Status | Profile | Support | Status | Profile | Support | Status | Profile | Support | Status | Profile | Support | Status | Profile | Support | Additional information |
| 1 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": administrativeState | {2 9 3 2 7 31} | ENUMERATED | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 2 | "Recommendation M.3100": alarmSeverityAssignmentProfilePointer | {0 0 13 3100 0 7 5} | CHOICE | c2 | c2 |  | c2 | c2 |  | c2 | c2 |  | - | - |  | - | - |  | - | - |  |  |
| 3 | "Recommendation M.3100": alarmStatus | {0 0 13 3100 0 7 6} | ENUMERATED | - | - |  | c9 | c9 |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 4 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": allomorphs | {2 9 3 2 7 50} | SET OF CHOICE | - | - |  | o | o |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 5 | "Recommendation M.3100": characteristicInformation | {0 0 13 3100 0 7 8} | OBJECT IDENTIFIER | - | - |  | o | o |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 6 | "Recommendation M.3100": crossConnectionObjectPointer | {0 0 13 3100 0 7 16} | CHOICE | c5 | c5 |  | c5 | c5 |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 7 | "Recommendation M.3100": currentProblemList | {0 0 13 3100 0 7 17} | SET OF SEQUENCE | c9 | c9 |  | c9 | c9 |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 8 | "Recommendation M.3100": downstreamConnectivityPointer | {0 0 13 3100 0 7 19} | CHOICE | m | m |  | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  |  |
| 9 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": nameBinding | {2 9 3 2 7 63} | OBJECT IDENTIFIER | o | o |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 10 | "Recommendation M.3100": networkLevelPointer | {0 0 13 3100 0 7 31} | CHOICE | o | o |  | o | o |  | o | o |  | - | - |  | - | - |  | - | - |  |  |
| 11 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectClass | {2 9 3 2 7 65} | CHOICE | - | - |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 12 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": operationalState | {2 9 3 2 7 35} | ENUMERATED | - | - |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 13 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": packages | {2 9 3 2 7 66} | SET OF OBJECT IDENTIFIER | o | o |  | c6 | c6 |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 14 | "Recommendation M.3100": supportableClientList | {0 0 13 3100 0 7 39} | SET OF CHOICE | c8 | c8 |  | c8 | c8 |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 15 | "Recommendation M.3100": supportedByObjectList | {0 0 13 3100 0 7 40} | SET OF CHOICE | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 16 | "Recommendation M.3100": tTPId | {0 0 13 3100 0 7 48} | CHOICE | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 17 | "OP1 Library Vol. 4": trmDataMode | {1 2 124 360501 1 313} | CHOICE | m | m |  | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  |  |
| 18 | "OP1 Library Vol. 4": trmFunctionalInterface | {1 2 124 360501 1 317} | CHOICE | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 19 | "OP1 Library Vol. 4": trmVoiceMode | {1 2 124 360501 1 322} | CHOICE | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 20 | "OP1 Library Vol. 1": typeText | {1 3 14 2 2 4 59} | GraphicString | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  | - | - |  |  |
| 21 | "Recommendation M.3100": upstreamConnectivityPointer | {0 0 13 3100 0 7 49} | CHOICE | m | m |  | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  |  |
| 22 | "Recommendation M.3100": userLabel | {0 0 13 3100 0 7 50} | GraphicString | m | m |  | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  |  |
| 23 | "NMF038, Issue 1": userIdentifier | {1 2 124 360501 1 327} | GraphicString | m | m |  | m | m |  | m | m |  | - | - |  | - | - |  | - | - |  |  |

Table C.4.5  bMGroomedCircuitTP Attribute group support

There are no attribute groups specified for this managed object class.

Table C.4.6  bMGroomedCircuitTP Action support

| Index | Action type template label | Value of object identifier for action type | Constraints and values | Status | Profile | Support | Additional information | Subindex | Action field name label | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "OP1 Library Vol. 4": trmAbort | {1 2 124 360501 6 11} |  | m | m |  |  |  |  |  |  |  |  | 2 |
| 2 | "OP1 Library Vol. 4": trmGetStatus | {2 124 360501 6 14}1 |  | m | m |  |  | 2.1 | Status | Reply Syntax INTEGER | m | m |  |  |
| 3 | "OP1 Library Vol. 4": trmLocalLoop | {1 2 124 360501 6 15} |  | m | m |  |  | 3.1 | LoopSetUp | Reply Syntax BOOLEAN | m | m |  |  |

Table C.4.7  bMGroomedCircuitTP Notification support

|  |  |  |  |  |  | Support |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | Notification type template label | Value of object identifier for notification type | Constraints and values | Status | Profile | Confirmed | Nonconfirmed | Additional information | Subindex | Notification field name label | Value of object identifier of attribute type associated with field | Constraints and values | Status | Profile | Support | Additional information |
| 1 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": attributeValueChange | {2 9 3 2 10 1} |  | c3 | c3 |  |  |  | 1.1 | AttributeValueChangeInfo |  | Information Syntax SEQUENCE | c3 | c3 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.2 | attributeIdentifierList | {2 9 3 2 7 8} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.2.1 | globalForm | - | OBJECT IDENTIFIER | c:o.1 | c:o.1 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.2.2 | localForm | - | INTEGER | c:o.1 | c:o.1 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3 | attributeValueChangeDefinition | {2 9 3 2 7 10} | SET OF SEQUENCE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.1 | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.2 | c:o.2 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.1.2 | localForm | - | INTEGER | c:o.2 | c:o.2 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.2 | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.3.3 | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.4 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.2 | nonSpecificForm | - | OCTET STRING | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.3 | c:o.3 |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.5.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.6 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 1.1.7.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 2 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": communicationsAlarm | {2 9 3 2 10 2} |  | c9 | c9 |  |  | "Recommendation Q.821:1992": correlatedRecordNameParameter "Recommendation Q.821:1992": logRecordIdParameter "Recommendation Q.821:1992": suspectObjectListParameter | 2.1 | AlarmInfo |  | Information Syntax SEQUENCE | c9 | c9 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.1 | probableCause | {2 9 3 2 7 18} | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.1.1 | globalValue | - | OBJECT IDENTIFIER | c:o.4 | c:o.4 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.1.2 | localValue | - | INTEGER | c:o.4 | c:o.4 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.2 | specificProblems | {2 9 3 2 7 27} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.2.1 | OBJECT IDENTIFIER | - | OBJECT IDENTIFIER | c:o.5 | c:o.5 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.2.2 | INTEGER | - | INTEGER | c:o.5 | c:o.5 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.3 | perceivedSeverity | {2 9 3 2 7 17} | ENUMERATED | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.4 | backedUpStatus | {2 9 3 2 7 11} | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5 | backUpObject | {2 9 3 2 7 40} | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.2 | nonSpecificForm | - | OCTET STRING | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.6 | c:o.6 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.5.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.6 | trendIndication | {2 9 3 2 7 30} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7 | thresholdInfo | {2 9 3 2 7 29} | SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.1 | triggeredThreshold | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.7 | c:o.7 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.1.2 | localForm | - | INTEGER | c:o.7 | c:o.7 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.2 | observedValue | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.2.1 | integer | - | INTEGER | c:o.8 | c:o.8 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.2.2 | real | - | REAL | c:o.8 | c:o.8 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3 | thresholdLevel | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1 | up | - | SEQUENCE | c:o.9 | c:o.9 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1 | high | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.1 | integer | - | INTEGER | c:o.10 | c:o.10 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.1.2 | real | - | REAL | c:o.10 | c:o.10 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2 | low | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.1 | integer | - | INTEGER | c:o.11 | c:o.11 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.1.2.2 | real | - | REAL | c:o.11 | c:o.11 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2 | down | - | SEQUENCE | c:o.9 | c:o.9 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1 | high | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.1 | integer | - | INTEGER | c:o.12 | c:o.12 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.1.2 | real | - | REAL | c:o.12 | c:o.12 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2 | low | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.1 | integer | - | INTEGER | c:o.13 | c:o.13 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.3.2.2.2 | real | - | REAL | c:o.13 | c:o.13 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.7.4 | armTime | - | GeneralizedTime | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.8 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.2 | nonSpecificForm | - | OCTET STRING | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.14 | c:o.14 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.9.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10 | stateChangeDefinition | {2 9 3 2 7 28} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.1 | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.15 | c:o.15 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.1.2 | localForm | - | INTEGER | c:o.15 | c:o.15 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.2 | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.10.3 | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11 | monitoredAttributes | {2 9 3 2 7 15} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.1 | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.16 | c:o.16 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.1.2 | localForm | - | INTEGER | c:o.16 | c:o.16 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.11.2 | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.12 | proposedRepairActions | {2 9 3 2 7 19} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.12.1 | OBJECT IDENTIFIER | - | OBJECT IDENTIFIER | c:o.17 | c:o.17 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.12.2 | INTEGER | - | INTEGER | c:o.17 | c:o.17 |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.13 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 2.1.14.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 3 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectCreation | {2 9 3 2 10 6} |  | c4 | c4 |  |  |  | 3.1 | ObjectInfo |  | Information Syntax SEQUENCE | c4 | c4 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2 | attributeList | {2 9 3 2 7 9} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.1 | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.18 | c:o.18 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.1.2 | localForm | - | INTEGER | c:o.18 | c:o.18 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.2.2 | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.3 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.2 | nonSpecificForm | - | OCTET STRING | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.19 | c:o.19 |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.4.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.5 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 3.1.6.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 4 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": objectDeletion | {2 9 3 2 10 7} |  | c4 | c4 |  |  |  | 4.1 | ObjectInfo |  | Information Syntax SEQUENCE | c4 | c4 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2 | attributeList | {2 9 3 2 7 9} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.1 | attributeId | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.20 | c:o.20 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.1.2 | localForm | - | INTEGER | c:o.20 | c:o.20 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.2.2 | attributeValue | - | ANY DEFINED BY attributeId | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.3 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.2 | nonSpecificForm | - | OCTET STRING | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.21 | c:o.21 |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.4.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.5 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 4.1.6.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |
| 5 | "Rec. X.721 ¦ ISO/IEC 10165-2 : 1992": stateChange | {2 9 3 2 10 14} |  | c7 | c7 |  |  |  | 5.1 | StateChangeInfo |  | Information Syntax SEQUENCE | c7 | c7 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.1 | sourceIndicator | {2 9 3 2 7 26} | ENUMERATED | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.2 | attributeIdentifierList | {2 9 3 2 7 8} | SET OF CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.2.1 | globalForm | - | OBJECT IDENTIFIER | c:o.22 | c:o.22 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.2.2 | localForm | - | INTEGER | c:o.22 | c:o.22 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3 | stateChangeDefinition | {2 9 3 2 7 28} | SET OF SEQUENCE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.1 | attributeID | - | CHOICE | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.1.1 | globalForm | - | OBJECT IDENTIFIER | c:o.23 | c:o.23 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.1.2 | localForm | - | INTEGER | c:o.23 | c:o.23 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.2 | oldAttributeValue | - | ANY DEFINED BY attributeID | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.3.3 | newAttributeValue | - | ANY DEFINED BY attributeID | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.4 | notificationIdentifier | {2 9 3 2 7 16} | INTEGER | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5 | correlatedNotifications | {2 9 3 2 7 12} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.1 | correlatedNotifications | {2 9 3 2 7 12} | SET OF INTEGER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2 | sourceObjectInst | - | CHOICE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.1 | distinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.1.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.1.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.2 | nonSpecificForm | - | OCTET STRING | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.3 | localDistinguishedName | - | SEQUENCE OF SET OF SEQUENCE | c:o.24 | c:o.24 |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.3.1 | AttributeType | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.5.2.3.2 | AttributeValue | - | ANY | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.6 | additionalText | {2 9 3 2 7 7} | GraphicString | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7 | additionalInformation | {2 9 3 2 7 6} | SET OF SEQUENCE | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7.1 | identifier | - | OBJECT IDENTIFIER | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7.2 | significance | - | BOOLEAN | c:o | c:o |  |  |
|  |  |  |  |  |  |  |  |  | 5.1.7.3 | information | - | ANY DEFINED BY identifier | c:m | c:m |  |  |

Table C.4.8  bMGroomedCircuitTP Parameter support

| Index | Parameter template label | Value of object identifier for parameter | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | "Recommendation Q.821:1992": correlatedRecordNameParameter | {0 0 17 821 0 5 2} | EVENT-INFO communicationsAlarm | c9 | c9 |  |  |
| 2 | "Recommendation Q.821:1992": logRecordIdParameter | {0 0 17 821 0 5 1} | EVENT-INFO communicationsAlarm | c9 | c9 |  |  |
| 3 | "Recommendation Q.821:1992": suspectObjectListParameter | {0 0 17 821 0 5 3} | EVENT-INFO communicationsAlarm | c9 | c9 |  |  |

Table C.4.9  bMGroomedCircuitTP Conditions

| Condition number | Condition | Reference |
| --- | --- | --- |
| c1 | IF 1.4.3/1 THEN m ELSE - | "the resource represented by the managed object is capable of being administratively placed in and out of service" |
| c2 | IF 1.4.3/2 THEN m ELSE - | "the tmnCommunicationsAlarmInformationPackage package is present AND the managed object supports configuration of alarm severities" |
| c3 | IF 1.4.3/4 THEN m ELSE - | "the attributeValueChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c4 | IF 1.4.3/7 THEN m ELSE - | "the objectCreation and objectDeletion notifications defined in Recommendation X.721 are supported by an instance of this managed object class" |
| c5 | IF 1.4.3/8 THEN m ELSE - | "the termination point can be flexibly assigned, (i.e. cross connected)." |
| c6 | IF 1.4.3/11 THEN m ELSE - | "any registered package, other than this package has been instantiated" |
| c7 | IF 1.4.3/12 THEN m ELSE - | "the stateChange notification defined in Recommendation X.721 is supported by an instance of this managed object class" |
| c8 | IF 1.4.3/13 THEN m ELSE - | "the object class can support more than one type of client" |
| c9 | IF 1.4.3/15 THEN m ELSE - | "the communicationsAlarm notification (as defined in Recommendation X.721) is supported by this managed object" |
| c10 | IF 1.4.3/20 THEN m ELSE - | "the name binding used to create an instance of this object class requires this attribute." |

Table C.4.10  bMGroomedCircuitTP Name Binding support

| Index | Name binding template label | Value of object identifier for name binding | Constraints and values | Status | Profile | Support | Additional information | Subindex | Operation | Constraints and values | Status | Profile | Support | Additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | bMGroomedCircuitTP-managedElement | {1 2 124 360501 4 168} | Superior class: "Recommendation M.3100": managedElement | o | o |  |  | 1.1 | Create support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.2 | Create with reference object |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.3 | Create with automatic instance naming |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.4 | Delete support |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.5 | Delete only if no contained objects |  | - | - |  |  |
|  |  |  |  |  |  |  |  | 1.6 | Delete contained objects |  | - | - |  |  |
| 2 | "Recommendation M.3100": trailTerminationPointSource-managedElement | {0 0 13 3100 0 6 23} | Superior class: "Recommendation M.3100": managedElement AND SUBCLASSES | o | o |  |  | 2.1 | Create support |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.2 | Create with reference object |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.3 | Create with automatic instance naming |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.4 | Delete support |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.5 | Delete only if no contained objects |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 2.6 | Delete contained objects |  | c:x | c:x |  |  |
| 3 | "Recommendation M.3100": trailTerminationPointSink-managedElement | {0 0 13 3100 0 6 24} | Superior class: "Recommendation M.3100": managedElement AND SUBCLASSES | o | o |  |  | 3.1 | Create support |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 3.2 | Create with reference object |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 3.3 | Create with automatic instance naming |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 3.4 | Delete support |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 3.5 | Delete only if no contained objects |  | c:m | c:m |  |  |
|  |  |  |  |  |  |  |  | 3.6 | Delete contained objects |  | c:x | c:x |  |  |

# ANNEX D - MANAGED OBJECT CLASSES (GDMO)

## D.1 GDMO Definitions for OP0 Specification

-- Below are the object templates for the three extension objects used

-- (termination Point, terminating Equipment, and Groomed Circuit).

-- GDMO definitions follow:

--

--  1.0  MANAGED OBJECT CLASSES

--  1.1  Termination Point M-Object

terminationPoint MANAGED OBJECT CLASS

DERIVED FROM    function;

CHARACTERIZED BY

terminationPointPackage PACKAGE

BEHAVIOUR  terminationPointBehaviour;     -- see clause 2.2.5.2

ATTRIBUTES

dataMode                  GET-REPLACE,

flowControlType        GET,

functionalInterface     GET,

terminatingType         GET,

voiceMode                 GET;

--          equipmentNames           GET,

--          functionType                  GET,

--          networkNames              GET,

--          typeText                        GET,

--          userLabels                    GET-REPLACE,

ACTIONS  abort,

getStatus,

ll,

rl;

;;

REGISTERED AS   {bt-objectClass    37};

--  1.2  Terminating Equipment M-Object

terminatingEquipment MANAGED OBJECT CLASS

DERIVED FROM    equipment;

CHARACTERIZED BY

terminatingEquipmentPackage PACKAGE

BEHAVIOUR  terminatingEquipmentBehaviour;  --  see clause 2.2.5.2

ATTRIBUTES

gender                      GET-REPLACE,

physicalInterface       GET;

--          equipmentType          GET,

--          functionNames          GET,

--          networkNames          GET,

--          productLabel             GET-REPLACE,

--          serialNumber            GET-REPLACE,

--          typeText                    GET,

--          userLabels                GET-REPLACE;

;;

REGISTERED AS  {bt-objectClass    38};

--  1.3  Groomed Circuit M-Object

groomedCircuit MANAGED OBJECT CLASS

DERIVED FROM      circuit;

CHARACTERIZED BY

groomedCircuitPackage PACKAGE

BEHAVIOUR     groomedCircuitBehaviour;    --  see clause 2.2.5.2

ATTRIBUTES

circuitProfile                  GET-REPLACE,

encryptionRouting         GET-REPLACE,

fibreRouting                  GET-REPLACE,

terrestrialRouting          GET-REPLACE;

ACTIONS  grooming,

trace;

NOTIFICATIONS   groomed;

;;

REGISTERED AS  {bt-objectClass    39};

--  2.0  BEHAVIOURS

--  2.1 Termination Point Behaviour

terminationPointBehaviour BEHAVIOUR

DEFINED AS

!The function managed object class behaviour [4] is inherited by the termination point managed object class.  The functionType attribute shall be set to 'terminationPoint (4)' for all instances of this managed object class.

If the dataMode attribute has a value other than NULL, the voiceMode attribute must be set to NULL, and the data BOOLEAN must be TRUE and voice BOOLEAN must be FALSE in the terminatingType attribute.

Similarly if the voiceMode attribute has a value other than NULL, the dataMode attribute must be set to NULL, and the voice BOOLEAN must be TRUE and data BOOLEAN must be FALSE in the terminatingType attribute.

Test actions have been incorporated into the class definition in order to enable testing of ports (local loops) and circuits (remote loops).!

;

-- REGISTERED AS   {ccitt(0) network-operator(3)  bt(2342) behaviour(10)

-- terminationPoint(0)};

--  2.2 Terminating Equipment Behaviour

terminatingEquipmentBehaviour BEHAVIOUR

DEFINED AS

!The equipment managed object class behaviour [4] is inherited by the terminatingEquipment managed object class.!

;

-- REGISTERED AS   {ccitt(0) network-operator(3)  bt(2342)  behaviour(10)

-- terminatingEquipment(1)};

--  2.3 Groomed Circuit Behaviour

groomedCircuitBehaviour BEHAVIOUR

DEFINED AS

!The circuit managed object class behaviour [4] is inherited by the groomedCircuit managed object class.

Trace and grooming actions are specified for this managed object class. If the requesting CME does not provide a value with the grooming action, then the responding CME will assume that the action is to be initiated immediately. The specified time sent with a groomingAction may be overridden by criteria detailed in a selected circuit profile.

The groomed notification indicates that grooming has taken place.!

;

-- REGISTERED AS   {ccitt(0)  network-operator(3)  bt(2342)   behaviour(10)

-- groomedCircuit(2)};

--  3.0  NAME BINDINGS

--  3.1 Groomed Circuit Name Binding

groomedCircuit-nb-1  NAME BINDING

SUBORDINATE OBJECT CLASS  groomedCircuit;

NAMED BY

SUPERIOR OBJECT CLASS network;

WITH ATTRIBUTE  circuitID;

CREATE  WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE  ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {bt-namebinding        70};

--  3.2 Termination Point #1 Name Binding

terminationPoint-nb-1  NAME BINDING

SUBORDINATE OBJECT CLASS  terminationPoint;

NAMED BY

SUPERIOR OBJECT CLASS network;

WITH ATTRIBUTE  functionID;

REGISTERED AS  {bt-namebinding       71};

--  3.3 Termination Point #2 Name Binding

terminationPoint-nb-2  NAME BINDING

SUBORDINATE OBJECT CLASS  terminationPoint;

NAMED BY

SUPERIOR OBJECT CLASS equipment;

WITH ATTRIBUTE  functionID;

REGISTERED AS  {bt-namebinding       124};

--  3.4 Terminating Equipment #1 Name Binding

terminatingEquipment-nb-1  NAME BINDING

SUBORDINATE OBJECT CLASS  terminatingEquipment;

NAMED BY

SUPERIOR OBJECT CLASS   network;

WITH ATTRIBUTE   equipmentID;

REGISTERED AS {bt-namebinding           72};

--  3.5 Terminating Equipment #2 Name Binding

terminatingEquipment-nb-2  NAME BINDING

SUBORDINATE OBJECT CLASS  terminatingEquipment;

NAMED BY

SUPERIOR OBJECT CLASS   equipment;

WITH ATTRIBUTE   equipmentID;

REGISTERED AS  {bt-namebinding            73};

--  4.0  ATTRIBUTES

circuitProfile  ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.CircuitProfile;

MATCHES FOR EQUALITY;

BEHAVIOUR circuitProfileBehaviour;

REGISTERED AS {bt-attributeType     200};

circuitProfileBehaviour    BEHAVIOUR

DEFINED AS

!The circuitProfile attribute is a set-valued attribute of the groomedCircuit managed object class.!

;

dataMode ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.DataMode;

MATCHES FOR EQUALITY;

BEHAVIOUR dataModeBehaviour;

REGISTERED AS {bt-attributeType           201};

dataModeBehaviour    BEHAVIOUR

DEFINED AS

!The dataMode attribute is a single-valued attribute of the terminationPoint managed object class.!

;

encryptionRouting  ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.Category;

BEHAVIOUR encryptionRoutingBehaviour;

REGISTERED AS {bt-attributeType          202};

encryptionRoutingBehaviour    BEHAVIOUR

DEFINED AS

!The encryptionRouting attribute is a single-valued attribute of the groomedCircuit managed object class.!

;

fibreRouting  ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.Category;

BEHAVIOUR fibreRoutingBehaviour;

REGISTERED AS {bt-attributeType              203};

fibreRoutingBehaviour     BEHAVIOUR

DEFINED AS

!The fibreRouting attribute is a single-valued attribute of the groomedCircuit managed object class.!

;

flowControlType ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.FlowControlType;

MATCHES FOR EQUALITY;

BEHAVIOUR flowControlTypeBehaviour;

REGISTERED AS {bt-attributeType            204};

flowControlTypeBehaviour      BEHAVIOUR

DEFINED AS

!The flowControlType attribute is a single-valued attribute of the terminationPoint managed object class.!

;

functionalInterface ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.FunctionalInterface;

MATCHES FOR EQUALITY;

BEHAVIOUR functionalInterfaceBehaviour;

REGISTERED AS {bt-attributeType           205};

functionalInterfaceBehaviour      BEHAVIOUR

DEFINED AS

!The functionalInterface attribute is a single-valued attribute of the terminationPoint managed object class.!

;

gender ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.Gender;

MATCHES FOR EQUALITY;

BEHAVIOUR genderBehaviour;

REGISTERED AS {bt-attributeType            206};

genderBehaviour    BEHAVIOUR

DEFINED AS

!The gender attribute is a single-valued attribute of the terminatingEquipment managed object class.!

;

physicalInterface ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.PhysicalInterface;

MATCHES FOR EQUALITY;

BEHAVIOUR physicalInterfaceBehaviour;

REGISTERED AS {bt-attributeType          207};

physicalInterfaceBehaviour    BEHAVIOUR

DEFINED AS

!The physicalInterface attribute is a single-valued attribute of the terminatingEquipment managed object class.!

;

terrestrialRouting  ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.Category;

BEHAVIOUR terrestrialRoutingBehaviour;

REGISTERED AS {bt-attributeType             208};

terrestrialRoutingBehaviour    BEHAVIOUR

DEFINED AS

!The terrestrialRouting attribute is a single-valued attribute of the groomedCircuit managed object class.!

;

terminatingType ATTRIBUTE

WITH ATTRIBUTE SYNTAX BandwidthModelModule.TerminatingType;

MATCHES FOR EQUALITY;

BEHAVIOUR terminatingTypeBehaviour;

REGISTERED AS {bt-attributeType                     209};

terminatingTypeBehaviour    BEHAVIOUR

DEFINED AS

!The terminatingType attribute is a single-valued attribute of the terminationPoint managed object class.!

;

voiceMode ATTRIBUTE

WITH ATTRIBUTE SYNTAX  BandwidthModelModule.VoiceMode;

MATCHES FOR EQUALITY;

BEHAVIOUR voiceModeBehaviour;

REGISTERED AS {bt-attributeType              210};

voiceModeBehaviour    BEHAVIOUR

DEFINED AS

!The voiceMode attribute is a single-valued attribute of the terminationPoint Record managed object class.!

;

-- 5.0  ACTIONS

abort     ACTION

REGISTERED AS {bt-action        9};

getStatus     ACTION

WITH REPLY SYNTAX BandwidthModelModule.Status;

REGISTERED AS {bt-action         10};

ll     ACTION

WITH REPLY SYNTAX BandwidthModelModule.LoopSetUp ;

REGISTERED AS  {bt-action          12};

rl     ACTION

WITH INFORMATION SYNTAX BandwidthModelModule.RemoteTerminationPoint;

WITH REPLY SYNTAX BandwidthModelModule.LoopSetUp;

REGISTERED AS  {bt-action           14};

-- dataParameters ACTION     (used for extendedDataCircuit)

--     WITH INFORMATION SYNTAX BandwidthModelModule.MeasurementPeriod;

--     WITH REPLY SYNTAX BandwidthModelModule.DataParameters;

-- REGISTERED AS  {bt-action        15};

grooming ACTION

WITH INFORMATION SYNTAX BandwidthModelModule.Grooming;

REGISTERED AS  {bt-action          16};

trace ACTION

WITH REPLY SYNTAX BandwidthModelModule.Trace;

REGISTERED AS  {bt-action           17};

-- END

-- asn.1 definitions

-- Module

BandwidthModelModule {ccitt(0) network-operator(3) bt(2342) modules(0) bandwidthModelModule(13)}

DEFINITIONS IMPLICIT TAGS ::= BEGIN

IMPORTS

Bandwidth, Name, Integer8

FROM FORUM-TYPES-1 {forum-modules(0) types-1(5)}

ObjectInstance

FROM FORUM-CMIP {forum-modules(0) cmip-version2(1)};

-- EXPORTS everything

Analogue ::=     SEQUENCE {

connection      Connection,

signalling      Signalling,

rxLevel         RxLevel,

txLevel         TxLevel

}

Async ::=        SEQUENCE {

characterSize   CharacterSize,

parity          Parity,

stopBits        StopBits

}

Category ::=     ENUMERATED{

dontCare        (0),

preferNot       (1),

preferred       (2),

required        (3),

requiredNot     (4)

}

ChannelAssociationInTS16 ::= BOOLEAN

--       FALSE not in TS16

--       TRUE in TS16

CharacterSize ::=        ENUMERATED{

unspecified   (0),

five          (1),

six           (2),

seven         (3),

eight         (4),

nine          (5)

}

CircuitProfile ::=       SET OF CHOICE{

circuitProfileName    [0] Name,

circuitProfile        [1] SEQUENCE{

cpType   Integer8,

cpValue  REAL}

}

Companding ::=   ENUMERATED{

notApplicable   (0),

g711mulaw       (1),

g711Alaw        (2)

}

Connection ::=   ENUMERATED{

x2wire  (0),

x4wire  (1)

}

DataMode ::=     CHOICE{

dataFormat      [0] DataFormat,

notApplicable   [1] NULL

}

DataFormat::=   SEQUENCE{

rate    Rate,

format  Format

}

DataParameters ::=       SEQUENCE{

ber         Parameter,

efs         Parameter,

ers         Parameter,

ses         Parameter,

dgm         Parameter,

availTime   Parameter,

totalTime   Parameter

}

Digital ::=      SEQUENCE {

encoding              Encoding,

channelAssociation    ChannelAssociationInTS16

}

EchoCancellation ::= BOOLEAN

Encoding ::=     INTEGER{

unspecified    (0),

x32kADPCM      (1),

x64kPCM        (2),

x16VAPC        (250),

x8VAPC         (251),

byteAligned    (252),

switched56K    (253),

x56k           (254),

clearChannel   (255)

}(0..255)

FlowControlType ::= CHOICE{

notApplicable  [0] NULL,

inband         [1] Inband,

outband        [2] Outband

}

Format ::=       CHOICE{

async   [2] Async,

sync    [3] Sync

}

FrameFormat ::=  INTEGER{

unspecified  (0),

hdlc         (1),

sdlc         (2),

x25          (3),

x75          (4)

}(0..255)

FunctionalInterface ::= CHOICE{

notApplicable   [0] NULL,

interface       [1] INTEGER{

x21         (0),

x21bis      (1),

g703-g704   (2),

atm         (250),

atm-aal     (251),

ocu-dp      (252),

v24         (253),

x24         (254),

unknown     (255)

}(0..255)

}

Gender ::=       ENUMERATED{

unknown (0),

dce     (1),

dte     (2)

}

Grooming ::=     CHOICE{

scheduled       GeneralizedTime,

now             NULL

}

Inband ::=       ENUMERATED{

dc2     (0),

dc3     (1)

}

InterfaceLogic ::=  ENUMERATED {

unknown    (0),

false      (1),

true       (2)

}

Level ::=        SEQUENCE{

nominal         REAL,

maxValue        REAL,

minValue        REAL,

minVariation    REAL

}

--       minVariation default is 0.5dB.

LoopSetUp ::=    BOOLEAN

MeasurementPeriod ::= SEQUENCE{

startTime       GeneralizedTime,

stopTime        GeneralizedTime

}

Mode ::= CHOICE{

analogue   [0] Analogue,

digital    [1] Digital

}

Outband ::=  SEQUENCE{

clearToSend             InterfaceLogic,

dataCarrierDetect       InterfaceLogic,

dataSetReady            InterfaceLogic,

dataTerminalReady       InterfaceLogic,

ringIdentifier          InterfaceLogic,

readyToSend             InterfaceLogic

}

Parameter ::=    CHOICE{

unavailable     NULL,

pValue          REAL

}

Parity ::=       ENUMERATED{

odd     (0),

even    (1),

mark    (2),

space   (3),

none    (4)

}

PhysicalInterface ::= INTEGER{

v24              (0),

v11              (1),

v35              (2),

x4wto            (3),

lgs              (4),

lge              (5),

mrd              (6),

ocu-dp           (7),

e-m              (8),

sonet            (241),

taxi             (242),

aal5             (243),

aal1             (244),

v35-v28          (245),

rs-232-c         (246),

mIL-STD-118-114  (247),

tI-601           (248),

g703unbalanced   (249),

g703balanced     (250),

v11-nnc          (251),

d24              (252),

dsx-1            (253),

japaneseTTC      (254),

unknown          (255)

}(0..255)

Rate ::= CHOICE{

rate    Bandwidth,

rates   SEQUENCE{

incoming     Bandwidth,

outgoing     Bandwidth}

}

RemoteTerminationPoint  ::=  ObjectInstance

Result ::=       BOOLEAN

RxLevel ::=      Level

Status ::=       INTEGER{

idle     (0),

ll       (1),

llTest   (2),

rl       (3),

rlTest   (4)

}(0..255)

StopBits ::=     ENUMERATED{

x1      (0),

x1-5    (1),

x2      (2)

}

Sync ::= SEQUENCE{

frameFormat     FrameFormat,

characterSet    CharacterSize

}

TxLevel ::=      Level

Signalling ::=   INTEGER{

x2280           (0),

x2600           (1),

dTMF            (2),

e-MI            (3),

e-MII           (4),

e-MIII          (5),

e-MIV           (6),

e-MV            (7),

loopDisconnect  (8),

plrI            (9),

plrII          (10),

to             (11),

ls             (12),

gs             (13),

plar           (14),

rmtext         (15),

ec             (16),

lse-m          (17),

gse-m          (18),

cbw            (19),

lcdc           (20),

dpt            (21),

lgsmrd         (22),

lgeDPT         (236),

lgeLCDC        (237),

lgsLCDC        (238),

lgeEC          (239),

lgsEC          (240),

lgsPLAR        (241),

lgsCBW         (242),

lgegs          (243),

lgsgs          (244),

lgsgsE-M       (245),

lgslsE-M       (246),

lgels          (247),

lgsls          (248),

lgeRE          (249),

lgsRE          (250),

lgeIntl1       (251),

lgsIntl1       (252),

bypassNosig    (253),

bypassSig      (254),

unknown        (255)

}(0..255)

TestResults ::=  ENUMERATED {

pass  (0),

fail  (1)

}

TerminatingType ::= SEQUENCE{

data       BOOLEAN,

voice      BOOLEAN,

analogue   BOOLEAN,

digital    BOOLEAN

}

Trace ::= SEQUENCE OF Name

VoiceCompression ::= ENUMERATED{

unspecified  (0),

x8k          (1),

x16k         (2),

x32k         (3),

x64k         (4)

}

VoiceMode ::=    CHOICE{

voiceMode          [0] SEQUENCE{

companding         [1] Companding,

echoCancellation   [2] EchoCancellation,

voiceCompression   [3] VoiceCompression,

mode               [4] Mode},

notApplicable      [5] NULL

}

END

--  asn.1 definitions

-- OBJECT IDENTIFIERS

-- Module

BandwidthModelUSEFULDefinitions {ccitt(0) network-operator(3) bt(2342) modules(0) bandwidthModelUSEFULDefinitions(42)}

DEFINITIONS ::=

BEGIN

-- EXPORTS Everything

ccitt-networkoperator   IDENTIFIER    ::= {ccitt 3}

bt                      IDENTIFIER    ::= {ccitt-networkoperator   2342}

-- BT categories of information object

bt-modules              IDENTIFIER    ::={bt 0}

bt-attributeType        IDENTIFIER    ::={bt 3}

bt-notifications        IDENTIFIER    ::={bt 4}

bt-objectClass          IDENTIFIER    ::={bt 5}

bt-namebinding          IDENTIFIER    ::={bt 6}

bt-action               IDENTIFIER    ::={bt 7}

bt-behaviour            IDENTIFIER    ::={bt 10}

-- BT Modules

--bt-bandwidthModelUSEFULDefinitions OBJECT IDENTIFIER ::={bt-modules 42}

bt-bandwidthModelModule            OBJECT IDENTIFIER    ::={bt-modules 13}

END

## D.2 GDMO Definitions for OP2 Specification

#LIBRARY "NMF038, Issue 1"

bMGroomedCircuitTP MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100": trailTerminationPointBidirectional;

CHARACTERIZED BY	bMGroomedCircuitTPPackage PACKAGE

BEHAVIOUR

bMGroomedCircuitTPBehaviour BEHAVIOUR

DEFINED AS!  This managed object class represents service access points in the

network.  The managed object classes represent resources used to terminate

Groomed Circuit resources.  In some circumstances (dependent upon the underlying

technology) GMGroomedCircuitTP's may be created and deleted across the interoperable interface otherwise they are established through a provisioning process and therefore may not be

created or deleted over the interoperable interface.!;;

ATTRIBUTES

"Rec. X.721 | ISO/IEC 10165-2 : 1992": administrativeState GET,

"Recommendation M.3100":tTPId GET,

"Recommendation M.3100": userLabel GET-REPLACE,

"Recommendation M.3100": downstreamConnectivityPointer GET-REPLACE,

"Recommendation M.3100": upstreamConnectivityPointer GET-REPLACE,

"OP1 Library Vol. 1": typeText GET,

"OP1 Library Vol. 4": trmDataMode GET-REPLACE,

"OP1 Library Vol. 4": trmVoiceMode GET,

"OP1 Library Vol. 4": trmFunctionalInterface GET,

"NMF038, Issue 1":userIdentifier GET-REPLACE;

ACTIONS	"OP1 Library Vol. 4":trmAbort,

"OP1 Library Vol. 4":trmLocalLoop,

"OP1 Library Vol. 4":trmGetStatus;

; -- terminate PACKAGE template

; -- terminate CHARACTERIZED BY clause

REGISTERED AS	{forum-objectClass 75};

bMFacilityTP  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100": connectionTerminationPointBidirectional;

CHARACTERIZED BY	bMFacilityTPPackage PACKAGE

BEHAVIOUR

bMFacilityTPBehaviour BEHAVIOUR

DEFINED AS!  This managed object class represents static internal access points

in the network that terminate Facility resources.  In some circumstances (dependent upon the underlying technology) bMFacilitTP's may be created and deleted across the interoperable interface otherwise they are established through a provisioning process and therefore may not be

created or deleted over the interoperable interface.!;;

ATTRIBUTES

"Rec. X.721 | ISO/IEC 10165-2 : 1992": administrativeState GET,

"Rec. X.721 | ISO/IEC 10165-2 : 1992": operationalState GET,

"Recommendation M.3100":cTPId GET,

"Recommendation M.3100": downstreamConnectivityPointer GET-REPLACE,

"Recommendation M.3100": upstreamConnectivityPointer GET-REPLACE,

"Recommendation M.3100": userLabel GET-REPLACE,

"OP1 Library Vol. 1": typeText GET,

"OP1 Library Vol. 4": trmFunctionalInterface GET,

"OP1 Library Vol. 4": trmPhysicalInterface GET,

"NMF038, Issue 1":userIdentifier GET-REPLACE;

; -- terminate PACKAGE template

; -- terminate CHARACTERIZED BY clause

REGISTERED AS	{forum-objectClass 76};

bMGroomedCircuit MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100": trailR1;

CHARACTERIZED BY

commonProfilePackage,

tDMProfilePackage,

frameRelayPackage,

aTMProfilePackage,

bMGroomedCircuitPackage PACKAGE

BEHAVIOUR

bMGroomedCircuitBehaviour BEHAVIOUR

DEFINED AS!  This managed object class represents Groomed Circuit which are used

to represent end to end bandwidth services in the network.  The Groomed Circuits

can automatically reroute around network failures, be restored to an optimal

routing (groomed), tested (by application of loopbacks on end points of the

connection) and have routing profile attributes.  Groomed Circuits may be created

and deleted across the interoperable interface.!;;

ATTRIBUTES

"Recommendation M.3100": userLabel GET-REPLACE,

"Recommendation M.3100": serverConnectionList GET-REPLACE,

"Recommendation M.3100": a-TPInstance GET-REPLACE,

"Recommendation M.3100": z-TPInstance GET-REPLACE,

"OP1 Library Vol. 4": circuitType GET,

"OP1 Library Vol. 1": typeText GET,

"OP1 Library Vol. 4": trmEncryptionRouting GET,

"OP1 Library Vol. 4": trmFibreRouting GET,

"OP1 Library Vol. 4": trmTerrestrialRouting GET,

"NMF038, Issue 1":userIdentifier GET-REPLACE;

ACTIONS	"OP1 Library Vol. 4":trmCircuitGrooming,

"OP1 Library Vol. 4":trmCircuitTrace,

"OP1 Library Vol. 4":trmGetStatus;

; -- terminate PACKAGE template

; -- terminate CHARACTERIZED BY clause

REGISTERED AS	{forum-objectClass 77};

bMFacility MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100": connectionR1;

CHARACTERIZED BY

bMFacilityPackage PACKAGE

BEHAVIOUR

bMFacilityBehaviour BEHAVIOUR

DEFINED AS!  This managed object class represents static bandwidth resources, or

'pipes', in the network (can be a physical or logical entity).  In some circumstances (dependent upon the underlying technology) 's may be created and deleted across the interoperable interface otherwise they are established through a provisioning process and therefore may not be created or deleted.!;;

ATTRIBUTES

"Recommendation M.3100": userLabel GET-REPLACE,

"Recommendation M.3100": a-TPInstance GET-REPLACE,

"Recommendation M.3100": z-TPInstance GET-REPLACE,

"OP1 Library Vol. 4": trmCircuitProfile GET,

"OP1 Library Vol. 4": circuitBandwidth GET,

"OP1 Library Vol. 4": facilityType GET,

"OP1 Library Vol. 1": typeText GET,

"OP1 Library Vol. 4": trmFunctionalInterface GET,

"OP1 Library Vol. 4": trmPhysicalInterface GET,

"NMF038, Issue 1":userIdentifier GET-REPLACE;

; -- terminate PACKAGE template

; -- terminate CHARACTERIZED BY clause

REGISTERED AS	{forum-objectClass 78};

-- 2. Packages

commonProfilePackage PACKAGE

ATTRIBUTES

commonProfile GET-REPLACE;

REGISTERED AS {forum-package 127};

tDMProfilePackage PACKAGE

ATTRIBUTES

tDMProfile GET-REPLACE;

REGISTERED AS {forum-package 128};

frameRelayPackage PACKAGE

ATTRIBUTES

frameRelayProfile GET-REPLACE;

REGISTERED AS {forum-package 129};

aTMProfilePackage PACKAGE

ATTRIBUTES

aTMProfile GET-REPLACE;

REGISTERED AS {forum-package 130};

-- 3. Name Bindings

bMGroomedCircuitTP-managedElement	NAME BINDING

SUBORDINATE OBJECT CLASS

bMGroomedCircuitTP;

NAMED BY SUPERIOR OBJECT CLASS

"Recommendation M.3100": managedElement;

WITH ATTRIBUTE "Recommendation M.3100":tTPId;

REGISTERED AS {forum-nameBinding 168};

bMFacilityTP-bMGroomedCircuitTP	NAME BINDING

SUBORDINATE OBJECT CLASS

bMFacilityTP;

NAMED BY SUPERIOR OBJECT CLASS

bMGroomedCircuitTP;

WITH ATTRIBUTE "Recommendation M.3100":cTPId;

REGISTERED AS {forum-nameBinding 169};

bMGroomedCircuit-network	NAME BINDING

SUBORDINATE OBJECT CLASS

bMGroomedCircuit;

NAMED BY SUPERIOR OBJECT CLASS

"Recommendation M.3100": network;

WITH ATTRIBUTE "Recommendation M.3100":trailId;

REGISTERED AS {forum-nameBinding 170};

bMFacility-network	NAME BINDING

SUBORDINATE OBJECT CLASS

bMFacility;

NAMED BY SUPERIOR OBJECT CLASS

"Recommendation M.3100": network;

WITH ATTRIBUTE "Recommendation M.3100":connectionId;

REGISTERED AS {forum-nameBinding 171};

-- 4. Attribute Types

userIdentifier	ATTRIBUTE

WITH ATTRIBUTE SYNTAX CommonProfileModule.UserIdentifier;

MATCHES FOR EQUALITY;

BEHAVIOUR

userIdentifierBehaviour BEHAVIOUR

DEFINED AS! The userIdentifier holds a text string that is used by the

UMS to identify the object.!;;

REGISTERED AS {forum-attribute 327};

commonProfile	ATTRIBUTE

WITH ATTRIBUTE SYNTAX CommonProfileModule.CommonProfile;

MATCHES FOR EQUALITY;

BEHAVIOUR

commonProfileBehaviour BEHAVIOUR

DEFINED AS! The commonProfile attribute specifies common routing characteristics

for end to end circuits.!;;

REGISTERED AS {forum-attribute 328};

tDMProfile	ATTRIBUTE

WITH ATTRIBUTE SYNTAX ProfileModule.TDMProfile;

MATCHES FOR EQUALITY;

BEHAVIOUR

tDMProfileBehaviour BEHAVIOUR

DEFINED AS! The tDMProfile attribute specifies routing characteristics for TDM

end to end circuits.!;;

REGISTERED AS {forum-attribute 329};

frameRelayProfile	ATTRIBUTE

WITH ATTRIBUTE SYNTAX FrameRelayProfileModule.FrameRelayProfile;

MATCHES FOR EQUALITY;

BEHAVIOUR

frameRelayProfileBehaviour BEHAVIOUR

DEFINED AS! The frameRelayProfile attribute specifies routing characteristics for

frame relay end to end circuits.!;;

REGISTERED AS {forum-attribute 340};

aTMProfile	ATTRIBUTE

WITH ATTRIBUTE SYNTAX ATMProfileModule.ATMProfile;

MATCHES FOR EQUALITY;

BEHAVIOUR

aTMProfileBehaviour BEHAVIOUR

DEFINED AS! The aTMProfile attribute specifies routing characteristics for ATM

end to end circuits.!;;

REGISTERED AS {forum-attribute 341};

-- 5. ASN.1 Modules, ASN.1 Definitions for attributes.

CommonProfileModule {forum-module 19}

DEFINITIONS ::= BEGIN

IMPORTS

CircuitProfile

FROM FORUM-ASN1-2 {forum-module trm(12)};

CommonProfile ::=

SEQUENCE

{

reroute         		Reroute,

backupBandwidth         ReserveBackupBandwidth,

circuitProfile          	CircuitProfile

}

Reroute ::= BOOLEAN

-- FALSE do not reroute around network failures

-- TRUE try to reroute around network failures

ReserveBackupBandwidth ::= BOOLEAN

-- FALSE do not reserve backup bandwidth

-- TRUE reserve backup bandwidth

UserIdentifier ::= GraphicString(SIZE(0..256) )

END

ProfileModule {forum-module 20}

DEFINITIONS ::= BEGIN

IMPORTS

CircuitBandwidth

FROM FORUM-ASN1-1  {forum-module release1(11)}

CircuitProfile

FROM FORUM-ASN1-2 {forum-module trm(12)};

TDMProfile ::=

SEQUENCE

{

bandwidth               	CircuitBandwidth,

circuitProfile          	CircuitProfile

}

END

FrameRelayProfileModule {forum-module 21}

DEFINITIONS ::= BEGIN

IMPORTS

DigitalBandwidthValue

FROM FORUM-ASN1-1  {forum-module release1(11)}

CircuitProfile

FROM FORUM-ASN1-2 {forum-module trm(12)};

FrameRelayProfile ::=

SEQUENCE

{

bc			Bc,

be			Be,

cir			CIR,

circuitProfile          	CircuitProfile,

dlcia			DLCI,

dlciz			DLCI,

useatm			UseATM

}

CIR ::= DigitalBandwidthValue

-- Committed Information Rate

Bc ::= DigitalBandwidthValue

-- Committed Burst Rate

Be ::= DigitalBandwidthValue

-- Excess Burst Rate

DLCI ::= Integer16

-- frame relay virtual endpoints

UseATM ::= BOOLEAN

-- used for Frame Relay over ATM situations

Integer16        ::= INTEGER(0..65535)

END

ATMProfileModule {forum-module 22}

DEFINITIONS ::= BEGIN

IMPORTS

DigitalBandwidthValue

FROM FORUM-ASN1-1  {forum-module release1(11)}

CircuitProfile

FROM FORUM-ASN1-2 {forum-module trm(12)};

ATMProfile ::=

SEQUENCE

{

vpia		VPI,

vpiz		VPI,

vcia		VCI,

vciz		VCI,

sir		SIR,

pir		PIR,

alm		AlmSurveillance,

qos		QOS,

circuitProfile	CircuitProfile

}

VPI ::= Integer16

-- virtual path endpoint

VCI ::= Integer16

-- virtual circuit endpoint

SIR ::= DigitalBandwidthValue

-- Sustained Information Rate

PIR ::= DigitalBandwidthValue

-- Peak Information Rate

AlmSurveillance ::= BOOLEAN

-- FALSE do not generate alarm if VP layer connectivity is lost

-- TRUE generate alarm if VP layer connectivity is lost

QOS ::= INTEGER{

serviceQualityLevel1(1),

serviceQualityLevel2(2)

}  (0..255)

Integer16        ::= INTEGER(0..65535)

END

-- 10.1  Forum Assigned Object Identifiers

FORUM-USEFUL-DEFINITIONS-2

{forum-module usefulDefinitions-2(18)}

DEFINITIONS ::=

BEGIN

-- EXPORTS everything

forum  OBJECT IDENTIFIER ::= { iso(1) member-body(2) 124 forum(360501) }

-- NMF categories of information object

forum-module             OBJECT IDENTIFIER   ::= {forum modules(0)}             -- 1-94

forum-attribute          OBJECT IDENTIFIER   ::= {forum attributes(1)}          -- 1-94

forum-groupAttribute     OBJECT IDENTIFIER   ::= {forum groupAttributes(2)}     -- 1-94

forum-objectClass        OBJECT IDENTIFIER   ::= {forum objectClasss(3)}        -- 1-94

forum-nameBinding        OBJECT IDENTIFIER   ::= {forum nameBindings(4)}        -- 1-94

forum-notification       OBJECT IDENTIFIER   ::= {forum notifications(5)}       -- 1-94

forum-action             OBJECT IDENTIFIER   ::= {forum actions(6)}             -- 1-94

forum-applicationContext OBJECT IDENTIFIER   ::= {forum applicationContexts(7)} -- 1-94

forum-abstractSyntax     OBJECT IDENTIFIER   ::= {forum abstractSyntax(8)}      -- 1-94

forum-specificError      OBJECT IDENTIFIER   ::= {forum 9}  -- Note that specificError and

forum-parameter          OBJECT IDENTIFIER   ::= {forum parameters(9)}  -- parameter share same arc. 1-94

forum-package            OBJECT IDENTIFIER   ::= {forum packages(10)}           -- 1-94

forum-ensemble           OBJECT IDENTIFIER   ::= {forum ensembles(11)}          -- 1-94

forum-functionalUnit     OBJECT IDENTIFIER   ::= {forum functionalUnits(12)}    -- 1-94

END  --  FORUM-USEFUL-DEFINITIONS-2

# ANNEX E - MIGRATION INFORMATION

This Ensemble specifies interfaces based on two distinctly different sets of standards.  The first is based on Network Management Forum Release 1 (OMNIPoint 0) standards while the second is based on Recommendation M.3100 from the ITU-T.  The OMNIPoint 0 interface specification has been implemented by several companies and is in active use.  In order to migrate between the specifications a  translation function between the  models, or a complete cutover to the new model is  required.  Translation between the two specifications is beyond the scope of this Ensemble.  A cutover from one model two another is theoretically straight forward however it does present considerable operational undertaking.

A cut over is aided by the fact that two interfaces share the same high level concepts (as is shown by this Ensemble, see Section 1.2).  The Resources (Section 2.3) and Functions (Section 2.4) defined are common to both specifications.  This means that the high level design and concepts are retained; they are mapped onto different managed objects.  As well some attributes are common to both.  Since the look and feel of the interface is maintained,  migrating between the two interfaces becomes a procedure which requires some effort but is straight forward.

Systems which use Recommendation M.3100 managed objects may be able to re-use all or part of their implementations (allomorphism is also possible)  if they are interfacing to an implementation of the OP2 specification.  All the objects defined in this interface use base,  or subclasses of base,  M.3100 objects.  See table F.1 for potential re-use and allomorphism with M.3100 objects.

Table E.1: BME Resources and MO classes

|  | OP2 Specification |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| M.3100 Object Class | Uses Base Object | Uses sub-Class Object |  | Sub-Class Name |  |
| network | X |  |  |  |  |
| managedElement | X |  |  |  |  |
| equipment | X |  |  |  |  |
| equipmentHolder | X |  |  |  |  |
| circuitPack | X |  |  |  |  |
| trailTerminationPointBiDirectional |  |  | X |  | bMGroomedCircuitTP |
| connectionTerminationPointBiDirectional |  |  | X |  | bMFacilityTP |
| trailR1 |  |  | X |  | bMGroomedCircuit |
| connectionR1 |  |  | X |  | bMFacility |

# ANNEX F - REFERENCES

1.	Network Management Forum, Forum 025, Network Management Forum Ensemble Concepts and Format, Issue 1,  August 1992.

2.	ITU-T|ISO/IEC:  Recommendation X.722|ISO/IEC 10165-4, Information Technology - Open System Interconnection - Structure of Management Information:  Guidelines for the Definition of Managed Objects, 1992.

3.	Network Management Forum, Forum 017, Reconfigurable Circuit Service: Configuration Management Ensemble, Issue 1, August 1992.

4.	Network Management Forum, Forum 018, Reconfigurable Circuit Service: AlarmSurveillance Ensemble, Issue 1, August 1992.

5.	ITU-T Recommendation M.3100, Generic Network Management Information Model, 1995.

6.	Network Management Forum, Forum 006, Issue 1.3, Volume 1 Supplement, Issue 1.1, August 1991

7.	Network Management Forum,  Forum 006, Forum Library - Volume 4:  OMNIPoint 1 Definitions, Issue 1.0, August 1992.

8.	ITU-T, CCITT Recommendation X.721 (1992) | ISO/IEC 10165-2 Information Technology - Open Systems Interconnection - Structure of Management Information: Definition of Management Information, 1992.

9.	Network Management Forum, Forum 020,  OMNIPoint 1 Conformance Requirements, Issue 1.0, August 1992.

10.	Network Management Forum, NMF 035 Switch Interconnection Management Ensemble,  Issue 1.0, October 1995.

11.	Network Management Forum, NMF 033 Customer Administration Configuration Management Ensemble, Issue 1.0,  October 1995.

12.	Network Management Forum,  CS302,  TMN Basic Management Platform Component Set, Issue 1.0,  October 1995.

13.	ISO/IEC ISP 12060-2,  Information Technology - International Standardization Profiles AOM2n OSI Management - Management Functions - Part 2:  AOM212 - Alarm Reporting and State Management Capabilities, June 1, 1994.

14.	ISO/IEC ISP 12060-4,  Information Technology - International Standardization Profiles AOM2n OSI Management - Management Functions - Part 4:  AOM221 - General Event Report Management, June 1, 1994.

15.	ISO/IEC ISP 12060-5,  Information Technology - International Standardization Profiles AOM2n OSI Management - Management Functions - Part 5:  AOM231 - General Log Control, June 1, 1994.

16.)	ITU-T,  CCITT Recommendation M.3010.  Maintenance:  Telecommunications Management Network - Principles for a Telecommunications Management Network.

Instructions for MCS proforma are specified in ITU-TS Rec. X.724 | ISO 10165-6.