EC 233 ELECTROMAGNETIC FIELDS	(ECE III SEMESTER)

UNIT II - STATIC MAGNETIC FIELD

2  MARKS QUESTION & ANSWERS

- What is the force on a charge, moving in a uniform magnetic field?

The force on a charge Q moving in a uniform magnetic field B with velocity v is given by

F=Q(v×B)        N

F=BQvsinθ     N

where,  is the angle between the direction of B and the direction in which the charge moves.

- What is the force experienced by a current carrying element in a uniform magnetic field?

The force experienced by a current carrying element Idl in a 	uniform magnetic field B is given by

F=I(dl×B)     N

F=BIlsinθ     N

where,  is the angle between the direction of B and the direction of 	current in the conductor.

- Give the Lorentz force equation.

The Lorentz force equation gives the force on a charge Q moving in 	a region where both the electric field E and magnetic field B are present.

F=Q(E+v×B)      N

where v is the velocity with which the charge moves in the field.

- Define magnetic flux density.

The magnetic flux per unit area is called the magnetic flux density 	B.

B=φA	  Wb/m2 (or) Tesla

- State Biot-Savart’s law.

The Biot-Savart law states that at any point P the magnitude 	of the magnetic field produced by the differential element dl is

- Proportional to the product of current, the magnitude of the differential length, and the sine of the angle lying between the element and a line connecting the element to the point P where the field is desired,

- Inversely proportional to the square of the distance from the differential element to the point P.

- Directly proportional to the constant of the medium () and

- Directed normal to the plane containing the differential element and the line drawn from the filament to the point P.

B=μIdlsinθ4πr2      Wb/m2

- State Ampere’s law for a magnetic field.

The Ampere’s law states that the line integral of H around a single closed path is equal to the current enclosed.

It can also be stated as the line integral of B around a single closed path is equal to the permeability of the medium times the current enclosed.

H⋅dl=I

B⋅dl=μI

- What is the force between two current carrying conductors?

The force between the two conductors carrying current I1 and I2 separated by a distance r is given by

F=μ0I1I22πr     N

- Give the relation between magnetic flux density and magnetic field intensity.

The permeability of any medium is the ratio of the magnetic flux 	density B to the magnetic field intensity H.

μ=BH    H/m

- State the Gauss’s law for magnetic fields.

The integral of the magnetic flux density B over a closed surface is 	zero.  This is called the Gauss’s law for magnetic fields.

B⋅dS=0

where, dS is the normal component of the surface.

- What is the torque on a current carrying loop?

The torque, or moment, of a force is a vector whose magnitude is 	the product of the magnitudes of the vector force, the vector lever arm, 	and the sine of the angle between these two vectors.  The direction of the 	vector torque is normal to both the force and lever arm.

T=R×F  Nm

where, R is the vector lever arm, F is the force vector.

- What is the torque on a planar coil?

The torque on a planar coil of any size in a uniform magnetic field is 	the product of the magnitudes of magnetic moment ‘m’, magnetic flux 	density B and the sine of the angle between these two.  It is given by

T=m×B   Nm

- Define magnetic moment.

The magnetic moment ‘m’ is defined as the product of the loop 	current and the vector area of the loop.  It is given by,

m=IA  Am2

where A is the vector area.

- Define permeability with respect to the field cell.

The permeability of a medium may be defined as the inductance per unit length of a transmission line cell filled with this medium.

μ=Ld     H/m

where, L is the inductance of a field cell and d is the length of the field cell.

- Give the expressions relating B and H with the current density J.

B=μ04πJ×arr2dv

curlB=μ0J

curlH=J

- Give the expressions relating magnetic vector potential with the current density J.

−∇2A=μJ

A=μ4πJrdr     Wb/m

- Give the relation between B and magnetic vector potential.

B=CurlA

- What is the magnetic field at any point due to a infinitely long conductor carrying current?

B=μ0I2πd    Wb/m2

H=I2πd     A/m

where, d is distance between the conductor and the point where the field 	is required.

- What is the magnetic field at any point due to a finite length conductor carrying current?

B=μ0I4πd[cosθ1−cosθ2]   Wb/m2

H=I4πd[cosθ1−cosθ2]    A/m

- What is the magnetic field at any point on the axis of a circular coil carrying current?

B=μ0Ia22(a2+d2)32   Wb/m2

H=Ia22(a2+d2)32     A/m

where ‘a’ is the radius of the circle.

- What is the magnetic field at the centre of the circular coil carrying current?

B=μ0I2a  Wb/m2

H=I2a    A/m

- What is the magnetic field at any point on the axis of a solenoid carrying current?

B=μ0NI2l[cosθ1−cosθ2]   Wb/m2

H=I2l[cosθ1−cosθ2]     A/m

- What is the magnetic field at a point midway on the axis of the solenoid carrying current?

B=μ0NI2(l/2)2+a2      Wb/m2

H=NI2(l/2)2+a2    A/m

- What is the magnetic field at any end of the axis of the solenoid carrying current?

B=μ0NI2l2+a2       Wb/m2

H=NI2l2+a2          A/m