## Human Computer Interface

## Week 4

## Software Tools

### Interface-Building Tools

Terminology varies depending on vendor. Popular terms include:

- Rapid Prototyper

- User Interface Builder

- User Interface Management System

- User Interface Development Environment

- Rapid Application Developer

Designers can write programs with easy-to-understand terms:

on mouseUp

play "boing" 
wait for 3 seconds 
visual effect wipe left very fast to black 
click at 150,100 
type "goodbye"

end mouseUp

Software engineering tools - Java

Class Test {

public static void main (String[] args) {

for (int i = 0; i < args.length; i++)

System.out.print(i == 0 ? args[i] : " " + args[i]);

System.out.println();

}

}

Software engineering tools - JavaScript

<HEAD>

<SCRIPT LANGUAGE="JavaScript">

<!-- to hide script contents from old browsers

function square(i) {

document.write("The call passed ", i ," to

the function."<BR>")

return i * i

}

document.write("The function returned ", square(5),".")

// end hiding contents from old browsers -->

</SCRIPT>

</HEAD>

<BODY>

<BR>

All Done.

</BODY>

Upon loading the web page it produces this output:

The call passed 5 to the function.

The function returned 25.

All Done.

# Interaction Devices

## 1. Keyboard Layouts

#### Keys

- 1/2 inch square keys

- 1/4 inch spacing between keys

- slight concave surface

- matte finish to reduce glare finger slippage

- 40- to 125-gram force to activate

- 3 to 5 millimeters displacement

- tactile and audible feedback important

- certain keys should be larger (e.g. ENTER, SHIFT, CTRL)

- some keys require state indicator, such as lowered position or light indicator (e.g. CAPS LOCK)

- key labels should be large, meaningful, permanent

- some "home" keys may have additional features, such as deeper cavity or small raised dot, to help user locate their fingers properly (caution - no standard for this)

#### Function keys

- typically simply labeled F1, F2, etc, though some may also have meaningful labels, such as CUT, COPY, etc.

- users must either remember each key's function, identify them from the screen's display, or use a template over the keys in order to identify them properly

- can reduce number of keystrokes and errors

- meaning of each key can change with each application

- placement on keyboard can affect efficient use

- special-purpose displays often embed function keys in monitor bezel

- lights next to keys used to indicate availability of the function, or on/off status

- frequent movement between keyboard home position and mouse or function keys can be disruptive to use

- alternative is to use closer keys (e.g. ALT or CTRL) and one letter to indicate special function

#### Cursor movement keys

- up, down, left, right

- some keyboards also provide diagonals

- best layout is natural positions

- inverted-T positioning allows users to place their middle three fingers in a way that reduces hand and finger movement

- cross arrangement better for novices than linear or box

- typically include typamatic (auto-repeat) feature

- important for form-fillin and direct manipulation

- other movements may be performed with other keys, such as TAB, ENTER, HOME, etc.

## 2. Pointing Devices

Pointing devices are applicable in six types of interaction tasks:

1. Select:

- user chooses from a set of items.

- used for traditional menu selection, identification of a file in a directory, or marking of a part in an automobile design.

2. Position:

- user chooses a point in a one-, two-, three-, or higher-dimensional space

- used to create a drawing, to place a new window, or to drag a block of text in a figure.

3. Orient:

- user chooses a direction in a two-, three-, or higher-dimensional space.

- direction may simply rotate a symbol on the screen, indicate a direction of motion for a space ship, or control the operation of a robot arm.

4. Path:

- user rapidly performs a series of position and orient operations.

- may be realized as a curving line in a drawing program, the instructions for a cloth cutting machine, or the route on a map.

5. Quantify:

- user specifies a numeric value.

- usually a one-dimensional selection of integer or real values to set parameters, such as the page number in a document, the velocity of a ship, or the amplitude of a sound.

6. Text:

- user enters, moves, and edits text in a two-dimensional space. The

- pointing device indicates the location of an insertion, deletion, or change.

- more elaborate tasks, such as centering; margin setting; font sizes; highlighting, such as boldface or underscore; and page layout.

### Direct-control pointing devices

#### lightpen

- enabled users to point to a spot on a screen and to perform a select, position, or other task

- it allows direct control by pointing to a spot on the display

- incorporates a button for the user to press when the cursor is resting on the desired spot on the screen

- lightpen has three disadvantages: users' hands obscured part of the screen, users had to remove their hands from the keyboard, and users had to pick up the lightpen

#### touchscreen

- allows direct control touches on the screen using a finger

- early designs were rightly criticized for causing fatigue, hand-obscuring-the-screen, hand-off-keyboard, imprecise pointing, and the eventual smudging of the display

- lift-off strategy enables users to point at a single pixel

- the users touch the surface

- then see a cursor that they can drag around on the display

- when the users are satisfied with the position, they lift their fingers off the display to activate

- can produce varied displays to suit the task

- are fabricated integrally with display surfaces

### Indirect pointing devices

#### mouse

- the hand rests in a comfortable position, buttons on the mouse are easily pressed, even long motions can be rapid, and positioning can be precise

#### trackball

- usually implemented as a rotating ball 1 to 6 inches in diameter that moves a cursor

#### joystick

- are appealing for tracking purposes

#### graphics tablet

- a touch-sensitive surface separate from the screen

#### touchpad

- built-in near the keyboard offers the convenience and precision of a touchscreen while keeping the user's hand off the display surface

### 3. Comparisons of pointing devices

#### Human-factors variables

- speed of motion for short and long distances

- accuracy of positioning

- error rates

- learning time

- user satisfaction

#### Other variables

- cost

- durability

- space requirements

- weight

- left- versus right-hand use

- likelihood to cause repetitive-strain injury

- compatibility with other systems

#### Some results

- direct pointing devices faster, but less accurate

- graphics tablets are appealing when user can remain with device for long periods without switching to keyboard

- mouse is faster than isometric joystick

- for tasks that mix typing and pointing, cursor keys a faster and are preferred by users to a mouse

- muscular strain is low for cursor keys

## 4. Speech Recognition, Digitization, and Generation

Speech recognition still does not match the fantasy of science fiction:

- demands of user's working memory

- background noise problematic

- variations in user speech performance impacts effectiveness

- most useful in specific applications, such as to benefit handicapped users

### Discrete word recognition

- recognize individual words spoken by a specific person; can work with 90- to 98-percent reliability for 20 to 200 word vocabularies

- Speaker-dependent training, in which the user repeats the full vocabulary once or twice

- Speaker-independent systems are beginning to be reliable enough for certain commercial applications

- been successful in enabling bedridden, paralyzed, or otherwise disabled people to broaden the horizons of their life

- also useful in applications with at least one of the following conditions:

  - speaker's hands are occupied

  - mobility is required

  - speaker's eyes are occupied

  - harsh or cramped conditions preclude use of keyboard

- voice-controlled editor versus keyboard editor

  - lower task-completion rate

  - lower error rate

- use can disrupt problem solving

## 5. Speech Recognition, Digitization, and Generation

Continuous-speech recognition

Not generally available:

- difficulty in recognizing boundaries between spoken words

- normal speech patterns blur boundaries

- many potentially useful applications if perfected

Speech store and forward

Voice mail users can

- receive messages

- replay messages

- reply to caller

- forward messages to other users, delete messages

- archive messages

Systems are low cost and reliable.

#### Speech Generation

Michaelis and Wiggins (1982) suggest that speech generation is "frequently preferable" under these circumstances:

- The message is simple.

- The message is short.

- The message will not be referred to later.

- The message deals with events in time.

- The message requires an immediate response.

- The visual channels of communication are overloaded.

- The environment is too brightly lit, too poorly lit, subject to severe vibration, or otherwise unsuitable for transmission of visual information.

- The user must be free to move around.

- The user is subjected to high G forces or anoxia

### Audio tones, audiolization, and music

Sound feedback can important:

- to confirm actions

- offer warning

- for visually-impaired users

- music used to provide mood context, e.g. in games

- can provide unique opportunities for user, e.g. with simulating various musical instruments

## 6. Image and Video Displays

The visual display unit (VDU) has become the primary source of feedback to the user from the computer.

The VDU has many important features, including:

- Rapid operation

- Reasonable size

- Reasonable resolution

- Quiet operation

- No paper waste

- Relatively low cost

- Reliability

- Highlighting

- Graphics and animation

Possible health concerns:

- visual fatigue

- stress

- radiation exposure

## 7. Display devices

monochrome displays

- are adequate, and are attractive because of their lower cost

RGB shadow-mask displays

- small dots of red, green, and blue phosphors packed closely

Raster-scan cathode-ray tube (CRT)

- electron beam sweeping out lines of dots to form letters

- refresh rates 30 to 70 per second

Liquid-crystal displays (LCDs)

- voltage changes influence the polarization of tiny capsules of liquid crystals

- flicker-free

- size of the capsules limits the resolution

Plasma panel

- rows of horizontal wires are slightly separated from vertical wires by small glass-enclosed capsules of neon-based gases

Light-emitting diodes (LEDs)

- certain diodes emit light when a voltage is applied

- arrays of these small diodes can be assembled to display characters

The technology employed affects these display attributes:

- Size

- Refresh rate

- Capacity to show animation

- Resolution

- Surface flatness

- Surface glare from reflected light

- Contrast between characters and background

- Brightness

- Flicker

- Line sharpness

- Character formation

- Tolerance for vibration

Each display technology has advantages and disadvantages with respect to these attributes. Users should expect these features:

- User control of contrast and brightness

- Software highlighting of characters by brightness

- Underscoring, reverse video, blinking (possibly at several rates)

- Character set (alphabetic, numeric, special and foreign characters)

- Multiple type styles (for example, italic, bold) and fonts

- Shape, size, and blinking rate of the cursor

- User control of cursor shape, blinking, and brightness

- Scrolling mechanism (smooth scrolling is preferred)

- User control of number of lines or characters per line displayed

- Support of negative and positive polarity (light on dark or dark on light)

### Digital photography and scanners

- Many name-brand suppliers (e.g. Sony, Kodak, Nikon, Canon)

- Downloadable to PCs

- Can obtain PhotoCD from standard 35-mm slides

- Scanners have dropped significantly in price

- High-resolution color scanners now available for reasonable price

- Optical Character Recognition (OCR) software also available

### Digital video

- 12-inch videodisks store up to 54,000 still images or 30 minutes of motion video

- widely used for

  - museums

  - travel facilities

  - education

  - industrial training

- CD-ROMs can provide up to

  - 600 megabytes of textual or numeric data

  - 6000 graphic images

  - 1 hour of music

  - 6 to 72 minutes of motion video (depending on quality)

- MPEG video cameras now available

### Projectors, heads-up displays, helmet-mounted displays

## 8. Printers

These are the important criteria for printers:

- Speed

- Print quality

- Cost

- Compactness

- Quiet operation

- Use of ordinary paper (fanfolded or single sheet)

- Character set

- Variety of typefaces, fonts, and sizes

- Highlighting techniques (boldface, underscore, and so on)

- Support for special forms (printed forms, different lengths, and so on)

- Reliability

#### dot-matrix printers

- print more than 200 characters per second, have multiple fonts, can print boldface, use variable width and size, and have graphics capabilities

#### inkjet printers

- offer quiet operation and high-quality output

#### thermal printers or fax machines

- offer quiet, compact, and inexpensive output on specially coated papers

#### laser printers

- operate at 30,000 lines per minute

#### color printers

- allow users to produce hardcopy output of color graphics, usually by an inkjet approach with three colored and black inks

#### photographic printers

- allow the creation of 35-millimeter or larger slides (transparencies) and photographic prints